3GPP TSG RAN WG1 #108-e	R1-2202093

e-Meeting, February 21th – March 3rd, 2022

Agenda item:	8.3.3

Source: 	Lenovo

Title: 	Intra-UE multiplexing enhancement for IIoT/URLLC

Document for:	Discussion

# Introduction

During RAN1 #107bis-e meeting, RAN1 made following agreements regarding intra-UE multiplexing enhancement for Industrial Internet of Things (IIoT) and ultra-reliable and low latency communication (URLLC) [1]:

Agreement

For resolving collision of PUCCHs of different priorities without repetitions within a time unit, Step 2.1 consists of the following sub-steps:

- Step 2.1-1: Determine a reference PUCCH resource

- Step 2.1-2: Select O PUCCH resource(s) overlapping with the reference PUCCH resource.

- Step 2.1-3: Apply Rel-17 intra-UE multiplexing/dropping rules to resolve overlapping among the reference PUCCH resource and O PUCCH resource(s).

- Step 2.1-4: Loop Step 2.1-1) ~ Step 2.1-3) until there are no overlapping PUCCHs in the time unit.

- FFS details

Agreement

For resolving collision of PUCCHs of different priorities without repetition within a time unit, down-select from the following options:

- Option 1:

  - The reference PUCCH resource is determined as in Rel-15, i.e. based on the starting symbol and duration

  - In step 2.1-2, select up to one PUCCH resource overlapping with the reference PUCCH resource according to Rel-15 pseudo code

- Option 2:

  - The reference PUCCH resource is determined as in Rel-15, i.e. based on the starting symbol and duration

  - In step 2.1-2, select all the PUCCH resources overlapping with the reference PUCCH resource according to Rel-15 pseudo code

- Option 3:

  - The reference PUCCH resource is determined by prioritizing HP PUCCH over LP PUCCH on top of Rel-15 rules

  - In step 2.1-2, select all the PUCCH resources overlapping with the reference PUCCH resource according to Rel-15 pseudo code

- Option 4:

  - The reference PUCCH resource is determined by prioritizing LP PUCCH carrying HARQ-ACK on top of Rel-15 rules

  - In step 2.1-2, If a LP PUCCH carrying HARQ-ACK overlaps with multiple HP PUCCHs and one of the HP PUCCH includes HARQ-ACK, only select the HP PUCCH including HARQ-ACK in step 2.1-2; otherwise, select all the PUCCH resources overlapping with the reference PUCCH resource according to Rel-15 pseudo code

FFS: Details on time units for all options

Working Assumption

For resolving collision of PUCCHs of different priorities without repetition within a time unit, the time unit of HP HARQ-ACK is used. For a LP PUCCH overlapping with multiple time units, down-select from:

- Alt. 1: the LP PUCCH is associated with the first time unit with overlapping HP PUCCH(s)

- Alt. 2: the LP PUCCH is associated with the first time unit with overlapping HP PUCCH with HARQ-ACK if any. Otherwise, the LP PUCCH is associated with the first time unit with overlapping HP PUCCH(s).

- Alt. 3: the LP PUCCH is associated with the last time unit with overlapping HP PUCCH(s)

Agreement

When a PUCCH carrying HP SR with PF0/1 overlaps with a PUCCH carrying LP HARQ-ACK with PF2/3/4:

- For positive SR, transmit SR on the SR PUCCH resource and drop HARQ-ACK.

- For negative SR, transmit HARQ-ACK only on the HARQ-ACK PUCCH resource.

Note: It was agreed to support multiplexing a LP HARQ-ACK and a HP SR into a PUCCH for some HARQ-ACK/SR PF combinations in Rel-17.

In this document, we discuss remaining issues for multiplexing uplink control information (UCI) of different priorities on PUCCH and on PUSCH.

# Multiplexing UCI of different priorities in PUCCH

When a LP PUCCH with HARQ-ACK overlaps with multiple time units of HP HARQ-ACK, the LP PUCCH should be associated with the earliest time unit with overlapping HP PUCCHs for multiplexing UCI of different priorities. Once the association time unit for multiplexing is determined, Rel-15 rules to resolve overlapping PUCCHs within a time unit can be reused for resolving overlapping PUCCHs of different priorities within a time unit. That is, the reference PUCCH resource can be determined as in Rel-15, i.e. based on the starting symbol and duration, and UE can select all the PUCCH resources overlapping with the reference PUCCH resource according to Rel-15 pseudo code.

Proposal 1: When a LP PUCCH with HARQ-ACK overlaps with multiple time units of HP HARQ-ACK, the LP PUCCH, for multiplexing, is associated with the earliest time unit with overlapping HP PUCCHs. Rel-15 rules to resolve overlapping PUCCHs within a time unit is reused for resolving overlapping PUCCHs of different priorities within a time unit.

If overlapping PUCCH transmissions of a HP SR (i.e. positive SR), 1-bit HP HARQ-ACK, and 1-bit LP HARQ-ACK from a UE would occur in a slot/sub-slot, the UE may, first, determine a PUCCH resource for multiplexing the HP SR and the 1-bit HP HARQ-ACK according to Rel-15 rules (i.e. Subclause 9.2.5.1 of TS 38.213) and secondly, multiplex the 1-bit LP HARQ-ACK with the HP UCI (i.e. HP HARQ-ACK and HP SR if the determined PUCCH resource is PF0; HP HARQ-ACK if the determined PUCCH resource is PF1) in the determined PUCCH resource with treating the 1-bit LP HARQ-ACK as a HP HARQ-ACK bit.

Proposal 2: For overlapping PUCCH transmissions of a HP SR (i.e. positive SR), 1-bit HP HARQ-ACK, and 1-bit LP HARQ-ACK, UE multiplexes LP HARQ-ACK into a PUCCH resource for the HP SR and the HP HARQ-ACK by treating the LP HARQ-ACK as a HP HARQ-ACK bit.

When a PUCCH carrying HP SR with PF0/1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0/1, similar to LP HARQ-ACK with PF2/3/4, a UE can transmit SR on the SR PUCCH resource for positive SR and drop HARQ-ACK. For negative SR, the UE transmits HARQ-ACK only on the HARQ-ACK PUCCH resource. If LP HARQ-ACK is multiplexed in a HP SR PUCCH resource of PF0, it would be difficult for gNB to configure one PUCCH PRB with different cyclic shifts for multiple UEs. Considering that SR PUCCH resources are semi-statically configured and gNB may have to configure many URLLC UEs with frequent SR resources of PF0 for low latency SR, multiplexing the LP HARQ-ACK in a HP SR PUCCH resource of PF0 would degrade system efficiency significantly.

Proposal 3: When a PUCCH carrying HP SR with PF0/1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0/1, similar to the case of LP HARQ-ACK with PF2/3/4, a UE can transmit SR on the SR PUCCH resource for positive SR and drop HARQ-ACK.

When a PUCCH carrying HP SR and HP HARQ-ACK with PF0/1 overlaps with a PUCCH carrying LP HARQ-ACK and when the total number of HARQ-ACK bits including HP HARQ-ACK bits and LP HARQ-ACK bits is larger than 2, UE determines a PUCCH resource to multiplex HP UCI bits of OUCI=OACK+1 (i.e. 1 bit for SR) and LP HARQ-ACK bits by determining a PUCCH resource set of priority index 1 based on the total number of HP UCI bits and LP HARQ-ACK bits and determining a PUCCH resource from the PUCCH resource set based on a PUCCH resource indicator  (PRI) (if HP HARQ-ACK includes dynamic HARQ-ACK) or by determining a PUCCH resource from PUCCH resources provided by sps-PUCCH-AN-List based on the total number of HP UCI bits and LP HARQ-ACK bits (if HP HARQ-ACK includes only SPS HARQ-ACK).

Proposal 4: When a PUCCH carrying HP SR and HP HARQ-ACK with PF0/1 overlaps with a PUCCH carrying LP HARQ-ACK and when the total number of HARQ-ACK bits including HP HARQ-ACK bits and LP HARQ-ACK bits is larger than 2, UE determines a PUCCH resource of priority index 1 based on HP UCI bits of OUCI=OACK+1 (i.e. 1 bit for SR) and LP HARQ-ACK bits.

# Multiplexing of CG-UCI and HARQ-ACK of different priorities in CG-PUSCH

According to the current Rel-17 TS 38.213 (V17.0.0), for a configured grant (CG) PUSCH transmission, a UE multiplexes CG-UCI in the CG PUSCH transmission if the UE is provided by betaOffsetCG-UCI a IoffsetCG-UCI value, from a set of values, with the mapping defined in Table 9.3-1 of TS 38.213. If the UE is provided cg-UCI-Multiplexing and multiplexes HARQ-ACK information in the PUSCH transmission, the UE jointly encodes the HARQ-ACK information and the CG-UCI and determines a number of resources for multiplexing the combined information in a PUSCH using βoffsetHARQ-ACK which provides indexes Ioffset,1HARQ-ACK and Ioffset,2HARQ-ACK for the UE to use if the UE multiplexes up to 11, and more than 11 combined information bits, respectively.

If HARQ-ACK information to be multiplexed with CG-UCI in a CG PUSCH consists of HARQ-ACK information of different priorities, LP HARQ-ACK and HP HARQ-ACK are separately encoded to guarantee necessary reliability for different priorities. Thus, the CG-UCI should be jointly encoded with either the LP HARQ-ACK or the HP HARQ-ACK. A priority of the CG-UCI can be determined based on a priority of the CG PUSCH. That is, CG-UCI of a CG PUSCH of priority index 0 is jointly encoded with LP HARQ-ACK, while CG-UCI of a CG PUSCH of priority index 1 is jointly encoded with HP HARQ-ACK.

Proposal 5: If UE is provided cg-UCI-Multiplexing and multiplexes HARQ-ACK of different priorities in a CG PUSCH, CG-UCI is jointly encoded with HP HARQ-ACK for the CG PUSCH of priority index 1, and CG-UCI is jointly encoded with LP HARQ-ACK for the CG PUSCH of priority index 0.

# Conclusion

In summary, we observe and propose the followings for intra-UE multiplexing enhancement for Rel-17 IIoT/URLLC:

- Proposal 1: When a LP PUCCH with HARQ-ACK overlaps with multiple time units of HP HARQ-ACK, the LP PUCCH, for multiplexing, is associated with the earliest time unit with overlapping HP PUCCHs. Rel-15 rules to resolve overlapping PUCCHs within a time unit is reused for resolving overlapping PUCCHs of different priorities within a time unit.

- Proposal 2: For overlapping PUCCH transmissions of a HP SR (i.e. positive SR), 1-bit HP HARQ-ACK, and 1-bit LP HARQ-ACK, UE multiplexes LP HARQ-ACK into a PUCCH resource for the HP SR and the HP HARQ-ACK by treating the LP HARQ-ACK as a HP HARQ-ACK bit.

- Proposal 3: When a PUCCH carrying HP SR with PF0/1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0/1, similar to the case of LP HARQ-ACK with PF2/3/4, a UE can transmit SR on the SR PUCCH resource for positive SR and drop HARQ-ACK.

- Proposal 4: When a PUCCH carrying HP SR and HP HARQ-ACK with PF0/1 overlaps with a PUCCH carrying LP HARQ-ACK and when the total number of HARQ-ACK bits including HP HARQ-ACK bits and LP HARQ-ACK bits is larger than 2, UE determines a PUCCH resource of priority index 1 based on HP UCI bits of OUCI=OACK+1 (i.e. 1 bit for SR) and LP HARQ-ACK bits.

- Proposal 5: If UE is provided cg-UCI-Multiplexing and multiplexes HARQ-ACK of different priorities in a CG PUSCH, CG-UCI is jointly encoded with HP HARQ-ACK for the CG PUSCH of priority index 1, and CG-UCI is jointly encoded with LP HARQ-ACK for the CG PUSCH of priority index 0.

-------------------------- Start of Text Proposal for TS 38.213 ----------------------------------------------------------------

## 9.3	UCI reporting in physical uplink shared channel

## *** Unchanged text is omitted ***

For a PUSCH transmission that is configured by a ConfiguredGrantConfig and includes CG-UCI, the UE multiplexes CG-UCI in the PUSCH transmission if the UE is provided by betaOffsetCG-UCI a IoffsetCG-UCI value, from a set of values, with the mapping defined in Table 9.3-1. If the UE is provided cg-UCI-Multiplexing and multiplexes HARQ-ACK information of a same priority in the PUSCH transmission, as described in clauses 9 and 9.2.5, the UE jointly encodes the HARQ-ACK information and the CG-UCI [5, TS 38.212] and determines a number of resources for multiplexing the combined information in a PUSCH using βoffsetHARQ-ACK which provides indexes Ioffset,1HARQ-ACK and Ioffset,2HARQ-ACK for the UE to use if the UE multiplexes up to 11, and more than 11 combined information bits, respectively.

If the UE is provided cg-UCI-Multiplexing and multiplexes HARQ-ACK information of different priorities in the PUSCH transmission of priority 0, as described in clauses 9 and 9.2.5, the UE jointly encodes the HARQ-ACK information of priority 0 and the CG-UCI [5, TS 38.212] and separately encodes the HARQ-ACK information of priority 1 and determines a number of resources for multiplexing the combined information in a PUSCH using βoffsetHARQ-ACK and βoffsetHARQ-ACK,1.

If the UE is provided cg-UCI-Multiplexing and multiplexes HARQ-ACK information of different priorities in the PUSCH transmission of priority 1, as described in clauses 9 and 9.2.5, the UE jointly encodes the HARQ-ACK information of priority 1 and the CG-UCI [5, TS 38.212] and separately encodes the HARQ-ACK information of priority 0 and determines a number of resources for multiplexing the combined information in a PUSCH using βoffsetHARQ-ACK and βoffsetHARQ-ACK,0.

-------------------------- End of Text Proposal for TS 38.213 ----------------------------------------------------------------

# References

- “RAN1 Chairman’s Notes”, RAN1#107bis-e meeting, January 17th – 25th, 2022.