3GPP TSG RAN WG1 Meeting #108-e	R1- 2202097

February 21st – March 3rd, 2022

Title:	[Draft] Reply LS to RAN4 on PEMAX for NR-V2X

Response to:	R4-2120047

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	Qualcomm [RAN1]

To:	RAN4

Cc:

Contact person:

Name:	Gabi Sarkis

E-mail Address:	gsarkis@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# Overall Description

RAN1 thanks RAN2 for their LS on PEMAX for NR-V2X and would like to provide replies to the following issues:

Issue 1: It is RAN4 understanding that the parameter to limit the transmitted power of PSSCH/PSCCH for V2X can be used in both in-coverage and out-of-coverage. RAN4 would like to check with RAN1 and RAN2 which parameter (sl-maxTxPower, sl-MaxTransPower, SL-TxPower) is the correct one to be used to fulfill the purpose?

A UE must comply with both sl-MaxTxPower and sl-MaxTransPower regardless of whether the parameter is used for In-Coverage (IC) or out-of-coverage (OC).

sl-MaxTxPower limits the maximum transmission power based on measured CBR. RAN1 specifications (subclause 16.2.1 of TS 38.213) incorporate this parameter in the calculation of PMAX,CBR.

sl-MaxTransPower limits the maximum transmission power for the resource pool. RAN1 specifications (subclause 16.2.1 of TS 38.213) assume this value should be incorporated in the calculation of PCMAX that is performed in TS 38.101-1.

Issue 2: RAN4 also had some discussion on whether the parameter should be associated with or without a serving cell on the NR V2X carrier, and three options are proposed: - Option 1: The parameter can be associated either with a serving cell or without a serving cell, and it can be configured separately with p-max for Uu - Option 2: The parameter can be associated either with a serving cell or without a serving cell, when the parameter is associated with a serving cell, PEMAX,c is the smaller value given by this parameter for SL and p-max for Uu of that serving cell. - Option 3: when UE is associated with a serving cell on the NR V2X carrier, p-max is used for serving cell c; when the UE is not associated with a serving cell on the NR V2X carrier, the parameter given for SL in RAN2 specification is used. RAN4 would like to check which option is aligned with the RAN1 and RAN2 specification.

RAN1 notes that all options are compatible with RAN1 specifications and assumes RAN4 will make the final decision.

# Actions

To RAN4:

Action: RAN1 kindly asks RAN4 to take the above reply into account for further work.

# Dates of Next TSG-RAN1 Meetings:

RAN1 #109-e			16 May – 27 May 2022	   			Electronic