3GPP TSG RAN WG1 #108-e									R1-2202104

e-Meeting, February 21st – March 3rd, 2022

Title:	Draft Reply to RAN4 LS on SRS for multi-RTT positioning

Response to:       R4-2202680

Release:	Rel-17

Work Item:	NR_pos_enh-Core

Source:	Qualcomm Incorporated [RAN1]

To:	RAN4

CC:	RAN2, RAN3

# Contact Person:

# Name:	Alexandros Manolakos

E-mail Address:	amanolak@qti.qualcomm.com

# 1. Overall Description:

RAN1 thanks RAN4 for their LS related to the SRS for multi-RTT positioning and would like to provide a reply to the following question:

RAN4 kindly requests RAN1 to confirm whether Rel-15 SRS are also applicable for UE Rx-Tx time difference measurement and gNB Rx-Tx time difference measurement, and if so, from which release.

With regards to NR Rel-16 release,

- According to the TS 38.822 NR UE Feature list, the Feature Group related to the support of UE Rx-Tx measurement report (FG 13-11) has as prerequire the FG “support of SRS for Positioning” (FG 13-8).

- A subset of related agreements and working assumptions during NR Rel-16 WI is provided below:

Agreement(RAN1#99):

Support reuse of Rel-15 SRS resource set for NR UL RTOA, AoA and gNB RSRP measurements for positioning in NR.

- Note: There is no impact to specifications managed by RAN1

- Note: There is no impact to specifications managed by RAN4 for UE requirements

- Note: No new UE behaviour is expected

Agreement (RAN1#98):

- gNB Rx-Tx time difference is defined with respect to the subframe timing associated with the UE

- Multiple SRS resources for positioning purposes can be used to determine the received UL subframe timing of the first arrival path of the UE.

- FFS: The resource ID(s) or resource set ID(s) used for determining the timing of the UE and possibly the Rx beam used at the gNB in the gNB Rx-Tx time difference measurements can be requested for reporting in the measurement report.

Working assumption (RAN1#98Bis):

A UE can be configured to report multiple Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL PRS resource/resource set. The DL PRS resource/resource sets can be in different positioning frequency layers

- FFS: Reporting of SRS for positioning resource/resource set ID corresponding to a UE Rx-Tx time difference measurement

- Note: This agreement does not introduce any new behavior for the transmission of SRS for positioning.

With regards to NR Rel-17,

- A subset of related agreements are provided below:

Agreement (RAN1 #106-e):

Reporting of one gNB Rx-Tx time difference and multiple UL-AOAs measurements for the first arrival path per SRS resource for positioning in a single gNB report to LMF is supported

- The above measurements are associated with SRS resource ID which is also reported to LMF

- FFS: Reporting of RSRP for the first arrival path

Agreement:

- For the first arrival path measurements on SRS for positioning resource,

  - gNB can report to LMF the following set of measurements {one SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA}

  - gNB can report to LMF the following set of measurements {one SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one-gNB Rx-Tx time difference}

  - FFS additional option: gNB can report to LMF the following set of measurements {multiple SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA, one-gNB Rx-Tx time difference}

  - All gNB measurements above are associated with SRS resource ID and timestamp, which are also reported to LMF

- For the first arrival path measurements on SRS for MIMO resource,

  - gNB can report to LMF the following set of measurements {one SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA}

  - FFS: gNB can report to LMF the following set of measurements {multiple SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA}

  - All gNB measurements above are associated with SRS resource ID and timestamp, which are also reported to LMF

  - Note: The operation of SRS for MIMO is transparent to the UE

In summary, Rel-15 SRS is not applicable for UE Rx-Tx time difference measurement and gNB Rx-Tx time difference measurement in neither NR Rel-16 release, nor NR Rel-17 release.

# 2. Actions to RAN4:

RAN1 respectfully requests RAN4 to take the abovementioned information into account.

# 3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #109-e	May 2022 		      		e-meeting

TSG RAN WG1 Meeting #110	August 2022 			     Toulouse,FR