3GPP TSG RAN WG1 Meeting #108-e		R1-2202111

# E-meeting, February 21st – March 3rd, 2022

Title:	Draft Reply LS to RAN2 on the SDT BWP Configuration for RedCap UE

Response to:	R2-2201828

Release:	Release 17

Work Item:	NR_SmallData_INACTIVE-Core

AI:	5.2

Source:	Qualcomm [RAN1]

To:	RAN2

Contact person:	Jing Lei

leijing@qti.qualcomm.com

Send any Reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall Description

RAN1 would like to thank RAN2 for the LS on the following agreements:

- RAN2 confirms that SDT will be configured only on initial BWP and there is no L1 ACK feedback for CG-SDT. - RAN2 agreed that there is no restriction on the candidate values of CG period for CG-SDT.

Besides, RAN1 would like to thank RAN2 for the following question on SDT resource configuration in the BWP of RedCap UE:

RAN2 would like RAN1 to confirm that the separate BWP in case of REDCAP may still be considered as the initial BWP and SDT resources can hence be configured on this BWP for REDCAP UEs.

For the above question, RAN1 would like to inform RAN2 that both RA-SDT and CG-SDT resources can be configured for RedCap UE in the initial BWP separately configured for RedCap UE [2]. More specifically,

- For a cell that allows a RedCap UE to access, network can configure a separate initial DL/UL BWP for RedCap UE, wherein the supported BW for the separate initial DL/UL BWP can have any values up to the maximum UE BW, and this applies to both TDD and FDD (including FD-FDD and HD-FDD).

- For both RA-SDT and CG-SDT, a RedCap UE with valid TA timer can transmit PRACH/PUSCH/PUCCH in an initial UL BWP separately configured for RedCap UE.

- If SSB and CORESET#0 are included in the initial DL BWP separately configured for RedCap UE, SDT resources (including CSS and USS sets for CG-SDT, or CSS sets for RA-SDT) can be configured in the separate initial DL BWP of RedCap UE.

- If SSB or the entire CORESET#0 are not included in the initial DL BWP separately configured for RedCap UE, SDT resources can still be configured in the separate initial DL BWP for RedCap UE based on UE capability.

- If the separate initial DL BWP of RedCap UE is configured with CSS sets for RA-SDT but not for paging, the RedCap UE is not required to monitor paging PDCCH when performing RA-SDT in the separate initial DL BWP.

# 2	Actions to RAN2:

ACTION: RAN1 respectfully asks RAN2 to take the above mentioned information into account.

# 3	Dates of next TSG RAN WG1 meetings

TSG RAN WG1 Meeting #109-e		16 – 27 May 2022, Electronic

# References

- R2-2201828, “Reply LS on the L1 aspects of small data transmission,” 3GPP TSG RAN2#116bis-e, January 2022.

- Chairman’s Notes, 3GPP RAN1#107e, November 2021.