3GPP TSG RAN WG1 Meeting #108-e    	            R1-2202113

E-meeting, February 21st – March 3rd, 2022

Source:	Qualcomm Incorporated

Title:	Clarification on PDCCH monitoring for Case 1-2

Agenda Item:	7.1

Document for: 	Discussion and Decision

# Background

In this contribution, we propose a clarification on PDCCH monitoring capability for Case 1-2.

# Discussion

At the RAN1#91 meeting, following agreements have been made:

Agreements: - For information, the following cases are clarified: - Case 1: PDCCH monitoring periodicity of 14 or more symbols - Case 1-1: PDCCH monitoring on up to three OFDM symbols at the beginning of a slot - Case 1-2: PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot - For a given UE, all search space configurations are within the same span of 3 consecutive OFDM symbols in the slot - Case 2: PDCCH monitoring periodicity of less than 14 symbols - Note: this includes the PDCCH monitoring of up to three OFDM symbols at the beginning of a slot

The Case 1-2 was intended to support DSS operations. In DSS, LTE-CRS is mapped on some symbols and these symbols are not available for NR-PDCCH monitoring. Therefore, Case 1-2 enables a UE to monitor PDCCH on a single span of three contiguous OFDM symbols that is not the first three consecutive OFDM symbols in a slot. The corresponding UE capability, pdcch-MonitoringSingleOccasion, has been specified for SCS 15kHz in TS 38.306 as follows [1].

| pdcch-MonitoringSingleOccasion Indicates whether the UE supports receiving PDCCH in a search space configured to be monitored within a single span of any three contiguous OFDM symbols in a slot with the capability of supporting at least 44 blind decodes in a slot for 15 kHz subcarrier spacing. | UE | No | No | FR1 only |
| --- | --- | --- | --- | --- |

However, PDCCH monitoring cannot be such flexible. According to TS 38.213 Table 13-11, the first symbol index of a PDCCH monitoring occasion for Type-0 CSS set in FR1 is {0, 1, 2, or NsymbCORESET}, where NsymbCORESET is the number of symbols for CORESET #0. Moreover, in DSS scenarios, LTE-CRS is present on symbols #0 and #4 when the number of CRS antenna ports is 1 or 2, and on symbols #0, #1, and #4 when the number of CRS antenna ports is 4. Therefore, the available symbols for NR PDCCH monitoring in Case 1-2 with the UE capability pdcch-MonitoringSingleOccasion are limited up on consecutive symbols #1 – #3 in a slot in reality.

(a) LTE-CRS 2 ports				(b) LTE-CRS 4 ports

Fig.1	Symbols available for NR-PDCCH monitoring on a DSS carrier

There are some other aspects indicating that single span of PDCCH monitoring on any symbols in a slot does not make sense:

- Single span of PDCCH monitoring occasion in a middle/end of a slot does not enable PDSCH scheduling over the entire slot with a single DCI format when K0 = 0.

- As far as we know, there is no real deployment that requires PDCCH monitoring in a middle/end of a slot for DSS operation.

Based on the discussion above, we propose to clarify for Rel-16 that the UE capability pdcch-MonitoringSingleOccasion is to enable PDCCH monitoring associated with any search space configurations within the same span of three consecutive OFDM symbols that are within the first four OFDM symbols of a slot. The TP to TS 38.306 can be as follows.

| pdcch-MonitoringSingleOccasion Indicates whether the UE supports receiving PDCCH in a search space configured to be monitored within a single span of any three contiguous OFDM symbols that are within the first four OFDM symbols in a slot with the capability of supporting at least 44 blind decodes in a slot for 15 kHz subcarrier spacing. | UE | No | No | FR1 only |
| --- | --- | --- | --- | --- |

Proposals:

- Agree to clarify for Rel-16 that the UE capability pdcch-MonitoringSingleOccasion is to enable PDCCH monitoring associated with any search space configurations within the same span of three consecutive OFDM symbols that are within the first four OFDM symbols of a slot for 15kHz subcarrier spacing.

- Send a LS to RAN2 to capture the following change to the description of pdcch-MonitoringSingleOccasion in TS 38.306 for Rel-16.

  - Indicates whether the UE supports receiving PDCCH in a search space configured to be monitored within a single span of any three contiguous OFDM symbols that are within the first four OFDM symbols in a slot with the capability of supporting at least 44 blind decodes in a slot for 15 kHz subcarrier spacing.

# Conclusion

In this contribution, we propose the following:

Proposals:

- Agree to clarify for Rel-16 that the UE capability pdcch-MonitoringSingleOccasion is to enable PDCCH monitoring associated with any search space configurations within the same span of three consecutive OFDM symbols that are within the first four OFDM symbols of a slot for 15kHz subcarrier spacing.

- Send a LS to RAN2 to capture the following change to the description of pdcch-MonitoringSingleOccasion in TS 38.306 for Rel-16.

  - Indicates whether the UE supports receiving PDCCH in a search space configured to be monitored within a single span of any three contiguous OFDM symbols that are within the first four OFDM symbols in a slot with the capability of supporting at least 44 blind decodes in a slot for 15 kHz subcarrier spacing.

# Reference

- 3GPP TS 38.306.