3GPP TSG RAN WG1 #108-e	R1-2202121

e-Meeting, February 21th – March 3rd, 2022

Agenda item:	7.2.6

Source: 	Qualcomm Incorporated

Title: 	Discussion on coefficients partition in eT2 CSI

Document for:	Discussion/Decision

# Introduction

Enhanced Type II codebook and Enhanced port-selection Type II codebook are introduced Rel-16. In CSI feedback based on these two codebooks, the actual number of coefficients, i.e., KNZ, is reported by UE. These KNZ coefficients are partitioned into two groups. However, the spec is unclear how to partition the coefficients when KNZ≤2*ν, where ν is the rank. In this contribution, we will discuss this issue and propose solution to clarify it in the spec.

# Issue 2: Coefficient partition

In reporting of the linear combination coefficients, the total KNZ non-zero coefficients are partitioned into two groups, wherein the first KNZ/2-ν coefficients are said to have higher priority and are packed firstly (omitted secondly) in Group 1, while the remaining KNZ/2 coefficients are said to have lower priority and are packed secondly (omitted firstly) in Group 2. However, if KNZ<2*ν, the value KNZ/2-ν could be negative and it causes ambiguity in partitioning.

In RAN1 #107-e, the same principle of coefficients partitioning was adopted for Rel-17 FeType II PS codebook, and the aforementioned issue was resolved with the following agreement:

Agreement

For Rel-17 PS codebook:

- Alt 2(updated #2): UCI Group 1 includes the max(0, KNZ2-υ)  highest priority elements of i2,4,l and the max(0, KNZ2-υ)  highest priority elements of i2,5,l (l=1,…,υ). UCI Group 2 includes the min(KNZ-v, KNZ2)   lowest priority elements of i2,4,l and the min(KNZ-v, KNZ2) lowest priority elements of i2,5,l (l=1,…,υ)

This agreement implies that when KNZ/2-ν<0, there is no coefficient partition and all coefficients are reported in UCI part 2. Hence, since both Rel-16 eType II codebook and Rel-17 FeType II PS codebook share many design principles in common, it is preferred reuse the same approach to align the UE behaviour in these two codebooks. Based the discussion, we propose

Proposal: For Rel-16 Enhanced Type II and Enhanced Type II port-selection, clarify that UCI Group 1 includes the max(0, KNZ2-υ)  highest priority elements of i2,4,l and the max(0, KNZ2-υ)  highest priority elements of i2,5,l (l=1,…,υ). UCI Group 2 includes the min(KNZ-v, KNZ2)   lowest priority elements of i2,4,l and the min(KNZ-v, KNZ2) lowest priority elements of i2,5,l (l=1,…,υ)

Following text proposal can be considered for TS38.214 spec:

### 5.2.3 CSI reporting using PUSCH

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

-	For Enhanced Type II reports, for a given CSI report n, each reported element of indices  i2,4,l i2,5,l and i1,7,l, indexed by l,i and f, is associated with a priority value Pril,i,f=2⋅L⋅υ⋅πf+υ⋅i+l, with π(f)=min⁡(2⋅n3,lf,2⋅N3-n3,l(f)-1) with l=1,2,…,υ, i=0,1,…,2L-1, and f=0,1,…,Mυ-1, and where n3,l(f) is defined in Clause 5.2.2.2.5. The element with the highest priority has the lowest associated value Pril,i,f. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where

-	Group 0 includes indices i1,1, i1,2 and i1,8,l (l=1,…,υ).

-	Group 1 includes indices i1,5 (if reported), i1,6,l (if reported), the υ2LMυ-KNZ/2 highest priority elements of i1,7,l,  i2,3,l, the  max(0, KNZ2-υ) KNZ/2-υ highest priority elements of i2,4,l and the  max(0, KNZ2-υ) KNZ/2-υ highest priority elements of i2,5,l (l=1,…,υ).

-	Group 2 includes the KNZ/2 lowest priority elements of i1,7,l, the min(KNZ-v, KNZ2) KNZ/2 lowest priority elements of i2,4,l and the min(KNZ-v, KNZ2) KNZ/2 lowest priority elements of i2,5,l (l=1,…,υ).

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

Following text proposal can be considered for TS38.212 spec:

### 6.3.2 Uplink control information non PUCSH

### 6.3.2.1.2 CSI only

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

Table 6.3.2.1.2-5A: Mapping order of CSI fields of one CSI report, CSI part 2 of codebookType=typeII-r16 or typeII-PortSelection-r16

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | PMI fields X1, from left to right as in Tables 6.3.2.1.2-1A/2A, if reported |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2, from left to right, as in Tables 6.3.2.1.2-1A/2A: i2,3,l:l=1,…,υ, i1,5, i1,6,l:l=1,…,υ and max(0, KNZ2-υ)(KNZ2-υ)×3 highest priority bits of i2,4,l:l=1,…,υ,max(0, KNZ2-υ)(KNZ/2-υ)×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*2LMυ-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on function Pril,i,f defined in clause 5.2.3 of TS38.214, if reported |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2, from left to right, as in Tables 6.3.2.1.2-1A/2A: min(KNZ-v, KNZ2)KNZ/2×3 lowest priority bits of i2,4,l:l=1,…,υ, min(KNZ-v, KNZ2)KNZ/2×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on function Pril,i,f defined in clause 5.2.3 of TS38.214, if reported |

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

# Conclusion

In this contribution, we discuss issue in coefficients partition in enhanced Type II and enhanced Type II port-selection codebook. Following is the proposal and we suggest the text proposal in Section 2.

Proposal: For Rel-16 Enhanced Type II and Enhanced Type II port-selection, clarify that UCI Group 1 includes the max(0, KNZ2-υ)  highest priority elements of i2,4,l and the max(0, KNZ2-υ)  highest priority elements of i2,5,l (l=1,…,υ). UCI Group 2 includes the min(KNZ-v, KNZ2)   lowest priority elements of i2,4,l and the min(KNZ-v, KNZ2) lowest priority elements of i2,5,l (l=1,…,υ)