3GPP TSG-RAN WG1 Meeting #108-e	R1-2202128

e-Meeting, February 21th – March 3rd, 2022

Agenda item:	8.1.4

Source: 	Qualcomm Incorporated

Title: 	Remaining Details of mTRP CSI

Document for:	Discussion/Decision

# Editorial correction

We noticed two issues in the following part of 38.214:

-	If the higher layer parameter csi-ReportMode is set to 'Mode2', one CRI k1 (k1≥0) is reported, which corresponds to the (k1+1)-th entry of the M+N resources or Resource Pairs in the corresponding CSI-RS Resource Set for channel measurement, and (k1+1)-th entry of the associated resources in the corresponding CSI-IM Resource Set, if configured. The first M codepoints of the CRI correspond to resources associated to Group 1 and Group 2. The last N codepoints of the CRI correspond to the N configured Resource Pairs. The UE shall report one RI, one PMI, one LI, if configured, and one CQI conditioned on CRI k1 if k1<M; or two RIs, two PMIs, two LIs, if configured, associated to the resource in Group 1 and the resource in Group 2, respectively, of the (k1+1)-th Resource Pair, and one or two CQIs, otherwise.

- Regarding the yellow highlighted part, one or two CQIs should be for the case of sTRP (if k1<M) and one CQI should be for NCJT CSI for a resource pair.

- Regarding the pink highlighted part, it should be the (k1+1-M)-th Resource Pair as the first M values of k1 correspond to sTRP CSI measurement hypotheses and do not correspond to a resource pair.

Hence, we have the following proposal to address the two issues above.

Proposal 1: Adopt the following TP to correct the two editorial issues mentioned above:

============TP for 38.214 Section 5.2.1.4.2 ====================================

--Unchanged part omitted------------------------

-	If the higher layer parameter csi-ReportMode is set to 'Mode2', one CRI k1 (k1≥0) is reported, which corresponds to the (k1+1)-th entry of the M+N resources or Resource Pairs in the corresponding CSI-RS Resource Set for channel measurement, and (k1+1)-th entry of the associated resources in the corresponding CSI-IM Resource Set, if configured. The first M codepoints of the CRI correspond to resources associated to Group 1 and Group 2. The last N codepoints of the CRI correspond to the N configured Resource Pairs. The UE shall report one RI, one PMI, one LI, if configured, and one or two CQIs conditioned on CRI k1 if k1<M; or two RIs, two PMIs, two LIs, if configured, associated to the resource in Group 1 and the resource in Group 2, respectively, of the (k1+1-M)-th Resource Pair, and one CQI, otherwise.

===============================================================

# Conclusion

In this contribution, we proposed:

Proposal 1: Adopt the following TP to correct the two editorial issues mentioned above:

============TP for 38.214 Section 5.2.1.4.2 ====================================

--Unchanged part omitted------------------------

-	If the higher layer parameter csi-ReportMode is set to 'Mode2', one CRI k1 (k1≥0) is reported, which corresponds to the (k1+1)-th entry of the M+N resources or Resource Pairs in the corresponding CSI-RS Resource Set for channel measurement, and (k1+1)-th entry of the associated resources in the corresponding CSI-IM Resource Set, if configured. The first M codepoints of the CRI correspond to resources associated to Group 1 and Group 2. The last N codepoints of the CRI correspond to the N configured Resource Pairs. The UE shall report one RI, one PMI, one LI, if configured, and one or two CQIs conditioned on CRI k1 if k1<M; or two RIs, two PMIs, two LIs, if configured, associated to the resource in Group 1 and the resource in Group 2, respectively, of the (k1+1-M)-th Resource Pair, and one CQI, otherwise.

===============================================================