3GPP TSG RAN WG1 Meeting #108-e	R1- 2202131

February 21st – March 03rd, 2022

Agenda item:	8.2.4

Source: 	Qualcomm Incorporated

Title: 	Beam management for new SCSs

Document for:	Discussion/Decision

- Introduction

The following agreements have been achieved for beam management for new SCSs in RAN1 #107-bis-e.

Conclusion Multiple SRIs for multi-PUSCHs by a single DCI are not supported in Rel-17. Agreement For maxNumberRxTxBeamSwitchDL, reuse the following candidate values {4, 7, 14} of 120 kHz in FR2-1 for 120 kHz in FR2-2. Agreement Support the value of BeamAppTime_r17 for 120 kHz SCS in FR2-2 as the same value for 120 kHz in FR2-1. Support the value of BeamAppTime_r17 for 480 kHz SCS as 4 times its value for 120 kHz. Support the value of BeamAppTime_r17 for 960 kHz SCS as 8 times its value for 120 kHz. R1-2200783 [Draft] LS on a minimum guard period between two SRS resources for antenna switching InterDigital Final LS is endorsed in R1-2200796, changing “kindly” to “respectfully”. The TP below for section 5.1.5 of TS38.214v17.0.0 is endorsed ============================== Start of TP for TS 38.214 =============================== 5.1.5 Antenna ports quasi co-location *** Unchanged text omitted *** When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a the corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL. *** Unchanged text omitted *** If the PDCCH carrying the scheduling DCI is received on one component carrier, and a the PDSCH scheduled by that DCI is on another component carrier: - The timeDurationForQCL is determined based on the subcarrier spacing of the scheduled PDSCH. If µPDCCH < µPDSCH an additional timing delay d2μPDSCH2μPDCCH is added to the timeDurationForQCL, where d is defined in 5.2.1.5.1a-1, otherwise d is zero; - When the UE is configured with enableDefaultBeamForCCS, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, or if the DL DCI does not have the TCI field present, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell. *** Unchanged text omitted *** ============================== End of TP for TS 38.214 =============================== Conclusion From RAN1 perspective, there is no consensus to enhance beam management for shared spectrum operation in Rel-17.

- Beam Switch Related Issues

In RAN1 #107be, it is decided that the value of beam switch gap will depend on RAN4’s feedback. On the application scenarios, our preference is that the beam switch gap should be applied to all the 7 cases listed in the following proposal with motivations described below.

- For Case 1, the motivation is for any two DL signals with different beams specified by NW or spec to have the beam switch gap in between;

- For Case 2, the motivation is that UE may use corresponding refined Rx beam for SSB or CSI-RS without specified QCL-TypeD. So beam switch gap is also needed between the signal with specified QCL-TypeD and the other signal without specified QCL-TypeD;

- For Case 3, the motivation is similar to Case 2;

- For Case 4, the motivation is that UE is expected to perform Rx beam sweep across CSI-RS resources in the same resource set with higher layer parameter Repetition set to ‘ON’, where the CSI-RS resources are indicated by the same TCI state, which is mapped to the refined Rx beam;

- For Case 5, the motivation is for any two UL signals with different beams specified by NW or spec to have the beam switch gap in between;

- For Case 6, the motivation is that UE may associate a fixed Tx beam with an SRS resource for BM without specified spatial relation for UL BM based on Tx beam sweep. So beam switch gap is also needed between the signal with specified spatial relation and SRS without specified spatial relation;

- For Case 7, the motivation is similar to Case 6.

Proposal 1: Support UE capability for the beam switch time gap, required between two signals in each of the following cases.

- Case 1:

  - Any two signal that have different specified QCL-TypeD assumptions

- Case 2:

  - One signal has specified QCL-TypeD assumption, while the other is SSB or CSI-RS without specified QCL-TypeD assumption

- Case 3:

  - The two signals are two CSI-RS resources without specified QCL-TypeD assumption

- Case 4:

  - The two signals are two CSI-RS resources with same specified QCL-TypeD assumption in a resource set with higher layer parameter Repetition set to ‘ON’

- Case 5:

  - Any two signals that have different specified spatial relations

- Case 6:

  - One signal has specified spatial relation assumption, while the other is SRS without specified spatial relation

- Case 7:

  - The two signals are two SRS resources without specified spatial relation

- Note:

  - The above “specified” means the beam indication is either explicitly provided by gNB or implicitly determined by spec.

In RAN1 #107be, LS is sent to RAN4 for the guard symbols between two SRS resources for antenna switching for both 480 and 960 kHz. In our view, we prefer to scale the number of symbols for 120 kHz accordingly, i.e. the number of symbols is 8 and 16 for 480 and 960 kHz, based on the existing number of 2 for 120 kHz.

Proposal 2: Support that the number of guard symbols between two SRS resources for antenna switching is 8 for 480 kHz, and 16 for 960 kHz.

- Conclusion

In this contribution we discussed enhancements for Rel-17 beam management for higher band. Below are the proposals:

Proposal 1: Support UE capability for the beam switch time gap, such that UE is NOT expected to be scheduled/configured with a signal within the beam switch action time before and after another signal if the two signals correspond to one of the following cases.

- Case 1:

  - Any two signal that have different specified QCL-TypeD assumptions

- Case 2:

  - One signal has specified QCL-TypeD assumption, while the other is SSB or CSI-RS without specified QCL-TypeD assumption

- Case 3:

  - The two signals are two CSI-RS resources without specified QCL-TypeD assumption

- Case 4:

  - The two signals are two CSI-RS resources with same specified QCL-TypeD assumption in a resource set with higher layer parameter Repetition set to ‘ON’

- Case 5:

  - Any two signals that have different specified spatial relations

- Case 6:

  - One signal has specified spatial relation assumption, while the other is SRS without specified spatial relation

- Case 7:

  - The two signals are two SRS resources without specified spatial relation

- Note:

  - The above “specified” means the beam indication is either explicitly provided by gNB or implicitly determined by spec.

Proposal 2: Support that the number of guard symbols between two SRS resources for antenna switching is 8 for 480 kHz, and 16 for 960 kHz.

- References

- RP-202925 Updated higher band WID