3GPP TSG-RAN WG1 #108-e	R1- 2202132

e-Meeting, Febuary 21st – March 3rd , 2022

Agenda item:	8.2.5

Source: 	Qualcomm Incorporated

Title: 	PDSCH and PUSCH enhancements for 52.6-71GHz band

Document for:	Discussion/Decision

# Introduction

In RAN #90e, a Rel-17 work item for NR operation in a frequency regime between 52.6GHz and 71GHz has been approved [1]. As a part of the work item, the study on the PDSCH/PUSCH potential enhancements with new agreed numerologies, i.e., 120kHz, 480kHz, and 960kHz was conducted. In this contribution, we discuss some important aspects needed to complete the design for FR2-2.

# Discussion

## Multi-PDSCH/PUSCH with single grant

### HARQ-ACK enhancements

#### Type-2 HARQ-ACK codebook enhancements

Regarding type-2 HARQ-ACK codebook enhancements for multi-PDSCH scheduling, we reached the following agreements in RAN1 #107-e [7].

Agreement For multi-PDSCH scheduling with a single DCI - Introduce a new RRC parameter, e.g., numberOfHARQ-BundlingGroups, to configure the number of HARQ bundling groups with value range {1, 2, 4} for type-2 HARQ-ACK codebook per serving cell. - If the RRC parameter is not configured for a serving cell, time domain bundling for type-2 HARQ-ACK codebook is not enabled for the serving cell. - The maximum number of PDSCHs allocated to each bundling group is ceil(NPDSCH,MAX/NHBG) where NHBG is the number of bundling groups configured by numberOfHARQ-BundlingGroups for a serving cell and NPDSCH,MAX is the maximum configured number of PDSCHs for the serving cell. - The PDSCHs corresponding to [configured or valid] SLIVs in a TDRA row index indicated by multi-PDSCH scheduling DCI are allocated to the bundling groups, e.g., if NHBG =4, NPDSCH,MAX =8, and 5 PDSCHs are scheduled, then 2/1/1/1 PDSCHs are assigned to each group, by reusing CBG grouping method. - For a group that is empty or is filled with only invalid PDSCH(s), HARQ-ACK bits for the bundling group is set to NACK (same principle as when no time bundling configured) - Logical AND operation is applied to across all valid PDSCHs within the same bundling group to generate 1 HARQ-ACK bit per group, at least for 1-TB case - If the number of HARQ bundling groups is configured as 1 for a serving cell, HARQ-ACK bits corresponding to any DCI for the serving cell belong to the first sub-codebook. - At least for 1-TB case, if the number of HARQ bundling groups is configured as larger than 1 for a serving cell, HARQ-ACK bits corresponding to multi-PDSCH scheduling case (which implies a multi-PDSCH DCI schedules more than one PDSCH) for the serving cell belong to the second sub-codebook, - Where the number of HARQ-ACK bits corresponding to a multi-PDSCH DCI is determined based on the maximum of Q value across all serving cells within the same PUCCH cell group, and Q=maximum configured number of PDSCHs for a cell without numberOfHARQ-BundlingGroups configured or Q=number of configured HARQ bundling groups for a cell with numberOfHARQ-BundlingGroups configured

In our view, the bundling process should include only the valid PDSCHs, not the configured SLIVs as this will allow having better resolution for feedback bits. For example, assume 4 configured SLIVs and two bundling groups, however, the last two SLIVs are invalid due to collision with semi-static UL symbols. If the bundling process is based on the configured SLIV, then, the first bundling group will contain the two valid PDSCHs, and for the second bundling group a NACK will be sent corresponding to the two invalid SLIVs. On the other hand, if the bundling process is based on the valid SLIVs, the first bundling group will carry only the A/N of the first valid PDSCH, while the second bundling group will carry only the A/N of the second valid PDSCH.

Proposal 1: For generating type-2 HARQ-ACK codebook, the formation of the bundling groups should be based on the valid SLIVs

As agreed in WI, the PDSCHs that overlaps with semi-static UL symbols will be skipped. Under certain configuration scenarios, we end up a DCI that schedule multiple PDSCHs, all of them except one are overlapping with semi-static UL symbols, i.e., only one PDSCH will be transmitted. In such scenarios, a reasonable design will feedback the survived PDSCH in sub codebook dedicated for fallback DCIs and single PDSCH grant scheduled by a DCI that can schedule multiple PDSCHs as this will reduce the feedback size especially if no time domain bundling is configured.

Proposal 2: If all PDSCHs scheduled by a DCI that schedules multi-PDSCHs (TDRA row has multiple SLIVs) except one PDSCH will not be transmitted due to overlap with semi-static UL symbols, then A/N bit of the valid PDSCH will be carried in the codebook of fallback and single-PDSCH grants.

For example, in the CR of TS 38.213, the following change can be made

… - the UE determines a first HARQ-ACK sub-codebook based on each detected DCI format scheduling one valid PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, or SPS PDSCH receptions, if any, and - the UE determines a second HARQ-ACK sub-codebook based on each detected DCI format scheduling more than one valid PDSCH reception, and …

### mTRP with intra-slot repetitions

The case of “tdmSchemeA” with a multi-PDSCH grant needs special handling. For example, in Rel. 16, for a single PDSCH grant, the mapping type of both repetitions must be type B, as mentioned in [TS 38.214, 6]: “The UE expects the PDSCH mapping type indicated by DCI field 'Time domain resource assignment' to be mapping type B, and the indicated PDSCH mapping type is applied to both PDSCH transmission occasions.” Now, with multi-PDSCH scheduling with a single DCI, each TDRA row can have up to 8 PDSCHs and the mapping type is indicated per PDSCH. Following the legacy rule, and to allow scheduling a UE in mTRP mode with tdmSchemeA, gNB must configure entries in the TDRA table where all the SLIVs have mapping type B which can limit the flexibility of scheduling. We can overcome this by introducing new a rule for multi-PDSCH grant. For example, when the UE is scheduled with multi-PDSCH grant and tdmSchemeA, it can always assume that the mapping type is B regardless of the mapping type of each SLIV defined in the TDRA table, or it can follow the mapping type indicated by the TDRA table for the first repetition and it assumes mapping type B for the second repetition. This will provide enough flexibility when configuring the TDRA table and scheduling a multi-PDSCH grant with tdmSchemeA.

Proposal 3: In the case of multi-PDSCH scheduling via a single DCI with 'tdmSchemeA', consider one of the following options

- Option 1: UE assumes PDSCH mapping Type B for first and second repetitions of each TB regardless of the mapping type for each SLIV of the indicated TDRA row.

- Option 2: UE applies the mapping type of each SLIV as indicated by the TDRA assignment field to the first repetition of the corresponding PDSCH and assumes PDSCH mapping Type B for the second repetition of each PDSCH.

In addition, we need also to handle the overlapping between the scheduled PDSCHs (two repetitions of each SLIV) and the semi-static UL symbols. In Rel. 16, there is no special handling of such overlapping for single PDSCH grants, i.e., the conflicting of dynamic PDSCH with semi-static UL symbols is treated as an error case. For multi-PDSCH scheduling with single DCI, it is assumed that the PDSCHs that overlap with semi-static UL symbols are assumed to be not valid and the HARQ process number increment will be skipped for such PDSCHs. In the case of multi-PDSCH scheduling via a single DCI with ‘tdmSchemeA’, the semi-static UL symbols may overlap with one or more of the scheduled PDSCHs, the overlap can occur over the first or/and second repetition of each TB. In order to ensure enough scheduling flexibility and simplify the generation of HARQ-ACK codebook type-1, two options can be considered. Option 1: If the first or/and the second repetition of any TB overlaps with semi-static UL symbols, the corresponding PDSCH will be skipped, and Option 2: If the first repetition of any TB overlaps with semi-static UL symbols, the corresponding PDSCH will be skipped, and if there is only overlap between the second repetition and semi-static UL symbols, the second repetition will be skipped but the PDSCH is assumed to be valid, i.e., UE should report A/N bit for such PDSCH.

Proposal 4: In the case of multi-PDSCH scheduling via a single DCI with 'tdmSchemeA', consider one of the following options to handle the overlap with semi-static UL symbols

- Option 1: If one of the repetitions of the PDSCH collides with semi-static UL symbols, the corresponding PDSCH is considered as not valid

- Option 2: If the first repetition of the PDSCH collides with semi-static UL symbols, the corresponding PDSCH is considered as not valid

  - On the other hand, if only the second repetition of the PDSCH collides with semi-static UL symbol, the PDSCH is still considered valid

## Out-of-order Rules

In RAN1 #106bis-e and #107bis-e, we have made the following agreement and conclusion, respectively, about the out-of-order scheduling.

Agreement: For two multi-PDSCH (or two multi-PUSCH) scheduling DCIs, UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCI to lead to out-of-order scheduling. - FFS: whether to allow OOO scheduling for the following two cases: - for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH) - for the case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV - Note: The above FFS aspect applies only to multi-PDSCH and multi-PUSCH scheduling with single DCI ### Conclusion: - UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCIs to lead to out-of-order scheduling, also for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH). - This may not have specification impact. - Note: It is separately discussed whether the scheduled PDSCHs (or PUSCHs or SLIV) is based on configured SLIV or valid SLIV.

In our view, we do not see a strong motivation to allow grants with overlapping spans, even for the case where one of the grants is a single PDSCH/PUSCH grant and the other one is a multi-PDSCH/PUSCH grant. Also, for the case where the DCIs end in the same symbol, if the scheduled PDSCHs by different DCIs can be interlaced in the time domain, this will complicate the UE processing the HARQ-ACK bookkeeping, and we do not see the need of allowing such behaviour. While the first sub-bullet of the FFS is not allowed by the legacy rules of OOO, the second sub-bullet will be allowed with Rel. 16 OOO rules, therefore, we propose to add an additional rule to disallow any to DCIs that scheduled PDSCHs (PUSCHs) from having an overlapping span, i.e., all the allocations by the first DCI should be finished before the first allocation of the second DCI even if the two DCIs are finished in the same symbol.

Proposal 5: The UE does not expect to be scheduled with two DCIs that schedule DL (UL) data allocations with overlapping spans, where the span of the allocations scheduled by one DCI is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV by the same DCI.

For example, the following addition can be made to TS 38.214

… For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH that ends later than symbol i. In a given scheduled cell, the UE is not expected to be scheduled with two DCIs of either format 1_1 or format 1_0 with overlapping data spans, where the span of the allocations scheduled by one DCI is defined from the beginning of the first scheduled valid SLIV till the end of the last scheduled valid SLIV by the same DCI. … For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol i. In a given scheduled cell, the UE is not expected to be scheduled with two DCIs of either format 0_1 or format 0_0 with overlapping data spans, where the span of the allocations scheduled by one DCI is defined from the beginning of the first scheduled valid SLIV till the end of the last scheduled valid SLIV by the same DCI. …

Another issue is how to determine the out-of-order cases if some of the scheduling SLIVs overlap with semi-static symbols of the opposite direction. Our understanding is that the out-of-order rules are defined for PDSCHs/PUSCHs therefore, we should only take into account the valid allocations when determining the out-of-order cases.

Proposal 6: The out-of-order rules should be applied on the valid allocations only.

# Conclusion

Proposal 1: For generating type-2 HARQ-ACK codebook, the formation of the bundling groups should be based on the valid SLIVs

Proposal 2: If all PDSCHs scheduled by a DCI that schedules multi-PDSCHs (TDRA row has multiple SLIVs) except one PDSCH will not be transmitted due to overlap with semi-static UL symbols, then A/N bit of the valid PDSCH will be carried in the codebook of fallback and single-PDSCH grants.

Proposal 3: In the case of multi-PDSCH scheduling via a single DCI with 'tdmSchemeA', consider one of the following options

- Option 1: UE assumes PDSCH mapping Type B for first and second repetitions of each TB regardless of the mapping type for each SLIV of the indicated TDRA row.

- Option 2: UE applies the mapping type of each SLIV as indicated by the TDRA assignment field to the first repetition of the corresponding PDSCH and assumes PDSCH mapping Type B for the second repetition of each PDSCH.

Proposal 4: In the case of multi-PDSCH scheduling via a single DCI with 'tdmSchemeA', consider one of the following options to handle the overlap with semi-static UL symbols

- Option 1: If one of the repetitions of the PDSCH collides with semi-static UL symbols, the corresponding PDSCH is considered as not valid

- Option 2: If the first repetition of the PDSCH collides with semi-static UL symbols, the corresponding PDSCH is considered as not valid

Proposal 5: The UE does not expect to be scheduled with two DCIs that schedule DL (UL) data allocations with overlapping spans, where the span of the allocations scheduled by one DCI is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV by the same DCI.

Proposal 6: The out-of-order rules should be applied on the valid allocations only.

# References

- RP-202925, “Revised WID: Extending current NR operation to 71GHz”, RAN #90e, CMCC

- RP-193259, “New SID: Study on supporting NR from 52.6GHz to 71GHz”, RAN #86, Intel Corporation

- R1-2110640 “Summary #3 of PDSCH/PUSCH enhancements (Scheduling/HARQ)”, RAN1 #106bis-e, Moderator (LG Electronics)

- R1-2110512 “Discussion summary #2 of [106bis-e-NR-52-71GHz-05]”, RAN1 #106bis-e, Moderator (vivo)

- ETSI TS 138 331, “Radio Resource Control (RRC)” V16.1.0 (2020-07)

- ETSI TS 138 214, “Physical layer procedures for data” V16.2.0 (2020-07)

- R1-2112714 “Summary #4 of PDSCH/PUSCH enhancements (Scheduling/HARQ)”, RAN1 #107-e, Moderator (LG Electronics)

- R1-2200744 “Summary #2  of PDSCH/PUSCH enhancements (Scheduling/HARQ)”, RAN1 #107bis-e, Moderator (LG Electronics)