3GPP TSG RAN WG1 #108-e																	R1-2202141

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.5.2

Source: 	Qualcomm Incorporated

Title: 	Maintenance on UL-AOA positioning

Document for:		Discussion and Decision

# Introduction

At RAN1 #107-e meeting, the following agreements were reached with regards to this sub-agenda:

Agreement - ARP location is associated with UL measurements for NR Positioning (UL AOA, UL-RTOA, UL SRS-RSRP, UL SRS-RSRPP and gNB Rx-Tx time difference measurements) - Use of ARP ID for potential overhead reduction in NRPPa signaling is up to RAN3 - Send LS to RAN3 to enable relevant signaling in RAN3 specification R1-2112739 [DRAFT] LS on ARP location association with UL measurements for NR positioning Moderator (Intel) Final LS endorsed in R1-2112740. Agreement - Definition for UL SRS-RSRPP: - Definition - UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response, where UL SRS-RSRPP for 1st path delay is the power corresponding to the first detected path. - Working assumption: For frequency range 1, the reference point for the UL SRS-RSRPP shall be the antenna connector of the gNB. For frequency range 2, UL SRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. - FFS: For frequency range 1 and 2, if receiver diversity is in use by the gNB, the reported UL SRS-RSRPP value shall not be lower than the corresponding UL SRS-RSRPP of any of the individual receiver branches - FFS: Note: First and additional paths RSRP, when provided in the same report, use the same RX branch(es) selected for the first arrival path and for the UL SRS-RSRP if the UL SRS-RSRP is reported - FFS: whether/how to capture the note in the specifications Send LS to RAN4 R1-2112743 [DRAFT] LS on UL SRS-RSRPP definition Moderator (Intel) Final LS endorsed in R1-2112744. Agreement Apply the following changes to the definition for UL SRS-RSRPP in the previous agreement: Definition UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response, where UL SRS-RSRPP for 1st path delay is the power corresponding to the first detected path. - Working assumption: For frequency range 1, the reference point for the UL SRS-RSRPP shall be the antenna connector of the gNB. For frequency range 2, UL SRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. - FFS: For frequency range 1 and 2, if receiver diversity is in use by the gNB, the reported UL SRS-RSRPP value shall not be lower than the corresponding UL SRS-RSRPP of any of the individual receiver branches - FFS: Note: First and additional paths RSRP, when provided in the same report, use the same RX branch(es) selected for the first arrival path and for the UL SRS-RSRP if the UL SRS-RSRP is reported - FFS: whether/how to capture the note in the specifications Note: The following two options are supported by gNB to LMF: - Option 1 (RX diversity for the first path UL SRS-RSRPP) - The same RX branch(es) as applied for the first path UL SRS-RSRPP measurements are used for the additional paths UL SRS-RSRPP measurements if those are provided together - For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements, then reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches - Option 2 (RX diversity for UL SRS-RSRP) - The same RX branch(es) as applied for UL SRS-RSRP measurements are used for UL SRS-RSRPP measurements (i.e., the first and additional paths UL SRS-RSRPP if those are provided)

In this paper, we present our views on the remaining issue related to enhancements on UL AoA positioning.

# Multiple path-RSRP for multiple UL-AoAs

The following proposal was discussed during RAN1 107 meeting:

Proposal 3.4-2 - Select one of the following alternatives - Alt.1 - For the first arrival path measurements on SRS for positioning resource, gNB can report to LMF multiple sets of the following measurement tuples {one SRS-RSRPP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA or one gNB Rx-Tx time difference} - For the first arrival path measurements on SRS for MIMO resource, gNB can report to LMF multiple sets of the following measurement tuples {one SRS-RSRPP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA or one-gNB Rx-Tx time difference} - Alt.2 - For the first arrival path measurements on SRS for positioning resource, gNB can report to LMF multiple sets of the following set of measurements {multiple SRS-RSRPP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA or one gNB Rx-Tx time difference} - For the first arrival path measurements on SRS for MIMO resource, gNB can report to LMF multiple sets of the following set of measurements {multiple SRS-RSRPP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA or one-gNB Rx-Tx time difference} - Above gNB measurements are associated with SRS resource ID and timestamp, which are also reported to LMF

From the above alternatives, we support Alt. 2 with a minor modification:

Proposal 1: Support the following:

- For the first arrival path measurements on SRS for positioning resource, gNB can report to LMF multiple sets of the following set of measurements {multiple SRS-RSRPP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA or one gNB Rx-Tx time difference}

- For the first arrival path measurements on SRS for MIMO resource, gNB can report to LMF multiple sets of the following set of measurements {multiple SRS-RSRPP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA or one-gNB Rx-Tx time difference}

With the above proposal, a gNB may inform an LMF, for a specific TOA, the power of each UL-AoA appearing at that TOA, whereas Alternative 1 only allows to provide the total power of each path across all the angular response.

# Conclusions

We make the following proposal:

Proposal 1: Support the following:

- For the first arrival path measurements on SRS for positioning resource, gNB can report to LMF multiple sets of the following set of measurements {multiple SRS-RSRPP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA or one gNB Rx-Tx time difference}

- For the first arrival path measurements on SRS for MIMO resource, gNB can report to LMF multiple sets of the following set of measurements {multiple SRS-RSRPP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA or one-gNB Rx-Tx time difference}

# References

- RP-202900, “New WID on NR Positioning Enhancements”, CATT, Intel Corporation, Ericsson; 3GPP TSG RAN Meeting #90e, Electronic Meeting, December 7 - 11, 2020.