3GPP TSG RAN WG1 #108-e			R1-2202161

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.12.2

Source:	Qualcomm Incorporated

Title:	Maintenance on UE feedback for Multicast RRC_CONNECTED UEs

Document for:	Discussion and Decision

# Introduction

In this document, we will present our views on the remaining details of UE feedback and retransmission to improve reliability for Multicast RRC_CONNECTED UEs.

# Discussion

## Multiplexing HARQ-ACK feedback of unicast and multicast

Agreement

When UE is configured with different codebook types for unicast and multicast and when UE is scheduled to multiplex HARQ-ACK for unicast and HARQ-ACK for multicast with the same priority in the same PUCCH slot,

- UE generates two separate sub-codebooks for unicast and multicast respectively and then concatenates them by appending sub-codebook for multicast to the sub-codebook for unicast.

  - Note: The PUCCH resource for transmitting the codebook is based on the last unicast DCI.

  - FFS: when Type-3 HARQ-ACK codebook or enhanced Type-2 codebook is used for unicast

  - Define a UE capability

RAN1 needs to discuss the remaining FFS is for the case when enhanced Type-2 codebook or Type-3 HARQ-ACK codebook is used for unicast.

If a UE is configured with enhanced Type-2 HARQ-ACK codebook for unicast and scheduled to multiplex HARQ-ACK feedback for unicast and multicast with the same priority in the same PUCCH slot, the UE generates two separate sub-codebooks for unicast and multicast respectively and then concatenates them by appending sub-codebook for multicast to the sub-codebook for unicast.

Proposal 1: If a UE is configured with enhanced Type-2 HARQ-ACK codebook for unicast and scheduled to multiplex HARQ-ACK feedback for unicast and multicast with the same priority in the same PUCCH slot, the UE generates two separate sub-codebooks for unicast and multicast respectively and then concatenates them by appending sub-codebook for multicast to the sub-codebook for unicast.

If a UE is configured with Type-3 HARQ-ACK codebook for unicast, there were two alternatives discussed in last RAN1 meeting:

    - Alt1: UE generate the Type 3 HARQ-ACK codebook including only unicast PDSCH scheduled by PDCCH with UE-specific RNTI, e.g., C-RNTI

    - Alt2: UE generate the Type 3 HARQ-ACK codebook including both unicast and multicast PDSCH scheduled by PDCCH with C-RNTI and one or multiple G-RNTIs

If the multicast and unicast are using different HPIDs, when their HARQ-ACK-feedbacks collide in the same PUCCH/PUSCH, Alt2 is straightforward to multiplex multicast and unicast feedback in the same Type 3 HARQ-ACK codebook. However, if multicast and unicast are dynamically sharing HPIDs, separate bits for unicast and multicast needs per HARQ process ID for Alt2. The feedback codebook size may be increased by times when multiple G-RNTIs are configured.

For Alt1, it would be too restrictive to always drop the multicast HARQ-ACK feedback. For unicast, the Type-3 HARQ-ACK codebook can be configured to count the feedback bits for CBG-based TBs. When the unicast feedback is not for CBG-based transmission, it is possible to apply Alt2 with affordable overhead. Whether to configure Alt1 or Alt2 can be up to gNB decision based on the unicast/multicast feedback configuration.

Proposal 2: If a UE is configured with type-3 HARQ-ACK codebook, UE can be configured to support Alt1 or Alt2

    - Alt1: UE generate the Type 3 HARQ-ACK codebook including only unicast PDSCH scheduled by PDCCH with UE-specific RNTI, e.g., C-RNTI

    - Alt2: UE generate the Type 3 HARQ-ACK codebook including both unicast and multicast PDSCH scheduled by PDCCH with C-RNTI and one or multiple G-RNTIs

## Multiplexing multicast HARQ-ACK feedback in PUSCH

Agreement

When PUCCH carrying multicast HARQ-ACK only overlaps with PUSCH with the same priority, support UL-DAI indicating the number of HARQ-ACK bits for multicast as defined in Rel-16 for unicast HARQ-ACK.

  - FFS it is applied to a single G-RNTI or applied to all configured G-RNTIs.

Agreement

Support multiplexing unicast and multicast HARQ-ACK onto the same PUSCH with the same priority for the following cases:

  - If the unicast and multicast HARQ-ACK codebooks are both Type-1

    - Option1-1: the 1-bit UL DAI with value “1” indicates multiplexing unicast and multicast HARQ-ACK codebooks onto the same PUSCH.

    - Option1-2: two bits UL DAI separately indicate whether multiplexing unicast and/or multicast HARQ-ACK codebooks onto the same PUSCH

      - FFS whether it is applied to a single G-RNTI or applied to all configured G-RNTIs.

  - If both unicast and multicast HARQ-ACK codebooks are Type-2, down-select from:

    - Option2-1: the 2-bit UL DAI is applicable to both HARQ-ACK codebooks.

    - Option2-2: 2-bit UL DAI(s) are included in DCI for multicast, in addition to the 2-bit UL DAI for unicast.

      - FFS whether a single UL DAI field is applied to all G-RNTIs, or separate UL DAI fields are applied for each configured G-RNTI.

  - FFS the unicast and multicast HARQ-ACK codebooks are different Types.

Unicast HARQ-ACK feedback can be multiplexed in PUSCH with UL DAI. The DAI field in UL grant indicate whether HARQ-ACK information is to be transmitted in the slot (Type-1 CB) or the number of HARQ-ACK information bits (Type-2 CB/enhanced Type-2 CB). In Rel-16, DCI format 0_1 can support 1, 2, 4, or 8bits in UL DAI and DCI format 0_2 can support 0, 2 or 4bis in UL DAI. We prefer to keep the maximum field size of UL DAI in UL grant as Rel-16.

Proposal 3: The maximum field size of UL DAI for multiplexing multicast feedback in PUSCH is same as that of Rel-16 UL DAI.

When PUCCH carrying multicast HARQ-ACK only overlaps with PUSCH with the same priority,

  - If multicast G-RNTI(s) is(are) configured with Type-1 codebook, candidate PDSCH occasions based on TDRA table are common for all G-RNTIs in a cell. A common 1-bit UL DAI can be applied to all G-RNTIs configured per cell per UE.

  - If multicast G-RNTI(s) is(are) configured with Type-2 codebook, it was agreed that the DL DAI is separately counted per G-RNTI. Therefore, separate 2-bit UL DAI field (VT-DAIUL) should be applied per G-RNTI, with the value of according to Table 9.1.3-2 of TS38.213. If there are 8-bit UL DAI in UL grant, we can support up to 4 G-RNTIs.

Proposal 4: For multiplexing multicast feedback only in the same PUSCH.

    - In case of Type-1 multicast HARQ-ACK codebook, 1-bit UL DAI is applied to all configured G-RNTIs.

    - In case of Type-2 multicast HARQ-ACK codebook, separate 2-bit UL DAI per G-RNTI is included in UL grant.

When a UE is configured to multiplex both unicast and multicast HARQ-ACK feedback with the same priority in the same PUSCH,

  - If the unicast and multicast HARQ-ACK codebooks are both Type-1,

    - Option 1-1 may be sufficient when common candidate PDSCH occasions are counted for unicast and multicast;

      - 0: nothing

      - 1: both unicast and multicast

    - Otherwise, Option 1-2 is more flexible to independently configure the unicast and multicast and codebook size is different when the TDMA/K1 for generating multicast type-1 codebook is different than that of unicast.

      - 00: nothing

      - 01: multicast only

      - 10: unicast only

      - 11: both unicast and multicast

  - If both multicast HARQ-ACK codebook is Type-2 and unicast HARQ-ACK codebook is Type-2 or enhanced Type-2,

    - Similar as DL DAI separately counted per C-RNTI and per G-RNTI, independent UL DAI is preferred for reliable protection.

    - gNB can split UL DAI bits to unicast and multicast without increasing the maximum field size of UL DAI.

  - If unicast and multicast have different HARQ-ACK codebooks and one of unicast or multicast HARQ-ACK codebook is Type-1,

    - Separate UL DAI should be applied for unicast and multicast.

    - For Type-1 codebook, 1-bit UL DAI is used and '0’ is added to match to 2bit;

    - Remaining UL DAI bits can be used for unicast or multicast.

Proposal 5: For multiplexing unicast and multicast feedback in the same PUSCH.

    - If the unicast and multicast HARQ-ACK codebooks are both Type-1,

    - Common 1-bit UL DAI (Option 1-1) can be applied when common candidate PDSCH occasions are counted for unicast and multicast; otherwise, separate 1-bit UL DAIs (Option 1-2) are applied for unicast and multicast.

    - If multicast HARQ-ACK codebook is Type-2 and unicast HARQ-ACK codebook is Type-2 or enhanced Type-2,

    - Separate UL DAI bits can be configured per unicast C-RNTI and per multicast G-RNTI.

    - If multicast HARQ-ACK codebook is Type-1 and unicast HARQ-ACK codebook is Type-2 or enhanced Type-2,

    - For multicast Type-1 codebook, 1-bit UL DAI is used and '0’ is added to match to 2 bits;

    - Remaining UL DAI bits can be used for the unicast.

    - If unicast HARQ-ACK codebook is Type-1 and multicast HARQ-ACK codebook is Type-2,

    - For unicast Type-1 codebook, 1-bit UL DAI is used and '0’ is added to match to 2 bits;

    - Remaining UL DAI bits can be used for the multicast.

## Fallback operation of Type-1 HARQ-ACK codebook for multicast

Agreement

Extending the fallback operation for Type-1 HARQ-ACK codebook to multicast PDSCH receptions.

- FFS how to handle the fallback operation for the case of multiple G-RNTIs/G-CS-RNTIs configured

- FFS how to handle the fallback operation for the case that PTP retransmission is used for PTM initial transmission.

For unicast, the fallback operation for unicast Type-1 HARQ feedback codebook is considered for one C-RNTI and SPS with one CS-RNTI. For multicast, a UE may be configured with multiple G-RNTIs and/or multiple G-CS-RNTIs. The multicast codebook type as well as TDRA table/K1 are configured for configured for all configured G-RNTIs/G-CS-RNTIs per cell per UE. However, the enabling/disabling of the multicast feedback can be different for dynamic grant multicast PDSCH per G-RNTI or SPS multicast PDSCH per G-CS-RNTI.

For a multicast PDSCH scheduled by DCI format 4_1, the feedback can be enabled/disabled per G-RNTI via unicast RRC signaling. If extending the fallback operation for Type-1 HARQ-ACK codebook to multicast reception, the fallback codebook can be applied to a PDSCH scheduled by any G-RNTI with feedback enabled. The UE does not need to generate fallback codebook for the G-RNTI with feedback disabled. No need to apply fallback operation per G-RNTI and gNB only applies the blind detection once for all the G-RNTI with feedback enabled, rather than doing it per G-RNTI respectively.

Similarly, for SPS multicast PDSCH without PDCCH, the feedback may be enabled/disabled per G-CS-RNTI via unicast RRC signaling or DCI format 4_2 associated with the G-CS-RNTI. The fallback codebook can be applied to the SPS multicast PDSCH(s) with any G-CS-RNTI configured with enabled feedback.

If NACK-only-based feedback can be configured for a G-RNTI or for a G-CS-RNTI with Type-1 feedback codebook, it is transformed to ACK/NACK-based feedback to generate the 1-bit fallback codebook.

Proposal 6: The fallback operation for Type-1 HARQ-ACK codebook can be applied to multicast PDCCH/PDSCH reception associated with G-RNTI/G-CS-RNTI with feedback enabled.

    - Agree on the TP#1 for Sect. 9.1.2 of TS38.213.

### TP#1 for Sect. 9.1.2 of TS38.213 ### 9.1.2 Type-1 HARQ-ACK codebook determination ## *** Unchanged text is omitted *** If a UE reports HARQ-ACK information in a PUCCH only for - a SPS PDSCH release indicated by DCI format 1_0 with counter DAI field value of 1, or - a PDSCH reception scheduled by DCI format 1_0 with counter DAI field value of 1 on the PCell, or - SPS PDSCH reception(s) associated with a CS-RNTI or - a SPS PDSCH release indicated by DCI format 4_1 with counter DAI field value of 1, associated with G-CS-RNTI configured with enabled feedback, or - a PDSCH reception scheduled by DCI format 4_1 with counter DAI field value of 1 on the PCell, associated with G-RNTI configured with enabled feedback, or - SPS PDSCH reception(s) associated with G-CS-RNTI configured with enabled feedback within the occasions for candidate PDSCH receptions as determined in clause 9.1.2.1, the UE determines a HARQ-ACK codebook only for the SPS PDSCH release or only for the PDSCH reception or only for one SPS PDSCH reception according to corresponding occasion(s) on respective serving cell(s), where the value of counter DAI in DCI format 1_0 or DCI format 4_1 is according to Table 9.1.3-1 and HARQ-ACK information bits in response to more than one SPS PDSCH receptions that the UE is configured to receive are ordered according to the following pseudo-code; otherwise, the procedures in clause 9.1.2.1 and clause 9.1.2.2 for a HARQ-ACK codebook determination apply. ## *** Unchanged text is omitted ***

## NACK-only-based HARQ-ACK feedback

In case of more than one/two bits NACK-only-based HARQ-ACK feedback in a PUCCH slot, RAN1 has listed alternatives as follows:

Agreement:

When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, further decide based on the following subset of alternatives (from previous agreement) with potential further down-selection:

- Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

- Alt2: Support sub-slot based PUCCH for this case.

- Alt3: Support UE transmitting more than one slot-based PUCCHs in the same PUCCH slot.

- Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission.

- Alt5: NACK-only bundling

For Alt1, the PUCCH for ACK/NACK-based feedback may not be available. So, we think Alt4 should be prioritized. The specific sequence can be adding the cyclic shift/scrambling on top of base PUCCH sequence. The maximum number of NACK-only-based HARQ-ACK feedback to be multiplexed in the same PUCCH slot may be subject to UE capability.

The UEs may monitor/feedback different G-RNTIs. From one UE perspective, it only needs to generate the feedback for the monitored multicast G-RNTIs. When more than one NACK-only-based feedback are available for transmission in the same PUCCH slot, the (2^X-1) orthogonal PUCCH resources can be allocated for up to X TBs of the monitored G-RNTI(s). If there are more than X TBs using NACK-only based feedback in a PUCCH slot, gNB should configure a PUCCH or PUSCH resource for UE to multiplex the more than X feedback bits by transforming NACK-only into the ACK / NACK HARQ bit.

Proposal 7: For RRC_CONNECTED UEs, when more than one NACK-only based feedback are available for transmission in the same PUCCH slot,

    - for up to X TBs with NACK-only feedback, support Alt4 to select one from up to (2^X-1) PUCCH sequences based on cyclic shift/scrambling;

    - for more than X TBs with NACK-only feedback, support Alt1 to use a configured UE-specific PUCCH/PUSCH resource for multiplexing

## FDMed Type-1 HARQ-ACK feedback codebook

Agreement

For the Type-1 codebook construction for FDM-ed unicast and multicast via Opt 4 (from the previous agreement), when UE is configured with multiple G-RNTIs and UE is configured with fdmed-Reception-Multicast, the sub-codebook for multicast consists of the sub-codebooks for each G-RNTI by appending one to another in ascending order of G-RNTI value.

- The sub-codebook for each G-RNTI is generated per the k1 and TDRA configurations for the same G-RNTI as the legacy procedure.

- FFS: whether/how to reduce the Type-1 codebook size when multiple G-RNTIs are configured.

- Note: The maximum number of G-RNTI(s) configured to UE for the FDMed unicast and multicast Type-1 codebook is up to UE capability which will be discussed in UE features.

Based on the above agreement, it is clear that a sub-codebook per G-RNTI will be generated respectively and appended on to another, which is not dependent on same or different TDRA tables. Referring to NR Rel-16 multi-TRP feature, a sub-codebook per TRP is generated even if their TDRA table is common.

However, in previous RAN1 meeting, companies raised the concern on the codebook size if multiple G-RNTIs. We think RAN1 agreement should not be reverted. It can be up to gNB scheduling to configure the required PUCCH resources and choose whether the FDMed pattern is applied. We propose the following changes for 9.1.2 of 38.213 to reflect the RAN1 agreement on FDM-ed Type-1 codebook pattern:

Proposal 8: Endorse TP#2 for TS38.213 to reflect the RAN1 agreement of the FDMed Type-1 HARQ-ACK feedback pattern.

### TP#2 for TS38.213:

### 9.1.2	Type-1 HARQ-ACK codebook determination

## *** Unchanged text is omitted ***

If a UE is provided fdmed-Reception-Multicast and the UE is configured to monitor PDCCH for detection of unicast DCI formats and to monitor PDCCH for detection of multicast DCI format associated with a G-RNTI

-	a serving cell is placed in a unicast set SU of NcellsDL,U serving cells if the UE is configured to monitor PDCCH for DCI formats 1_0/1_1/1_2 for scheduling on serving cell c, and

-	a serving cell is placed in a multicast set SM,G-RNTI of NcellsDL,M, G-RNTI serving cells if the UE is configured to monitor PDCCH for DCI formats 4_1/4_2 associated with a G-RNTI for scheduling on serving cell c, and

-	serving cells are placed in a set by concatenating the unicast set followed by the other multicast sets according to an ascending order of the corresponding G-RNTI

the UE generates a Type-1 HARQ-ACK codebook for the set SU and the set SM of serving cells separately by setting NcellsDL=NcellsDL,U and NcellsDL=NcellsDL,M in the following pseudo-code. The UE concatenates the HARQ-ACK codebook generated for the set SU followed by the HARQ-ACK codebook generated for the set SM to obtain a total number of OACK HARQ-ACK information bits.

## *** Unchanged text is omitted ***

# 3. Conclusion

In this contribution, we discussed various aspects related to NR multicast reliability with the following proposals:

For multiplexing HARQ-ACK feedback of unicast and multicast

Proposal 1: If a UE is configured with enhanced Type-2 HARQ-ACK codebook for unicast and scheduled to multiplex HARQ-ACK feedback for unicast and multicast with the same priority in the same PUCCH slot, the UE generates two separate sub-codebooks for unicast and multicast respectively and then concatenates them by appending sub-codebook for multicast to the sub-codebook for unicast.

Proposal 2: If a UE is configured with type-3 HARQ-ACK codebook, UE can be configured to support Alt1 or Alt2

    - Alt1: UE generate the Type 3 HARQ-ACK codebook including only unicast PDSCH scheduled by PDCCH with UE-specific RNTI, e.g., C-RNTI

    - Alt2: UE generate the Type 3 HARQ-ACK codebook including both unicast and multicast PDSCH scheduled by PDCCH with C-RNTI and one or multiple G-RNTIs

For multiplexing multicast HARQ-ACK feedback in PUSCH

Proposal 3: The maximum field size of UL DAI for multiplexing multicast feedback in PUSCH is same as that of Rel-16 UL DAI.

Proposal 4: For multiplexing multicast feedback only in the same PUSCH.

    - In case of Type-1 multicast HARQ-ACK codebook, 1-bit UL DAI is applied to all configured G-RNTIs.

    - In case of Type-2 multicast HARQ-ACK codebook, separate 2-bit UL DAI per G-RNTI is included in UL grant.

Proposal 5: For multiplexing unicast and multicast feedback in the same PUSCH.

    - If the unicast and multicast HARQ-ACK codebooks are both Type-1,

    - Common 1-bit UL DAI (Option 1-1) can be applied when common candidate PDSCH occasions are counted for unicast and multicast; otherwise, separate 1-bit UL DAIs (Option 1-2) are applied for unicast and multicast.

    - If multicast HARQ-ACK codebook is Type-2 and unicast HARQ-ACK codebook is Type-2 or enhanced Type-2,

    - Separate UL DAI bits can be configured per unicast C-RNTI and per multicast G-RNTI.

    - If multicast HARQ-ACK codebook is Type-1 and unicast HARQ-ACK codebook is Type-2 or enhanced Type-2,

    - For multicast Type-1 codebook, 1-bit UL DAI is used and '0’ is added to match to 2 bits;

    - Remaining UL DAI bits can be used for the unicast.

    - If unicast HARQ-ACK codebook is Type-1 and multicast HARQ-ACK codebook is Type-2,

    - For unicast Type-1 codebook, 1-bit UL DAI is used and '0’ is added to match to 2 bits;

    - Remaining UL DAI bits can be used for the multicast.

For fallback operation of Type-1 HARQ-ACK codebook for multicast

Proposal 6: The fallback operation for Type-1 HARQ-ACK codebook can be applied to multicast PDCCH/PDSCH reception associated with G-RNTI/G-CS-RNTI with feedback enabled.

    - Agree on the TP#1 for Sect. 9.1.2 of TS38.213.

For NACK-only-based feedback,

Proposal 7: For RRC_CONNECTED UEs, when more than one NACK-only based feedback are available for transmission in the same PUCCH slot,

    - for up to X TBs with NACK-only feedback, support Alt4 to select one from up to (2^X-1) PUCCH sequences based on cyclic shift/scrambling;

    - for more than X TBs with NACK-only feedback, support Alt1 to use a configured UE-specific PUCCH/PUSCH resource for multiplexing

For Type-1 HARQ-ACK feedback codebook,

Proposal 8: Endorse TP#2 for TS38.213 to reflect the RAN1 agreement of the FDMed Type-1 HARQ-ACK feedback pattern.

# References

[1] R1-2112933, “CR of TS38.213: Introduction of multicast-broadcast services in NR”, Samsung, RAN1#107-e, November 11th – 19th, 2021, 2021.