3GPP TSG RAN WG1 #108-emeeting	     R1-2202180

e-Meeting, Feb. 21st – Mar. 3rd, 2022

Agenda item:	7.2.11

Source: 	Qualcomm Incorporated

Title: 	Rel-16 UL MIMO Coherence Capability

Document for:	Discussion/Decision

# Introduction

In [1], RAN4 has sent their agreements on UL MIMO coherence for Rel-16 UL Tx switching between two carriers to RAN1 and RAN2. They asked RAN1 to check if there was any RAN1 specification impact due to the RAN4 agreements and asked RAN2 to take into consideration the RAN4 agreements on UL MIMO coherence related UE capability to introduce UE capability signaling.

In RAN#93e, UL MIMO coherence capability was submitted and discussed with following observation and conclusion.

Observation: Some confusion as to the relation with the referenced RAN4 LS. Most companies prefer that this is discussed at WG level. conclusion: No conclusion at RAN#93e. Expect that this can be discussed at WG level (based on company contributions there).

In RAN1-107e, this has been discussed and we feel most of the UE & Chipset vendors are supportive as this per band per band combination feature could fully utilize the UE’s coherence design to achieve better UL MIMO performance.

Following the RAN plenary conclusion, we submit our views in this paper.

# Discussion

The UL coherence depends on the UE transmitter architecture. More precisely, it depends on whether a common oscillator signal is applied for up-conversion in the different Tx chains or not.

The same UL MIMO coherence capability in a given band likely does not hold across different band combinations even for the same band. For example, the UE may use a different set of antennas for the UL transmission for a given band in one band combination compared to the antennas used for the same band in a different band combination. A common example for this is that the UE may support 4Tx UL MIMO in one band when no UL CA is configured but the same UE may fall back to 2Tx UL MIMO only in the exact same band when UL CA is configured. Note that this was the very reason why the supported number of UL MIMO layers is signaled per band per combination. Obviously, a UE that can support coherent 2Tx UL MIMO may fall into any of the three coherence categories for 4Tx UL MIMO (i.e. it can be either coherent, partial coherent, or non-coherent for 4Tx UL MIMO). The latter is the case, for example, if the 1st and 2nd chains are associated with a common oscillator, while the 3rd and 4th chains are not.

Due to this consideration, the UL MIMO coherence capability needs to be signaled per band per band combination.

Unfortunately, the current specification doesn’t correctly capture this.

The LS from RAN4 is trying to solve the coherence issue when the UE reports and is configured with UL Tx switching. Although we agree that the capability for coherence with UL Tx switching should be per-band and per-band combination, we believe there is a more fundamental issue, as explained above, regardless of the configuration of UL Tx switching. Hence, we think that RAN1 should agree to introduce UL coherence as a per band per band combination capability before making any further decision on UL Tx switching.

Proposal: A new UE capability indicating UL MIMO coherence per band per band combination is introduced (regardless of UL Tx switching) in Rel-16.

During the email discussion in RAN1 meeting, some infra vendors mentioned about the impact on gNB complexity. We understand the concern that the gNB may have to do a comparison of different band combination options and choose a CA configuration according to some metric. We understand the concern, although the gNB doesn't actually have to do this, i.e. it could just select a band combination first and apply whatever capability is reported for that band combination. However, at the same time, forcing the UE to under-report is making the gNB to always end up with the worst possible choice. We can’t see any benefit in that. Changing the capability structure is not going to change the UE design or the underlying UE capabilities. There seems to be a bit of a naivete in thinking that non-coherent UE design can be forced to be coherent by restricting the reporting granularity. Obviously, in the vast majority of cases, this will work in the opposite direction. The reduced granularity will force the UE to under-report.  To promote UL MIMO coherence to take off, we would suggest companies to be positive to the proposal.

# Conclusions

In this paper, we discussed the UL coherence which should be a per-band and per-band combination capability. However, current specification doesn’t correctly capture this.  To make a general correction, we make the following proposal.

Proposal: A new UE capability indicating UL MIMO coherence per band per band combination is introduced (regardless of UL Tx switching) in Rel-16.

# Reference

- R1-2106431, LS on UL MIMO coherence for Tx switching between two carriers, RAN1-106e.

- RP-212108, UL MIMO coherence capabilities, RAN#93e.