3GPP TSG RAN WG1 Meeting #108-e	R1-2202186

e-Meeting, February 21st – March 3rd, 2022

Source:	Sharp

Title:	Clarification on simultaneous PSFCH transmission and reception

Agenda Item:	7.2.4

Document for:	Discussion and Decision

Introduction

In this document, we discuss one issue related to simultaneous PSFCH transmission/reception in 5G V2X.

Discussion

The following is an excerpt from the latest version (i.e. v16.8.0) of TS 38.213 (Rel-16).

#### 16.2.4.2 Simultaneous PSFCH transmission/reception If a UE - would transmit Nsch,Tx,PSFCH PSFCHs and receive Nsch,Rx,PSFCHPSFCHs, and - transmissions of the Nsch,Tx,PSFCH PSFCHs would overlap in time with receptions of the Nsch,Rx,PSFCH PSFCHs the UE transmits or receives only a set of PSFCHs corresponding to the smallest priority field value, as determined by a first set of SCI format 1-A and a second set of SCI format 1-A [5, TS 38.212] that are respectively associated with the Nsch,Tx,PSFCH PSFCHs and the Nsch,Rx,PSFCH PSFCHs. If a UE would transmit Nsch,Tx,PSFCH PSFCHs in a PSFCH transmission occasion, the UE transmits NTx,PSFCH PSFCHs corresponding to the smallest NTx,PSFCH priority field values indicated in all SCI formats 1-A associated with the PSFCH transmission occasion.

Regarding the two highlighted sentences above, our understanding is that there is no ambiguity in the second one (NTx,PSFCH PSFCHs corresponding to the smallest NTx,PSFCH priority field values), where

- The NTx,PSFCH PSFCHs are chosen from the Nsch,Tx,PSFCH PSFCHs with ascending order of priority field values, and;

- There could be multiple priority field values associated with the NTx,PSFCH PSFCHs. In the example illustrated in Table 1, NTx,PSFCH=3, i.e. PSFCH#1, PSFCH#2, and PSFCH#3 are transmitted, where PSFCH#1 and PSFCH#2 are both associated with priority field value 1, and PSFCH#2 is associated with priority field value 2. As another example, it is of course also possible, depending on the power constraints specified in clause 16.2.3 of TS 38.213 and UE capability/implementation, that NTx,PSFCH=2, i.e. PSFCH#1 and PSFCH#2 are transmitted, and are both associated with priority field value 1.

Table 1 “NTx,PSFCH PSFCHs corresponding to the smallest NTx,PSFCH priority field values”

(Nsch,Tx,PSFCH=4)

| PSFCH for transmission | Priority field value |
| --- | --- |
| PSFCH#1 | 1 |
| PSFCH#2 | 1 |
| PSFCH#3 | 2 |
| PSFCH#4 | 5 |

It is unclear to us, though, whether there is a common understanding in RAN1 regarding the first highlighted sentence (a set of PSFCHs corresponding to the smallest priority field value). Our interpretation is that, with a singular form of the word “value” in that sentence, all PSFCHs in the above mentioned set of PSFCHs have to be associated with a same/single priority field value. In the example illustrated in Table 2, since the smallest priority field value “1” (among all priority field values respectively associated with the 9 triggered PSFCHs) corresponds to PSFCH#1 and PSFCH#2, both for transmission, the UE shall perform PSFCH transmission (rather than PSFCH reception), and the set of PSFCHs would be {PSFCH#1, PSFCH#2} (or a subset thereof, depending on the power constraints specified in clause 16.2.3 of TS 38.213 and UE capability/implementation).

Table 2 “A set of PSFCHs corresponding to the smallest priority field value”

(Nsch,Tx,PSFCH=4, Nsch,Rx,PSFCH=5)

| PSFCH for transmission | Priority field value |  | PSFCH for reception | Priority field value |
| --- | --- | --- | --- | --- |
| PSFCH#1 | 1 |  | PSFCH#5 | 2 |
| PSFCH#2 | 1 |  | PSFCH#6 | 3 |
| PSFCH#3 | 2 |  | PSFCH#7 | 4 |
| PSFCH#4 | 5 |  | PSFCH#8 | 6 |
|  |  |  | PSFCH#9 | 6 |

With the above interpretation, the spec is not in line with the following RAN1 agreements where it says the highest priority (i.e. smallest priority field value) associated with TX, e.g. value “1” in Table 2, and that associated with RX, e.g. value “2” in Table 2, are compared against each other in order to determine whether to perform TX or RX, and there is no such a restriction that only PSFCH(s) associated with the highest priority can be transmitted (in case it is determined to perform TX).

Agreements: For the prioritization between PSFCH TX and PSFCH RX, - When the UE is required to transmit more than one PSFCH, the highest priority of the associated PSCCH/PSSCH is used for prioritization of the PSFCH transmission. - When the UE is required to receive more than one PSFCH, the highest priority of the associated PSCCH/PSSCH is used for prioritization of the PSFCH reception.

In our understanding, in most cases the above-mentioned restriction prohibits the UE from transmitting multiple PSFCHs even if the UE is capable of doing so (with only one rare exception, i.e. when the NTx,PSFCH PSFCHs determined without such a restriction happen to be associated with a same, smallest, priority field value), making it meaningless for the UE to support simultaneous transmission of multiple PSFCHs. In fact, as can be seen in the earlier discussion of the second highlighted sentence (and the second paragraph of clause 16.2.4.2 as a whole) in this document, such a restriction was never intended in the spec.

We would thus like to check whether the above interpretation of the first highlighted sentence (and the corresponding restriction on UE behaviour) is a common understanding in RAN1. If yes we propose to fix the issue with the TP in the Appendix.

If the above interpretation of the first highlighted sentence is instead not a common understanding in RAN1, we would like to propose to discuss and reach a conclusion on the UE behaviour regarding that sentence in TS 38.213.

Conclusion

In this contribution, we discuss one issue related to simultaneous PSFCH transmission/reception in 5G V2X, and propose a TP to fix the issue.

References

RP-190766, “New WID on 5G V2X with NR sidelink”, LG Electronics, Huawei, 3GPP RAN#83.

Appendix: TP for TS 38.213

-------------------------------------------- Start of TP -------------------------------------------

< Unchanged parts are omitted >

#### 16.2.4.2	Simultaneous PSFCH transmission/reception

If a UE

-	would transmit Nsch,Tx,PSFCH PSFCHs and receive Nsch,Rx,PSFCHPSFCHs, and

-	transmissions of the Nsch,Tx,PSFCH PSFCHs would overlap in time with receptions of the Nsch,Rx,PSFCH PSFCHs

the UE transmits or receives only a set of PSFCHs containing a PSFCH corresponding to the smallest priority field value, as determined by a first set of SCI format 1-A and a second set of SCI format 1-A [5, TS 38.212] that are respectively associated with the Nsch,Tx,PSFCH PSFCHs and the Nsch,Rx,PSFCH PSFCHs.

< Unchanged parts are omitted >

-------------------------------------------- End of TP --------------------------------------------