3GPP TSG RAN WG1 Meeting #108-e	R1-2202187

e-Meeting, February 21th – March 3rd, 2022

Source:	Sharp

Title:	Clarification on the condition to trigger re-evaluation in mode 2

Agenda Item:	7.2.4

Document for:	Discussion and Decision

Introduction

In the preparation phase email discussion [107-e-Prep-AI7.2.4] for Rel-16 NR V2X maintenance in RAN1#107-e meeting, for the potential issue on the condition to trigger re-evaluation in mode 2 (issue#20 in the summary), it was suggested by the Moderator that the proponent comes up with a concrete proposal for RAN1 conclusion in a future meeting. In this document, as a follow-up of [107-e-Prep-AI7.2.4], we share our views on the issue and propose a conclusion for common understanding.

Discussion

Clarification on the condition for re-evaluation

For sensing in Rel-16 NR V2X, the resource selection window is defined as [n+T1, n+T2] and selection of T1 is up to UE implementation under the condition 0≤T1≤Tproc,1SL. After performing Step 1)-Step 7) as specified in 8.1.4 of TS38.214, the remaining set SA is reported to MAC layer for resource selection, shown as follows.

1) A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot t'ySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource, where - selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL , where Tproc,1SL is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP; - if T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots). The total number of candidate single-slot resources is denoted by Mtotal. [… …] 7) If the number of candidate single-slot resources remaining in the set SA is smaller than X⋅Mtotal, then Th(pi,pj) is increased by 3 dB for each priority value Th(pi,pj) and the procedure continues with step 4. The UE shall report set SA to higher layers.

It is noticed that MAC layer would randomly select the resources from SA which is indicated by PHY, shown as follows. For the cases when T1<Tproc,1SL, the set SA may include some single-slot resources within the interval [n+T1, n+Tproc,1SL] and MAC layer may select among these resources for the selected sidelink grant, denoted as {ri}.

4> randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier.

According to 5.22.1.2a of TS38.321 for re-evaluation and pre-emption, any resource(s) of the selected sidelink grant is re-evaluated at the moment of T3(i.e. Tproc,1SL) before the slot where the SCI first indicates the resource(s), shown as follows.

- 5.22.1.2a Re-evaluation and Pre-emption A resource(s) of the selected sidelink grant for a MAC PDU to transmit from multiplexing and assembly entity is re-evaluated by physical layer at T3 before the slot where the SCI indicating the resource(s) is signalled at first time as specified in clause 8.1.4 of TS 38.214 [7].

For resource(s) {ri}, the moment to trigger re-evaluation as mentioned above is slotri-Tproc,1SL. Since slotri is within the time interval [n+T1, n+Tproc,1SL], it would result in the moment of triggering re-evaluation, i.e. slotri-Tproc,1SL comes before slot n, and slot n is the slot when MAC triggers for initial resource selection procedures. Thus, for the resources within the time interval [n+T1, n+Tproc,1SL] which are selected by MAC layer for the selected sidelink grant, it shall not be triggered for the re-evaluation. The following conclusion is proposed for common understanding.

Proposal 1: It is RAN1 understanding that a resource(s) of the selected sidelink grant to be first signalled in slot m shall not be triggered for re-evaluation in slot r which is T3 before slot m if slot r is not later than slot n, where slot n is the slot when MAC layer triggers for initial resource selection for the selected sidelink grant.

Conclusion

In this contribution, we discuss one remaining issue in mode 2 and make the following proposal:

Proposal 1: It is RAN1 understanding that a resource(s) of the selected sidelink grant to be first signalled in slot m shall not be triggered for re-evaluation in slot r which is T3 before slot m if slot r is not later than slot n, where slot n is the slot when MAC layer triggers for initial resource selection for the selected sidelink grant.