3GPP TSG RAN WG1 #108-e	R1-2202189

e-Meeting, February 21st – March 3rd, 2022

Source:	Sharp

Title:	Initial access aspects

Agenda Item:	8.2.1

Document for:	Discussion and Decision

- Introduction

In this contribution, we provide discussions on initial access aspects of supporting NR from 52.6GHz to 71 GHz.

- DRS related aspects

The following agreement was achieved during RAN1 107-e [1]:

Agreement - Support DBTW with 480 and 960 kHz SCS. - For licensed and unlicensed operation, support 64 candidate SSB positions in a half frame - Working assumption: Use 2 bits for Q: - SubcarrierSpacingCommon - spare bit in MIB - Send LS to RAN2 for confirming the use of the spare bit in MIB - The use of 2 bits for Q can be revisited if RAN2 tells RAN1 that the spare bit cannot be used Agreement - Same NSSBQCL values using the same set of signaling bits are supported for 120, 480, and 960 kHz. - Supported values of NSSBQCL: {16, 32, 64} - Note: - For operation with shared spectrum channel access, any supported value of NSSBQCL can be indicated and value < 64 indicates DBTW enabled - UE is expected to be configured with NSSBQCL=64 in licensed operations - For operation with and without shared spectrum channel access, NSSBQCL=64 indicates that the SS/PBCH block index and the candidate SS/PBCH block index have a one-to-one mapping relationship.

In RAN2 116bis-e meeting, it has been decided not to use the spare bit in MIB for Q indication. Correspondingly, a comprised solution at RAN1 could be indicating Q = {32, 64} with 1 MIB payload bit (i.e., subCarrierSpacingCommon), but we believe that is too restrictive and also not aligned with the agreement supporting Q values of {16, 32, 64}.

Partial Q indication by MIB could be a solution to alleviate the problem. More specifically, subCarrierSpacingCommon = ‘scs30or120’ indicates Q = 64 and subCarrierSpacingCommon = ‘scs15or60’ indicates Q = {16, 32}, which means the gNB may transmit SSB with ether Q = 16 or 32. That is, only a part of Q information is indicated by MIB and the complete Q information (or remained Q information) can be indicated by SIB1. Before decoding SIB1, when subCarrierSpacingCommon = ‘scs15or60’ is indicated, for the reception of SSB, the UE makes QCL assumption of Q = 32. While for the reception of Type0 PDCCH, the UE makes QCL assumption of Q = 16.

If gNB actually transmits SSB and Type0 PDCCH according to Q = 16, the UE will perform soft combining with candidate SSBs “a” and “a+32”, as shown in Figure 1. The cost is that there may be some decoding performance loss because candidate SSBs “a+16” is not included in the soft combining. UE’s QCL assumption for Type0 PDCCH is Q = 16, aligned with that actually used by the gNB.

Figure 1: SSB reception when the gNB actually transmits according to Q = 16

If gNB actually transmits SSB and Type0 PDCCH according to Q = 32, the UE will just receive candidate SSBs “a” and “a+32” and performs soft combining. For the reception of Type0 PDCCH, the UE will perform extra blind decoding (e.g., blind decoding #2 in Figure 2 in addition to blind decoding #1 and blind decoding #3) since the UE assumes Q = 16 for Type0 PDCCH reception. For blind decoding #2, the UE just decodes nothing, which does not affect Type0 PDCCH reception.

Figure 2: Type0 PDCCH reception when the gNB actually transmits according to Q = 32

By letting UE make different assumptions on Q for SSB reception and Type0 PDCCH reception, three Q values of {16, 32, 64} can be indicated by 1 MIB bit.

Proposal 1: Use 1 MIB payload bit to partially indicate Q values of {16, 32, 64} and the full indication for Q can be complemented by SIB1. Adopt the following text proposal.

Text proposal for TS38.213 V17.0.0:

4 Synchronization procedures 4.1 Cell search [omitted text] For operation with shared spectrum channel access in FR2-2, a UE assumes that SS/PBCH blocks in a serving cell that are within a same discovery burst transmission window or across discovery burst transmission windows are quasi co-located with respect to average gain, quasi co-location 'typeA' and 'typeD' properties, when applicable, if a value of imodNSSBQCL is same among the SS/PBCH blocks, where i is the candidate SS/PBCH block index. NSSBQCL is either provided by ssb-PositionQCL or, if ssb-PositionQCL is not provided, obtained from a MIB provided by a SS/PBCH block according to Table 4.1-2. The UE can determine an SS/PBCH block index according to i modNSSBQCL. The UE assumes that within a discovery burst transmission window, a number of transmitted SS/PBCH blocks on a serving cell is not larger than NSSBQCL and a number of transmitted SS/PBCH blocks with a same SS/PBCH block index is not larger than one. If subCarrierSpacingCommon in the MIB indicates ‘scs15or60’, the UE assumes that NSSBQCL = 16 for PDCCH monitoring in the Type0-PDCCH CSS set in Clause 13. Table 4.1-2: Mapping between subCarrierSpacingCommon to NSSBQCL for operation with shared spectrum channel access in FR2-2 subCarrierSpacingCommon; NSSBQCL. scs15or60; 32. scs30or120; 64. [omitted text]

- Conclusion

In this contribution, we provide the following observations and proposals:

Proposal 1: Use 1 MIB payload bit to partially indicate Q values of {16, 32, 64} and the full indication for Q can be complemented by SIB1. Adopt the text proposal.

Text proposal for TS38.213 V17.0.0

- References

  - 3GPP RAN1#107-e Chairman’s note, November 2021

  - R2-2201033 Consideration on RRC and MAC running CR, January 2022