3GPP TSG RAN WG1 #108-e	R1-2202197

e-Meeting, February 21st – March 3rd, 2022

Source:	Sharp

Title:	TB processing over multi-slot PUSCH

Agenda Item:	8.8.1.2

Document for:	Discussion and Decision

Background

RAN1 specification for Rel-17 CovEnh has been published in [1-4]. We discuss issues for finalization of the work.

List of issues in the current specification

List of issues in the current specification is as follows. We provide a solution to solve the issues.

- Starting coded bit determination

- Support for TBoMS with Type-1 configured grant

- Editorial issues

  - Starting coded bit determination

In the FL summary at the last meeting [6], the following discussion points are provided.

- Aspect 1: The definition of G and E for TBoMS

- Aspect 2: The value of G and E for TBoMS (or, alternatively, the value of H)

- Aspect 3: Handling of the filler bits in TBoMS

For Aspect 1, it is proposed to redefine G as the total number of coded bits available for transmission of the transport block in a slot. We are basically fine with the proposal. However, the redefined G is the same as the one for repetition type-A in Rel-15/16 specification.

For PUSCH repetition in Rel-15/16, PHY layer determines the number of repetitions and report it to the MAC layer. This procedure is defined as follows in TS38.214 of Rel-15.

When transmitting PUSCH scheduled by DCI format 0_1 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1, if the UE is configured with pusch-AggregationFactor, the same symbol allocation is applied across the pusch-AggregationFactor consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the pusch-AggregationFactor consecutive slots applying the same symbol allocation in each slot. The redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, …pusch-AggregationFactor -1, is determined according to table 6.1.2.1-2.

As highlighted above, the UE shall repeat the TB across K (i.e., pusch-AggregationFactor for Rel-15 PUSCH repetition) consecutive slots. This implies that the PHY layer reports the number K to the MAC layer for TB repetition. Upon reception of the report from the PHY layer, the MAC layer performs bundling of K uplink grants as highlighted yellow.

When the MAC entity is configured with pusch-AggregationFactor > 1, the parameter pusch-AggregationFactor provides the number of transmissions of a TB within a bundle of the dynamic grant. After the initial transmission, pusch-AggregationFactor – 1 HARQ retransmissions follow within a bundle. When the MAC entity is configured with repK > 1, the parameter repK provides the number of transmissions of a TB within a bundle of the configured uplink grant. After the initial transmission, HARQ retransmissions follow within a bundle. For both dynamic grant and configured uplink grant, bundling operation relies on the HARQ entity for invoking the same HARQ process for each transmission that is part of the same bundle. Within a bundle, HARQ retransmissions are triggered without waiting for feedback from previous transmission according to pusch-AggregationFactor for a dynamic grant and repK for a configured uplink grant, respectively. Each transmission within a bundle is a separate uplink grant after the initial uplink grant within a bundle is delivered to the HARQ entity.

Further, the MAC layer is supposed to instruct the PHY layer to generate a transmission according to each uplink grant. Therefore, the MAC layer instructs the PHY layer K transmissions for PUSCH repetition in Rel-15. The MAC layer is connected to the PHY layer through transport layer. Therefore, each transmission refers to TB of UL-SCH.

If a transmission is instructed from the MAC layer, the PHY layer triggers encoding procedure specified in TS38.212 of the TB delivered from the transport layer. Therefore, each encoding procedure corresponds to each repetition (i.e., each slot). As a consequence, the number of available bits G for UL-SCH transmission refers to available bits for UL-SCH in each slot.

Observation 1: The number of available bits G for UL-SCH transmission refers to available bits for UL-SCH in each slot in Rel-15.

On the other hand, In Rel-17 specification, it is specified that the UE shall repeat the TB N * K times for a TBoMS according to the following description.

For TB processing over multiple slots: - For unpaired spectrum, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot. - For paired spectrum or supplementary uplink band, the same symbol allocation is applied across the N∙K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K consecutive slots applying the same symbol allocation in each slot.

Therefore, the MAC layer should bundle N * K uplink grants, each with an instruction of the transmission of UL-SCH. Therefore, even for TBoMS, each transmission corresponds to a slot.

Observation 2: For TBoMS, the number of available bits G for UL-SCH transmission refers to available bits for UL-SCH in each slot from N * K slots.

With the above two observations, it is clear that no redefinition is necessary for G.

Observation 3: Without any specification change, the number of available bits G for UL-SCH transmission refers to available bits for UL-SCH in each slot from N * K slots.

Regarding Aspect 2, interpretation 2 is correct. This is because G represents the length of the output sequence for UCI multiplexing procedure as well as the length of the rate-matching sequence (when the number of code blocks = 1). More specifically, 38.212 says “Denote the multiplexed data and control coded bit sequence as .” Therefore, in the example described in [6], there is no case that G = E = 6 but the output length from UCI multiplexing procedure = 10 as described in interpretation 1.

Observation 4: Interpretation 2 is correct because G represents the length of the output sequence for UCI multiplexing procedure as well as the length of the rate-matching sequence length (when the number of code blocks = 1).

For Aspect 3, at least from Sharp’s perspective, impacts on filler bits haven’t been fully analysed at the time of agreement achieved. Therefore, we don’t think that Direction 2 reverts the agreement. On the other hand, strictly speaking, Direction 1 is fully aligned with the agreement. In that sense, we are fine with either, and we slightly prefer Direction 1.

Observation 5: Impacts on filler bits haven’t been fully analysed at the time of agreement achieved. Therefore, it is not true that Direction 2 reverts the agreement. On the other hand, strictly speaking, Direction 1 is fully aligned with the agreement.

Proposal 1: Filler bits are considered to implement Option C as agreed in RAN plenary.

  - Support for TBoMS with Type-1 configured grant

At RAN1#106-e meeting [7], the following agreement was made for starting coded bit determination.

Agreement TBoMS is supported for both configured grant and dynamic grant.

On the other hand, TS38.214 has not been updated such that TBoMS for type-1 configured grant is supported since we haven’t agreed how to indicate the number of slots for a single TBoMS indicated for type-1 configured grant. Up to now, we have agreed that activation of TBoMS feature is achieved by a value N for indicating the number of slots for a single TBoMS indicated through time domain resource allocation field. Therefore, corresponding configuration for the value N is necessary in type-1 configured grant configuration.

Proposal 2: A value N for indicating the number of slots for a single TBoMS is configured in rrc-ConfiguredUplinkGrant for type-1 configured grant.

  - Editorial issues

As discussed in our contribution submitted under AI8.8.1.1, available slot determination procedure for TBoMS with configured grant should be specified in an appropriate section. We propose the following text proposal.

Proposal 3: Adopt Text proposal #1.

##### Text proposal#1 ##### 6.1.2.3.3 Transport Block repetition for uplink transmissions of TB processing over multiple slots with a configured grant The procedures described in this clause apply to PUSCH transmissions of TB processing over multiple slots with a Type 2 configured grant. The higher layer parameter repK-RV defines the redundancy version pattern to be applied to the repetitions. If the parameter repK-RV is not provided in the configuredGrantConfig, the redundancy version for uplink transmissions with a configured grant shall be set to 0. If the parameter repK-RV is provided in the configuredGrantConfig, the nth transmission occasion among N∙K transmissions occasions, n=0,1, …, N∙K -1, is associated with (mod((n-mod(n, N))/N,4)+1)th value in the configured RV sequence. When K=1, or when K>1 and the configured grant configuration is configured with startingFromRV0 set to 'off', the initial transmission of the transport block may only start at the first transmission ocasion of the N∙K transmission occasions. Otherwise, the initial transmission of the transport block may start at - The first transmission occasion of the N∙K transmission occasions if the configured RV sequence is {0,2,3,1}. - Any transmission occasion n associated with RV=0, and for which n mod N =0, if the configured RV sequence is {0,3,0,3} or {0,0,0,0}. If the UE determines that, for a transmission occasion, the number of symbols available in a slot for the PUSCH transmission of TB processing over multiple slots is smaller than transmission duration L, the UE does not transmit the PUSCH in the transmission occasion. For unpaired spectrum: - The UE determines N∙K slots for a PUSCH transmission of a TB processing over multiple slots, based on tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst. - A slot is not counted in the number of N∙K slots for PUSCH transmission of TB processing over multiple slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. For paired spectrum and SUL band: - For the case of non reduced capability half-duplex UE, the UE determines N∙K consecutive slots for a PUSCH transmission of a TB processing over multiple slots. - For the case of reduced capability half-duplex UE, the UE determines N∙K slots for a PUSCH transmission of a TB processing over multiple slots. A slot is not counted in the number of N∙K slots for a PUSCH transmission of a TB processing over multiple slots, if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. For Type 2 PUSCH transmission with a configured grant of TB processing over multiple slots, the UE shall transmit the TB across the N∙K slots determined for the PUSCH transmission applying the same symbol allocation in each slot. A Type 2 PUSCH transmission with a configured grant of TB processing over multiple slots is omitted in a slot according to the conditions in clause 9, clause 11.1 and clause 11.2A of [6, TS 38.213]. -------- Unchanged contents are omitted --------- end of text proposal

Conclusion

In this contribution, we have the following proposals:

Observation 1: The number of available bits G for UL-SCH transmission refers to available bits for UL-SCH in each slot in Rel-15.

Observation 2: For TBoMS, the number of available bits G for UL-SCH transmission refers to available bits for UL-SCH in each slot from N * K slots.

Observation 3: Without any specification change, the number of available bits G for UL-SCH transmission refers to available bits for UL-SCH in each slot from N * K slots.

Observation 4: Interpretation 2 is correct because G represents the length of the output sequence for UCI multiplexing procedure as well as the length of the rate-matching sequence length (when the number of code blocks = 1).

Observation 5: Impacts on filler bits haven’t been fully analysed at the time of agreement achieved. Therefore, it is not true that Direction 2 reverts the agreement. On the other hand, strictly speaking, Direction 1 is fully aligned with the agreement.

Proposal 1: Filler bits are considered to implement Option C as agreed in RAN plenary.

Proposal 2: A value N for indicating the number of slots for a single TBoMS is configured in rrc-ConfiguredUplinkGrant for type-1 configured grant.

Proposal 3: Adopt Text proposal #1.

References

- 3GPP TS38.211V17.0.0, December 2021

- 3GPP TS38.212V17.0.0, December 2021

- 3GPP TS38.213V17.0.0, December 2021

- 3GPP TS38.214V17.0.0, December 2021

- RAN#94-e draft meeting report (draft_MeetingReport_RAN_94e_211217_eom), December 2021

- R1-2200752, “Final FL summary of TB processing over multi-slot PUSCH (AI 8.8.1.2),” Moderator (Nokia, Nokia Shanghai Bell), January 2022

- RAN1 Chairman’s note at RAN1#107bis-e meeting, January 2022

- R1-2200500, “Enhancements on PUSCH repetition type A”, January 2022