3GPP TSG RAN WG1 #108-e                                     R1-2202213

e-Meeting, February 21st – March 3rd, 2022

Source:	ZTE

Title:	Discussion on UE features for IoT over NTN

Agenda Item:	8.16.14

Document for:   Discussion

- Introduction

The latest version of UE features list for IoT-NTN is provided in [1]. In this contribution, our views on some features are elaborated with corresponding updates.

- Discussion on the UE feature

As lists in Table 1, FG 2-1, FG 2-2 and FG2-3 are changed. Specifically, W.r.t FG 2-1, following revise is suggested:

Regarding FG 2-1:

- For component 7, it’s preferred to remove the bracket [7. For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops ]. Because according to the agreement in RAN1#106e[2],combination of both open and closed control loops are supported in NR-NTN and the relevant agreements from NR NTN are re-used for IoT NTN as working assumption.

- For component8, the bracket for “[in DL] [and] [in UL]” can be removed. According to the agreement in RAN1#107e[3], the pre-compensation for the DL Doppler experienced in service link is not supported in NR-NTN. In this case, the correction of experienced Doppler should include the impacts for both DL and UL.

- For components 10, an additional UE feature, i.e. A UE starts/restarts the validity timer at the epoch time of the assistance information, can be added to capture the agreements in RAN1#106bis [4].

- For component 15, it’s preferred to remove the bracket [15.delaying the starts of ra-ResponseWindow] to capture the IoT-NTN related agreements “start of ra-ResponseWindow is delayed by an offset” in RAN2#115e[5]. Since the legacy ra-ResponseWindow range may not be enough to cover different UEs in the IoT-NTN, the UE may determine the starts of ra-ResponseWindow with an offset after Msg 1 transmission, where the value of offset varies based on whether the UE can have access to the situation information(e.g.,location of the UE, an ephemeris of a satellite or a trajectory of a HAPS used by a BS, and a payload type of the satellite or the HAPS used by the BS).

- For FG 2-1, it’s preferred to add an additional component 17, i.e. UE applies cell specific K_mac in timing relationship enhancements, since the cell specific K_mac should also be applied in timing relationship enhancement similar as the cell specific K_offset. Besides, the sequence number of legacy component 17 should be correspondingly updated as component 18.

W.r.t the FG 2-1a, an additional UE feature “ The same value is used for segment durations for all PRACH preambles.” can be added according to the agreements in RAN1#106bis [4].

Moreover, according to the agreements in RAN1#107-e[3], UE can support the segment pre-compensation using two methods by UE implementation. In order to capture the relevant agreement[3], an additional component 2 should be added in FG2-1a to illustrate how to achieve the segmented pre-compensation. And the name of FG 2-1a can be revised as “segment pre-compensation”.

W.r.t the FG 2-2, it’s preferred to keep the component “UE receives UE specific K_offset” since the action of UE receiving is needed in the specification.

W.r.t the FG 2-3, according to the latest agreement [3], i.e. NR NTN solutions are a baseline for the  following UE-specific TA handling issues, the following component similar as the NR-NTN can be added in the FG 2-3: (1)The granularity of the reported TA is a slot.(2) As for the frequency of reports, the TA is reported when the offset between current information about UE specific TA and the last successfully reported information about UE specific TA is over the threshold.

Proposal-1: The updates on the UE features for IoT-NTN listed in Table-1 should be supported.

Table 1 UE features for IoT-NTN

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. LTE_NBIOT_eMTC_NTN | 2-1 | Basic IoT over NTN support | [7. For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops] 8.In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link [in DL] [and] [in UL] 10. Support a validity timer of UL synchronization is configured by the network. A UE starts/restarts the validity timer at the epoch time of the assistance information. [15.delaying the starts of ra-ResponseWindow] - UE receives cell specific K_mac] - UE applies cell specific K_mac in timing relationship enhancements 17.18. In RRC_IDLE state and RRC_CONNECTED state, pre-compensate the calculated frequency offset and TA in uplink transmissions |  | Yes | Release 17 UE cannot access [NTN/satellite] | [per UE/per band] | No | No | FFS: whether this feature group needs to be separate for eMTC and NB-IoT | Optional with capability signalling For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| 2. LTE_NBIOT_eMTC_NTN | 2-1a | Segment duration configuration pre-compensation | 1.Support segment duration configuration by the network The same value is used for segment durations for all PRACH preambles. 2.Support segment pre-compensation by using one of followings methods: 1)UE can drop/insert samples and puncture OFDM symbols 2)UE can blank subframes, slots, and repetition units |  | Yes | Release 17 UE cannot access [NTN/satellite] | [Per UE/per band] | No | No | For UEs support NSGO scenarios, it must indicate this FG is supported. | Optional with capability signalling [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| 2. LTE_NBIOT_eMTC_NTN | 2-3 | UE specific TA pre-compensation reporting | [Support reporting of information about the UE specific TA pre-compensation] - The granularity of the reported TA is a slot. - As for the frequency of reports, the TA is reported when the offset between current information about UE specific TA and the last successfully reported information about UE specific TA is over the threshold. | 2-1 | Yes | UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD | [per UE/per band] | No | No | UE-specific TA reporting is supported in IoT-NTN FFS: Detailed contents of report FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO | Optional with capability signalling FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported |
| 2. LTE_NBIOT_eMTC_NTN | 2-2 | Enhancing timing relationships using a time offset | 1.UE receives UE specific K_offset 2.UE applies UE specific K_offset in timing relationship enhancements | 2-1 [, 2-3] | Yes | UE does not know the offset to apply for UL transmission | [per UE/per band] | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO | Optional with capability signalling FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported |

# References

- R1-2111813 Moderator’s Summary of UE features for IoT over NTN, AT&T, NTT DOCOMO.

- Chair’s notes RAN1#106-e final.

- Chair’s notes RAN1#107-e final.

- Chair’s notes RAN1#106b-e final.

- R2-2109043 Summary of [AT115-e][037][IoT-NTN] User Plane Impact	OPPO