3GPP TSG RAN WG1 #108-e		    R1-2202236

e-Meeting, February 21st – March 3rd, 2022

Source: 	Transsion Holdings

Title:	Discussion on PEI design

Agenda item:	8. 7. 1. 1

Document for:	Discussion and decision

Introduction

In RAN1#107bis-e, following conclusion on paging enhancements were reached [1].

Agreement A new PEI-RNTI is supported for DCI format 2_7. The following text proposals are adopted: (Omit) Agreement For whether and how to accommodate PEI-O location determination for the case POnumPerPEI is smaller than Ns, decide one of the following alternatives - Alt-2: It is supported, and UE applies the single value in PEI-F_offset for the frame-level offset and the (flooris/POnumPerPEI+1) -th value out of Ns/POnumPerPEI configured values in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset Note: The number of PO mapping to one PEI should be multiple of Ns when POnumPerPEI is larger than Ns Agreement For PEI-O location determination: - Range of PEI-F_offset, in unit of frame, is 0, 1,2, …, 16 - Range of firstPDCCH-MonitoringOccasionOfPEI-O, in unit of symbol, is decided as one of the following alternatives: - Alt-1: The same value range as firstPDCCH-MonitoringOccasionOfPO Agreement For the value PEI-RNTI, decide one of the following: - Alt-1: PEI-RNTI is of fixed value, and value design is up to RAN2. Conclusion For a UE supporting R17 paging enhancement feature, it is up to UE implementation whether the UE monitors the MO(s) for a PEI. - If UE decides to not to monitor PEI, it has to monitor UE’s PO as defined in 38.304. - Note: No specification impact Agreement The PDCCH monitoring occasions defined by peiSearchSpace colliding with UL symbols are omitted from the determination of the PEI monitoring occasions. The PDCCH monitoring occasions for PEI which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for PEI in the PEI-O. Agreement The transmission power of PEI is based on the same determination method as paging PDCCH, and the following TP to TS 38.213 is adopted. (Omit) Agreement UE expects at most one PEI outcome from each PDCCH monitoring occasion of PEI-O, and the following text proposal is adopted.(Omit) Agreement If one PEI-O is associated with POs of 2 PFs, - The two PFs are consecutive PFs configured in SIB - FFS: two PFs are consecutive PFs within the same paging cycle - Note: As an example, SFN of the first PF of the PF(s) associated with the PEI-O can be obtained by: (SFN of UE’s PF) - iPO/Ns⋅T/N

But there are also some issues that have not made a consensus. In this contribution, we will provide our views on these issues.

Design of PDCCH-based PEI

Short message

In RAN1#107bis-e, the following proposed conclusion was discussed[1]:

Conclusion For Rel-17, inclusion of SI change indication or ETWS/CMAS notification in DCI format 2_7 is not supported.

Since the period of PEI is shorter than the period of SI change indication or ETWS/CMAS notification, if each DCI format 2_7 contains 2 bits of SI change indication or ETWS/CMAS notification information, it will causes bit waste. Furthermore, whether or not to monitor the PEI is up to UE implementation. Therefore, at certain times, the UE can obtain SI change indication or ETWS/CMAS notification information by not monitoring the PEI but monitoring the paging PDCCH in a conventional manner.

- For Rel-17, inclusion of SI change indication or ETWS/CMAS notification in DCI format 2_7 is

- not supported.

PEI bit position

In RAN1#107bis-e Email discussion, the following proposal was discussed:

Conclusion For Rel-17 DCI format 2_7, - Explicit starting position configuration for paging indication field or TRS availability indication field is not supported. - Paging indication field starts from the first bit, and TRS availability indication field starts from the next bit after the end of the paging indication field - UE does not expect the total size of paging indication filed and TRS availability indication field is larger than the DCI payload size

As discussed by most companies, the UE only needs to know how many POs are associated with a PEI, how many subgroups are associated with each PO, and whether to include a TRS availability indication. Then, using the mapping principle of PO subgroup followed by TRS availability indication, the UE can clearly parse the meaning of the DCI field without specifying the positions of the paging indication field and the TRS availability indication field in the PEI in the system message. In addition, this mapping principle also helps to save system message bits.

- Explicit starting position configuration for paging indication field or TRS availability indication field is not supported.

SearchSpaceId#0

In RAN1#107bis-e [1], companies have made the following working assumption,

Working assumption - SearchSpaceId = 0 can be configured for peiSearchSpace for the case of CORESET multiplexing pattern 2 or 3

The analysis of different CORESET multiplexing patterns is as follows:

Firstly, for some configurations of CORESET multiplexing pattern 1, there may be scenarios where the SSB precedes the associated PEI. If the UE uses the subsequent SSB in this scenario, the UE needs to buffer the PEI before the SSB processing is completed, which undoubtedly increases the complexity of the UE processing. However, if the UE uses the previous SSB in this scenario, it will remain awake until the PEI is detected, which is not conducive to power saving. Therefore, Search Space #0 of some configurations of multiplexing pattern 1 is not suitable for PEI detection. Search space #0 for multiplexing pattern 1 is not supported unless the base station can avoid such a configuration.

For CORESET multiplexing pattern 2 or 3, since the time domain interval between CORESET and SSB is small, the UE does not need to stay awake for a long time and wait for SSB processing. Therefore, for CORESET multiplexing pattern 2 or 3, configuring the search space of PEI as searchspace#0 can still achieve power saving gain.

The CORESET multiplexing patterns are shown in the following figure:

Figure：CORESET/SSB multiplexing patterns

- SearchSpaceId=0 can be configured for peiSearchSpace for the case of CORESET multiplexing pattern 2 or 3.

PF associated with POs

In RAN1#107bis-e [1], companies have made the following agreement,

Agreement If one PEI-O is associated with POs of 2 PFs, - The two PFs are consecutive PFs configured in SIB - FFS: two PFs are consecutive PFs within the same paging cycle - Note: As an example, SFN of the first PF of the PF(s) associated with the PEI-O can be obtained by: (SFN of UE’s PF) - iPO/Ns⋅T/N

According to the definition of paging in TS 38.304, the following table is about the number of PFs in one paging cycle, where T is the paging cycle, and N is the number of PFs in one paging cycle. It can be seen from the table that the number of PFs in one paging cycle is an even number. Therefore, if a PEI is associated with two consecutive PFs, these consecutive PFs must be in the same paging cycle.

Table : the number of PFs in one paging cycle

| N T | 32 | 64 | 128 | 256 |
| --- | --- | --- | --- | --- |
| T | 32 | 64 | 128 | 256 |
| T/2 | 16 | 32 | 64 | 128 |
| T/4 | 8 | 16 | 32 | 64 |
| T/8 | 4 | 8 | 16 | 32 |
| T/16 | 2 | 4 | 8 | 16 |

- Two PFs are consecutive PFs within the same paging cycle.

Conclusion

In this contribution, we discuss the design of PEI. We have the following proposals.

- For Rel-17, inclusion of SI change indication or ETWS/CMAS notification in DCI format 2_7 is

- not supported.

- Explicit starting position configuration for paging indication field or TRS availability indication field is not supported.

- SearchSpaceId=0 can be configured for peiSearchSpace for the case of CORESET multiplexing pattern 2 or 3.

- Two PFs are consecutive PFs within the same paging cycle.

References

Draft_Minutes_report_RAN1#107b-e_v020，3GPP TSG RAN1#107bis-e，2022-01