3GPP TSG RAN WG1 Meeting #108-e					           R1-2202258

e-Meeting, February 21st – March 3rd, 2022

______________________________________________________________________Agenda item: 8.11.1.2

Source: Moderator (LG Electronics)

Title: Feature lead summary #5 for AI 8.11.1.2 Inter-UE coordination for Mode 2 enhancements

Document for: Discussion and information

- Draft proposals for Wednesday’s GTW (March 2)

  - Scheme 1

FL’s observation of 6th email discussion (UE-B’s behavior when UE-B receives multiple resource sets from the same UE-A)

- Alt 1: Fraunhofer, Intel, OPPO, Apple, DCM, NEC, Futurewei, vivo, Ericsson(for progress), LGE(for progress), Qualcomm, InterDigital(for progress), Panasonic, Spreadtrum, Sharp(for progress), (15)

  - Comment:

    - Remove note: Fraunhofer, Intel, Futurewei, Sharp, (4)

    - For 3rd bullet, up to UE implementation whether or not to perform 1st sub-bullet and/or 2nd sub-bullet: vivo, (1)

- Alt 2: Nokia, LGE, InterDigital, Fujitsu, xiaomi, Sharp, Huawei, (7)

- Other Alt:

  - Samsung:

    - For preferred resources and non-preferred resources, this would be a combination of the two cases:

      - When UE is transmitting to UE-A, it can use the latest of preferred or non-preferred resources. We are also open to using the latest preferred resource and the latest non-preferred resources.

  - CATT:

    - For UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the same UE-A,

      - Alt 1-1: UE-B uses the latest received non-preferred resource set from the same UE-A subject to UE processing timeline for its resource selection for a TB to be transmitted to the UE-A.

      - Alt 1-2: it is up to UE-B implementation to use one or multiple of them in its resource (re)selection.

  - Huawei:

    - Align the same behavior between when UE-B receives multiple preferred resource sets from the same UE-A and when UE-B receives multiple non-preferred resource sets from the same UE-A

Updated Draft proposal 3-13:

Alt 1:

- For UE-B’s behavior when UE-B receives multiple preferred resource sets from the same UE-A,

  - Option 1: UE-B uses the latest received preferred resource set from the same UE-A subject to UE processing timeline for its resource selection for a TB to be transmitted to the UE-A.

    - Note: If latency bound of inter-UE coordination information is defined in RAN2, RAN1 will check the necessity of further refinement on the meaning of “the latest received preferred resource set”.

- For UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the same UE-A,

  - Option 3: UE-B determines a final non-preferred resource set by making union of all the received non-preferred resource sets from the same UE-A subject to UE processing timeline. UE-B uses the final non-preferred resource set for its resource selection.

- For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A,

  - It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

Alt 2:

- When UE-B receives multiple inter-UE coordination information from the same UE-A, it is up to UE-B implementation to use one or multiple of them in its resource (re)selection.

FL’s observation of 6th email discussion (UE-B’s behavior when UE-B receives multiple resource sets from the different UE-As)

- Alt 1: Intel, Apple, DCM, NEC, vivo, Ericsson(for progress), LGE(for progress), Qualcomm, InterDigital(for progress), Panasonic, Spreadtrum, Sharp(for progress), (12)

  - Comment:

    - For 3rd bullet, up to UE implementation whether or not to perform 1st sub-bullet and/or 2nd sub-bullet: vivo,

    - Add “none” for the 3rd bullet: Intel,

- Alt 2: Nokia, OPPO, LGE, InterDigital, ZTE, Fujitsu, xiaomi, Sharp, Huawei, (9)

- Other Alt:

  - Fraunhofer:

    - UE-B uses each received non-preferred resource set subject to UE processing timeline for its resource selection for each TB to be transmitted to each UE-A providing the non-preferred resource set

    - UE-B determines a final non-preferred resource set by making a union of all the received non-preferred resource sets from different UE-As subject to UE processing timeline. UE-B uses the final non-preferred resource set for its resource selection subject to the location of the UE-As

  - OPPO, Apple, CATT, (3):

    - For UE-B’s behaviour when UE-B receives multiple non-preferred resource sets from the different UE-As.

      - it is up to UE-B implementation to use one or multiple of them in its resource (re)selection

  - Futurewei:

    - For UE-B’s behaviour when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As,

      - Option 2: UE-B uses both the received preferred resource set and non-preferred resource set from different UE-As for its resource selection for a TB to be transmitted to the UE-A providing the preferred resource set. UE-B uses the received non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A providing the non-preferred resource set.

  - Samsung:

    - For UE-B’s behaviour when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As,

      - UE-B uses for its resource selection both of the following:

        - A final non-preferred resource set by making union of all the received non-preferred resource sets from different UE-As subject to UE processing timeline

        - A Preferred resource set from a UE-A (if any) subject to UE processing timeline when transmitting to the UE-A.

  - Huawei:

    - Align the same behavior between when UE-B receives multiple preferred resource sets from the different UE-As and when UE-B receives multiple non-preferred resource sets from the different UE-As

Draft proposal 3-14:

Alt 1:

- For UE-B’s behaviour when UE-B receives multiple preferred resource sets from the different UE-As,

  - Option 1: UE-B uses each received preferred resource set subject to UE processing timeline for its resource selection for each TB to be transmitted to each UE-A providing the preferred resource set.

    - It is up to UE-B’s implementation to avoid the case where UE-B uses the preferred resource sets overlapping in the same slot simultaneously for its resource selection for different TBs to be transmitted to different UE-As providing the preferred resource sets

- For UE-B’s behaviour when UE-B receives multiple non-preferred resource sets from the different UE-As.

  - Option 1: UE-B determines a final non-preferred resource set by making union of all the received non-preferred resource sets from different UE-As subject to UE processing timeline. UE-B uses the final non-preferred resource set for its resource selection.

- For UE-B’s behaviour when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As,

  - it is up to UE-B implementation to use one or multiple of them in its resource (re)selection

Alt 2:

- When UE-B receives multiple inter-UE coordination information from the different UE-As, it is up to UE-B implementation to use one or multiple of them in its resource (re)selection.

FL’s observation of 6th email discussion (Criteria on which received preferred non-preferred resource set(s) can be actually taken into account in UE-B’s resource selection)

- Option 1: Feedback type (e.g., request- or condition- based, preferred or non-preferred resource sets)

  - Supported by Intel, (1)

  - It is not concrete criterion: OPPO, (1)

  - Not supported by LGE, Qualcomm, (2)

- Option 2: Feedback source ID

  - Supported by Fraunhofer, Intel, Sharp, (3)

  - It is not concrete criterion: OPPO, (1)

  - Not supported by LGE, Qualcomm, (2)

- Option 3: Feedback generation time

  - Supported by Fraunhofer, Intel, CATT, (3)

  - It is not concrete criterion: OPPO, (1)

  - Not supported by LGE, Qualcomm, (2)

- Option 4: Overlap ratio of resource selection windows

  - Supported by Fraunhofer, Intel, (2)

  - It is not concrete criterion: OPPO, (1)

  - Not supported by LGE, Qualcomm, (2)

- Option 5: Radio range or geographical distance from the source of inter-UE coordination feedback

  - Supported by Fraunhofer, Intel, NEC, Ericsson, xiaomi, CATT, (6)

  - It is not concrete criterion: OPPO, (1)

  - Not supported by LGE, Qualcomm, (2)

- Option 6: Priority level used for generation of inter-UE coordination feedback

  - Supported by Fraunhofer, Intel, NEC, (3)

  - It is not concrete criterion: OPPO, (1)

  - Not supported by LGE, Qualcomm, (2)

- Option 7: Alignment between parameter(s) of preferred resource set and transmission requirement of a TB to be transmitted by UE-B

  - Supported by Fraunhofer, NEC, vivo, (3)

  - It is not concrete criterion: OPPO, (1)

  - Not supported by LGE, Qualcomm, (2)

- Option 8: Up to UE implementation (i.e., not specify the filtering criteria)

  - Supported by LGE, InterDigital, ZTE, xiaomi, Huawei, (5)

  - Not supported by Qualcomm, (1)

- Comments:

  - Discuss 3-13/3-14 first: DCM, (1)

  - Prefer to keep previous conclusion: OPPO, Qualcomm, Samsung, Huawei, (4)

Draft conclusion 6-3:

There is no consensus in RAN1 on defining additional criteria of filtering the received preferred resource set(s) and/or non-preferred resource set(s) to be taken into account in UE-B’s resource selection

FL’s observation of 6th email discussion (Additional UE-B behavior to handle the case when it is not possible that the number of candidate single-slot resources after applying the received non-preferred resource set as per the existing agreement meets the requirement of X*M_total)

- Alt 1: Fraunhofer, Nokia, Apple, DCM, NEC, Futurewei, vivo(for progress), LGE, Qualcomm, InterDigital, ZTE, Panasonic, Fujitsu, Spreadtrum, CATT, (15)

- Alt 2: Intel, OPPO, Ericsson, LGE, xiaomi, Huawei, (6)

- Other:

  - Fruanhofer, Ericsson:

    - If UE-B has its own resource set information, it uses this candidate resource set as fallback

    - Otherwise, It is up to UE-B’s implementation whether/how to take the received non-preferred resource set in its resource selection to meet this requirement

  - Samsung:

    - It is up to UE-B’s implementation whether/how to take the received non-preferred resource set in its resource selection to meet this requirement

Updated Draft proposal 3-16:

Alt 1:

For the case when it is not possible that the number of candidate single-slot resources after applying the received non-preferred resource set as per the existing agreement meets the requirement of X*M_total in step 7),

- It is up to UE-B’s implementation whether/how to take the received non-preferred resource set in its resource selection after step 6) to meet this requirement

Alt 2:

For the case when it is not possible that the number of candidate single-slot resources after applying the received non-preferred resource set as per the existing agreement meets the requirement of X*M_total in step 7),

- UE-B does not take the received non-preferred resource set in its resource selection after step 6).

FL’s observation of 6th email discussion (Cast type of inter-UE coordination information when both a SCI format 2-C and MAC CE are used)

- Support: Fraunhofer(for progress), OPPO, DCM, NEC, Futurewei, vivo, Ericsson, LGE, Qualcomm, InterDigital, Samsung(for progress), ZTE, Panasonic, Fujitsu, Spreadtrum, xiaomi, Sharp, CATT, Huawei, (19)

- Not support: Nokia, (1)

- Comments:

  - Keep “only”: OPPO, DCM, Qualcomm, ZTE, Sharp, (5)

Updated Draft proposal 3-8:

- For inter-UE coordination information transmission, only when the cast type of inter-UE coordination information is unicast regardless of whether or not it is multiplexed with other data, a SCI format 2-C  can be used in addition to MAC CE

FL’s observation of 6th email discussion (UE-A’s behavior of determining a priority value of inter-UE coordination information transmission triggered by a condition other than explicit request reception if the priority value is not (pre)configured)

- Keep the sub-bullet: Intel, DCM, LGE, Qualcomm, InterDigital, Panasonic, Huawei, (7)

- Remove the sub-bullet: Fraunhofer, OPPO, Apple, DCM, NEC, Futurewei, vivo, Ericsson, LGE, Samsung, ZTE, Panasonic, Fujitsu, Spreadtrum, xiaomi, Sharp, CATT, (17)

Updated Draft conclusion 3-11:

In RAN1, no further decision is necessary for UE-A’s behavior of determining a priority value of inter-UE coordination information transmission triggered by a condition other than explicit request reception if the priority value is not (pre)configured

- It is up to RAN2 whether/how to additionally handle this case (e.g., defining default priority value)

FL’s observation of 6th email discussion (Condition when Option B can be used for preferred resource set)

- Support: Fraunhofer, OPPO, Apple, NEC, Futurewei, vivo, LGE(for progress), Qualcomm, InterDigital, Panasonic, Spreadtrum, xiaomi, Sharp(for progress), Huawei, (14)

- Not support: DCM, Ericsson, Samsung, ZTE, (4)

  - Remove Option 3: DCM, Ericsson, Samsung, ZTE, (4)

  - Remove Option 2: Ericsson, Samsung, (2)

- Other

  - DCM: Add following as a compromise

    - Option 3 is allowed only in resource pool (pre-)configured with random selection

  - (pre)configuration enables/disables Option 2 and/or Option 3

    - Supported by Fraunhofer(for progress), Qualcomm(for progress), Sharp, (3)

Draft proposal 3-17:

Alt 1:

For Scheme 1, Option B can be used for preferred resource set when at least one of following condition is met:

- Option 1: UE-B does not have a capability of performing sensing/resource exclusion.

- Option 2: UE-B performs random resource selection.

- Option 3: UE-B has a capability of performing sensing/resource exclusion, but UE-B determines not to perform sensing/resource exclusion by its implementation.

Alt 2:

For Scheme 1, Option B can be used for preferred resource set when at least one of following condition is met:

- Option 1: UE-B does not have a capability of performing sensing/resource exclusion.

- Option 2: UE-B performs random resource selection.

- Option 3: UE-B has a capability of performing sensing/resource exclusion, but UE-B determines not to perform sensing/resource exclusion by its implementation.

  - It is allowed only in resource pool (pre)configured with random resource selection

FL’s observation of 6th email discussion (2nd SCI format(s) than can be used to schedule retransmission of a TB containing inter-UE coordination information initially scheduled by a SCI format 2-C)

- Alt 1: OPPO, DCM, vivo, LGE, InterDigital, Fujitsu, Sharp, (7)

- Alt 2: Fraunhofer, Apple, Futurewei, vivo, Ericsson, Qualcomm, ZTE, Panasonic, Spreadtrum, xiaomi, Sharp, CATT, Huawei, (13)

- Comments:

  - Samsung: a SCI format 1-A indicates how to derive TBS

Updated Draft conclusion 3-20:

Alt 1:

- No consensus on any restriction on a 2nd SCI format that can be used for retransmission of inter-UE coordination information MAC CE initially scheduled by a SCI format 2-C.

  - Note: the same TBS should be ensured between initial transmission and retransmission(s) for the same inter-UE coordination information

Alt 2:

No consensus on 2nd SCI format other than SCI format 2-C that can be used for retransmission of inter-UE coordination information MAC CE initially scheduled by a SCI format 2-C

- Note: the same TBS should be ensured between initial transmission and retransmission(s) for the same inter-UE coordination information

FL’s observation of 6th email discussion (Cast type of inter-UE coordination information transmission triggered by a condition other than explicit request reception)

- Alt 1: Nokia, DCM, NEC, Futurewei, vivo(for progress), Ericsson, Qualcomm, InterDigital, ZTE, Panasonic, Fujitsu, Spreadtrum, Sharp, CATT, (14)

- Alt 2: Fraunhofer, Apple, LGE, xiaomi, Huawei, (5)

- Alt 2 with following note: OPPO, ZTE, Spreadtrum, CATT, (4)

  - Note: When IUC is multiplexed with other data, the cast type of IUC must be the same as the cast type of the data

- Others:

  - Intel: pre-defined cast type is used

  - Samsung: not support unicast

Updated Draft conclusion 3-12:

Alt 1:

- For transmission of inter-UE coordination information with non-preferred resource set triggered by a condition other than explicit request reception,

  - When it is not multiplexed with other data, UE-A determines its cast type by implementation

Alt 2:

- For transmission of inter-UE coordination information with non-preferred resource set triggered by a condition other than explicit request reception,

  - UE-A determines its cast type by implementation

  - Note: When inter-UE coordination information is multiplexed with other data, the cast type of inter-UE coordination information is the same as the cast type of the data

FL’s observation of 5th email discussion (Restrictions to the IUC mechanism to address the power saving operation)

- Support: Fraunhofer, InterDigital, Qualcomm, LGE, DCM, NEC, Futurewei, Apple, Spreadtrum, xiaomi, Panasonic, vivo, Samsung, OPPO, Fujitsu, Huawei, ZTE, (17)

- Not support: Intel, Ericsson, CATT, (3)

Updated Draft conclusion 3-19:

No consensus on supporting specific enhancement in Rel-17 on UE-A’s behavior not triggering inter-UE coordination information generation when the amount of sensing performed by UE-A is below a certain threshold

FL’s observation of 5th email discussion (Latency bound of inter-UE coordination information)

- Support: InterDigital, Qualcomm, LGE, DCM, Apple, Ericsson, Panasonic, Samsung, OPPO, Fujitsu, Huawei, ZTE, (12)

- Not support: Intel, Futurewei, xiaomi, CATT, (4)

  - Conclusion is not needed: Intel, CATT, vivo, (3)

FL’s observation of 6th email discussion (Latency bound of inter-UE coordination information)

- Further comments:

  - RAN1 needs to discuss latency bound issue: Intel, Futurewei, vivo, xiaomi, (4)

Draft conclusion 3-9:

- For latency bound of inter-UE coordination information transmission, RAN1 relies on RAN2’s decision as per LS R1-2200880 from RAN2

- Draft proposals for 7th email discussion (Due: March 3rd 4:59am UTC)

  - Scheme 1

FL’s observation of 6th email discussion (selection window for inter-UE coordination information transmission)

- For 1st bullet,

  - Alt 1: Intel, OPPO, Ericsson, LGE, Sharp, (5)

  - Alt 2: Apple, Futurewei, vivo, Fujitsu, CATT, (5)

  - Other alt:

    - n’+T’_2 < n+T_1: Apple, Futurewei, Qualcomm, InterDigital, (4)

  - Neither: Samsung, Huawei, (2)

- For 2nd bullet,

  - Support: Intel, Apple, vivo, Ericsson, InterDigital, Fujitsu, Sharp, CATT, (8)

  - Not support: OPPO, Futurewei, LGE, Samsung, Huawei, (5)

    - Futurewei: n’+T_2 < n+T_1

- Up to UE implementation to ensure the set of resources are within [n+T_1, n+T_2] subject to Rel-16 specification: Samsung, Huawei, (2)

- Up to RAN2 decision: ZTE, (1)

Draft conclusion 3-10-3:

For a resource selection window for transmitting inter-UE coordination information in Scheme 1,

- For inter-UE coordination information triggered by UE-B’s explicit request,

  - Alt 1:

    - (n+T_1) ≤ (n’+T’_1)

    - (n’+T’_2) ≤ (n+T_2)

where

      - (n+T_1) – Start slot of resource selection window for determining the set of resources

      - (n+T_2) – End slot of resource selection window for determining the set of resources

      - (n’+T’_1) – Start slot of resource selection window used for inter-UE coordination information transmission

      - (n’+T’_2) – End slot of resource selection window used for inter-UE coordination information transmission

  - Alt 2:

    - (n’+T’_1 – Tproc,0 – Tproc,1 – T2,min) ≤ (n+T_1)

where

      - (n+T_1) – Start slot of resource selection window for determining the set of resources

      - (n’+T’_1) – Start slot of resource selection window used for inter-UE coordination information transmission

  - Alt 3:

    - (n’+T’_2) ≤ (n+T_1)

where

      - (n+T_1) – Start slot of resource selection window for determining the set of resources

      - (n’+T’_2) – End slot of resource selection window used for inter-UE coordination information transmission

- For inter-UE coordination information triggered by a condition other than explicit request reception,

  - (n'+T_1) < (n+T_1)

where

    - (n+T_1) – Start slot of resource selection window for determining the set of resources

    - (n’+T_1) – Start slot of resource selection window used for inter-UE coordination information transmission

Q7-1: FL understands that according the following existing agreements, we don’t need to define the notation of “n” itself separately. This means that the terms of (n+T_1) and (n+T_2) themselves should be directly used to discuss the relationship between a resource selection window for determining the set of resources and a resource selection window for determining TX resources of inter-UE coordination information.

- Agreement: - For determining preferred resource set in Scheme 1, when inter-UE coordination information transmission is triggered by a condition other than explicit request reception, - Values of following parameters are (pre)configured for a resource pool. If there is no (pre)configuration, UE-A determines by its implementation the values of the following parameters - prio_TX - L_subCH - P_rsvp_TX - UE-A determines by its implementation values of following parameters - n+T_1, n+T_2 - FFS: Whether/how to support (pre)configuration of n+T_1 and n+T_2 - Note that it is up to RAN2 decision whether/how the values of these parameters are provided by PC5-RRC signaling from UE-B to UE-A and UE-A uses the received information to determine the preferred resource set - Agreement: - For Scheme 1, when the inter-UE coordination information transmission is triggered by UE-B’s explicit request, - Starting/Ending time locations of resource selection window is provided by UE-B’s explicit request - Starting/Ending time locations of resource selection window is a form of combination of DFN index and slot index

Company provides which alternative is supported.

- Notations:

  - (n+T_1) – Start slot of resource selection window for determining the set of resources

    - For inter-UE coordination information triggered by UE-B’s explicit request, this value of (n+T_1) is provided by UE-B’s request as per the existing agreement

    - For inter-UE coordination information triggered by a condition other than explicit request reception, this value of (n+T_1) is determined by UE-A’s implementation as per the existing agreement

  - (n+T_2) – End slot of resource selection window for determining the set of resources

    - For inter-UE coordination information triggered by UE-B’s explicit request, this value of (n+T_2) is provided by UE-B’s request as per the existing agreement

    - For inter-UE coordination information triggered by a condition other than explicit request reception, this value of (n+T_2) is determined by UE-A’s implementation as per the existing agreement

  - (n’+T’_1) – Start slot of resource selection window used for inter-UE coordination information transmission

  - (n’+T’_2) – End slot of resource selection window used for inter-UE coordination information transmission

  - n' is the slot where UE procedure of determining TX resources of inter-UE coordination information is triggered

- For inter-UE coordination information triggered by UE-B’s explicit request,

  - Alt 1-1:

    - (n+T_1) ≤ (n’+T’_1)

    - (n’+T’_2) ≤ (n+T_2)

  - Alt 1-2:

    - (n’+T’_1 – Tproc,1 – T2,min) ≤ (n+T_1)

  - Alt 1-3:

    - (n’+T’_2) < (n+T_1)

  - Alt 1-4:

    - (n‘+T‘_1) < (n+T_1)

  - Alt 1-5:

    - (n’+T’_1 – Tproc,1) ≤ (n+T_1)

  - Alt 1-6:

    - Up to UE-A’s implementation

- For inter-UE coordination information triggered by a condition other than explicit request reception,

  - Alt 2-1:

    - (n'+T’_1) < (n+T_1)

  - Alt 2-2:

    - (n’+T’_2) < (n+T_1)

  - Alt 2-3:

    - (n'+T’_1) ≥ (n+T_1)

  - Alt 2-4:

    - Up to UE-A’s implementation

| Company | Alternative(s) for inter-UE coordination information triggered by UE-B’s explicit request | Alternative(s) for inter-UE coordination information triggered by a condition other than explicit request reception | Comments |
| --- | --- | --- | --- |

FL’s observation of 6th email discussion (Sensing window for determining the set of resources)

- Support: OPPO, DCM, NEC, Ericsson, LGE, InterDigital, ZTE, Panasonic, Fujitsu, Spreadtrum, xiaomi, CATT, Huawei, (13)

  - Add “initially” before “determining the set of resources”: Huawei, (1)

- Not support: Intel, Futurewei, vivo, Sharp, (4)

  - Futurewei: [ (n+T_1) – T_0 – T’_1 determined by UE-A, (n+T_1) – T_proc,0 –T_proc,1-T2min- T’_1 determined by UE-A ]

  - Vivo: For inter-UE coordination information triggered by UE-B’s explicit request, n+T_1 and n+T_2 are provided by the request determined according to Rel-16 TS 38.214 Section 8.1.4. With n > = n’, where n’ is the slot in which inter-UE coordination information generation is triggered and n is selected so that  [n+T_1, n+T2] is located within the selection window provided by the request

- Comments:

  - Samsung: revise wording as follows:

    - The sensing window for determining the set of resources in Scheme 1, is determined relative to the resource selection window of the set of resources by re-using the Rel-16 sensing window design.

Updated Draft conclusion 3-10-1:

For sensing window for determining the set of resources in Scheme 1,

- No further change is supported. Note that the sensing window for determining the set of resources is already derived based on the location n+T_1 and n+T_2 used for determining the set of resources in TS38.214 section 8.1.4, i.e., sensing window is defined by the range of slots [ (n+T_1) – T_0 – T_1 determined by UE-A, (n+T_1) – T_proc,0 – T_1 determined by UE-A ].

  - For inter-UE coordination information triggered by UE-B’s explicit request, n+T_1 and n+T_2 are provided by the request.

  - For inter-UE coordination information triggered by a condition other than explicit request reception, n+T_1 and n+T_2 are determined by UE-A’s implementation.

Q7-2: FL understands that according the following existing agreements, we don’t need to define the notation of “n” itself separately. This means that the terms of (n+T_1) and (n+T_2) themselves should be directly used to discuss the definition of sensing window for determining the set of resources.

- Agreement: - For determining preferred resource set in Scheme 1, when inter-UE coordination information transmission is triggered by a condition other than explicit request reception, - Values of following parameters are (pre)configured for a resource pool. If there is no (pre)configuration, UE-A determines by its implementation the values of the following parameters - prio_TX - L_subCH - P_rsvp_TX - UE-A determines by its implementation values of following parameters - n+T_1, n+T_2 - FFS: Whether/how to support (pre)configuration of n+T_1 and n+T_2 - Note that it is up to RAN2 decision whether/how the values of these parameters are provided by PC5-RRC signaling from UE-B to UE-A and UE-A uses the received information to determine the preferred resource set - Agreement: - For Scheme 1, when the inter-UE coordination information transmission is triggered by UE-B’s explicit request, - Starting/Ending time locations of resource selection window is provided by UE-B’s explicit request - Starting/Ending time locations of resource selection window is a form of combination of DFN index and slot index

Company provides which alternative is supported.

For sensing window for determining the set of resources in Scheme 1,

- Notations:

  - For inter-UE coordination information triggered by UE-B’s explicit request, the values of (n+T_1) and (n+T_2) are provided by the request as per the existing agreement.

  - For inter-UE coordination information triggered by a condition other than explicit request reception, the values of (n+T_1) and (n+T_2) are determined by UE-A’s implementation as per the existing agreement.

  - T’’_1 is up to UE-A’s implementation under 0 <= T’’_1 <= Tproc,1

  - (n’+T’_1) – Start slot of resource selection window used for inter-UE coordination information transmission

  - n' is the slot where UE procedure of determining TX resources of inter-UE coordination information is triggered

- Alt 1:

  - No further change is supported. Note that the sensing window for determining the set of resources is already derived based on the location (n+T_1) and (n+T_2) used for determining the set of resources in TS38.214 section 8.1.4, i.e., sensing window is defined by the range of slots [ (n+T_1) – T_0 – T’’_1, (n+T_1) – T_proc,0 – T’’_1].

- Alt 2:

  - Sensing window for determining the set of resources is defined by the range of slots [ (n+T_1) – T_0 – T’’_1, (n+T_1) – T_proc,0 – T’’_1 –T_proc,1 – T2min ]

- Alt 3:

  - Sensing window for determining the set of resources is defined by the range of slots [ (n+T_1) – T_0 – T’’_1, (n’+T’_1) – T_proc,0  –T’’_ 1 ]

| Company | Alternative(s) | Comments |
| --- | --- | --- |

FL’s observation of 6th email discussion (re-evaluation of the set of resources)

- Support: Intel, NEC, Qualcomm, InterDigital, Huawei, (5)

- Not support: OPPO, Apple, DCM, Futurewei, vivo, Samsung, ZTE, Fujitsu, Spreadtrum, xiaomi, Sharp, CATT, (12)

  - OPPO, Samsung, ZTE (3), : leave resource set update at slot n’-T_3 to UE implementation

  - Fujitsu (1): (pre)configuration enables/disables the re-evaluation procedure

- Comments:

  - Huawei:

    - Sensing window for updating the set of resources: [m - T3 - T0, m - T3 - Tproc,0)

    - Resource selection window for updating the set of resources: [m- T3 + T1, m - T3 +T2]

Q7-3: Company provide which alternative is preferred.

Draft proposal or conclusion 3-10-2:

Alt 1:

There is no consensus in RAN1 on specifying UE-A’s behavior of updating the preferred or non-preferred resource set before its inter-UE coordination information transmission

Alt 2:

For Scheme 1,

- UE-A is required It is up to UE-A’s implementation to update the set of resources at slot n’ m-T_3, where slot n’ m is the slot in which UE-A sends the inter-UE coordination information is transmitted

  - It is up to UE-A’s implementation to perform additional updates before or after slot n’ m -T_3

| Company | Alternative(s) | Comments |
| --- | --- | --- |

- Summary of contributions

  - Scheme 1

- Finalization of contents and containers of UE-A’s inter-UE coordination information and UE-B’s explicit request, including determination of destination UE(s) for UE-A’s inter-UE coordination information and UE-B’s explicit request

  - Remaining details on determining preferred resource set

    - If inter-UE coordination information is triggered by a condition rather than request reception

      - Setting of resource selection window

        - T_1 and T_2 are (pre)configured and slot n is a slot when UE-A start to process the sensing and resource selection [Futurewei,3] (1)

        - T_2-T_1 is (pre)configured [Intel,14] (1)

      - Minimum number of candidate single-slot resources for feedback [Intel,14] (1)

      - No further change is supported [OPPO,6] [CMCC,17] [ZTE,29] (3)

    - Further consideration on modification of T_scal [Sharp,23] (1)

  - Remaining details on bit field size of contents of inter-UE coordination information

    - Reference slot indication

      - 10+ ceil( log2(10*2^u)) where u is 0, 1, 2, 3 for SCS of 15, 30, 60, 120, respectively

        - Supported by [Huawei,1] [CATT,7] [Apple,15] [Samsung,20] [LGE,26] [ZTE,29] (6)

    - Slot offset for first resource location

      - Ceil(log2(N_slot_offset)) where N_slot_offset is the number of entries in the (pre)configured values set from [0, 255]

        - Supported by [Huawei,1] (1)

      - Ceil(log2(maximum value of slot offset))

        - Supported by [DCM,9] [Apple,15] (2)

      - 8 bits

        - Supported by [Samsung,20](for TRIV other than first TRIV) [ZTE,29](for TRIV other than first TRIV in a SCI format 2-C) (2)

      - 0 bit

        - Supported by [Samsung,20](for first TRIV) [ZTE,29](for first TRIV) (2)

      - Ceil(log2(maximum value of slot offset/31))

        - Supported by [LGE,26] (1)

    - Resource set indication for each combination

      - Up to 26 bits [Huawei,1] [Panasonic,5] [CATT,7] [DCM,9] [Apple,15] [Samsung,20](for non-preferred resource set) [LGE,26] (7)

      - Up to 22 bits [Samsung,20](for preferred resource set) [ZTE,29] (2)

    - Resource set type

      - Always 1 bit

        - Supported by [Huawei,1] [CATT,7] [LGE,26] (3)

      - 0 bit if request contains “resource set type indication” and if condition-based IUC is disabled. Otherwise, 1 bit.

        - Supported by [Apple,15] (1)

  - Remaining details on first resource location indication of each TRIV

    - Maximum value of slot offset for the first resource location indication

      - 16

        - Supported by [Apple,15](for SCI format 2-A as a baseline) (1)

      - 32

        - Supported by [Qualcomm,22](when SCI format 2-C is used) (1)

      - 256

        - Supported by [Huawei,1] [CATT,7](for 2nd SCI) [DCM,9] [Apple,15](for SCI format 2-A as a baseline) (4)

      - 1023

        - Supported by [ZTE,29] (1)

      - 4092

        - Supported by [OPPO,6] (1)

      - 8000

        - Supported by [CATT,7](for MAC CE) [LGE,26] (2)

      - 8192

        - Supported by [Futurewei,3] [Samsung,20] (2)

          - Possible values of (pre)configured maximum value is form of 2^k -1 [Futurewei,3] [Samsung,20]

      - Maximum reservation periodicity configured in the pool * 2^u

        - Supported by [Qualcomm,22](when MAC CE only is used) (1)

    - Granularity of slot offset

      - 1

        - Supported by [CATT,7] [DCM,9] [Apple,15] [Qualcomm,22] (4)

      - 31

        - Supported by [LGE,26] (1)

      - Candidates themselves are (pre)configured

        - Supported by [Huawei,1] (1)

      - Determined by the bit field size for indicating slot offset and SCS (e.g., 1, 2, 4, 8, 16, 32)

        - Supported by [Samsung,20] (1)

    - Whether or not UE-A provide preferred or non-preferred resources for each first resource location

      - Supported with additional indicating the lowest subchannel index of each first resource

        - [OPPO,6] [ETRI,13] [Apple,15] (3)

      - Supported with additional indicating the lowest subchannel index of first resource of a first combination

        - Supported by [Intel,14] (1)

      - Not support

        - [Huawei,1] (1)

    - Further consideration on modifying the definition of reference slot [ETRI,13] [Intel,14] (2)

  - Remaining details on bit field size of contents of an explicit request

    - Starting and ending time locations of a resource selection window

      - 2*{10+ ceil( log2(10*2^u))} where u is 0, 1, 2, 3 for SCS of 15, 30, 60, 120, respectively

        - Supported by [Huawei,1] [CATT,7] (2)

    - Resource set type

      - 0 or 1 bit as per (pre)configuration

        - Supported by [Apple,15] (1)

  - Details on a SCI format 2-C

    - SCI fields design

      - SCI fields for a SCI format 2-A

        - Supported by [Huawei,1] [DCM,9] [Apple,15] [Xiaomi,19] [ITL,25] [LGE,26] [Ericsson,27] [ZTE,29] (8)

          - [vivo,4] [Panasonic,5]: Cast type is not included for an explicit request

      - SCI fields for both a SCI format 2-A and 2-B

        - Supported by [Panasonic,5] [CATT,7] [Intel,14] [Samsung,20] (4)

    - Condition of that a SCI format 2-C can be used as container of inter-UE coordination information

      - Keep N<=3 (i.e., remove square brackets)

        - Supported by [LGE,26] [Ericsson,27] (2)

          - [LGE,26]: Add “UE does not expect that the total payload size of a SCI format 2-C with N=3 exceeds 140 bits” as a note

      - N<=2

        - Supported by [CATT,7] [DCM,9] [Apple,15] (3)

      - Remove N parts

        - Supported by [Intel,14] (1)

      - Both N<=3 and N<=2

        - Supported by [Samsung,20] (1)

      - Further consideration on additional condition of that a SCI format 2-C can be used [Intle,14] [Qualcomm,22]

        - [Intel,14]: a SCI format 2-C can be used for preferred resource set

        - [Qualcomm,22]: a SCI format 2-C can be used for the case when other data is not multiplexed with inter-UE coordination information

  - Cast type(s) of inter-UE coordination information transmission with preferred resource set triggered by a condition other than explicit request reception on top of unicast

    - Neither groupcast nor broadcast

      - Supported by [vivo,4] [Panasonic,5] [OPPO,6] [DCM,9] [Spreadtrum,11] [CMCC,17] [Samsung,20] [LGE,26] [Ericsson,27] [Mitsubishi,28] [ZTE,29] (11)

    - Groupcast

      - Supported by [Futurewei,3] [Fraunhofer,30] (2)

    - Groupcast and broadcast

      - Supported by [Intel,14] (1)

    - Up to RAN2/SA2 decision

      - Supported by [Huawei,1] (1)

  - Latency bound of inter-UE coordination information transmission triggered by UE-B’s explicit request

    - Supported by [vivo,4] [CATT,7] [Intel,14] [Apple,15] [Xiaomi,19] [Qualcomm,22] [Sharp,23] [ITL,25] [Fraunhofer,30] (9)

      - PC5-RRC configured

        - Supported by [vivo,4] (1)

      - (pre)configured

        - Supported by [CATT,7] [Intel,14] [Xiaomi,19] (3)

      - Indicated by UE-B’s request

        - Supported by [CATT,7] [Apple,15] [Sharp,23] [ITL,25] [Fraunhofer,30] (5)

      - 8 slots

        - Supported by [Qualcomm,22](for standalone inter-UE coordination information) (1)

      - Derived based on the starting time of resource selection window provided by UE-B’s request

        - Supported by [Sharp,23] (1)

  - Further consideration on modifying UE-A’s procedure for determining a set of resources [Nokia,2] [vivo,4] [CATT,7] [Intel,14] [ASUSTeK,16] [Fraunhofer,30] (6)

  - Further consideration on additional contents of the inter-UE coordination information in Scheme 1 [InterDigital,10] [Intel,14] [ASUSTeK,16] (3)

  - Further consideration on differentiating supported cast type for each condition of non-preferred resource set [OPPO,6] [CMCC,17] [Mitsubishi,28] (3)

    - Up to UE-A’s implementation [DCM,9] (1)

  - Further consideration on additional contents of the request for the inter-UE coordination information in Scheme 1 [Nokia,2] [Fujitsu,8] (2)

  - Further consideration on specifying additional details on Condition 1-A-2/1-B-2/2-A-2 [vivo,4] [Intel,14] (2)

  - Further consideration on additional condition for determining a set of resources [Nokia,2]

  - Further consideration on parameter setting for determining the non-preferred resource set [Futurewei,3]

  - Further consideration on modifying re-evaluation/pre-emption operation considering the received non-preferred resource set [vivo,4]

  - Further consideration on using UE-A’s resource reservation period as coordination information [vivo,4]

  - Further consideration on modifying the cast type of request signaling [Intel,14]

  - Further consideration on modifying the cast type of inter-UE coordination information transmission triggered by an explicit request [Intel,14]

  - Further consideration on modifying the cast type of inter-UE coordination information transmission triggered by a condition other than explicit request reception [Xiaomi,19]

  - Further consideration on the case when only a SCI format 2-C is used as a container of inter-UE coordination information and/or its request [Samsung,20]

  - Further consideration on the possibility of that different parameters of the request are transmitted by a SCI format 2-C and MAC CE [Intel,14]

  - Further consideration on modifying interpretation rule for TRIV [ASUSTeK,16]

- Finalization of behavior of UE-B receiving resource set(s) from UE-A(s)

  - UE-B’s behavior when UE-B receives multiple inter-UE coordination information from the same UE-A

    - UE-B uses the latest inter-UE coordination information in its resource selection

      - Supported by [Panasonic,5] [LGE,26](for preferred resource set) (2)

    - UE-B determines one of them by implementation to use in its resource selection

      - Supported by [LGE,26] (1)

  - UE-B’s behavior when UE-B receives multiple inter-UE coordination information from the different UE-As

    - For preferred resource set,

      - UE-B uses one inter-UE coordination information for each UE-A

        - Supported by [Panasonic,5] [DCM,9] (2)

      - UE-B uses multiple inter-UE coordination information in its resource selection

        - Supported by [Apple,15] [Samsung,20] (2)

      - UE-B determines one of them by implementation to use in its resource selection

        - Supported by [LGE,26] (1)

    - For non-preferred resource set,

      - UE-B uses multiple inter-UE coordination information in its resource selection

        - Supported by [Panasonic,5] [DCM,9] [Apple,15] [Samsung,20] [Qualcomm,22] (5)

      - UE-B determines one of them by implementation to use in its resource selection

        - Supported by [LGE,26] (1)

    - For preferred resource set and non-preferred resource set,

      - UE-B uses preferred resource later

        - Supported by [DCM,9] (1)

      - UE-B determines one of them by implementation to use in its resource selection

        - Supported by [LGE,26] (1)

      - Not supported by [Samsung,20] (1)

  - Further consideration on modifying UE-B’s resource selection procedure based on the received set of resources [Nokia,2] [vivo,4] [CATT,7] [Fujitsu,8] [ITL,25] (5)

    - [Nokia,2]: Overlapping portion dependent resource exclusion

    - [vivo,4]: Restrict maximum number of resource exclusion, change the definition of M_total

    - [CATT,7]: Additional candidate single-slot resource ratio

    - [Fujitsu,8] [ITL,25]: Canceling a subset of resource exclusion

  - Further clarification on the condition for using Option B [DCM,9] [Qualcomm,22] [Ericsson,27] (3)

    - [DCM,9]: UE that does not support sensing/resource exclusion, UE that supports sensing/resource exclusion but performs random selection for the corresponding transmission

    - [Qualcomm,22]: UE that supports sensing/resource exclusion but does not perform sensing/resource exclusion

    - [Ericsson,27]: UE that does not support sensing

  - Further consideration on specifying cast type(s) of UE-B’s transmission that can use inter-UE coordination information [CATT,7] [Qualcomm,22] [Mitsubishi,28] (3)

  - Further considering on specifying a condition of skipping the received inter-UE coordination information [Intel,14] [Sharp,23]

  - Further consideration on specifying format translation from the received set of resources to candidate single-slot resources [Intel,14]

- Finalization of when and with which information UE-A generates and/or transmits an inter-UE coordination information, including triggering based on condition(s) other than an explicit request

  - Sensing window for determining the set of resources

    - Sensing window prior to the transmission time (slot n) of UE-A’s inter-UE coordination information

      - Supported by [Huawei,1] [OPPO,6] [CATT,7] [Xiaomi,19] (4)

        - [n-T_0-T_proc,1, n-T_proc,0-T_proc,1]: [Huawei,1] [CATT,7] [Xiaomi,19] (3)

        - [n-T_0-T_ 3, n-T_proc,0-T_ 3]: [OPPO,6] (1)

    - Sensing window prior to the resource selection window for transmitting UE-A’s inter-UE coordination information

      - Supported by [Intel,14]

        - [?, n-T_proc,0-T_proc1] where n is the beginning of the resource selection window: [Intel,14]

    - No additional spec change is needed for sensing window for determining the set of resources

      - Supported by [LGE,26]

  - Further consideration on additional condition triggering inter-UE coordination information [Nokia,2] [Intle,14] [Samsung,20] [Ericsson,27] [Fraunhofer,30] (5)

- Finalization of when UE-B generates and/or transmits an explicit request

  - Further consideration on additional condition triggering an explicit request for inter-UE coordination information [vivo,4] [Intel,14] [NEC,18] [Ericsson,27] (4)

- Finalization of resource selection and/or multiplexing with sidelink transmissions for UE-A’s inter-UE coordination information and UE-B’s explicit request

  - Further consideration on additional restriction on inter-UE coordination information transmission [Intel,14] [Qualcomm,22] [Lenovo,24] [Ericsson,27] (4)

    - [Intel,14]: Resource selection window for inter-UE coordination information transmission is inside of a resource selection window for determining the set of resources

    - [Qualcomm,22] [Ericsson,27]: For inter-UE coordination information transmission without multiplexing with other data, retransmission is not supported

    - [Qualcomm,22]: For inter-UE coordination information transmission without multiplexing with other data, the number of subchanel is 1 and a remaining PDB is 8 slots

    - [Lenovo,24]: The ending time of a resource selection window for inter-UE coordination information transmission is not after the starting time of a resource selection window for determining the set of resources

  - Further consideration on multiplexing inter-UE coordination information, an explicit request, and data in a PSSCH [Intel,14]

  - Further consideration on updating UE-A’s resource (re)selection procedure for its transmission based on UE-A’s inter-UE coordination information [ASUSTeK,16]

  - Further consideration on dedicated resources for inter-UE coordination information transmission [ITL,25]

- Finalization of prioritization of inter-UE coordination information and explicit request

  - Further consideration on default priority value for inter-UE coordination information triggered by a condition rather than request reception [Huawei,1] [Futurewei,3] [DCM,9] [Intel,14] [CMCC,17] (5)

    - Up to UE-A’s implementation [Huawei,1] [Futurewei,3] [CMCC,17](for preferred resource set) (3)

    - Up to UE-A’s implementation with (pre)configured lower limit of priority value [Panasonic,5] (1)

    - Fixed to 8 [DCM,9] (1)

    - Same as priority value of indicated by other UE’s SCI [CMCC,17](for non-preferred resource set) (1)

    - Not supported by [OPPO,6] (1)

  - Scheme 2

- Finalization of determination of PSFCH resource/index for conflict indication

  - Frequency and code domain resources derived by

    - m_CS

      - 0

        - Supported by [Futurewei,3] [OPPO,6] [DCM,9] [Intel,14] [Samsung,20] [Qualcomm,22] [LGE,26] [Ericsson,27] [ZTE,29] (9)

          - [Intel,14]: it up to UE implementations whether/how to set the reservation period in the re-selected resource

          - [Qualcomm,22]: UE A sends PSFCH conflict indicator to UE B if a resource conflict is detected in the next SPS period

      - Based on target TB (0 for current TB, 6 for next TB(s))

        - Supported by [Huawei,1] [Nokia,2] [CATT,7] [InterDigital,10] [Spreadtrum,11] [Apple,15] (6)

    - m_0 determination based on PSFCH resource index

      - In the same way as specified in TS 38.213 Section 16.3

        - Supported by [Huawei,1] [Panasonic,5](when different PRB is used) [CATT,7] [DCM,9] [Spreadtrum,11] [Intel,14] [Apple,15] [Samsung,20](when the different PRB is used) [Qualcomm,22] [LGE,26] [Ericsson,27] [ZTE,29] (12)

      - A value of m_0 is (pre)configured

        - Supported by [Panasonic,5](when the same PRB is used) (1)

      - Circular offset is additionally applied to values of m_0 as specified in TS38.213 Section 16.3

        - Supported by [Samsung,20](when the same PRB is used) (1)

    - Case when the same PRB is used for both SL HARQ-ACK feedback and a resource conflict indication

      - Supported by [Futurewei,3] [Samsung,20] (2)

      - UE does not expect it [Huawei,1] [CATT,7] [Qualcomm,22] [ZTE,29] (4)

- Finalization of behavior of UE-B receiving a conflict indication from UE-A

  - Further consideration on UE-B’s behavior for handling a resource conflict in periodic reserved resources [Huawei,1] [Nokia,2] [CATT,7] [InterDigital,10] [Spreadtrum,11] [Apple,15] (6)

    - Not supported by [Futurewei,3] [OPPO,6] [DCM,9] [Intel,14] [Samsung,20] [LGE,26] [Ericsson,27] (7)

  - Further consideration on skipping the received resource conflict indication [Nokia,2] [OPPO,6] [Fujitsu,8] [Ericsson,27] (4)

  - Further consideration on specifying conditions to skip a transmission of a resource conflict indication [Nokia,2] [Fujitsu,8] [Intel,14] (3)

  - Further clarification on the next reserved resources subject to processing time budget [DCM,9] (1)

- Finalization of prioritization of conflict indication

  - Further consideration on modifying executing order of prioritization of PSFCH for a resource conflict [ETRI,13] [Apple,15] [Xiaomi,19] (3)

    - [ETRI,13] [Xiaomi,19]: PSFCH TX/TX or TX/RX prioritization is performed first

    - [Apple,15]: prioritization between PSFCH TX or RX and other channel(s) is performed first

  - Further consideration on modifying prioritization rule for PSFCH TX of SL HARQ-ACK feedback and a resource conflict indication [ETRI,13] (1)

  - Further consideration on the issue due to imbalanced prioritization between PSFCH TX and RX for a resource conflict indication [Apple,15] (1)

- Finalization of how to determine UE-B among UEs scheduling conflicting TBs, including whether/how to handle, or differently handle, the case when at least one of UEs scheduling conflicting TBs doesn’t support Scheme 2

  - Based on a second UE flag (i.e., whether UE scheduling a conflict TB can be UE-B or not)  indicated by UE-B’s SCI format 1-A: [Huawei,1] [Futurewei,3] [Panasonic,5] [OPPO,6] [CATT,7] [DCM,9] [InterDigital,10] [Apple,15] [Sharp,23] [LGE,26] (10)

    - UE pairing for selecting UE-B considers only UEs transmitting SCI format 1-A with Second UE flag of 1

      - Supported by [Huawei,1] [Panasonic,5] [OPPO,6] [CATT,7] [DCM,9] [LGE,26] (6)

    - Drop PSFCH TX when the selected UE-B does not support Scheme 2 after applying the existing WA for selecting UE-B

      - Supported by [Nokia,2] [Sharp,23] [Ericsson,27] (3)

    - At least one of UEs scheduling conflicting TBs does not support scheme 2, all other UEs transmitting SCI format 1-A with a second flag of 1 are UE-Bs

      - Supported by [Futurewei,3] [Apple,15] (2)

  - Based on whether PSFCH occasion(s) for resource conflict indication is passed or not: [Huawei,1] [OPPO,6] [Fujitsu,7] [LGE,26] (4)

    - UE pairing for selecting UE-B considers only UEs whose PSFCH occasions for a resource conflict indication are not yet passed

      - Supported by [Huawei,1] [OPPO,6] [Fujitsu,7] [LGE,26] (4)

  - Based on priority value of UE-B’s transmission [InterDigital,10] (1)

  - Further consideration on specifying cast type of UE-B’s transmission that can receive a resource conflict indication [Futurewei,3] [CATT,7] [Fujitsu,8] (3)

  - Further consideration on tie-breaking for the case when conflicting TBs have the same priority [Futurewei,3] [Fujitsu,8] (2)

    - Up to UE implementation [Intel,14]

- Others

  - Further restrict or expand on the condition to be UE-A and/or UE-B [Nokia,2] [vivo,4] [Intel,14] [Ericsson,27] [Mitsubishi,28] (5)

  - Further consideration on modifying condition for determining a resource conflict [Nokia,2] [Fujitsu,8] [Intel,14] [Lenovo,24] (4)

  - Further consideration on ID sharing mechanism between UE-A and UE-B [Nokia,2]

  - Further consideration on modifying signaling granularity of enabling/disabling/controlling inter-UE coordination scheme [vivo,4]

  - Further consideration on specifying executing order for the case when multiple UE-B’s reserved resources are collided [vivo,4]

  - Further consideration on ensuring the time difference between successive UE-B’s reserved resources fulfil the processing time budget [CATT,7]

  - Further clarification on UE-A’s behavior when the case when one of SCI(s) scheduling the same reserved resources does not fulfill the processing time budget [Fujitsu,8]

  - Further consideration on modifying re-evaluation/pre-emption procedure without using inter-UE coordination information [Intel,14]

  - Further consideration on modifying UE-B’s resource (re)selection procedure based on a SCI format 1-A [Qualcomm,22]

  - Further consideration on inter-UE coordination with mode 1 operation [Lenovo,24]

  - Further consideration on power-saving UE with inter-UE coordination information [Ericsson,27]

- Reference

- R1-2200964	Inter-UE coordination in sidelink resource allocation	Huawei, HiSilicon

- R1-2200981	Inter-UE coordination for Mode 2 enhancements	Nokia, Nokia Shanghai Bell

- R1-2200983	Discussion on techniques for inter-UE coordination	FUTUREWEI

- R1-2201112	Remaining issues on mode-2 enhancements	vivo

- R1-2201182	Inter-UE coordination for Mode 2 enhancements	Panasonic Corporation

- R1-2201255	Inter-UE coordination in mode 2 of NR sidelink	OPPO

- R1-2201336	Remaining issues on Inter-UE coordination for Mode 2 enhancements	CATT, GOHIGH

- R1-2201438	Discussion on inter-UE coordination for Mode 2 enhancements	Fujitsu

- R1-2201495	Remaining issues on sidelink resource allocation for reliability and latency	NTT DOCOMO, INC.

- R1-2201531	Discussions on remaining issues for Mode 2 inter-UE coordination	InterDigital, Inc.

- R1-2201558	Discussion on inter-UE coordination in sidelink resource allocation	Spreadtrum Communications

- R1-2201585	Discussion on inter-UE coordination for Mode 2 enhancements	Sony

- R1-2201617	Discussion on inter-UE coordination for Mode 2 enhancements	ETRI

- R1-2201716	Remaining opens of sidelink inter-UE coordination schemes	Intel Corporation

- R1-2201785	Remaining Issues of Inter-UE Coordination	Apple

- R1-2201820	Remaining issues on V2X mode 2 enhancements	ASUSTeK

- R1-2201874	Remaining issues on inter-UE coordination for mode 2 enhancement	CMCC

- R1-2201907	Discussion on mode 2 enhancements	NEC

- R1-2201920	Discussion on inter-UE coordination	Xiaomi

- R1-2202032	On Inter-UE Coordination for Mode2 Enhancements	Samsung

- R1-2202086	Discussion on Mode 2 enhancements	MediaTek Inc.

- R1-2202159	Reliability and Latency Enhancements for Mode 2	Qualcomm Incorporated

- R1-2202202	Discussion on inter-UE coordination for mode 2 enhancements	Sharp

- R1-2202231	Inter-UE coordination for Mode 2 enhancements	Lenovo, Motorola Mobility

- R1-2202245	Inter-UE coordination for mode 2 enhancements	ITL

- R1-2202253	Discussion on inter-UE coordination for Mode 2 enhancements	LG Electronics

- R1-2202263	Details on mode 2 enhancements for inter-UE coordination	Ericsson

- R1-2202356	Inter-UE coordination for enhanced resource allocation	Mitsubishi Electric RCE

- R1-2202377	Remaining issues on the inter-UE coordination	ZTE, Sanechips

- R1-2202483	Inter-UE coordination for Mode 2 enhancements	Fraunhofer HHI

- Appendix

  - Conclusions made in RAN1#103-e meeting

- Conclusion:

  - The schemes of inter-UE coordination in Mode 2 are categorized as being based on the following types of “A set of resources” sent by UE-A to UE-B:

    - UE-A sends to UE-B the set of resources preferred for UE-B’s transmission

        - e.g., based on its sensing result

    - UE-A sends to UE-B the set of resources not preferred for UE-B’s transmission

        - e.g., based on its sensing result and/or expected/potential resource conflict

    - UE-A sends to UE-B the set of resource where the resource conflict is detected

    - FFS: details of resource conflict, e.g., including type of resource conflict

    - FFS: details of sensing operation at UE-A side

    - FFS: which type(s) of resource set information is(are) beneficial/feasible to which cast type(s)

    - Note: these different types may be used in combination with each other

  - From RAN1 perspective, further study on the feasibility/benefit of inter-UE coordination is required

  - Send an LS to RAN plenary

    - Final LS in R1-2009841

- Conclusion:

  - For the schemes of inter-UE coordination identified as feasible/beneficial, at least the following aspects are further discussed.

    - How/when UE-A determines the contents of ”A set of resources”, including consideration of UL scheduling

    - When UE-A sends ”A set of resources” to UE-B, including which UE(s) sends it

    - How UE-A and UE-B are determined

    - How UE-A sends ”A set of resources” to UE-B, including container used for carrying it, implicitly or explicitly or both

    - How/when/whether UE-B receives “A set of resources” and takes it into account in the resource selection for its own transmission

    - How/whether to define the relationship between support/signaling of inter-UE coordination and cast type

  - Conclusions made in RAN1#104-e meeting

- Conclusion:

  - RAN1 concludes that the inter-UE coordination in Mode 2 is feasible, and is beneficial (e.g., reliability, etc.) compared to Rel-16 Mode 2 RA, and thus recommends specification of the feature.

    - The detailed observations can be found in the attachment of the LS

- Draft LS in R1-2102165, along with the attachment R1-2102166, is approved (with a typo fix)

  - Final LS in R1-2102168

  - Agreements made in RAN1#104bis-e meeting

- Agreement:

  - Support the following schemes of inter-UE coordination in Mode 2:

    - Inter-UE Coordination Scheme 1:

      - The coordination information sent from UE-A to UE-B is the set of resources preferred and/or non-preferred for UE-B’s transmission

        - FFS details including a possibility of down-selection between the preferred resource set and the non-preferred resource set, whether or not to include any additional information other than indicating time/frequency of the resources within the set in the coordination information

      - FFS condition(s) in which Scheme 1 is used

    - Inter-UE Coordination Scheme 2:

      - The coordination information sent from UE-A to UE-B is the presence of expected/potential and/or detected resource conflict on the resources indicated by UE-B’s SCI

        - FFS details including a possibility of down-selection between the expected/potential conflict and the detected resource conflict

      - FFS condition(s) in which Scheme 2 is used

- Agreement:

  - Study further to determine the conditions for UEs to be UE-A(s)/UE-B(s) for inter-UE coordination:

    - Details include applicable scenario(s)/inter-UE coordination scheme(s)

    - E.g., only UE(s) among the intended receiver(s) of UE-B can be a UE-A, any UE can be a UE-A, high-layer configured, etc.

      - Including the possibility of being subject to certain conditions and/or capability

- Agreement:

  - When UE-B receives the inter-UE coordination information from UE-A, consider at least one of the following options (with details FFS including possibly down-selecting/merging one or more of the options below, applicable scenario(s)/condition(s) for each option, UE behavior) for UE-B’s to take it into account in the resource (re)-selection for its own transmission

    - For scheme 1:

      - Option 1-1: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on both UE-B’s sensing result (if available) and the received coordination information

      - Option 1-2: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based only on the received coordination information

      - Option 1-3: UE-B’s resource(s) to be re-selected based on the received coordination information

      - Option 1-4: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on the received coordination information

    - For scheme 2:

      - Option 2-1: UE-B can determine resource(s) to be re-selected based on the received coordination information

      - Option 2-2: UE-B can determine a necessity of retransmission based on the received coordination information

  - Agreements made in RAN1#106-e meeting

- Agreement:

  - For scheme 1, the following inter-UE coordination information signalling from UE-A is supported. FFS details including condition(s)/scenario(s) under which each information is enabled to be sent by UE-A and used by UE-B.

    - Set of resources preferred for UE-B’s transmission

    - Set of resources non-preferred for UE-B’s transmission

- Agreement:

  - For scheme 2, the following inter-UE coordination information signalling from UE-A is supported. FFS details including condition(s)/scenario(s) under which each information is enabled to be sent by UE-A and used by UE-B

    - Presence of expected/potential resource conflict on the resources indicated by UE-B’s SCI

      - FFS: UE behaviour when the presence of expected/potential resource conflict is detected by the transmitter

    - FFS: Whether to additionally support the presence of detected resource conflict on the resources indicated by UE-B’s SCI

- Agreement:

  - In scheme 1, the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination information transmission triggered by an explicit request in Mode 2:

    - A UE that sends an explicit request for inter-UE coordination information can be UE-B

    - A UE that received an explicit request from UE-B and sends inter-UE coordination information to the UE-B can be UE-A

    - (Working assumption) At least a destination UE of a TB transmitted by UE-B can be UE A

    - The above feature can be enabled or disabled or controlled by (pre-)configuration

      - FFS: Details on how to support this, including (pre-)configuration signaling granularity

    - FFS: Additional details and conditions on UE-A and UE-B

  - (Working Assumption) In scheme 1, the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination information transmission triggered by a condition other than explicit request reception in Mode 2:

    - A UE that satisfies the condition mentioned in the main bullet and sends inter-UE coordination information is UE-A

    - A UE that received inter-UE coordination information from UE-A and uses it for resource (re-)selection is UE-B

    - The above feature can be enabled or disabled or controlled by (pre-)configuration

      - FFS: Details on how to support this, including (pre-)configuration signaling granularity

    - FFS: Additional details and conditions on UE-A and UE-B

- Agreement:

  - In scheme 2, at least the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination transmission triggered by a detection of expected/potential resource conflict(s) in Mode 2:

    - A UE that transmitted PSCCH/PSSCH with SCI indicating reserved resource(s) to be used for its transmission, received inter-UE coordination information from UE-A indicating expected/potential resource conflict(s) for the reserved resource(s), and uses it to determine resource re-selection is UE-B

    - A UE that detects expected/potential resource conflict(s) on resource(s) indicated by UE-B’s SCI sends inter-UE coordination information to UE-B, subject to satisfy one of the following conditions, is UE-A

      - (Working assumption) At least a destination UE of one of the conflicting TBs, i.e., TBs to be transmitted in the expected/potential conflicting resource(s)

        - Whether a non-destination UE of a TB transmitted by UE-B can be UE-A is (pre-)configured

      - FFS: Additional details and condition(s) on UE-A and UE-B

    - The above feature can be enabled or disabled or controlled by (pre-)configuration

      - FFS: Details on how to support this, including (pre-)configuration signaling granularity

    - FFS: Definition of expected/potential resource conflict(s) and other details (if any)

- Agreement:

  - In scheme 2, the following UE-B’s behavior in its resource (re)selection is supported when it receives inter-UE coordination information from UE-A:

    - UE-B can determine resource(s) to be re-selected based on the received coordination information

      - UE-B can reselect resource(s) reserved for its transmission when expected/potential resource conflict on the resource(s) is indicated

        - FFS: Other details (if any)

- Agreement:

  - In scheme 1, at least following UE-B’s behavior in its resource (re-)selection is supported when it receives inter-UE coordination information from UE-A:

    - For preferred resource set, the following two options are supported:

      - Option A): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information

        - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set in combination with its own sensing result

          - UE-B uses in its resource (re-)selection, resource(s) not belonging to the preferred resource set when condition(s) are met

            - FFS: Details of condition(s)

          - This option is supported when UE-B performs sensing/resource exclusion

          - FFS: Other details (if any)

      - Option B): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based only on the received coordination information

        - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set

          - This option is supported at least when UE-B does not support sensing/resource exclusion

            - FFS: Whether the support is conditional or UE capability

          - FFS: Other details (if any)

      - FFS: Other option(s), and other details (if any)

    - For non-preferred resource set,

      - UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information

        - UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set

          - FFS: Details including

            - Whether/how UE-B can use in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set, definition of the overlap, and other details (if any)

            - When UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set

        - FFS: UE-B reselects in its resource (re-)selection, resource(s) to be used for its transmission when the resource(s) are fully/partially overlapping with the non-preferred resource set

      - FFS: Other option(s), and other details (if any)

- Agreement:

  - In scheme 2, at least the following is supported to determine inter-UE coordination information:

    - Among resource(s) indicated by UE-B’s SCI, UE-A considers that expected/potential resource conflict occurs on the resource(s) satisfying at least one of the following condition(s):

      - Condition 2-A-1:

        - Other UE’s reserved resource(s) identified by UE-A are fully/partially overlapping with resource(s) indicated by UE-B’s SCI in time-and-frequency

        - FFS: Other details (if any)

        - FFS: Whether/how to specify additional criteria and other details (if any) including signaling details of conflict indication

      - (Working Assumption) Condition 2-A-2:

        - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

          - FFS: Other details (if any)

      - FFS: Other condition(s)

    - FFS: Other details (if any)

- Agreement:

  - In scheme 1, at least the following is supported to determine inter-UE coordination information of preferred resource set:

    - UE-A considers any resource(s) satisfying all the following condition(s) as set of resource(s) preferred for UE-B’s transmission

      - Condition 1-A-1:

        - Resource(s) excluding those overlapping with reserved resource(s) of other UE identified by UE-A whose RSRP measurement is larger than a RSRP threshold

          - FFS: Other details (if any)

      - FFS: Condition 1-A-2:

        - Resource(s) excluding slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B

          - FFS: Other details (if any)

      - FFS: Condition 1-A-3:

        - Resource(s) satisfying UE-B’s traffic requirement (if available)

          - FFS: Other details (if any)

      - FFS: Other condition(s)

    - FFS: Other details (if any)

- Agreement:

  - In scheme 1, at least the following is supported to determine inter-UE coordination information of non-preferred resource set:

    - UE-A considers any resource(s) satisfying at least one of the following condition(s) as set of resource(s) non-preferred for UE-B’s transmission

      - Condition 1-B-1:

        - Reserved resource(s) of other UE identified by UE-A from other UEs’ SCI (including priority field) and RSRP measurement

          - FFS: Other details (if any)

      - FFS: Condition 1-B-2:

        - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B

          - FFS: Other details (if any)

      - FFS: Other condition(s)

    - FFS: Other details (if any)

  - Agreements made in RAN1#106bis-e meeting

- Agreement:

  - For Scheme 2, PSFCH format 0 is used to convey the presence of expected/potential resource conflict on reserved resource(s) indicated by UE-B’s SCI

- Agreement:

  - For Condition 2-A-1 of Scheme 2, down-select one or more of following additional criteria to determine resource(s) where expected/potential resource conflict occurs

    - Option 1: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a RSRP threshold according to the priorities included in the SCI:

      - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations

      - Strive to reuse Rel-16 specification wherever possible

    - Option 2: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is within a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

      - FFS: Whether the threshold depends on priority

    - Option 3: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) and the other UE is within a distance threshold of UE-B as determined by both UEs’ SCIs.

    - Option 4: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

      - FFS: Whether the threshold depends on priority

    - FFS: In case of collisions of resources for two UEs having TBs with UE A as destination UE, if needed

- Working Assumption

  - For Condition 1-B-1 of Scheme 1, the following two options are supported

    - Option 1: Reserved resource(s) of other UE(s) identified by UE-A whose RSRP measurement is larger than a (pre)configured RSRP threshold which is determined by at least priority value indicated by SCI of the UE(s)

    - Option 2: Reserved resource(s) of other UE identified by UE-A whose RSRP measurement is smaller than a (pre)configured RSRP threshold which is determined by at least priority value indicated by SCI of the UE(s) when UE-A is a destination of a TB transmitted by the UE(s)

- Working Assumption

  - For Scheme 1 with non-preferred resource set, support following condition:

    - Condition 1-B-2:

      - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

- Agreement:

  - For Condition 1-A-1 of Scheme 1, the set of resources preferred for UE-B’s transmission is a form of candidate single-slot resource as specified in Rel-16 TS 38.214 Section 8.1.4

    - When the inter-UE coordination information transmission is triggered by UE-B’s explicit request, the candidate single-slot resource(s) are determined in the same way according to Rel-16 TS 38.214 Section 8.1.4 with at least following parameters provided by signaling from UE-B. FFS whether or not to apply RSRP threshold increase in Step 7) of Rel-16 TS 38.214 Section 8.1.4.

      - Priority value to be used for PSCCH/PSSCH transmission

        - It replaces prio_TX

      - Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot

        - It replaces L_subCH

      - Resource reservation interval

        - It replaces P_rsvp_TX

      - FFS: Starting/ending time location of resource selection window

    - FFS : In addition to Rel-16 procedure, use inter-UE coordination information from other UEs

      - If there is no consensus in RAN1#106bis-e, no further discussions for Rel-17

- Conclusion:

  - No consensus that UE-A uses inter-UE coordination information from other UEs when it determines the preferred resource set for Condition 1-A-1 of Scheme 1.

- Working Assumption

  - For Scheme 1 with preferred resource set, support following condition:

    - Condition 1-A-2:

      - Resource(s) excluding slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

      - This can be disabled by RRC (pre-)configuration

- Agreement:

  - For allocating PSFCH resources in Scheme 2, at least following can be (pre)configured separately from those for SL HARQ-ACK feedback.

    - Set of PRBs for PSFCH transmission/reception (sl-PSFCH-RB-Set)

- Agreement:

  - For Scheme 2,

    - Index of a PSFCH resource for inter-UE coordination information transmission is determined in the same way according to Rel-16 TS 38.213 Section 16.3 with at least following modification

      - P_ID is L1-Source ID indicated by UE-B’s SCI

      - M_ID is 0

    - FFS: How to set m_CS

    - FFS: How to set m_0

    - FFS: Whether M_ID can be (pre)configured

  - Agreements made in RAN1#107-e meeting

- Agreement:

  - A resource pool level (pre-)configuration uses either of the following options

    - Option 1: PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted

      - Reuse PSSCH-to-PSFCH timing as specified in TS 38.213 Section 16.3 to determine the PSFCH occasion for resource conflict indication

      - Time gap between the PSFCH and a slot where expected/potential resource conflict occurs is larger than or equal to T_3

    - Option 2: PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI

      - UE-A transmits the PSFCH in a latest slot that includes PSFCH resources for inter-UE coordination information and is at least T_3 slots of the resource pool before the PSSCH resource indicated by UE-B’s SCI in which expected/potential resource conflict occurs

      - FFS: How to account for processing timeline

    - Note that it is possible not to configure either option1 or option 2.

- Agreement:

  - For Condition 1-A-2 of Scheme 1, the set of resources preferred for UE-B’s transmission is a form of candidate single-slot resource as specified in Rel-16 TS 38.214 Section 8.1.4

    - UE-A excludes candidate single-slot candidate(s) belonging to “slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation” after Step 6) of TS 38.214 Section 8.1.4

- Agreement:

  - When PSFCH TX/RX for Scheme 2 is overlapping with LTE SL TX/RX and/or UL in a UE, reuse prioritization rule as specified in TS 38.213 Section 16.2.4.1 and 16.2.4.3.1.

- Conclusion:

  - For Scheme 2, the values of the following parameters are the same as those for SL HARQ-ACK feedback in the same resource pool

    - Period of PSFCH resources (sl-PSFCH-Period)

    - Number of cyclic shift pairs used for a PSFCH transmission that can be multiplexed in a PRB (sl-NumMuxCS-Pair)

    - Number of PSFCH resources available for multiplexing information in a PSFCH transmission (sl-PSFCH-CandidateResourceType)

- Agreement:

  - For Scheme 1, a resource pool level (pre-)configuration can enable one of the following alternatives:

    - Alt 1 (Working Assumption): MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B.

      - For the indication of resource set, the following is supported:

        - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

          - First resource location of each TRIV is separately indicated by the inter-UE coordination information

        - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used.

          - FFS: UE capability details

          - 2nd SCI is UE RX optional

    - Alt 2: MAC CE is used as the container of inter-UE coordination information transmission from UE A to UE B.

      - For the indication of resource set, the following is supported:

        - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

          - First resource location of each TRIV is separately indicated by the inter-UE coordination information

    - FFS: Whether/How to use resource reservation information as coordination information

- Working Assumption:

  - A resource pool level (pre-)configuration can enable one of the following options:

    - Option 1:

      - For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs

        - For the case when UE-A is a destination UE of a TB transmitted by UE-B

          - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a RSRP threshold according to the priorities included in the SCI:

            - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations for UE-B and other UE respectively

        - For the case when UE-A is a destination UE of a TB transmitted by another UE

          - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a RSRP threshold according to the priorities included in the SCI:

            - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations for other UE and UE-B respectively

    - Option 4:

      - For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs

        - For the case when UE-A is a destination UE of a TB transmitted by UE-B

          - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

        - For the case when UE-A is a destination UE of a TB transmitted by another UE

          - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of the resource(s).

      - Support of Option 4 is subject to UE capability

    - FFS: Whether/how RSRP threshold depends on priority, MCS, overlap

- Agreement:

  - For Scheme 1 with non-preferred resource set,

    - Physical layer at UE-B excludes in its resource (re-)selection, candidate single-slot resource(s) obtained after Step 6) of Rel-16 TS 38.214 Section 8.1.4 overlapping with the non-preferred resource set

- Agreement:

  - For Condition 1-A-1 of Scheme 1, when UE-A determines the set of resources preferred for UE-B’s transmission, apply RSRP threshold increase in the same way according to Rel-16 TS 38.214 Section 8.1.4.

    - FFS: Whether/how to introduce the maximum limit of RSRP threshold increase

- Agreement:

  - For Scheme 1, at least following parameters are provided by UE-B’s request:

    - Priority value to be used for PSCCH/PSSCH transmission

    - Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot

    - Resource reservation interval

- Agreement:

  - For Scheme 2, when PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI,

    - Time gap between the PSFCH and SCI(s) scheduling conflicting TBs is larger than or equal to X value.

      - FFS: Details of X

- Working Assumption:

  - For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A, for each pair of UEs scheduling the conflicting TBs, a UE with the higher priority value is UE-B.

    - FFS whether/how to set additional condition for UE-A to send PSFCH.

    - Conclude on whether/how to handle, or differently handle, the case when at least one of UEs scheduling conflicting TBs doesn’t support Scheme 2 at the subsequent meetings

- Agreement:

  - For inter-UE coordination information triggered by an explicit request in Scheme 1,

    - UE-A uses a TX resource pool used for UE-B’s request transmission to determine the set of resources and to transmit the set of resources to UE-B

- Agreement:

  - For inter-UE coordination information triggered by a condition rather than request reception in Scheme 1,

    - UE-A transmitting in a resource pool provides inter-UE coordination information associated with the same resource pool

  - Agreements made in RAN#94-e meeting

- Agreement:

  - RAN1 is tasked to complete the remaining normative work for Rel-17 NR sidelink enhancement by Q1 of 2022

    - All RAN1 decisions that impact other WGs should be finalized in RAN1#107bis-e

  - Use the list of open issues provided RP-212880 (status report of WI: NR sidelink enhancement) as a starting point for technical discussions in RAN1.

    - This does not mean that all the issues included in the list are considered essential or the list is complete

    - RAN1 should not spend additional effort to further refine the list

  - Agreements made in RAN1#107bis-e meeting

- Agreement:

  - For Scheme 1, when the inter-UE coordination information transmission is triggered by UE-B’s explicit request,

    - Starting/Ending time locations of resource selection window is provided by UE-B’s explicit request

      - Starting/Ending time locations of resource selection window is a form of combination of DFN index and slot index

- Agreement:

  - When PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI, time gap between the PSFCH and SCI(s) scheduling conflicting TBs is larger than or equal to X value

    - X = sl-MinTimeGapPSFCH

  - UE does not transmit the conflict indicator or receive the conflict indicator if the timeline is not satisfied

- Agreement:

  - For Scheme 1, a resource pool level (pre-)configuration can enable one of the following alternatives:

    - (Working assumption) Alt1: MAC CE and 2nd SCI are used as the container of an explicit request transmission from UE-B to UE-A

      - A single format SCI 2-C is used for inter-UE coordination information and request

        - 1 bit in format 2-C is used to indicate whether the SCI is used for request to coordination information or for conveying coordination information

      - SCI 2-C is UE RX optional

      - It is up to UE implementation to additionally use 2nd SCI (for UE-B).

    - Alt2: MAC CE is used as the container of an explicit request transmission from UE-B to UE-A

- Conclusion:

  - For Scheme 2, there is no consensus to support indication of the following

    - Condition type of a resource conflict

    - Time location of a resource conflict

- Agreement:

  - Alt 2-1

    - For Scheme 2,

      - The PHY layer reports S_A after Step 7) of TS 38.214 Section 8.1.4 to higher layer.

      - When UE-B receives a conflict indicator for resource(s) indicated by its SCI,

        - PHY layer at UE-B reports resources overlapping with the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

          - If (pre)configured, the PHY layer reports resources in a slot including the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

        - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources.

      - FFS: Whether/How the conflict in periodic transmission is indicated by UE-A and handled by UE-B

- Agreement:

  - For PSFCH TX/RX or TX/TX prioritization in Scheme 2,

    - Priority value of PSFCH TX for a resource conflict indication is the smallest priority value of the conflicting TBs

    - Priority value of PSFCH RX for a resource conflict indication is priority value indicated by UE-B’s SCI

    - For PSFCH TX/RX or TX/TX prioritization between SL HARQ-ACK feedback(s) and resource conflict indication(s), PSFCH TX/RX for SL HARQ-ACK feedback is always prioritized over PSFCH TX/RX for a resource conflict indication

- Agreement:

  - For Scheme 1, unicast is supported for an explicit request transmission for inter-UE coordination information

    - Unicast is used for the inter-UE coordination information transmission triggered by the explicit request

- Working Assumption:

  - For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception

    - Groupcast/Broadcast for non-preferred resource set, FFS for preferred resource set

      - FFS: Under which conditions groupcast/broadcast can be supported

    - Unicast

      - FFS: Under which conditions unicast can be supported

- Agreement:

  - For determining preferred resource set in Scheme 1, the value of Cresel is determined by UE-A according to Rel-16 procedure.

    - This information is not conveyed to/from UE-B

    - When inter-UE coordination information is triggered by UE-B’s request, P_rsvp_TX used for determining SL_RESOURCE_RESELECTION_COUNTER according to Rel-16 procedure is provided by resource reservation interval indicated by UE-B’s request

- Agreement:

  - For the indication of resource set in Scheme 1, the value of Sl-MaxNumPerReserve is fixed to 3.

- Agreement:

  - The following working assumption is confirmed with modification in RED.

    - MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B.

      - For the indication of resource set, the following is supported:

        - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

          - First resource location of each TRIV is separately indicated by the inter-UE coordination information

        - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used.

          - FFS: UE capability details

          - 2nd SCI is UE RX optional

          - The field size of the indication of resource set in a SCI format 2-C is determined by [N=3]

- Agreement:

  - For inter-UE coordination information transmission in Scheme 1,

    - Inter-UE coordination information can be multiplexed with other data only if the source/destination ID pair is the same

      - Retransmission of the TB carrying inter-UE coordination information is supported

  - For explicit request transmission in Scheme 1,

    - Explicit request can be multiplexed with other data only if the source/destination ID pair is the same

      - Retransmission of the TB carrying request is supported

- Agreement:

  - For inter-UE coordination triggered by an explicit request in Scheme 1, whether or not to transmit the inter-UE coordination information upon the request reception is determined by UE-A’s implementation subject to the following procedures.

    - Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control

- Agreement:

  - For inter-UE coordination triggered by a condition rather than request reception in Scheme 1,

    - A resource pool level (pre-)configuration can enable one of the following alternatives:

      - Alt 1: it is up to UE-A’s implementation whether or not to trigger the inter-UE coordination information generation.

      - Alt 2: the inter-UE coordination information generation can be triggered only when UE-A has data to be transmitted together with the inter-UE coordination information to UE-B

    - Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the inter-UE coordination information triggered by a condition.

- Agreement:

  - For inter-UE coordination triggered by UE-B’s explicit request in Scheme 1,

    - A resource pool level (pre-)configuration can enable one of the following alternatives:

      - Alt 1: it is up to UE-B’s implementation whether or not to trigger the request generation

      - Alt 2: the request generation can be triggered only when UE-B has data to be transmitted to UE-A

    - Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the request transmission.

- Agreement:

  - For Scheme 1 with preferred resource set Option A,

    - MAC layer selects resources using S_A and the received preferred resource set

      - MAC layer firstly selects resources for transmissions within the intersection of S_A and the preferred resource set until it becomes impossible to select a resource within the intersection under the constraint defined in Rel-16.

        - It is up to the UE whether to use the preferred resource set from SCI format 2-C and/or MAC CE

      - After this, if the number of selected resources is smaller than the required number of transmissions for a TB, MAC layer selects resources for the remaining transmissions outside the intersection but inside S_A under the constraint defined in Rel-16.

- Agreement:

  - For Scheme 1 with preferred resource set Option B,

    - MAC layer selects resources belonging to the received preferred resource set under the constraint defined in Rel-16

      - It is up to the UE whether to use the preferred resource set from SCI format 2-C and/or MAC CE

- Agreement:

  - For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as indicated by UE-B’s explicit request.

    - For the case when inter-UE coordination information is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the inter-UE coordination information and data

- Agreement:

  - For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of explicit request is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as that of a TB to be transmitted by UE-B.

    - For the case when the explicit request is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the explicit request and data

- Agreement:

  - For inter-UE coordination information triggered by a condition other than explicit request reception in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration.

    - FFS: Otherwise, the priority value is determined by UE-A’s implementation.

    - For the case when inter-UE coordination information is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the inter-UE coordination information and data

- Agreement:

  - For sidelink transmission carrying inter-UE coordination information in Scheme 1,

    - UE-A performs its resource (re)selection according to the same procedure in TS 38.214 Section 8.1.4 to transmit the inter-UE coordination information to UE-B.

  - For sidelink transmission carrying request in Scheme 1,

    - UE-B performs its resource (re)selection according to the same procedure in TS 38.214 Section 8.1.4 to transmit the request for the inter-UE coordination information to UE-A if UE-B performs sensing/resource exclusion. Otherwise, at least UE-B can perform random selection

  - Note: RAN1 does not pursue specific enhancement of Rel-17 resource (re)selection for the transmission of inter-UE coordination information and its request.

- Working assumption:

  - First resource location of each TRIV is a slot offset with respect to a reference slot

    - Alt 2:

      - The slot offset is the number of logical slots from the reference slot

        - The value range of slot offsets is from 0 to maximum value that is (pre)configurable up to [256]

          - FFS: The detailed value range including granularity

        - Slot offset for each TRIV to indicate the set of resources is separately indicated by inter-UE coordination information

    - For the reference slot,

      - The reference slot is the slot indicated by the inter-UE coordination information in a form of combination of DFN index and slot index

- Agreement:

  - For determining preferred resource set in Scheme 1, when inter-UE coordination information transmission is triggered by a condition other than explicit request reception,

    - Values of following parameters are (pre)configured for a resource pool. If there is no (pre)configuration, UE-A determines by its implementation the values of the following parameters

      - prio_TX

      - L_subCH

      - P_rsvp_TX

    - UE-A determines by its implementation values of following parameters

      - n+T_1, n+T_2

    - FFS: Whether/how to support (pre)configuration of n+T_1 and n+T_2

    - Note that it is up to RAN2 decision whether/how the values of these parameters are provided by PC5-RRC signaling from UE-B to UE-A and UE-A uses the received information to determine the preferred resource set

- Agreement:

  - For inter-UE coordination information is triggered by UE-B’s request,

    - A resource pool level (pre-)configuration can enable one of the following alternatives:

      - Alt 1:

        - Resource set type to be provided by inter-UE coordination information transmission is determined by UE-A’s implementation and its information is indicated by UE-A’s inter-UE coordination information

          - UE-A’s inter-UE coordination information indicates either preferred resource set or non-preferred resource set

      - Alt 2:

        - Resource set type to be provided by inter-UE coordination information transmission is indicated by UE-B’s request

          - UE-B’s request indicates either preferred resource set or non-preferred resource set

    - Note that it is up to RAN2 decision whether/how UE-B provides its support of sensing/resource exclusion to UE-A via PC5-RRC signaling and UE-A uses the received information to determine the type of resource set to be transmitted to UE-B

- Agreement:

  - For inter-UE coordination information is triggered by a condition other than explicit request reception,

    - Resource set type to be provided by inter-UE coordination information transmission is determined by UE-A’s implementation and its information is indicated by UE-A’s inter-UE coordination information

      - UE-A’s inter-UE coordination information indicates either preferred resource set or non-preferred resource set

- Working assumption:

  - For Scheme 2, (pre)configuration is supported to enable or disable that 1 LSB of reserved bits of a SCI format 1-A is used to indicate of whether UE scheduling a conflict TB can be UE-B or not.

    - FFS: UE-A's behavior for the case when at least one of UEs scheduling conflicting TBs is not capable of receiving the conflict indication

  - Agreements made in RAN1#108-e meeting

- Agreement:

  - For a slot offset that is (pre)configured to indicate the first resource location of each TRIV with respect to a reference slot,

    - Granularity of the slot offset is 1 logical slot

    - (Pre)configured maximum value of the slot offset is up to 8000

      - When both SCI format 2-C and MAC CE are used as the container of inter-UE coordination information, the maximum value of the slot offset is 255

      - When MAC CE only is used as the container of inter-UE coordination information, the maximum value of the slot offset is the (pre)configured maximum value

- Agreement:

  - A SCI format 2-C includes all the fields present in SCI format 2-A except cast type indicator

- Conclusion:

  - For cast type(s) of inter-UE coordination information with preferred resource set triggered by a condition other than explicit request reception

    - There is no consensus in RAN1 on the support of groupcast or broadcast for preferred resource set

- Agreement

  - For Scheme 2,

    - m_CS for a resource conflict indication for the next reserved resource indicated by the corresponding UE-B’s SCI for either current TB transmission or next TB transmission is 0

- Agreement

  - For Scheme 2, when UE-B receives a conflict indicator for resource(s) indicated by its SCI, it up to UE-B’s implementation whether/how to set the reservation periodicity in the re-selected resource.

- Agreement

  - For Scheme 2,

    - m_0 for a resource conflict indication is derived in the same way as specified for HARQ-ACK information in TS 38.213 Section 16.3

    - A UE expects that different PRBs are (pre)configured between conflict indication and HARQ-ACK information

- Agreement:

  - For Scheme 1, when both SCI format 2-C and MAC CE are used as the container of an explicit request for inter-UE coordination information, the same bit field size for the request in a SCI format 2-C is applied to MAC CE

- Agreement:

  - For Scheme 1, when MAC CE only is used as the container of an explicit request for inter-UE coordination information, the same bit field size for the request in a SCI format 2-C is applied to MAC CE

- Conclusion:

  - For inter-UE coordination operation in Rel-17, RAN1 understands that only UE(s) in mode 2 can be UE-A

    - Note that RAN1 does not pursue specific enhancement of Rel-17 inter-UE coordination operation for handling the case where UE(s) in mode 1 can be UE-A

- Agreement

  - Confirm the following working assumption with modification in RED

    - Working assumption made in RAN1#107bis-e:

      - First resource location of each TRIV is a slot offset with respect to a reference slot

        - Alt 2:

          - The slot offset is the number of logical slots from the reference slot

            - The value range of slot offsets is from 0 to maximum value that is (pre)configurable up to [8000256]

              - FFS: The detailed value range including granularity

          - Slot offset for each TRIV except for first TRIV to indicate the set of resources is separately indicated by inter-UE coordination information

            - Slot offset for first TRIV is 0

        - For the reference slot,

          - The reference slot is the slot indicated by the inter-UE coordination information in a form of combination of DFN index and slot index

- Agreement

  - MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B.

    - For the indication of resource set, the following is supported:

      - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

        - First resource location of each TRIV is separately indicated by the inter-UE coordination information

      - If N <= 2, MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If N > 2, only MAC CE is used.

        - FFS: UE capability details

        - 2nd SCI is UE RX optional

        - The field size of the indication of resource set in a SCI format 2-C is determined by N=2

- Agreement

  - For Scheme 1, each bit field size of a SCI format 2-C for inter-UE coordination information is given by following table:

    - Note that lowest subchannel index for the first resource location of each TRIV is separately indicated by inter-UE coordination information

| Field name | Field size (in bits) |
| --- | --- |
| Providing/requesting indicator | 1 |
| Resource combination(s) | 2*log2(N subchannel SLNsubchannel SL + 12Nsubchannel SL + 16)+9+Y Where N subchannel SL is provided by the higher layer parameter sl-NumSubchannel, Y=log2Nrsv_period with that Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResource is configured; Y=0 otherwise. |
| First resource location(s) | 8 |
| Reference slot location | 10+log2(10∙2μ) Where μ is 0, 1, 2, 3 for SCS of 15kHz, 30kHz, 60kHz, 120kHz, respectively. |
| Resource set type | 1 |
| Lowest subchannel indices for the first resource location of each TRIV | 2*log2NsubchannelSL where NsubchannelSL is provided by the higher layer parameter sl-NumSubchannel |
| (FFS) Actual number of resource combination | 1 Note: Support of this field is to be concluded by Feb 28. |

- Agreement

  - For Scheme 1, each bit field size of a SCI format 2-C for an explicit request for inter-UE coordination information is given by following table:

| Field name | Field size (in bits) |
| --- | --- |
| Providing/requesting indicator | 1 |
| Priority | 3 |
| Number of subchannels | log2(N subChannel SL) Where N subChannel SL is provided by the higher layer parameter sl-NumSubchannel |
| Resource reservation period | Y Where Y=log2Nrsv_periodwith that Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResoure is configured; Y=0 otherwise. |
| Resource selection window location | 210+log2(10∙2μ) Where μ is 0, 1, 2, 3 for SCS of 15kHz, 30kHz, 60kHz, 120kHz, respectively. |
| Resource set type | 1 bit if determineResourceSetTypeScheme1 is set to ‘UE-B’s request’, otherwise, 0 bit |

  - This agreement does not imply that new field requested by RAN2 cannot be further added.

- Agreement

  - For Scheme 1, when MAC CE only is used as the container of inter-UE coordination information, each bit field size for inter-UE coordination information is given by following table from RAN1’s perspective, and RAN1 understands that the maximum value of N resource combinations to be conveyed in inter-UE coordination information is bounded so that the total payload size of inter-UE coordination information leads not to exceed the size of TB including the MAC CE

    - Details (e.g., whether/how to separately indicate the value of N in the inter-UE coordination information, how to put the following fields into MAC CE and the related field sizes in MAC CE) are up to RAN2

| Field name | Field size (in bits) |
| --- | --- |
| Providing/requesting indicator | 1 |
| Resource combination(s) | N*log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16)+9+Y Where N subChannel SL is provided by the higher layer parameter sl-NumSubchannel, Y=log2Nrsv_periodwith that Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResoure is configured; Y=0 otherwise. |
| First resource location(s) | N-1*log2(X) Where X is provided by the (pre)configured maximum value of slot offset for the case when MAC CE only is used as a container of inter-UE coordination information |
| Reference slot location | 10+log2(10∙2μ) Where μ is 0, 1, 2, 3 for SCS of 15kHz, 30kHz, 60kHz, 120kHz, respectively. |
| Resource set type | 1 |
| Lowest subchannel indices for the first resource location of each TRIV | N*log2NsubchannelSL Where NsubchannelSL is provided by the higher layer parameter sl-NumSubchannel. |

- Conclusion:

  - There is no consensus in RAN1 on indicating actual number of resource combination in a SCI format 2-C for inter-UE coordination information.

    - Note: Different resource combinations can indicate the same set of resources for the case when only one resource combination is actually used

- Agreement

  - For Scheme 2,

    - The PHY layer reports S_A after Step 7) of TS 38.214 Section 8.1.4 to higher layer.

    - When UE-B receives a conflict indicator for resource(s) indicated by its SCI,

      - PHY layer at UE-B reports resources overlapping with the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

        - If (pre)configured, the PHY layer reports resources in a slot including the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

      - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources.

- Agreement

  - Confirm the following working assumption with modification in RED/BLUE. Note that the terminology of “indicationUEB flag” means the indication of whether UE scheduling a conflict TB can be UE-B or not.

    - Working Assumption:

      - For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A,

        - for each pair of UEs scheduling the conflicting TBs whose PSFCH occasions for resource conflict indication are not yet passed and indicationUEB flag is set to 1 if the higher parameter of indicationUEBScheme2 is (pre)configured to ‘Enabled’, a UE with the higher priority value is UE-B. When the UEs in the pair have the same priority value, UE-A determines one of the UEs to be UE-B by its implementation.

          - UE-A considers the SCIs received earlier than or equal to sl-MinTimeGapPSFCH before the PSFCH occasion for conflict indication when determining UE-B.

- Agreement

  - A UE performs PSFCH TX/RX or TX/TX prioritization between SL HARQ-ACK feedback(s) and resource conflict indication(s) first, and then the UE performs prioritization between prioritized PSFCH TX(s) or RX(s) and LTE SL TX/RX or UL by reusing prioritization rule as specified in TS 38.213 Section 16.2.4.1 and 16.2.4.3.1.

- Conclusion:

  - RAN1 does not pursue specific enhancement of Rel-17 inter-UE coordination operation for handling the overlapping between UL with SL-HARQ-ACK information and PSFCH for a conflict indication, i.e., there is no case in Rel-17 where the overlapping between UL with SL-HARQ-ACK information and PSFCH for a conflict indication occur at a UE performing inter-UE coordination operation

- Conclusion:

  - There is no consensus in RAN1 to further introduce enhancement in Rel-17 on Mode 2 resource selection procedure to ensure the timeline (i.e., minimum time gap between PSFCH and a slot where a SCI is transmitted of sl-MinTimeGapPSFCH, minimum time gap between PSFCH and a slot where expected/potential resource conflict occurs on PSSCH resource indicated by a SCI of T_3) for a conflict indication.

- Agreement

  - For Scheme 1, when both SCI format 2-C and MAC CE are used as the container of inter-UE coordination information, the same inter-UE coordination information is indicated in the SCI format 2-C and the MAC CE

    - Details (e.g., how to put the fields of SCI format 2C for inter-UE coordination information into MAC CE and the related field sizes in MAC CE) are up to RAN2

- Conclusion:

  - When PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted,

    - if there is a PSFCH occasion satisfying “the minimum time gap (sl-MinTimeGapPSFCH) between the PSFCH occasion and a slot where the SCI is transmitted” but not satisfying “the minimum time gap (T_3) between the PSFCH occasion and a slot of the earliest reserved PSSCH resource indicated by the corresponding SCI after the PSFCH occasion”,

      - the PSFCH occasion cannot be used by UE-A for a conflict indication for reserved PSSCH resource other than the earliest reserved PSSCH resource indicated by the corresponding SCI after the PSFCH occasion

- Agreement

  - (Pre)configuration of parameters related to n+T_1 and n+T_2 for determining the set of preferred resources in inter-UE coordination information triggered by a condition other than explicit request reception is not supported.

    - Note that T_2 is no smaller than T_2,min and 0 <= T_1 <= Tproc,1 as specified in TS 38.214 section 8.1.4.