3GPP TSG RAN WG1 #108-e			R1-2202265

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.16.11

Source:	Ericsson

Title:	UE features for NR sidelink enhancements

Document for:	Discussion, Decision

# 1	Introduction

In this paper, we provide some comments and modifications to the feature groups considered for NR SL Rel-17 by the moderator the after the RAN1#107bis-e meeting and included in R1-2200780 [1].

# 2	Feature group discussion

## 2.1	Feature group 32-2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-2 | Receiving NR sidelink of PSFCH/S-SSB | 1) UE can receive NR PSFCH/S-SSB. FFS whether other components will be included | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |

The feature group 32-2 indicates that a UE is able to receive the PSFCH and S-SSB. This FG is useful for indicating limited SL capabilities. We are supportive of including this FG since it is a new capability agreed in Rel-17 and it is aligned with the agreements and conclusions taken in RAN1. Moreover, in our view, no other components should be included in this FG.

No other components shall be included in the FG 32-2.

In our view, there is no need to split the capability further since both the S-SSB and the PSFCH are needed for a UE that has some basic SL functionalities.  The rationale is the following:

- The S-SSB reception is needed for a proper functionality of the SL feature due to the synchronization requirements. If the UE is not able to receive S-SSB it cannot operate out-of-coverage.

- The PSFCH reception is needed in order to monitor the success or failure of the UE previous transmission, and therefore, increase the reliability of the transmissions.

The FG 32-2 does not need to be split further. The S-SSB and PSFCH reception is not signalled in different FG.

## 2.2	Feature group 32-4

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can perform periodic-based partial sensing and resource allocation operation. 5) UE can perform contiguous partial sensing and resource allocation operation. 6) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 8) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 10) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 11) DL pathloss based open loop power control when mode 2 is configured by NR Uu FFS whether any other components should be added | [TBD] | [Yes] | [No] | UE does not support transmission according to the partial sensing and resource allocation | [Per band] | N.A. | N.A. | N.A. | Note: Random selection according to Rel-16 in the exceptional pool is supported. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Candidate values for B are {8,16} FFS how to define total SL processes if UE reports more than one FGs of 15-3, 32-4 and 32-4a Note: Component 6 is not required to be signalled in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Component-6 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-6 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-6 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) Note: Component 10 is only required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 11 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |

For the remaining unstable fields of FG 32-4, i.e., pre-requisite FGs, whether the gNB needs to know about the feature, signalling to other UEs about the feature and the granularity of the feature, we propose the following:

- Include as required pre-requisite the FG 32-2 if agreed

- Need for the gNB to know if the feature is supported: Yes

- Applicable to the capability signalling exchange between UEs (Sidelink WI only)”: No

- Granularity of the feature: Per band

For the FG 32-4, the remaining unstable fields are completed as follows:

- Include as required pre-requisite the FG 32-2 if agreed

- Need for the gNB to know if the feature is supported: Yes

- Applicable to the capability signalling exchange between UEs (Sidelink WI only)”: No

- Granularity of the feature: Per band

Moreover, in our view, for the note related to random resource selection, we think it is fine to keep the note as it is, without highlighting the wording “according to Rel-16”. Given the lack of agreements stating anything else, Rel-16 random resource selection is applied in the exceptional pool.

Remove the highlighted part for the wording “according to Rel-16” in the note related to random resource selection.

## 2.4	Feature group 32-4a

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-4a | Transmitting NR sidelink mode 2 with random resource selection | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with random resource selection configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 5) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 6) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 7) DL pathloss based open loop power control when mode 2 is configured by NR Uu FFS whether any other components should be added | [TBD] | [Yes] | [No] | UE does not support transmission according to the random resource selection and resource allocation | [Per band] | N.A. | N.A. | N.A. | Note: Random selection according to Rel-16 in the exceptional pool is supported. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Candidate values for B are {8,16} FFS how to define total SL processes if UE reports more than one FGs of 15-3, 32-4 and 32-4a Note: Component 4 is not required to be signalled in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Component-4 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-4 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-4 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) Note: Component 6 is only required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 7 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |

For the remaining unstable fields of 32-4a, i.e., pre-requisite FGs, whether the gNB needs to know about the feature, signalling to other UEs about the feature and the granularity of the feature, we propose the following:

- Include as required pre-requisite the FG 32-2 if agreed

- Need for the gNB to know if the feature is supported: Yes

- Applicable to the capability signalling exchange between UEs (Sidelink WI only)”: No

- Granularity of the feature: Per band

For the FG 32-4a, the remaining unstable fields are completed as follows:

- Include as required pre-requisite the FG 32-2 if agreed

- Need for the gNB to know if the feature is supported: Yes

- Applicable to the capability signalling exchange between UEs (Sidelink WI only)”: No

- Granularity of the feature: Per band

Regarding the note for random resource selection, in our view, the note regarding the random resource selection should be kept since a Rel-17 UE should be able to perform random resource selection in an exceptional pool as it was already supported in Rel-16.

Keep the note regarding the random resource selection in the exception resource pool.

## 2.5	Feature group 32-5a

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-5a | Inter-UE coordination scheme 1 in NR sidelink mode 2 | 1) UE can transmit and receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit and receive an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. FFS whether/how to split FG 32-5a into multiple FGs | [TBD] | [Yes] | [Yes] | UE does not support inter-UE coordination scheme 1 in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |

In our view, a UE which is performing Inter-UE coordination scheme 1 should be able to transmit (and receive) both types of resource sets, i.e., preferred and non-preferred set. The motivation is that based on the information contained on an enquiry from a Tx UE, the Rx UE may need to perform the operation for a preferred or a non-preferred set. Moreover, the UE operation to obtain a preferred or non-preferred set of resources is really similar. Therefore, we support that both are included in the FG definition.

A UE implementing FG 32-5a supports both preferred and non-preferred set of resources.

Regarding the prerequisite feature groups, for the FG 32-5a, the UE needs to support the feature groups which are related to the transmissions and reception of SL transmissions. In this case, we support to include the FG 15-1 and FG 15-3 as defined for Rel-16 UEs. Moreover, it is also possible that a UE involved in the inter-UE coordination scheme 1 can be performing partial sensing mechanism, and therefore, we propose to include it too.

Include in the pre-requisite feature groups field for the FG 32-5a the following FGs: 15-1, 15-3 and 32-4.

For the remaining unstable fields of 32-5a, i.e., whether the gNB needs to know about the feature, signalling to other UEs about the feature and the granularity of the feature, we propose the following:

- Need for the gNB to know if the feature is supported: Yes

- Applicable to the capability signalling exchange between UEs (Sidelink WI only)”: Yes

- Granularity of the feature: Per band

For the FG 32-5a, the remaining unstable fields are completed as follows:

- Need for the gNB to know if the feature is supported: Yes

- Applicable to the capability signalling exchange between UEs (Sidelink WI only)”: Yes

- Granularity of the feature: Per band

### 2.5.1	Addition of feature group related to 32-5a

Agreement For Scheme 1, a resource pool level (pre-)configuration can enable one of the following alternatives: - Alt 1 (Working Assumption): MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B. - For the indication of resource set, the following is supported: - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request. - First resource location of each TRIV is separately indicated by the inter-UE coordination information - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used. - FFS: UE capability details - 2nd SCI is UE RX optional - Alt 2: MAC CE is used as the container of inter-UE coordination information transmission from UE A to UE B. - For the indication of resource set, the following is supported: - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request. - First resource location of each TRIV is separately indicated by the inter-UE coordination information - FFS: Whether/How to use resource reservation information as coordination information

Regarding the reception of the 2nd stage SCI which contains the inter-UE coordination information as indicated in the agreement above, a new FG indicating this new optional UE optional is needed. The proposed FG description is included in the table below.

Introduce a new optional FG to indicate that a UE is capable of receiving a 2nd stage SCI containing the inter-UE coordination information.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-5a1 | Reception of Inter-UE coordination contained in 2nd stage SCI for Inter-UE coordination scheme 1 in NR sidelink mode 2 | UE can receive inter-UE coordination information of preferred resource set/non-preferred resource set in the 2nd stage SCI in its own resource (re-)selection in NR sidelink mode 2. | [32-5a] | [No] | [Yes] | UE does not support receiving inter-UE coordination information in 2nd stage SCI for scheme 1 in NR sidelink mode 2. | [Per UE] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |

## 2.6	Feature group 32-5b

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-5b-1 | Transmitting Inter-UE coordination scheme 2 in NR sidelink mode 2 | 1) UE can transmit inter-UE coordination information of presence of expected/potential resource conflict in NR sidelink mode 2. | 32-5b-2, TBD | [Yes] | [Yes] | UE does not support transmitting inter-UE coordination scheme 2 in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| 32. NR_SL_enh | 32-5b-2 | Receiving Inter-UE coordination scheme 2 in NR sidelink mode 2 | 1) UE can receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. | [TBD] | [Yes] | [Yes] | UE does not support receiving inter-UE coordination scheme 2 in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |

For the FG32-5b-1 and FG32-5b-1, in our view, both FGs need as pre-requisite the FGs which are related to the transmission and reception capability for the UEs (similar to the pre-requisites defined for the inter-UE coordination scheme 1). Therefore, we propose to include the FG 15-1 and FG 15-11 as defined for Rel-16 UEs. Moreover, it has not been disregarded that a UE involved in the inter-UE coordination scheme 2 can be performing partial sensing mechanism, and therefore, we propose to include it too. The rationale behind including these FGs is that the UEs involved in the inter-UE coordination scheme 2 shall be able to perform sensing, i.e., full sensing (FG 15-1) or partial sensing (FG 32-4a), in order to detect a collision and they should also be able to transmit PSFCH (FG 15-11).

Include in the pre-requisite feature groups field for the FG 32-5b-1 and FG 32-5b-2 the following FGs: 15-1, 15-11 and 32-4, along with the already included pre-requisite FGs.

# 3	Conclusion

In the previous sections we made the following observations:

No table of figures entries found.

Based on the discussion in the previous sections we propose the following:

Proposal 1	No other components shall be included in the FG 32-2.

Proposal 2	The FG 32-2 does not need to be split further. The S-SSB and PSFCH reception is not signalled in different FG.

Proposal 3	For the FG 32-4, the remaining unstable fields are completed as follows:

	Include as required pre-requisite the FG 32-2 if agreed

	Need for the gNB to know if the feature is supported: Yes

	Applicable to the capability signalling exchange between UEs (Sidelink WI only)”: No

	Granularity of the feature: Per band

Proposal 4	Remove the highlighted part for the wording “according to Rel-16” in the note related to random resource selection.

Proposal 5	For the FG 32-4a, the remaining unstable fields are completed as follows:

	Include as required pre-requisite the FG 32-2 if agreed

	Need for the gNB to know if the feature is supported: Yes

	Applicable to the capability signalling exchange between UEs (Sidelink WI only)”: No

	Granularity of the feature: Per band

Proposal 6	Keep the note regarding the random resource selection in the exception resource pool.

Proposal 7	A UE implementing FG 32-5a supports both preferred and non-preferred set of resources.

Proposal 8	Include in the pre-requisite feature groups field for the FG 32-5a the following FGs: 15-1, 15-3 and 32-4.

Proposal 9	For the FG 32-5a, the remaining unstable fields are completed as follows:

	Need for the gNB to know if the feature is supported: Yes

	Applicable to the capability signalling exchange between UEs (Sidelink WI only)”: Yes

	Granularity of the feature: Per band

Proposal 10	Introduce a new optional FG to indicate that a UE is capable of receiving a 2nd stage SCI containing the inter-UE coordination information.

Proposal 11	Include in the pre-requisite feature groups field for the FG 32-5b-1 and FG 32-5b-2 the following FGs: 15-1, 15-11 and 32-4, along with the already included pre-requisite FGs.

# 4	References

R1-2200780, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e”, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#107bis-e eMeeting, January 17th – January 27th, 2022.