3GPP TSG RAN WG1 #108	R1-2202270

e-Meeting, February 21st – 3rd March, 2022

Agenda item:		8.13.1

Source:	Nokia, Nokia Shanghai Bell

Title:	On cross-carrier scheduling from SCell to Pcell

Document for:		Discussion and Decision

# Introduction

RAN#94 in December closed the DSS WI for Rel-17. This document discusses the remaining critical issue of sSCell not available to make the feature usable together with SCell deactivation/dormancy.

# Discussion

## sSCell not available

RAN1#106 made the following agreement:

Agreement [RAN1#106] Specification supports dormant BWP operation on sSCell for a UE is configured CCS from sSCell to P(S)Cell.

RAN1#106bis had the following proposal under discussion, but no time to reach any agreement:

- For a UE configured for cross-carrier scheduling from sSCell to P(S)Cell, when sSCell is deactivated, a mechanism to support monitoring of additional PDCCH monitoring candidates/DCI formats on P(S)Cell is supported - The additional PDCCH monitoring candidates/DCI formats are not monitored on P(S)Cell when sSCell is activated - FFS whether the additional PDCCH monitoring candidates/DCI formats are in - additional SS set(s) that are not monitored on P(S)Cell when sSCell is activated - SS set(s) that are monitored on P(S)Cell when sSCell is activated

RAN1#107 again discussed the point, with the following progression in the moderator’s summary

### Discussion Point 9 - For UEs configured for cross-carrier scheduling from sSCell to P(S)Cell, please indicate your view on whether a mechanism to enable monitoring of additional PDCCH monitoring candidates/DCI formats on P(S)Cell when sSCell is deactivated should be supported for the following cases - Case 1: Supported for Type A Ues - Case 2: Supported for Type A and Type B Ues - Case 13: Supported only for Type A2 Ues (as described in discussion point 6) - Note: The additional PDCCH monitoring candidates/DCI formats are not monitored on P(S)Cell when sSCell is activated - Also, please indicate your preference for the detailed mechanism - M1: Additional SS sets monitored on P(S)Cell when sSCell is deactivated and not monitored when sSCell is activated - M2: SSSG switch on P(S)Cell triggered when sSCell is deactivated/activated - M3: Scaling factor α not applied for BD limit computation and overbooking handling on P(S)Cell, when sSCell is deactivated ### Proposal 9v2-1 - Scaling factor α (from RAN1#106bis-e agreement) is not applied for PDCCH monitoring on P(S)Cell when sSCell is deactivated ### Proposal 9v3-1 - Following is supported - Scaling factor α (from RAN1#106bis-e agreement) is not applied for PDCCH monitoring on P(S)Cell from slot n+X1 of P(S)Cell when sSCell deactivation is triggered in slot n of P(S)Cell - Scaling factor α (from RAN1#106bis-e agreement) is not applied for PDCCH monitoring on P(S)Cell from slot n+X2 when sSCell BWP switch to dormant BWP is triggered in slot n of P(S)Cell - X1 = [BWP switching delay specified for P(S)Cell SCS]; X2 = [BWP switching delay specified for P(S)Cell SCS];

In addition to SCell deactivation, SCell dormance as well as SCell RLF lead to the situation that the sSCell is unable to schedule any data on either the PCell or on the SCell. In such situations it would be useful to use the sSCell BD monitoring budget on the PCell without RRC reconfiguration. In our view, the scaling factor fall-back as suggested in RAN1#107 proposal 9v2-1 would be a welcomed but insufficient a solution when there is no mechanism change the actual SS configuraiton the UE is to apply, and the network would anyway be forced to use RRC reconfiguration. This would effectively render the X-CC from sSCell to PCell and SCell deactivation/dormancy for power saving purposes useless.

Proposal 1: Both the Type A and the Type B UE can be configured with a fallback search space set for monitoring PDCCH for self-scheduling on the P(S)Cell in case a fall-back mechanism is triggered due sSCell being considered not to able to transmit data.

Proposal 2: The conditions for sSCell is considered not to be able to transmit data are at least:

- sSCell is deactivated

- sSCell is dormant

- sSCell is experiencing a radio link failure

To enable the above framework the Search Space Group (SSG) framework could be re-used. The Search Spaces for CCS from Scell to PCell and PCell self scheduling when CCS is configured would belong to one search space group (SSG#1) and the fallback monitoring search spaces (e.g. on sef scheduling form PCell) would belong to a second search space group (SSG#2).

The switching between the SSG’s can be dfined based SCell Dormancy or CCS SCell RLF criteria . In both case the the UE and serving cells would switch from SSG#1 to SSG#2 upon the SCell Dormancy or RLF detection.

Proposal 3: Re-use SSG framework establishing at least Scell Dromancy and RLF as criterias for switching between SSGs.

# Conclusion

In this contribution the following observations and proposals are made:

Proposal 1: Both the Type A and the Type B UE can be configured with a fallback search space set for monitoring PDCCH for self-scheduling on the P(S)Cell in case a fall-back mechanism is triggered due sSCell being considered not to able to transmit data.

Proposal 2: The conditions for sSCell is considered not to be able to transmit data are at least:

- sSCell is deactivated

- sSCell is dormant

- sSCell is experiencing a radio link failure

Proposal 3: Re-use SSG framework establishing at least Scell Dromancy and RLF as criterias for switching between SSGs.

# References

- R1-2106356, Summary#5 of Email discussion [105-e-NR-DSS-01], Moderator (Ericsson), RAN1#105

- R1-2108661, Summary#5 of Email discussion [106-e-NR-DSS-01], Moderator (Ericsson), RAN1#106

- R1-2110664, Summary#4 of Email discussion [106bis-e-NR-DSS-01], Moderator (Ericsson), RAN1#106bis

- R1-2112884, Summary#5 of Email discussion [107-e-NR-DSS-01], Moderator (Ericsson), RAN1#107

- R1-2104185, On cross-carrier scheduling from SCell to Pcell, Nokia, Nokia Shanghai Bell

- R1-2107526, On cross-carrier scheduling from SCell to Pcell, Nokia, Nokia Shanghai Bell

- R1-2110376, On cross-carrier scheduling from SCell to Pcell, Nokia, Nokia Shanghai Bell

- R1-2110944, On cross-carrier scheduling from SCell to Pcell, Nokia, Nokia Shanghai Bell