3GPP TSG RAN WG1 #108-e			R1-2202274

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	5

Source:	Ericsson

Title:	Discussion on RAN2 LS to RAN1 on inter-UE coordination

Document for:	Discussion, Decision

# 1	Introduction

RAN1 received a LS from stating the agreements reached in RAN2 in aspects related to inter-UE coordination feature and which of the topics will be discussed in RAN2 and which ones will be mainly discussed in RAN1. The list of topics is the following:

1: Inter-UE coordination (IUC) issues (on which) RAN2 mainly relies on RAN1: - HARQ retransmission number for inter-UE coordination information - Information and length of information of IUC MAC CE. The information indicated in RAN1 LS should be taken into account as baseline. - UE-B procedure (e.g. final selection of resources) to the (non-)preferred resource set in IUC - Scheme 2 inter-UE coordination design - Condition for the UE-A to transmit IUC - Signaling design and trigger conditions for the request from UE-B to UE-A - Cast types (UC/GC/BC) of inter-UE coordination - Transmission of inter-UE coordination MAC CE on dedicated resource - L1 parameters/configurations for IUC in Uu RRC (including L1 configurations per resource pool) - Whether UE-A can be in mode1 or mode2 (interested companies are invited to raise/discuss the issue directly in RAN1) 2. IUC issues (on which) RAN2 starts discussion: - LCP for inter-UE coordination MAC CE, support for standalone inter-UE coordination MAC CE/multiplex MAC CE and MAC SDU in a MAC PDU - Timer to handle latency bound for inter-UE coordination - Priority value/priority order of inter-UE coordination MAC CE. RAN1 progress can be taken into account in phase-2 discussion. - HARQ feedback option of inter-UE coordination MAC CE 3. IUC in SL DRX is deprioritized in Rel-17 from RAN2 point of view

# 2	Discussion

In their LS, RAN2 have indicated that SL-DRX in Rel-17 is deprioritized from their point of view.

RAN2 has deprioritized power saving aspects, i.e., SL-DRX, in relationship with inter-UE coordination.

In our view, further work on the topic in RAN1 should not be considered. However, given that the use of inter-UE coordination is configured at pool lever while SL DRX is, to a large extent, configured at UE level, it is important that the specifications prevent the use of the combinations for which there is no support.

RAN1 needs to discuss how to capture the consequences of the RAN2 agreement on down-prioritizing IUC in SL DRX

As included in one of our contributions for RAN1#108-e meeting [1], by using the inter-UE coordination mechanism scheme 2 when UEs using power saving resource allocation mechanisms are involved, the possibility of avoiding a collision is increased which improves the overall system performance.

Inter-UE coordination scheme 2 improves the performance of all UEs even if only a few of them do full sensing.

Moreover, for scheme 1 it is important to consider the information that it is included in the inter-UE coordination information transmitted by UE-A and how accurate this information is based on the amount of sensing performed – which is directly connected with the resource allocation mechanism used, e.g., partial sensing or full sensing. Therefore, it is necessary to consider these cases when defining the procedures for inter-UE coordination scheme 1.

For inter-UE coordination scheme 1, RAN1 needs to consider the different resource allocation mechanism, e.g., full-sensing and partial sensing, performed by UE-A when transmitting the inter-UE coordination information to UE-B.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	RAN2 has deprioritized power saving aspects, i.e., SL-DRX, in relationship with inter-UE coordination.

Observation 2	Inter-UE coordination scheme 2 improves the performance of all UEs even if only a few of them do full sensing.

Observation 3	For inter-UE coordination scheme 1, RAN1 needs to consider the different resource allocation mechanism, e.g., full-sensing and partial sensing, performed by UE-A when transmitting the inter-UE coordination information to UE-B.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN1 needs to discuss how to capture the consequences of the RAN2 agreement on down-prioritizing IUC in SL DRX

# 4	References

R1-2202264, “Additional considerations on resource allocation for power saving and inter-UE coordination”, Ericsson, RAN1#108-e, eMeeting