3GPP TSG-RAN WG1 Meeting #108-e	R1-2202277

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.9.1

Source:	Ericsson

Title:	Support of 16-QAM for unicast in UL and DL in NB-IoT

Document for:	Discussion and Decision

# 1	Introduction

The core-part of the Work Item (WI) on “Additional enhancements for NB-IoT and LTE-MTC” [1] was recently finalized including the specification of the following enhancement for NB-IoT:

- Specify 16-QAM for unicast in UL and DL, including necessary changes to DL power allocation for NPDSCH and DL TBS. This is to be specified without a new NB-IoT UE category. For DL, increase in maximum TBS of e.g. 2x the Rel-16 maximum, and soft buffer size will be specified by modifying at least existing Category NB2. For UL, the maximum TBS is not increased. [NB-IoT] [RAN1, RAN4] - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT] [RAN2, RAN1, RAN4]

In this contribution we state our view on the topics intended to be discussed during the maintenance phase (i.e., CQI table switching, ΔTF power difference between QPSK and 16-QAM, and capturing the support of 16-QAM in DL during the PUR procedure in clause 16.4.1.5 of TS 36.213).

# 2	Support of 16-QAM for unicast in DL

## 2.1	CQI Table switching

In RAN1# 107-e, the following CQI table for 16-QAM in downlink was designed [2]:

| Reported value | NPDCCH repetition level | NPDSCH transport block error probability not exceeding 0.1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Modulation | Code rate x 1024 | Repetition | Efficiency |
| noMeasurement | No measurement reporting | Out of range |  |  |  |
| candidateRep-A | 1 | QPSK | 221 | 1 | 0.4316 |
| candidateRep-B | 2 | QPSK | 280 | 1 | 0.2737 |
| candidateRep-C | 4 | QPSK | 81 | 1 | 0.1579 |
| candidateRep-D | 8 | QPSK | 81 | 2 | 0.0789 |
| candidateRep-E | 16 | QPSK | 81 | 4 | 0.0395 |
| candidateRep-F | 32 | QPSK | 81 | 8 | 0.0198 |
| candidateRep-G | 1 | QPSK | 336.8 | 1 | 0.6579 |
| candidateRep-H | 1 | QPSK | 453.6 | 1 | 0.8860 |
| candidateRep-I | 1 | QPSK | 579.4 | 1 | 1.1316 |
| candidateRep-J | 1 | QPSK | 759 | 1 | 1.4825 |
| candidateRep-K | 1 | 16QAM | 487.3 | 1 | 1.9035 |
| candidateRep-L | 1 | 16QAM | 541.2 | 1 | 2.1140 |
| candidateRep-M | 1 | 16QAM | 658 | 1 | 2.5702 |
| candidateRep-N | 1 | 16QAM | 783.7 | 1 | 3.0614 |
| candidateRep-O | 1 | 16QAM | 837.6 | 1 | 3.2719 |

Towards the end of RAN1# 107-e, the possibility of switching between the Rel-17 CQI table and the legacy CQI table was discussed.

There were several proposals for performing the CQI Table switching such as using MAC CE, Rmax, RRC configuration, and “if 16QAM in DL is configured in msg4, then the UE should use the 16QAM CQI table, otherwise the UE will use the legacy table”.

Upon RAN1 completed the design of the Rel-17 CQI Table, there was a comment stating that the designed Rel-17 CQI table should be sufficient as to do not require a switching to the legacy table since for this feature we are talking about UEs configured with a small Rmax value, and the radio conditions of such a UEs cannot change so drastically as to require reports associated with a very large number of repetitions which are associated with the entries that did not remain available in the new CQI table (i.e., report entries from ”CandidateRep-G” onwards in the legacy table).

During RAN1# 107-e, it was mentioned that the designed Rel-17 CQI table should be sufficient as to do not require a switching to the legacy table since UEs should be configured with a small Rmax value, and the radio conditions of such UEs cannot change so drastically as to require reports tied to a very large number of repetitions.

16-QAM was designed to be used with 1 repetition, if due to a change in the radio conditions were necessary to switch to QPSK it seems that the CQI Table covers a reasonable margin of NPDCCH repetitions (up to 32). It seems that going beyond the number of repetitions covered by the CQI table would correspond to scenarios where only QPSK is usable without a possibility of going back to 16-QAM in short term. Moreover, we also need to consider a scenario that suddenly requires a very large number of repetitions may even result in a radio link failure (RLF) due that the UE is configured with a small Rmax value.

16-QAM was designed to be used with 1 repetition, if due to a change in the radio conditions were necessary to switch to QPSK, it seems that the Rel-17 CQI Table covers a reasonable margin of NPDCCH repetitions (up to 32 repetitions).

We need to consider that going beyond the number of repetitions (> 32 repetitions) covered by the Rel-17 CQI table, may even result in an RLF for a UE configured with a small Rmax value.

Thus, for a scenario requiring a large number of repetitions (i.e., > 32) a CQI table switching mechanism may result to be irrelevant, since in those scenarios a larger Rmax would need to be configured.

Based on the Rel-17 CQI table design which covers up to 32 repetitions for QPSK, introducing a table switching mechanism is no longer necessary.

## 2.2	16-QAM in DL during the PUR procedure in clause 16.4.1.5 of TS 36.213

In RAN1# 106-bis-e, the following agreements were reached touching upon the support of 16-QAM in DL during the PUR procedure.

Agreement Support 16-QAM for NPDSCH in PUR procedure - CSI report is not supported/expected during PUR procedure. Agreement To support 16-QAM for NPDSCH and NPUSCH in PUR procedure, - 16-QAM can be enabled/disabled by UE specific RRC signaling for NPDSCH and NPUSCH separately - The corresponding configurations and signaling details are up to RAN2

During the discussions post-RAN1#107-e, more specifically during the 2nd round of the Editor’s CRs discussions it was decided to leave until “Rel-17 Maintenance” capturing the way of supporting 16-QAM in DL during the PUR procedure in clause 16.4.1.5 of TS 36.213.

According with the endorsed version of the Editor’s CR, clause 16.4.1.5 is as follows:

#### 16.4.1.5 Modulation order and transport block size determination To determine the modulation order in the NPDSCH, the UE shall - if the UE is configured with higher layer parameter npdsch-16QAM-Config and the DCI is mapped onto the UE specific search space and the 4-bit "modulation and coding scheme" field () in the DCI is set to ‘1111’, - use modulation order, = 4 - otherwise - use modulation order, = 2.

The original proposal to capture the support of 16-QAM in DL during the PUR procedure was as follows:

#### 16.4.1.5 Modulation order and transport block size determination To determine the modulation order in the NPDSCH, the UE shall - if the UE is configured with higher layer parameter npdsch-16QAM-Config and the DCI is mapped onto the UE specific search space and the 4-bit "modulation and coding scheme" field () in the DCI is set to ‘1111’, except for the DCI CRC scrambled by PUR-RNTI and the UE not configured with higher layer parameter pur-DL-16QAM-Config, - use modulation order, = 4 - otherwise - use modulation order, = 2.

During an offline discussion, one other delegate proposed the following last-minute wording aiming at making clearer the statement capturing the support of 16-QAM in DL during the PUR procedure, however there was not time to discuss this proposal during the 2nd round of review of the Editor’s CR:

#### 16.4.1.5 Modulation order and transport block size determination To determine the modulation order in the NPDSCH, the UE shall - if the UE is configured with higher layer parameter npdsch-16QAM-Config and the DCI is mapped onto the UE specific search space given by C-RNTI and the 4-bit "modulation and coding scheme" field () in the DCI is set to ‘1111’, or if the UE is configured with higher layer parameter pur-DL-16QAM-Config and the DCI is mapped onto the UE specific search space given by PUR-RNTI and the 4-bit "modulation and coding scheme" field () in the DCI is set to ‘1111’ - use modulation order, = 4 - otherwise - use modulation order, = 2.

In our view, the wording above although lengthier makes the statement clearer. Based on the above wording proposal, in the Text Proposal (TP) below we made an attempt to simplify the sentence keeping its essence.

To incorporate the support of 16-QAM in DL during the PUR procedure in clause 16.4.1.5 of TS 36.213, the following TP in R1-2202281 is adopted:

------------------------------------------------------- Text Start ----------------------------------------------------------- #### 16.4.1.5 Modulation order and transport block size determination To determine the modulation order in the NPDSCH, the UE shall - if the UE is configured with higher layer parameter npdsch-16QAM-Config and the DCI is mapped onto the UE specific search space given by C-RNTI or if the UE is configured with higher layer parameter pur-DL-16QAM-Config and the DCI is mapped onto the UE specific search space given by PUR-RNTI, and the 4-bit "modulation and coding scheme" field () in the DCI is set to ‘1111’, - use modulation order, = 4 - otherwise - use modulation order, = 2. ------------------------------------------------------- Text End -----------------------------------------------------------

# 3	Support of 16-QAM for unicast in UL

## 3.1	Additional power control parameter for 16-QAM in UL

In RAN1# 105-e, the following was reached [3]:

Agreement Introduce a new term in uplink power control of NPUSCH using 16-QAM. FFS on the details.

In RAN1# 107-e [2], the following agreement was reached:

Agreement The following working assumption is confirmed. For the new term ∆TF,c introduced for power control of NPUSCH, - Reuse the LTE definition simplified for NB-IoT: ∆TF,ci=10log102BPRE∙Ks-1 for Ks=1.25 and ∆TF,ci=0 for Ks=0, where Ks is given by higher layer parameter deltaMCS-Enabled, and BPRE=KNRE where K is the code block size. - FFS: whether the new term applies to QPSK when configured with 16QAM, if it does not, whether an additional term is introduced to avoid jump between QPSK and 16QAM

RAN1 introduced into the UE’s transmit power control equation a new term (ΔTF) for 16-QAM in UL as to account for the larger number of bits per Resource Element (RE) that this higher order modulation scheme introduces. On this matter, it was pointed out the possibility of introducing a way to prevent a large power difference between QPSK and 16-QAM. During RAN1# 107-e, two proposals remained on the table 1) “Introducing ΔTF for QPSK” and 2) “Introducing an Offset acting on ΔTF for 16-QAM”.

A new term (ΔTF) for 16-QAM in UL was introduced as to account for the larger number of bits per RE that this higher order modulation scheme introduces.

Due to the introduction of ΔTF, it was pointed out the possibility of introducing a way to prevent a large power difference between QPSK and 16-QAM.

Two proposals remained considered to alleviate the power difference between QPSK and 16-QAM: 1) “Introducing ΔTF for QPSK” and 2) “Introducing an Offset acting on ΔTF for 16-QAM”.

The main concern with “1) Introducing ΔTF for QPSK” was that with such a solution the QPSK UL power control behavior will be different with and without 16-QAM configured.

“Introducing ΔTF for QPSK” has as a side effect QPSK resulting in an UL power control behavior that will be different with and without 16-QAM configured.

At some point, one company commented [4]:

“maybe we could revisit the origination of debate i.e., to solve the 6.5db jump. Actually, from terminal’s view, in terms of open loop such jump up to 6.5dB is very common, perhaps we could let it go”

Due that it was not possible to reach a consensus towards 1) or 2), at some point one company commented that “in terms of open loop such jump up to 6.5dB is very common, perhaps we could let it go”.

In our view, the WID’s objective was about introducing 16-QAM for NB-IoT and therefore we should not create side effects (i.e., different behaviors) from making modifications touching upon legacy modulation schemes.

Based on observation 12, any solution intended to alleviate the power difference between QPSK and 16-QAM should be based on a solution acting on 16-QAM elements (i.e., offset acting on ΔTF), otherwise is preferred to deal with a power difference between QPSK and 16-QAM.

If the power difference between QPSK and 16-QAM is to be alleviated, it should be based on a solution acting on 16-QAM elements (i.e., offset acting on ΔTF), otherwise is preferred to live with such a power difference between modulation schemes.

## 3.2	Clarifications for 16-QAM in UL on TS 36.212 and TS 36.213

### 3.2.1	TS 36.212 clause 6.3.2

The clarification on TS 36.212 clause 6.3.2 refers to incorporating the new modulation scheme (i.e., 16-QAM) introduced in Rel-17.

To incorporate the support of 16-QAM in UL into clause 6.3.2 of TS 36.212, the following TP in R1-2202280 is adopted:

### --------------------------------------------------------- Text Start --------------------------------------------------------- ### 6.3.2 Uplink shared channel Figure 6.3.2-1 shows the processing structure for the UL-SCH transport channel. Data arrives to the coding unit in the form of a maximum of one transport block over a number of resource units per UL cell. The number of resource units is scheduled according to [3]. The following coding steps can be identified: - CRC attachment - Channel coding - Rate matching Figure 6.3.2-1: Transport block processing for UL-SCH The CRC attachment, channel coding, and rate matching are performed according to clauses 5.2.2.1, 5.2.2.3, and 5.2.2.4, respectively, with the following differences: - In clause 5.1.4.1.2 in the calculation of , Qm is 1 for π/2-BPSK, 2 for π/4-QPSK and 4 for 16QAM, and rvidx = 0 or 2. In addition, after rate matching interleaving is applied per resource unit according to clauses 5.2.2.7 and 5.2.2.8 without any control information in order to apply a time-first rather than frequency-first mapping, where the input sequence to 5.2.2.7 is the portion of e for a resource unit instead of f, and where is the number of SC-FDMA symbols for NPUSCH in a UL resource unit as given in clause 10.1.2.3 of [2]. ------------------------------------------------------- Text End -----------------------------------------------------------

### 3.2.2	TS 36.213

#### 3.2.2.1	TS 36.213 clause 16.2.1.1.1

The new term in the UE’s transmit power control equation is also applicable for PUR, nonetheless the statement “NPUSCH (re)transmissions with 16QAM” does not encompass PUR, therefore an explicit statement about PUR should be added.

To clarify that ΔTF also applies for 16-QAM in UL for PUR, the following TP in R1-2202281 for clause 16.2.1.1.1 of TS 36.213 is adopted:

### ------------------------------------------------------- Text Start ---------------------------------------------------------- ##### 16.2.1.1.1 UE behaviour The setting of the UE Transmit power for a Narrowband Physical Uplink Shared Channel (NPUSCH) transmission is defined as follows. For FDD, if the UE is capable of enhanced random access power control [12], and it is configured by higher layers, and for TDD, enhanced random access power control shall be applied for a UE which started the random access procedure in the first or second configured NPRACH repetition level. ### ------------------------------------------------------- Text Omitted ------------------------------------------------------- - If NPUSCH (re)transmissions with 16QAM or NPUSCH (re)transmission corresponding to preconfigured uplink resource with 16QAM, - ΔTF,ci=10log102BPRE⋅Ks-1 for and ∆TF,c(i)=0 for where is given by the parameter deltaMCS-Enabled provided by higher layers for serving cell , and - BPRE=K/NRE where K is the code block size and NRE is the number of resource elements determined as NRE=(NsymbUL-1)NslotsULNscRUNRU where NsymbUL, NslotsUL, NscRU are defined in [3], and NRU is defined in section 16.5.1.1 - otherwise ∆TF,c(i)=0. ------------------------------------------------------- Text End -----------------------------------------------------------

#### 3.2.2.2	TS 36.213 clause 16.5.1.2

For the PUR case which in clause 16.5.1.2 is treated has an exception, it has been added a complementary statement to clarify from where the indices are obtained citing the corresponding Information Element.

To clarify from where the indices associated to (,) are obtained for 16-QAM in UL for PUR, the following TP in R1-2202281 for clause 16.5.1.2 of TS 36.213 is adopted:

### ------------------------------------------------------- Text Start ---------------------------------------------------------- #### 16.5.1.2 Modulation order, redundancy version and transport block size determination To determine the modulation order, redundancy version and transport block size for the NPUSCH, the UE shall first - read the "modulation and coding scheme" field () in the DCI or configured by higher layers for NPUSCH transmission using preconfigured uplink resource, and - read the "redundancy version" field () in the DCI or initiate with rvDCI=0 for NPUSCH transmission using preconfigured uplink resource, and - read the "resource assignment" field () in the DCI or configured by higher layers for NPUSCH transmission using preconfigured uplink resource, and - compute the total number of allocated subcarriers (), number of resource units (), and repetition number () according to Clause 16.5.1.1. ### ------------------------------------------------------- Text Omitted ------------------------------------------------------- The UE shall use (,) and Table 16.5.1.2-2 to determine the TBS to use for the NPUSCH. is given in Table 16.5.1.2-1 if , or ITBS=IMCS'+14 if NPUSCH with 16QAM except for NPUSCH transmission using preconfigured uplink resource in which case the corresponding indices are provided in PUR-Config-NB, otherwise. IMCS' is the value of the "modulation and coding scheme for 16QAM" in the DCI. ------------------------------------------------------- Text End -----------------------------------------------------------

# 5	Conclusion

In the previous sections we made the following observations for the support of 16-QAM for unicast in UL and DL for NB-IoT:

Observation 1	Towards the end of RAN1# 107-e, the possibility of switching between the Rel-17 CQI table and the legacy CQI table was discussed.

Observation 2	There were several proposals for performing the CQI Table switching such as using MAC CE, Rmax, RRC configuration, and “if 16QAM in DL is configured in msg4, then the UE should use the 16QAM CQI table, otherwise the UE will use the legacy table”.

Observation 3	During RAN1# 107-e, it was mentioned that the designed Rel-17 CQI table should be sufficient as to do not require a switching to the legacy table since UEs should be configured with a small Rmax value, and the radio conditions of such UEs cannot change so drastically as to require reports tied to a very large number of repetitions.

Observation 4	16-QAM was designed to be used with 1 repetition, if due to a change in the radio conditions were necessary to switch to QPSK, it seems that the Rel-17 CQI Table covers a reasonable margin of NPDCCH repetitions (up to 32 repetitions).

Observation 5	We need to consider that going beyond the number of repetitions (> 32 repetitions) covered by the Rel-17 CQI table, may even result in an RLF for a UE configured with a small Rmax value.

Observation 6	Thus, for a scenario requiring a large number of repetitions (i.e., > 32) a CQI table switching mechanism may result to be irrelevant, since in those scenarios a larger Rmax would need to be configured.

Observation 7	A new term (ΔTF) for 16-QAM in UL was introduced as to account for the larger number of bits per RE that this higher order modulation scheme introduces.

Observation 8	Due to the introduction of ΔTF, it was pointed out the possibility of introducing a way to prevent a large power difference between QPSK and 16-QAM.

Observation 9	Two proposals remained considered to alleviate the power difference between QPSK and 16-QAM: 1) “Introducing ΔTF for QPSK” and 2) “Introducing an Offset acting on ΔTF for 16-QAM”.

Observation 10	“Introducing ΔTF for QPSK” has as a side effect QPSK resulting in an UL power control behavior that will be different with and without 16-QAM configured.

Observation 11	Due that it was not possible to reach a consensus towards 1) or 2), at some point one company commented that “in terms of open loop such jump up to 6.5dB is very common, perhaps we could let it go”.

Observation 12	In our view, the WID’s objective was about introducing 16-QAM for NB-IoT and therefore we should not create side effects (i.e., different behaviors) from making modifications touching upon legacy modulation schemes.

Observation 13	Based on observation 12, any solution intended to alleviate the power difference between QPSK and 16-QAM should be based on a solution acting on 16-QAM elements (i.e., offset acting on ΔTF), otherwise is preferred to deal with a power difference between QPSK and 16-QAM.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Based on the Rel-17 CQI table design which covers up to 32 repetitions for QPSK, introducing a table switching mechanism is no longer necessary.

Proposal 2	To incorporate the support of 16-QAM in DL during the PUR procedure in clause 16.4.1.5 of TS 36.213, the following TP in R1-2202281 is adopted:

Proposal 3	If the power difference between QPSK and 16-QAM is to be alleviated, it should be based on a solution acting on 16-QAM elements (i.e., offset acting on ΔTF), otherwise is preferred to live with such a power difference between modulation schemes.

Proposal 4	To incorporate the support of 16-QAM in UL into clause 6.3.2 of TS 36.212, the following TP in R1-2202280 is adopted:

Proposal 5	To clarify that ΔTF also applies for 16-QAM in UL for PUR, the following TP in R1-2202281 for clause 16.2.1.1.1 of TS 36.213 is adopted:

Proposal 6	To clarify from where the indices associated to (,) are obtained for 16-QAM in UL for PUR, the following TP in R1-2202281 for clause 16.5.1.2 of TS 36.213 is adopted:

# 6	References

RP-201306, "WID revision: Additional enhancements for NB-IoT and LTE-MTC”, RAN #88e, Electronic Meeting, June 29th – July 3rd, 2020.

Session notes for 8.9 (Rel-17 enhancements for NB-IoT and LTE-MTC), Ad-hoc chair (CMCC), 3GPP TSG RAN WG1 Meeting #107-e, e-Meeting, November 11th – 19th, 2021.

Session notes for 8.9 (Rel-17 enhancements for NB-IoT and LTE-MTC), Ad-hoc chair (CMCC), 3GPP TSG RAN WG1 Meeting #105-e, e-Meeting, May 10th – 27th, 2021.

R1-2112651, “Feature lead summary #1 on 106bis-e-LTE-Rel17-NB-IoT-eMTC-01,” Moderator (Huawei), 3GPP TSG RAN WG1 Meeting #107-e, e-Meeting, November 11th – 19th, 2021.