3GPP TSG-RAN WG1 Meeting #108-e	R1-2202310

e-Meeting, February 21st – March 3rd, 2022

Title:	Draft LS reply on Enhanced TCI state indication for UE-specific PDCCH MAC CE

Response to:	R2-2201995/ R1-2200886

Release:	Rel-17

Work Item:	NR_feMIMO-Core

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

Name:	Mihai Enescu

E-mail Address:	mihai.enescu@nokia.com

# 1. Overall Description

RAN1 would like to thank RAN2 for their LS in R1-2200886 (R2-2201995) on Enhanced TCI state indication for UE-specific PDCCH MAC CE.

RAN1 discussed the question posed by RAN2 and would like to clarify the following:

Question: RAN2 would like to ask whether “Enhanced TCI state indication for UE specific PDCCH MAC CE” can be applied to CORESET zero or not.

Answer: There is an ongoing discussion in RAN1 on how to handle PDCCH candidates in CSS 0/0A/1/2 in SFN operation. If SFN operation is not applied for all search space types, there may be certain CORESETs (including CORESET#0) not activated with two TCI states. However, as RAN1 agreement below does not rule out signaling of two TCI states for CORESET#0, there may be nothing wrong with allowing “Enhanced TCI state indication for UE specific PDCCH MAC CE” applying to CORESET#0.

Related discussion in RAN1 is captured in the below agreements,

Agreement

Introduce enhanced MAC CE signaling for PDCCH activating two TCI states for SFN-based PDCCH transmission

- The corresponding MAC CE includes at least the following fields

  - Serving cell ID

  - CORESET ID

  - Two TCI state IDs

- FFS whether for CA scenario additionally support RRC configured set of the serving cells which can be addressed by a single MAC CE

- FFS whether or not enhanced MAC CE signaling is applicable to a CORESET configured with CORESETPoolindex

Send LS to RAN2 to inform about agreement on support of enhanced MAC CE for CORESET in Rel-17. LS is endorsed in R1-2104064

Agreement

For CSS associated with SFN CORESET, study the following alternatives and down-select in RAN1#107e:

- Alt 2: UE doesn’t expect PDCCH candidates in CSS to be associated with CORESET activated with two TCI states, except for CSS type 3 associated with CORESET configured with scheme 1

- Alt 3: If PDCCH candidates in CSS 0/0A/1/2/3 are associated with CORESET that activated with two TCI states, the first TCI state is applied for the CSS reception, except for CSS type 3 associated with CORESET configured with scheme 1.

  - For CSS type 3 associated with CORESET configured with scheme 1,  both TCI states can be applied for the CSS reception.

Agreement

If PDCCH candidates in CSS 3 are associated with CORESET that is activated with two TCI states and configured with enhanced SFN scheme 1 or TRP based pre-compensation, both TCI states can be applied for the CSS reception.

- FFS: Whether/How specification change is needed is up to the editor

2. Actions:

To RAN2 group:

ACTION: 	RAN1 would kindly ask RAN2 to take the above information into account.

3. Date of Next RAN1 Meetings:

RAN1#109-e	May 16th – May 27th, 2022		E-meeting

RAN1#110	August 22nd – August 26th, 2022		Toulouse, FR