3GPP TSG RAN WG1 #108-e	R1-2202329

e-Meeting, February 21st – March 3rd, 2022

Agenda item:		8.7.1.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Open issues on TRS information for IDLE/INACTIVE mode UEs

Document for:		Discussion and Decision

# Introduction

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1]

  - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1]

NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #100 meeting

  - Specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, minimizing system overhead impact [RAN1]

NOTE: Always-on TRS/CSI-RS transmission by gNodeB is not required

The release 17 work item [1] on UE power saving includes the following objective

In this contribution, assumptions and evaluation results relevant to the objective 1b are discussed.

# Behavior and signaling related to TRS/CSI-RS occassions

## Availability indication

The aspects related to presence indication or guarantee for the IDLE/Inactive UEs for potential TRS/CSI-RS occasions(s) was discussed in RAN1#104e with following agreement:

Agreements: For a cell with TRS/CSI-RS occasions configured for IDLE/Inactive UEs, IDLE/Inactive UE’s assumption on the availability of TRS/CSI-RS at the configured occasion(s) is informed to the idle/inactive UE based on explicit indication. - FFS details (e.g., the signalling, detailed information for the TRS/CSI-RS, etc.) - There is no intended blind detection of the presence/absence of TRS/CSI-RS at the UE side in this feature. That is, the UE assumes TRS/CSI-RS is not present if the network does not indicate it is available (or indicates it is unavailable).

### Validity duration and reference point

The reference point for the start of the validity duration was agreed in last meeting to match to the SFN of the first PF of the default paging cycle:

Agreement The reference point for start of the validity duration is SFN of the first PF from the current default DRX cycle where UE receives the availability indication - FFS: Whether the availability indication is transmitted [only once] during the validity duration

In RAN1#107bis-e, it was further clarified the UE interpretation and on the presence of the L1 indication:

Agreement UE can receive L1 based signaling for TRS availability indication before the expiration/end of validity duration associated with previous L1 based signaling for TRS availability indication - For each bit indicated as ‘1’ in the availability indication field of the current L1 based signaling TRS availability indication, the UE assumes the corresponding TRS resource set(s) are available from the reference point until the end of the validity duration associated with the current L1 based signaling indication. - For each bit indicated as ‘0’ in the availability indication field of the current L1 based signaling, the UE keeps the existing current assumption on the availability or unavailability of the corresponding TRS resource set(s). Note: the validity duration for different group of TRS resources sets correspond to different bits in the availability indication field can be different and are maintained independently. Agreement If SIB configures TRS resource, TRS availability indication field is present in DCI format 2_7 (if configured) with CRC scrambled by PEI-RNTI and DCI format 1_0 with CRC scrambled by P-RNTI

As described in 38.213, the reference frame for the start of the validity duration is determined by (SFN+PF_offset) mod T =0   (where T= default paging cycle) that corresponds to the frame the includes the PDCCH carrying the indication.

The aspect of ‘consistency of the indication’ has been discussed in past metings. Now following the agreements made, upon receiving a availability indication(s) with value ‘1’ (in the bit map), UE would assume that the TRS resource set (associated to the bit in the bitmap) would be present starting from the corresponding reference point (at (SFN+PF_offset) mod T =0) for the duration of the validity duration (i.e. N*default paging cycle). Hence, after receiving first availability indication, UE would not need further availability indications to confirm the availability. If a new L1 availability indication with value ‘1’ is receveived and the reception would occur within a different paging cycle, i.e. later reference point, UE would assume the availability starting from that default paging cycle till end of validity duration, as illustrated in Figure 1. If, alternatively an ‘0’ is received, the UE would keep its earlier assumption of the presence.

Figure 1. Illustration of TRS availability assumption based on the L1 availability indication.

It was also confirmed that both paging DCI and PEI (DCI format 2_7) can be configured to indicate the availability. The PEI monitoring occasions are expected to occur in a frame before PO (i.e. set by by PEI-F_offset) as illustrated by Figure 2. Hence, the reference frame for validity duration can be different for PEI and paging DCI could be different if the PEI-F_offset is larger than PF_offset and the UE is associated to the first PF (e.g. (SFN + PF_offset) mod T). Hence, PEI would for example indicate the availability for status for default DRX cycle n, while paging DCI would indicate the availability status for default paing cycle n+1. Thus this could result the L1 availability indication to be different for PEI and paging DCI.

Observation: If PO and associated PEI fall to different default paging cycles, the L1 availability indication could be different in the paging DCI and PEI.

Following the discussion above, this would not be a problem from UE perspective as UE can always determine it’s assumption of TRS availability on the timing of latest L1 availability indication with value ‘1’. Hence, if UE receives L1 availability indication with value ‘1’ in PEI, it can assume the availability based on the associated default paging cycle (i.e. that contains the indication) and correspondingly if UE receives paging DCI (e.g. based on PEI indication) it can determine the availability assumption accordingly.

Observation: Having different L1 indication value in paging DCI and associated PEI would not cause any UE ambiquity as UE can determine the availability assumption based on the latest L1 availabilty indication with value ‘1’.

It could also be considered that if the L1 availability is in PEI that the reference point for the validity duration is determined always based on the associated PO. This would ensure alignment of the indications (while it is not required), but could result an ambiquity case. In the case that paging cycle of the UE is longer than the default DRX cycle, and two PF, n and n+1, are associated to same PEI, it could be that the PF’s (and corresponding POs) fall to different default paging cycles and hence the availability status could be different for these and therefore ‘aligned’ indication would not be necessary. In principle there is no problem with this approach either as network can choose whether it indicates the availability in the first place. Hence, if TRS would not be present in the latter default paging cycle associated to PF n+1, network could choose to indicate no availability in the PEI (nor in the paging DCI) or choose to make the TRS present upon the PEI transmission or at the next default paging cycle. However, as the current approach would allow better flexibility, it is preferred.

Figure 2. Illustration of PEI monitoring configuration

# TRS/CSI-RS occasions configuration

The TRS configuration for IDLE/Inactive mode UEs has been discussed in the past meetings.

In RAN1#104e, discussion was started whether TRS resource set configuration is constrained within the initial BWP or not. In RAN1#104bis-e, it was agreed that the frequency location is not costrained by the initial DL BWP:

Agreement: IDLE/INACTIVE mode UE is not expected to receive TRS/CSI-RS outside the initial DL BWP. - Configuration of the frequency location of TRS/CSI-RS occasion(s) for idle/inactive UEs is not restricted by initial BWP.

This is reflected in 38.214v17.0.0 Section 5.1.6.1.1 as follows:

##### 5.1.6.1.1 CSI-RS for tracking [text omitted] - the bandwidth and the frequency location of the NZP CSI-RS resource, is given by the higher layer parameter [nrofRBs], [startingRB] and [frequencyDomainAllocation] in a [TRS-ResourceSet] and applies to all resources in a [TRS-ResourceSet]. The [frequencyDomainAllocation] configuration is not restricted by initial DL BWP.

As per RRC, the frequencyDomainAllocation indicates the location of the first RE within PRB {0,1,23}), not the configuration of the bandwidth and starting location. Thus to align the text in 38.214 with the agreement made in RAN1#104bis-e, change would be required.

Observation: RAN1#104bis-e agreement on initial BWP not restrictin TRS-ResourceSet configuration is not correctly captured in TS38.214

In this context, in Section 5.2.2.3.1 is captured for the CONNECTED mode operation that if the startingRB<NBWPstart, where startingRB is the starting CRB index of the CSI-RS resource provided by frequencyDomainAllocation and NBWPstart is the BWP starting CRB. I.e. the CSI-RS start position is aliged with BWP start position. Correspondingly, the bandwith of the CSI-RS is aligned with the active BWP (i.e. if nrofRBs>NBWPsize+NBWPstart-Ninitial RB,then NCSI-RSBW=NBWPsize+NBWPstart-Ninitial RB,).

##### 5.2.2.3.1 NZP CSI-RS [text omitted] The bandwidth and initial common resource block (CRB) index of a CSI-RS resource within a BWP, as defined in Clause 7.4.1.5 of [4, TS 38.211], are determined based on the higher layer parameters nrofRBs and startingRB, respectively, within the CSI-FrequencyOccupation IE configured by the higher layer parameter freqBand within the CSI-RS-ResourceMapping IE. Both nrofRBs and startingRB are configured as integer multiples of 4 RBs, and the reference point for startingRB is CRB 0 on the common resource block grid. If startingRB<NBWPstart, the UE shall assume that the initial CRB index of the CSI-RS resource is Ninitial RB=NBWPstart, otherwise Ninitial RB=startingRB. If nrofRBs>NBWPsize+NBWPstart-Ninitial RB, the UE shall assume that the bandwidth of the CSI-RS resource is NCSI-RSBW=NBWPsize+NBWPstart-Ninitial RB, otherwise NCSI-RSBW=nrofRBs. In all cases, the UE shall expect that NCSI-RSBW≥min⁡(24, NBWPsize).

It is good to note that Section 5.2.2.3.1 considers frequency allocation given freqBand within the CSI-RS-ResourceMapping IE, while Section 5.1.6.1.1 for RRC_IDLE or RRC_INACTIVE UE discusses about nrofRBs and startingRB within the TRS-ResourceSet IE, thus there should not be confusion, but in order to ensure proper understanding this could also be claried as a part of the TP. Hence following TP is proposed to be considered:

Proposal: Adopt following TP to 38.214 Section 5.1.6.1.1:

##### 5.1.6.1.1 CSI-RS for tracking [text omitted] - the bandwidth and the frequency location of the NZP CSI-RS resource, is given by the higher layer parameter [nrofRBs], [startingRB] and [frequencyDomainAllocation] in a [TRS-ResourceSet] and applies to all resources in a [TRS-ResourceSet]. The [frequencyDomainAllocation] configuration is Bandwidth, nrofRBs, and the initial CRB index, startingRB, of the NZP CSI-RS resource configured by TRS-ResourceSet are not restricted by initial DL BWP.

# Conclusion

In this contribution we have discussed the the aspects related to providing potential TRS/CSI-RS occasion(s) available in connected mode to IDLE/INACTIVE-mode UEs.

In section 2 we have considered the possible use cases as well the presence indication related behavior and assumptions. First we make following proposal and observation:-

Observation: If PO and associated PEI fall to different default paging cycles, the L1 availability indication could be different in the paging DCI and PEI.

Observation: Having different L1 indication value in paging DCI and associated PEI would not cause any UE ambiquity as UE can determine the availability assumption based on the latest L1 availabilty indication with value ‘1’.

In section 3 we discussed the remaining open items relared to the TRS occasion configuration for the IDLE/INACTIVE mode UEs:-

Observation: RAN1#104bis-e agreement on initial BWP not restrictin TRS-ResourceSet configuration is not correctly captured in TS38.214

Proposal: Adopt following TP to 38.214 Section 5.1.6.1.1:

##### 5.1.6.1.1 CSI-RS for tracking [text omitted] - the bandwidth and the frequency location of the NZP CSI-RS resource, is given by the higher layer parameter [nrofRBs], [startingRB] and [frequencyDomainAllocation] in a [TRS-ResourceSet] and applies to all resources in a [TRS-ResourceSet]. The [frequencyDomainAllocation] configuration is Bandwidth, nrofRBs, and the initial CRB index, startingRB, of the NZP CSI-RS resource configured by TRS-ResourceSet are not restricted by initial DL BWP.

# References

- Mediatek, ” New WI: UE Power Saving Enhancements”, RP-193239, December 2019

- R1-2112943, Introduction of Rel-17 UE power saving enhancements, CR to 38.212

- R1-2112936, Introduction of UE power savings enhancements in NR, CR to 38.213

- R1-2112957, Introduction of NR UE power savings enhancements, CR to 38.214