3GPP TSG RAN WG1 #108-e	                                           	      R1-2202365

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.7.2

Source:	NEC

Title:	Remaining issues on DCI-based power saving adaptation

Document for:	Discussion

# Introduction

In RAN1#107bis-e, the RAN1 discussion on Rel-17 DCI based power saving adaptation has been mostly concluded [1]. There are still some remaining issues. In this contribution, we provide our views on the remaining issues of ‘DCI based power saving adaptation’.

# Discussion

## DCI Format Configuration

The following agreements have been made in RAN1#107bis-e [1]:

Agreement

- For Case 3 (i.e., 3 SSSG switching), the following is supported

  - 2-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviours

    - ‘00’ is Beh 2

    - ‘01’ is Beh 2A

    - ‘10’ is Beh 2B

    - ‘11’ is reserved

Note that:

- ‘00’ is Beh 2: stop monitoring SS sets associated with SSSG#1 and SSSG#2 and monitoring  of SS sets associated to SSSG#0

- ‘01’ is Beh 2A: stop monitoring SS sets associated with SSSG#0 and SSSG#2 and monitoring  of SS sets associated to SSSG#1

- ‘10’ is Beh 2B: stop monitoring SS sets associated with SSSG#0 and SSSG#1 and monitoring of SS sets associated to SSSG#2

Agreement

The configuration of PDCCH monitoring adaptation is common for DCI format x-1 and x-2, neither Alt 1 nor Alt 2 is agreed,

- Alt 1: Support RRC parameter configured for the PDCCH monitoring adaptation indication field presence/absence in DCI format x-2.

- Alt 2:  The configuration of PDCCH monitoring adaptation is common for DCI format x-1 and x-2, except skipping duration for DCI format x-2 (i.e., PDCCHSkippingDurationList-r17-x-2 for DCI format x-2). The PDCCH monitoring adaptation indication field for DCI format x-2 is dependent on PDCCHSkippingDurationList-r17-x-2 if configured.

For DCI format x-2, the configuration is intended to give more flexibility than DCI format x-1. The presence/absense of PDCCH monitoring adaptation field in DCI format x-2 can be determined by RRC parameter. Different configuration including bit-width of the field in DCI format x_1 and x_2 would give gNB more flexibility of indicating PDCCH monitoring adaptation.

Proposal 1: Support RRC parameter configured for the PDCCH monitoring adaptation indication field presence/absence in DCI format x-2.

## PDCCH Skipping Configuration

In RAN1#107 e-meeting, the following agreements have been made:

Confirm the following working assumption (extracted from RAN1#107-bis agreements)

Agreement

- For value X in Beh 1A, candidate skipping values are

  - Up to [100ms] length is supported,

    - The X is configured and indicated in the unit of slot.

      - Working assumption for candidate values for X

        - {1,2,3,…,20,30, 40, 50, 60, 80, 100} for 15 kHz SCS,

        - {1,2,3,…,40, 60, 80, 100, 120,160,200} for 30 kHz SCS,

        - {1,2,3,…,80, 120, 160, 200, 240, 320,400} for 60kHz SCS,

        - {1,2,3,…,160, 240, 320,400, 480, 640,800} for 120kHz SCS

  - FFS: additional symbol level / PDCCH monitoring period level skipping duration

Working Assumption

- The bit length of the candidate skipping values and SSSG switching initial timer values in slots for 480kHz and 960kHz SCS are assumed to be the same as that for 120KHz SCS

For PDCCH monitoring adaptation for an active BWP in active time, timer-based SSSG switching is supported. During the inactivity time, the UE can move to less frequent PDCCH monitoring upon timer expiry. PDCCH skipping means stopping PDCCH monitoring for a duration X. It is proposed that X can be multiples of PDCCH monitoring periodicity. PDCCH monitoring skipping would be applied after UE receives the PDCCH monitoring adaptation control signaling bit(s) in DCI.

Proposal 2: Support PDCCH monitoring skipping duration as multiple of PDCCH monitoring periodicity.

A UE determines a PDCCH monitoring occasion on an active DL BWP from the PDCCH monitoring periodicity, the PDCCH monitoring offset, and the PDCCH monitoring pattern within a slot. For search space set , the UE determines that a PDCCH monitoring occasion(s) exists in a slot with number  [4, TS 38.211] in a frame with number  if nfNslotframe,μ+ns,fμ-op,smod kp,s=0. The UE monitors PDCCH candidates for search space set  for  consecutive slots, starting from slot , and does not monitor PDCCH candidates for search space set  for the next  consecutive slots. ([2])

After a PDCCH monitoring slot, it may not be necessary for a UE to monitor every consecutive slot for a transmission burst in higher SCS, and PDCCH monitoring may need to be skipped in multiple-slot steps during Ts. It may also be interpreted as a kind of periodic PDCCH skipping. The current spec would require a scheduling DCI to indicate a PDCCH monitoring skipping duration for each skipping.

Proposal 3: Support consecutive PDCCH monitoring in multi-slot steps during Ts for UE Power Saving.

# Conclusion

In this contribution, we discussed some remaining issues of Rel-17 PDCCH monitoring adaptation for UE power saving, and have the following proposals:

Proposal 1: Support RRC parameter configured for the PDCCH monitoring adaptation indication field presence/absence in DCI format x-2.

Proposal 2: Support PDCCH monitoring skipping duration as multiple of PDCCH monitoring periodicity.

Proposal 3: Support consecutive PDCCH monitoring in multi-slot steps during Ts for UE Power Saving.

# References

- [1] RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting #107bis-e, e-Meeting, January 17th – 25th, 2022.

- [2] 3GPP TS 38.213 NR Physical layer procedures for control