3GPP TSG RAN WG1 #108	R1-2202384

e-Meeting, Feb 21th – Mar 3rd, 2022

Agenda item:		8.7.1.1

Source:	Nordic Semiconductor ASA

Title:					On paging early indication

Document for:		Discussion and Decision

# Introduction

RAN#93 RAN guidance helped, and good progress has been achieved in RAN1#107 and 107b. The FL summary after 107b meeting can be found in [1].

# Signaling details for PDCCH PEI

## PEI SS set config

Working assumption - SearchSpaceId = 0 can be configured for peiSearchSpace for the case of CORESET multiplexing pattern 2 or 3

In the end of RAN1#107b, above WA has been approved. It is not clear why in the end agreed as working assumptions, because there is clearly no technical issue with supporting SS#0 in pattern 2 and 3 for peiSearchSpace (PEI SS), similarly as there is no technical issue with supporting SS#0 also for pattern 1.  The only aspect here is that gNB should not configure frame offset between PEI and PO to be zero, if both Paging and PEI SS are SS#0. However, this misconfiguration can be clearly avoided.

Other aspect is the case when peiSearchSpace is not configured. Same principle should apply as for any other CSS in pdcch-ConfigCommon in NR, i.e. if not configured UE does not monitor PEI in the initial DL BWP.

Proposal-1: For CORESET multiplexing pattern 2 and 3, peiSearchSpace can be configured to be 0. If peiSearchSpace is not configured, UE does not monitor PEI in the initial DL BWP.

- No RAN1 spec change required.

It remained open whether a gap between PEI and PO MOs should be guaranteed or not. Given that UE is allowed to fall back to direct PO monitoring, specifying a gap seem to be a waste of precious online time.

Proposal-2: Conclude that no minimum gap between PEI and PO is defined.

Final issue which remained open is the FFS in below agreement.

Agreement If one PEI-O is associated with POs of 2 PFs, - The two PFs are consecutive PFs configured in SIB - FFS: two PFs are consecutive PFs within the same paging cycle - Note: As an example, SFN of the first PF of the PF(s) associated with the PEI-O can be obtained by: (SFN of UE’s PF) - iPO/Ns⋅T/N

If PFs are in different paging cycles, there is very large gap between PFs. Therefore, we support the clarification that the consecutive PFs shall be within the same default paging cycle.

Proposal-3: Inform RAN2 that from RAN1 point of view, two PFs are consecutive PFs within the same paging cycle.

## Detection performance aspects

It has been shown by at least one company that if DCI format payload size is known for the PEI PDCCH, PDCCH can be detected as a sequence, even though the detection performance may not be fully optimal. On the other hand, if multiple subgroups are configured then detecting presence of PEI becomes challenging.

When PEI carries sub-grouping indication, if UE is guaranteed wide-band DMRS within CORESET#0 or commonCORESET during PEI monitoring occasion, those could be used in the similar way as TRS, since PDCCH DMRS form a known signal of comb structure. The density of PDCCH DMRS is exactly the same as that of TRS, i.e. 3RS per PRB, and DMRS present within entire CORESET can provide both time and frequency synchronization benefit, given that such DMRS are transmitted in two consecutive symbols of the CORESET.

Moreover, DMRS transmitted in entire CORESET:

- may still maintain the assumption that precoder is fixed within REG .

- from rate-matching point view, dynamic CORESET rate-matching is mandatory feature which can ensure there is no collision between DMRS in a CORESET#0/CommonCORESET and PDSCH for RRC-connected UEs. For IDLE UE, such collision must be avoided by gNB according to 38.214 sub-clause 5.1.4.

- gNB may multiplex PDCCHs for multiple UEs in the same MO

Observation-1: For Idle UEs, gNB must avoid overlapping of CORESET#0/CommonCORESET and PDSCH. For RRC connected UEs, gNB may configure UE to rate-match dynamically around entire CORESET#0/CommonCORESET (as mandatory feature).

Observation-2: When gNB indicates transmitting PDCCH DMRS in entire CORESET#0/CommonCORESET, the UE may use DMRS as sequence-based detection of PEI presence and/or to facilitate PDCCH DMRS for consequent finer-synchronization. There is clear benefit for UE.

Proposal-4: Consider introducing PDCCH DMRS transmitted in an entire CORESET#0/CommonCORESET configured by SIB1/MIB during PEI monitoring occasions to facilitate sequence-based detection of PEI presence and/or to facilitate PDCCH DMRS for consequent finer-synchronization.

- narrow-band/wide-band precoding can be assumed in CORESET#0/CommonCORESET, i.e. no change from R15/R16 required.

- introduce parameter to indicate that DMRS are transmitted by gNB in entire CORESET

## 10.4A PDCCH monitoring for early indication of paging A UE can be provided the following for detection of a DCI format 2_7 in RRC_IDLE state or in RRC_INACTIVE state [12, TS 38.331] - a search space set, by peiSearchSpace, to monitor PDCCH for detection of DCI format 2_7 according to a Type2A-PDCCH CSS set as described in clause 10.1 - a number of frames, by PEI-F_offset, from the start of a first paging frame of paging frames associated with a number of PDCCH monitoring occasions for DCI format 2_7 [17, TS 38.304] to the start of a frame - a number of symbols, by firstPDCCH-MonitoringOccasionOfPEI-O, from the start of the frame to the start of the first PDCCH monitoring occasion for DCI format 2_7 - a size, by payloadSizeDCI_format2_7 - a number of subgroups per paging occasion, NSGPO, by subgroupsNumPerPO - a number of paging occasions associated with the number of PDCCH monitoring occasions for DCI format 2_7, NPOPEI, by PONumPerPEI A paging indication field of DCI format 2_7 includes NPOPEI segments of K bits, where K=NSGPO if NSGPO>0 and K=1 if NSGPO is not provided or NSGPO=0. For a subgroup index iSG, 0≤iSG<K, a UE determines a value for the iPO⋅K+iSG bit in the paging indication field, where iPO=UE_IDmodN⋅NS+iSmodNPOPEI is a paging occasion index, and UE_ID, N, NS, and iS are defined in [17, TS 38.304]. When the value is '1', the UE monitors a paging occasion determined according to [17, TS 38.304]; otherwise, the UE is not required to monitor the paging occasion. If UE is provided with peiSearchSpace-DMRS, UE assumes that PDCCH DM-RS are transmitted in all REGs of CORESET in monitoring occasion in which peiSearchSpace is monitored.

# Conclusions

Regarding the PEI design we had the following observations and proposals:

Proposal-1: For CORESET multiplexing pattern 2 and 3, peiSearchSpace can be configured to be 0. If peiSearchSpace is not configured, UE does not monitor PEI in the initial DL BWP.

- No RAN1 spec change required.

Proposal-2: Conclude that no minimum gap between PEI and PO is defined.

Proposal-3: Inform RAN2 that from RAN1 point of view, two PFs are consecutive PFs within the same paging cycle.

Observation-1: For Idle UEs, gNB must avoid overlapping of CORESET#0/CommonCORESET and PDSCH. For RRC connected UEs, gNB may configure UE to rate-match dynamically around entire CORESET#0/CommonCORESET (as mandatory feature).

Observation-2: When gNB indicates transmitting PDCCH DMRS in entire CORESET#0/CommonCORESET, the UE may use DMRS as sequence-based detection of PEI presence and/or to facilitate PDCCH DMRS for consequent finer-synchronization. There is clear benefit for UE.

Proposal-4: Consider introducing PDCCH DMRS transmitted in an entire CORESET#0/CommonCORESET configured by SIB1/MIB during PEI monitoring occasions to facilitate sequence-based detection of PEI presence and/or to facilitate PDCCH DMRS for consequent finer-synchronization.

- narrow-band/wide-band precoding can be assumed in CORESET#0/CommonCORESET, i.e. no change from R15/R16 required.

- introduce parameter to indicate that DMRS are transmitted by gNB in entire CORESET

# References

- R1-2200787	Summary#4 of Maintenance on Paging Enhancement,	MediaTek Inc