3GPP TSG-RAN WG1 Meeting #108-e	R1-2202389

e-Meeting, February 21st– March 3rd 2022

Agenda Item:	8.5.1

Source:	Ericsson

Title:	Techniques mitigating Rx/Tx timing delays

Document for:	Discussion, Decision

- Introduction

The problem of UE Rx/Tx timing differences between different antenna panels was one of the major topics studied in the Rel. 17 positioning enhancement study item and was identified as a serious bottle neck in reaching the full potential of the large bandwidths available for mm-wave positioning. Several techniques for mitigating these problems were also evaluated e.g. in [3]. In the study report, methods, measurements, signaling, and procedures for improving positioning accuracy in the presence of the UE Rx/Tx timing delays, and/or gNB Rx/Tx timing delays were recommended for normative work [2] and at the RAN plenary in December 2020, a Rel. 17 WID for positioning enhancements [1] was approved including the following objective:

- Specify methods, measurements, signaling, and procedures for improving positioning accuracy of the Rel-16 NR positioning methods by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, including [RAN1]

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

Within the WI, agreements have been made to support reporting of Timing Error Group (TEG) associations of timing measurements and reference signal transmissions.  In this contribution, we provide Ericsson’s views on some remaining issues.

- Mitigating UE Rx timing errors for DL TDOA

## Multiple sequential RSTD measurements towards the same TRP utilizing different RX TEGs

At RAN1#107-e the following agreement was made

---------------------------------------------- RAN1#107-e agreement ---------------------------------------------

Agreement

Make the following modification on the previous agreement made in RAN#106bis-e:

- Subject to UE capability, support the LMF to request a UE to optionally measure the same DL PRS resource of a TRP with N different UE Rx TEGs and report the corresponding multiple RSTD measurements.

    - N=[2, 3, 4, 6, 8] (FFS: other values), where the maximum value of N depends on UE capability, and applies to all DL PRS positioning frequency layers

    - Note: If N is not explicitly included in the request, it is up to UE to determine the number of different UE Rx TEGs to measure the same DL PRS resource within its capability

  - The TRP can be either a “RSTD” reference TRP or a neighbour TRP

  - FFS: details of the signalling, procedures, and UE capability

  - The timestamps of the multiple RSTD measurements in the same measurement report can be the same or different.

  - Note: All RSTD measurements are relative to a single reference timing

- Support the LMF to request a TRP to optionally measure the same SRS resource of a UE with M different TRP Rx TEGs and report the corresponding multiple RTOA measurements.

  - M = [2, 3, 4, 6, 8] (FFS: other values)  applies to all configured SRS resources for positioning

  - Note: If M is not explicitly included in the request, it is up to TRP to determine the number of different TRP Rx TEGs to measure the same SRS resources for positioning

  - FFS: details of the signalling, procedures

  - The timestamps of the multiple RTOA measurements in the same measurement report can be the same or different.

---------------------------------------------- end RAN1#107-e agreement ---------------------------------------------

The agreement to support a UE to measure the same DL PRS resource of a TRP with N different UE Rx TEGs and report the corresponding multiple RSTD measurement, enables estimation of timing error differences between different TEGs and is very efficient in mitigation of UE Rx timing errors as shown by the simulation results in Figure 1.

We note that multiple RSTD measurements of the same DL PRS resource with different TEGs could be achieved in a number of different ways depending on scenario, UE capabilities and DL PRS configuration. A UE could perform multiple RSTD measurements using different UE RX TEGs based on

- Option 1:  different symbols in the same slot of the same DL PRS resource

  - Useful e.g. for indoor industrial scenarios where a two symbol staggered comb 2 signals is sufficient to achieve a good TOA measurement. Measurements with four different TEGS could thus, easily fit into one slot.

- Option 2:  different repetitions of the same DL PRS resource

  - Useful for scenarios where more symbols are needed to achieve a good TOA measurement.

- Option 3:  different occasions of the same DL PRS resource

  - For long DL PRS periodicity timing errors may drift (e.g. due to temperature dependence of filter group delays) but at least for short periodicities this could be applicable.

- Option 4:  Simultaneous reception of the same DL PRS resource

  - for UEs that have such capabilities

Option 1 is suitable when a small number of symbols is enough to achieve coverage such as e.g. in IIOT scenarios. Option 2 is useful when a full symbol or more is needed to achieve coverage. Option 3 could be useful when the time between consecutive DL PRS occasions is short enough for UE RX TEGs to be valid. Option 4 is very useful to save resources in case the UE has simultaneous PRS reception capabilities. Thus, we propose that all these alternatives are supported.  In UE capability discussion, a capability for Option 4 is already agreed to be supported (i.e., UE feature row 27-1-4a).  Details on which among the other three options (i.e., Options 1-3) applies to the UE feature row 27-1-4 are yet to be agreed.  Hence, we make the following proposal:

Downselect which among the following options apply to UE feature row 27-1-4:

- (1)  UE performs multiple RSTD measurements towards the same TRP based on different repetitions of the same DL PRS resource,

- (2)  UE performs multiple RSTD measurements towards the same TRP based on different symbols of the same DL PRS resource,

- (3)  UE performs multiple RSTD measurements towards the same TRP based on different occasions of the same DL PRS resource.

A UE that has the capability and receives the request should try to perform and report the measurements. Just as for RSTD measurements in general, there is no guarantee that the UE can hear the signal and perform the measurements. However, as long as the DL-PRS is configured appropriately and is received with high enough SINR for a given UE RX TEG association, the UE shall measure and report the corresponding RSTD measurement. This should be captured by RAN4 requirements

Inform RAN4 with an LS that RAN4 requirements should capture that, subject to UE capability, a UE configured to perform and report multiple RSTD measurements towards the same TRP, utilizing different UE RX TEGs, shall report one RSTD measurement for each UE RX TEG association for which the DL PRS is received with an appropriate configuration and with high enough SINR.

Figure 1 Utilization of antenna panel (or UE RX TEG) info is seen to give a very big improvement but still fails to fully mitigate the UE RX timing errors and to fulfill Rel. 17 positioning accuracy requirements in the InF-SH scenario. However, the combination of two techniques, 1) utilization of antenna panel (or UE RX TEG) info, and 2) sequentially performing one TOA estimate for each antenna panel towards the same TRP, result in complete mitigation of the UE RX timing errors and fulfillment of Rel. 17 positioning accuracy requirements.

More details of the methods described here are given in R1-2103771 [7].

- Mitigating UE Tx timing errors for UL TDOA

## UE TX TEG association reporting

At RAN1#104bis-e, RAN1#105-e and RAN1#106bis-e the following agreements were made

---------------------------------------------- RAN1#104bis-e agreement ---------------------------------------------

Agreement:

Support the following for mitigating UE Tx timing errors and/or TRP Rx timing errors for UL TDOA

- Support a TRP to provide the association information of RTOA measurements with TRP Rx TEG(s) to the LMF when the TRP reports the RTOA measurements to the LMF if the TRP has multiple Rx TEGs

- Support a UE to provide under capability the association information of UL SRS resources for positioning with Tx TEGs to the LMF if the UE has multiple Tx TEGs

  - FFS: Whether to support a UE to provide the association information of UL SRS resources for MIMO with Tx TEGs to the LMF if the UE has multiple Tx TEGs

  - FFS: Whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF;

- FFS: the details of the Signaling, procedures, and UE capability

---------------------------------------------- end RAN1#104bis-e agreement ---------------------------------------------

---------------------------------------------- RAN1#105-e agreement ---------------------------------------------

Agreement:

- For mitigating UE Tx timing errors for UL TDOA, support  one of the following options:

  - Option 1:

    - Subject to UE’s capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE has multiple Tx TEGs.

    - FFS: Support LMF to forward the association information provided by the UE to the serving and neighboring gNBs

  - Option 2:

    - Subject to UE’s capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE has multiple Tx TEGs.

    - Support the serving gNB to forward the association information provided by the UE to the LMF

    - FFS: Support LMF to forward the association information from the serving gNB for the UE to the neighboring gNBs

- FFS: UE should be able to report capability information related to Tx TEGs to LMF via LPP signaling

- Support gNB to report the associated SRS resource ID/resource set ID of the RTOA measurement to LMF

---------------------------------------------- end RAN1#105-e agreement ---------------------------------------------

---------------------------------------------- RAN1#106bis-e working assumption ---------------------------------------------

Working assumption:

- For mitigating UE Tx timing errors for UL TDOA, subject to UE’s capability, support the serving gNB to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE supports multiple UE Tx TEGs for UL TDOA.

  - The serving gNB should forward the association information provided by the UE to the LMF.

    - FFS: whether to support the serving gNB to forward the association information to the neighboring gNBs

  - UE should report its capability of supporting multiple UE Tx TEGs for UL TDOA to serving gNB.

- For mitigating UE Tx timing errors for Multi-RTT, subject to UE’s capability, support the LMF to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE supports multiple Tx TEGs for Multi-RTT.

  - FFS: whether to support the LMF to forward the association information to the serving and neighboring gNBs

  - UE should report its capability of supporting multiple UE Tx TEGs for Multi-RTT directly to the LMF.

- FFS: Mitigation of UE Tx timing errors when Multi-RTT, UL-TDOA and/or DL-TDOA are used.

---------------------------------------------- end RAN1#106bis-e agreement ---------------------------------------------

Regarding the FFS in the agreement at RAN1#104bis-e on support for SRS for MIMO

  - FFS: Whether to support a UE to provide the association information of UL SRS resources for MIMO with Tx TEGs to the LMF if the UE has multiple Tx TEGs

We note that large amounts of precious UL resources can be saved if UL SRS resources can be reused for multiple purposes. We therefore propose that

The UE can be configured by the gNB to send UE TX TEG association reports for all SRS types including SRS for MIMO.

At RAN1#106bis-e the following proposal was discussed but not agreed:

---------------------------------------------- RAN1#106bis-e proposal ---------------------------------------------

Proposal 3.2-2b

- The UE can be configured with a list of SRS resource sets for which UE TX TEG association reporting should be performed.

- It shall be possible to configure a UE with an SRS with a restriction for the UE to utilize a certain UE TX TEG when transmitting the SRS.

---------------------------------------------- end RAN1#106bis-e proposal ---------------------------------------------

The UE obviously needs to know for which UL SRSs to report the TX TEG association. Thus, we are pro to the first bullet (the second bullet is discussed in section 3.2):

The UE can be configured by the gNB with a list of SRS resource sets and SRS resources for which UE TX TEG association reporting should be performed. In case an SRS resource set is listed rather than an SRS resource then the UE should report the UE TX TEG association for all SRS resources in the SRS resource set.

## Transmission of one UL SRS from each UE antenna panel (or UE TX TEGs) towards the same TRP

Providing the TX TEG association information of UL SRS transmissions to the LMF gives a remarkable improvement in positioning accuracy in the InF-SH scenario but still fails to fully mitigate the UE TX timing errors and to fulfill Rel. 17 positioning accuracy requirements as can be seen in Figure 2. The reason for this failure is that one TRP per UE antenna panel is wasted since it’s used as a reference. Thus, as many measurements are lost as there are antenna panels.

Figure 2 Utilization of UE antenna panel (or UE TX TEG) info is seen to give a very big improvement but still fails to fully mitigate the UE TX timing errors and to fulfill Rel. 17 positioning accuracy requirements in the InF-SH scenario. However, the combination of two techniques, 1) utilization of UE antenna panel (or UE TX TEG) info, and 2) sequential transmission of one UL SRS from each UE antenna panel towards the same TRP, result in complete mitigation of the UE TX timing errors and fulfillment of Rel. 17 positioning accuracy requirements.

The solution is to configure the UE to transmit one UL SRS from each of it’s different UE antenna panels (or UE TX TEGs) ‘towards’ the same TRP. If the TRP can hear the SRSs transmitted from two UE different antenna panels (or TX TEGs) and perform the UL RTOA measurement for each of them, then the UE TX timing error difference between the two antenna panels (or UE TX TEGs) can be calculated as the difference between the two UL-RTOA measurements. Note that it’s enough that one TRP can do this to estimate the timing error difference. The LMF can use the estimated timing error differences to compensate for the timing error differences in positioning the UE and thus removes the need to waste measurements from some TRPs as references. Combining this method with knowledge of the UE TX TEG association of SRS transmissions, results in full mitigation of the UE TX timing errors and fulfillment of the Rel. 17 positioning accuracy requirements in the InF-SH scenario as can be seen in Figure 2.

In Rel. 16, there exist some capability to let the UE sweep an SRS over different TX beams and thus potentially also over different TEGs. This is accomplished by configuring an SRS resource set with multiple SRS resources. The SRS resources correspond to different beams and potentially different UE TX TEGs/antenna panels.

With a minor tweak, this mechanism could be utilized to make the UE sweep over UE TX TEGs over the SRS resources in a SRS resource set. In order for the gNB to know how many SRS resources to configure in each SRS resource set, the gNB also needs to know how many UE TX TEGs the UE is using. Thus, the UE also needs to report the number of UE TX TEGs as part of UE capabilities.

The gNB can then configure one SRS resource set for each TRP with a pathloss reference towards the SSB or a DL PRS transmitted by the TRP. Each SRS resource set would consist of as many SRS resources as there are UE TX TEGs (as reported as part of UE capabilities). Each SRS resource would be configured with a spatial relation towards the SSB or a DL PRS transmitted by the TRP.

The UE would then transmit each SRS resource with a different TX TEG, using the best beam possible given the selected TX TEG and the spatial relation. As already agreed, the UE would also report the UE TX TEG association of each SRS resource.

Support UE TX TEG sweeping over SRS resources for positioning in a SRS resource set configuration.

The UE shall report the number of UE TX TEGs as part of UE capabilities.

At RAN1#106bis-e the following proposal was discussed but not agreed:

---------------------------------------------- RAN1#106bis-e proposal ---------------------------------------------

Proposal 3.2-2b

- The UE can be configured with a list of SRS resource sets for which UE TX TEG association reporting should be performed.

- It shall be possible to configure a UE with an SRS with a restriction for the UE to utilize a certain UE TX TEG when transmitting the SRS.

---------------------------------------------- end RAN1#106bis-e proposal ---------------------------------------------

The second bullet (the first bullet is discussed in section 3.1) would allow for an alternative mechanism for UE TX TEG sweeping. It would give the NW the possibility to control how the UE sweeps UE TX TEGs. Since the NW already knows what UE TX TEG that will be used this also makes the reporting of UE TX TEGs redundant leading to a saving of UL resources. The restriction should be on the SRS resource level. We are therefore supportive of bullet 2 with the clarification that the restriction is configured on the SRS resource level:

It shall be possible to configure a UE with an SRS resource with a restriction for the UE to utilize a certain UE TX TEG when transmitting the SRS

At RAN1#106bis-e the following proposal was discussed but not agreed:

## Beam and antenna panel sweeping SRS

The number NSRS of SRS’s needed for the multi antenna panel SRS transmission scheme described in section 3.2 above is is the number NTRP of TRPs times the number Ndelay groups of UE TX TEGs at the UE, NSRS=NTRP∙Ndelay groups , as illustrated in Figure 3.

Figure 3 The number of SRS’s needed for the multi antenna panel scheme based on configuration to use certain delay groups is the number of TRPs times the number of delay groups at the UE.

An alternative multi antenna panel SRS transmission scheme would be to let the UE transmit SRSs sweeping over all UE TX TEGs and transmit beams, as illustrated in Figure 4. All TRPs would then try to receive all SRSs, but would typically only manage to receive and perform measurements based on some of them.

Figure 4 The number of SRS’s needed for the multi antenna panel scheme based on delay group and beam sweeping is the number of delay groups at the UE times the number of UE beams per delay group, i.e. the total number of UE beams utilizing all delay groups.

The number NSRS of SRS’s needed for the multi antenna panel scheme based on beam and delay group sweeping is simply the total number of UE transmit beams, which can be expressed as the number Nbeams per TEG of beams per UE TX TEG times the number NTEG of UE TX TEGs at the UE, NSRS=Nbeams per TEG∙NTEG .

Beam and UE TX TEG sweeping is thus more resource effective if there are more TRPs that can hear the UE than there are UE beams per UE TX TEG, i.e. if

NTRP>Nbeams per TEG .

According to the agreed common scenario parameters applicable for all scenarios, each FR2 antenna panel (i.e., each delay group) has 2x4 spatially separated antenna elements corresponding to 8 spatial non-oversampled DFT beams. In the InF scenario, there are 18 BSs, each consisting of a three-sector site and typically a UE can be heard by almost all BSs. Thus, the beam and UE TX TEG sweeping scheme is more resource efficient in this scenario.

This can be achieved by reusing the Rel. 16 SRS resource and resource set structure in a similar way as in section 3.2. In order to know how many SRS resources to configure the gNB also needs to know the total number N of UE beams, i.e.

N=i=1nr of UE TX TEGsNi

where Ni is the number of beams for UE TX TEG i. The total number of UE beams needed to sweep all UE TX TEGs and all directions should therefore be reported as part of UE capabilities.

The gNB can then configure one SRS resource set with a pathloss reference towards the serving cell SSB or a DL PRS transmitted by the serving cell. Each SRS resource set would consist of as many SRS resources as the total number of UE beams (as reported as part of UE capabilities). The SRS resources would be configured without spatial relations to indicate to the UE that sweeping should be performed over all directions.

The UE would then transmit each SRS resources with different beams and UE TX TEGs, sweeping over all UE TX TEGs and directions. As already agreed, the UE would also report the UE TX TEG association of each SRS resource.

To support this mechanism, we thus propose that

The total number of UE beams needed to sweep all UE TX TEGs and all directions should be reported as part of UE capabilities

An alternative way to accomplish the same thing would be define an SRS for positioning with multiple ports, using the ports to sweep over the different antenna panels and beams, i.e. using antenna switching as proposed by some companies. To achieve the multi TRP coverage needed for positioning, ports would need to be separated based on time division. Using TD to separate ports the difference between antenna switching and beam and antenna panel sweeping would be very small.

Beam and UE TX TEG sweeping can be achieved by using multiple SRSs for sweeping or by using multiple ports of one SRS for sweeping. The latter alternative may be viewed as antenna switching. To achieve the multi TRP coverage needed for positioning, ports would need to be separated based on time division (TD). Using TD to separate ports the difference between antenna switching and beam and antenna panel sweeping would be very small.

However, since the SRS for positioning is single port, we think it’s more simple to use multiple SRS resources to sweep over beams and antenna panels. This fits well with the Rel. 16 SRS resource set structure and also fits well with the UE TX TEG concept.

- Mitigating UE Tx/Rx timing errors for DL+UL positioning

## Timing drifts and timing adjustments

At RAN1#106bis-e the following proposal was made by feature lead but not agreed

---------------------------------------------- RAN1#106bis-e proposal ---------------------------------------------

(Round 3)Proposal 3.3-2a(H)

- Consider supporting one of the following alternatives related to the UE Rx-Tx time difference (decision to be made in RAN1#106b):

  - Option 1:

    - Subject to UE capability, the UE may report an additional UL Timestamp associated to a UE Rx-Tx measurement, corresponding to the timing of the uplink subframe of a positioning SRS.

    - Add the following to the UE Rx-Tx time difference definition (similar to the definition for HD-FDD UE in TS 36.214):

      - If the UE does not transmit SRS in subframe #j, and if the UE reports an additional timestamp for the positioning SRS associated to the measurement, it shall compensate for the difference in the transmit timing of uplink subframe #j and the transmission timing of the subframe containing positioning SRS.

  - Option 2:

    - Subject to a UE capability, a UE may optionally report Timing Adjustment (TA) change information

      - Option 2A: The TA change information is included in the UE Tx TEG report

      - Option 2B: The TA change information is included in the Rx-Tx measurement report

      - Note: TA change information corresponds to: Tx Timing change with a timestamp that this change occurred.

  - Option 3:

    - Send an LS to RAN4, requesting RAN4 to make the decision to select Option 1 or Option 2

- If RAN1 makes the decision to adopt either Option 1 or Option 2, send an LS to RAN4 to check if RAN4 has any issue to support the option.

---------------------------------------------- end RAN1#106bis-e proposal ---------------------------------------------

We don’t think either of the two proposals (option 1 or option 2) are in a shape where the decision could be left to RAN4. We think further discussions are needed to reach a working solution that is acceptable to all parties.

We are proponents of reporting of TA change information but will here try to formulate a compromise proposal which is closer to option 1.

The main problem with option 1 is that the UE doesn’t know which SRS the gNB will use to perform the gNB Rx-Tx time difference measurement that will be used together with a UE Rx-Tx time difference measurement to estimate the RTT. This can, however, be solved in the following manner. Instead of modifying the UE Rx-Tx time difference measurement definition, the UE can report a compensation term for each SRS resource configured by the LMF for transmission timing compensation. Thus, we make the following proposal

To mitigate transmission timing changes for multi-RTT measurements:

- Support the LMF to configure a UE (with required capabilities) with a list of SRS resource sets and SRS resources for each TRP for reporting of transmission timing compensation.

  - In case, an SRS resource set is listed, then that should be interpreted as including all SRS resources in the SRS resource set.

- Given a UE Rx-Tx time difference measurement reported in a multi RTT report, the UE should also report a transmission timing compensation for each SRS resource indicated for transmission timing compensation.

  - The transmission timing compensation is signaled together with two timestamps:

    - A first timestamp for the UL subframe #j closest in time to the DL subframe #i in which the DL PRS used for the UE Rx-Tx time difference measurement is received

    - A second timestamp for the subframe #k where the TX instance of the SRS resource occurs which is closest in time to the reception time of the DL PRS used for the UE Rx-Tx time difference measurement.

  - Transmission timing compensation is defined as the difference in transmission timing between the subframe #k and subframe #j.

  - Transmission timing is defined as the time between the transmission of UL subframe #i and the first detected path (in time) of the corresponding downlink subframe #i from the reference cell.

  - A transmission timing compensation in a multi-RTT report is only coupled to the two timestamps and is thus in the report not tied to a specific SRS resource or UE Rx-Tx time difference measurement.

Note that the transmission timing compensation can often be the same (i.e. based on the same two timestamps) for multiple combinations of UE Rx-Tx time difference measurements (towards different TRPs) and SRS resources. Since transmission timing compensation isn’t tied to a specific SRS resource or UE Rx-Tx time difference measurement in the multi-RTT report, reporting of duplicate transmission timing compensation is avoided.

This proposal avoids changing the definition of the UE Rx-Tx time difference measurement which was opposed by many companies. At the same time this proposal can achieve exactly the same transmission timing change mitigation as option 1. In addition, multiple SRS resources can be handled in such a way that UE Rx-Tx time difference measurement, gNB Rx-Tx time difference measurements and transmission timing compensation terms can be correctly matched by the LMF.

We note that our concerns regarding the limited possibilities for the UE to compensate for UE autonomous timing adjustments and TX timing changes are still valid. Since UE manufacturers are reluctant to report timing adjustments explicitly, we are, however, willing to leave for UE implementation how to estimate transmission timing compensation terms based on e.g. timing adjustments and reference cell TOA estimates.

We note, however, that the accuracy of the UEs estimation of transmission compensation terms depends on circumstances such as

- availability of SSB and TRS from transmission timing reference cell (i.e. serving cell)

- timing of SSB/TRS relative to DL PRS and UL SRS

- TRS, periodicity and bandwidth of TRS

- UE DRX mode

- UE transmission state changes causing changes in temperature and thus also UE oscillator frequency drifts

- Timing adjustments

We therefore think that the transmission timing compensation should be complemented with a measurement quality indication, giving a measure of the deviation of the transmission timing compensation from the true difference in transmission timing between subframe #k and subframe #j.

The transmission timing compensation should be reported by the UE to the LMF together with a quality estimate, giving a measure of the deviation of the transmission timing compensation from the true difference in transmission timing between UL subframe #k of UL SRS transmission and UL subframe #j closest in time to DL PRS reception.

In the subsection below we discuss in more detail how the UE can estimate transmission timing changes under different circumstances.

### UE estimation of transmission timing differences

Transmission timing is defined in 38.133:

The uplink frame transmission takes place  before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. For serving cell(s) in pTAG, UE shall use the SpCell as the reference cell for deriving the UE transmit timing for cells in the pTAG. For serving cell(s) in sTAG, UE shall use any of the activated SCells as the reference cell for deriving the UE transmit timing for the cells in the sTAG.

One may note here that if there is no carrier aggregation (CA) or dual connectivity (DC), the reference cell is simply the single serving cell. In case of CA/DC the reference cell is one of the serving cells.

The RAN4 requirements for TX timing are very relaxed but in practice the accuracy is much better or at least the drift with time is typically small.

To keep track of uplink frame timing the UE typically tunes an oscillator to the RX frequency of the reference cell (using e.g.  the CSI-RS for tracking transmitted from the reference cell for tuning) and use that oscillator as a UE clock. In addition, the UE regularly measures the time of arrival of the first path (again using e.g. the CSI-RS for tracking) to check the uplink frame timing relative to the DL frame timing. If a deviation from the stipulated relative timing is seen, then an autonomous timing adjustment of the UL frame timing is performed in accordance with rules described in 38.133. The UE also adjusts the UL frame timing after receiving a TA command, changing NTA.

The required accuracy of the uplink frame transmission timing is very low. It may deviate with ±Te  around the stipulated offset of NTA+NTA offset×Tc, where Te is between 3∙64∙Tc≈98ns and 12∙64∙Tc≈391ns depending on subcarrier spacing. These requirements have been set based on the initial frame timing e.g. after a DRX cycle under the assumption that there has been a SSB available to the UE within the previous 160ms. One may note that if it’s not initial frame timing and the UE is using a full bandwidth TRS (CSI-RS for tracking) to track frequency and time, the UE should be able to follow the stipulated frame timing with extremely much better precision. There are, however, no such requirements.

Tuning to the DL frequency means that the UE clock is affected by doppler shifts in such a way that it compensates for the movements of the UE to keep the UL TX  frame timing relative to the DL RX frame timing constant even though the propagation time changes. Still, due to imperfect tuning and/or disruptive channel events, such as blocking of the first path or unblocking of a new first path, the TX UL frame timing relative to the DL RX frame timing does change, eventually resulting in timing adjustments.

UE movements and the corresponding change in propagation time also result in misalignment of RX UL frames at the gNB, triggering the gNB to send a TA command to the UE.

So, what do we mean then with a change in TX UL frame timing? Clearly we don’t mean a change relative to the gNB time or some absolute time since the UE has no such time and isn’t even striving to achieve any such time. The UE time should be ‘dopplershifted’ relative to gNB time and absolute time. The UE should only strive to keep the UL TX  frame timing relative to the DL RX frame timing constant.

A change in TX UL frame timing must therefore mean a change in UL TX frame timing relative to the DL RX frame timing of the reference cell (as defined in 38.133 - not positioning reference cell). There are four types of such changes

- Timing adjustments due to a TA command from the gNB

- Changes due to disruptive channel events, such as blocking of the first path or unblocking of a new first path in the channel of the reference cell

- Changes due to imperfect tuning to the RX DL from the reference cell or drift of the UE oscillator frequency between DL frequency estimates used for tuning

- Autonomous timing adjustments to correct for changes of type 2 or type 3 above, once detected by the UE

Type 1, timing adjustments due to a TA command from the gNB, clearly need to be compensated for to achieve accurate RTT estimation when they occur between the RX of the DL PRS used for the UE Rx-Tx time difference measurement and the TX of an UL SRS used for the gNB Rx-Tx time difference measurement.

Type 2 and type 3 changes are not known by the UE until at some point the UE measures the TOA of the first path of the reference cell. When such a change is discovered the UE compensates for that change by performing a type 4 timing adjustment. According to requirements, adjustments are only needed to the extent that the deviation is kept within ±Te around the stipulated offset of NTA+NTA offset×Tc.

Let’s now discuss a few examples and check to what extent changes in TX timing can be compensated for. Below, we will for simplicity refer to the transmission timing reference cell as the serving cell.

Figure 5 Example 1 of events impacting a UE Rx-Tx time difference measurement and timing difference compensation

In the first example illustrated in Figure 5, the following events occur:

- The UE receives the DL PRS from the target TRP at time instance A, measures the TOA and use that to calculate the UE Rx-Tx time difference

- The UE receives the TRS from the serving cell at time instance B and measures the TOA

  - Based on the TOA the UE calculates the transmission timing offset ToffsetB. The UE decides not to perform a timing adjustment since the timing offset ToffsetB is close enough to the stipulated offset NTA+NTA offset×Tc.

- The UE receives the TRS from the serving cell at time instance C and measures the TOA

  - Based on the TOA the UE calculates the transmission timing offset ToffsetC. The UE decides to perform a timing adjustment since the timing offset ToffsetC is too far from the stipulated offset NTA+NTA offset×Tc.

- The UE performs a timing adjustment of size TTAD.

  - The natural value for the timing adjustment is TTAD=NTA+NTA offset×Tc-ToffsetC bringing the transmission timing offset back to the stipulated value but the UE may also perform a smaller adjustment

- The UE transmits the UL SRS with transmission timing offset ToffsetE

We can write the timing offset difference between time instance A and E as

ToffsetE-ToffsetA=TdriftA,B+TdriftB,C+TdriftC,E+TTAD

Here we note that TdriftB,C and TTAD are known to the UE and we can write

ToffsetE-ToffsetA=TdriftA,B+TdriftC,E+TCOMP

where

TCOMP=TdriftB,C+TTAD

We note that if the timing offset is on target in time instance B ( ToffsetB=NTA+NTA offset×Tc) and the UE performs the natural timing adjustment TTAD=NTA+NTA offset×Tc-ToffsetC in time instance E then

TCOMP=TdriftB,C+TTAD=ToffsetC-ToffsetB+TTAD=0

However, due to the very relaxed requirements for transmission timing the timing offset in time instance B could be far off target without necessarily triggering a timing adjustment. In such cases TCOMP could be large in comparison to the drifts TdriftA,B+TdriftC,E, and the compensation term could thus be useful.

Figure 6 Example 2a of events impacting a UE Rx-Tx time difference measurement and timing difference compensation

In the second example illustrated in Figure 6, the following events occur:

- The UE receives the TRS from the serving cell at time instance A and measures the TOA

  - Based on the TOA the UE calculates the transmission timing offset ToffsetA. The UE decides not to perform a timing adjustment since the timing offset ToffsetA is close enough to the stipulated offset NTA+NTA offset×Tc.

- The UE receives the DL PRS from the target TRP at time instance B, measures the TOA and use that to calculate the UE Rx-Tx time difference

- The UE receives the TRS from the serving cell at time instance C and measures the TOA

  - Based on the TOA the UE calculates the transmission timing offset ToffsetC. The UE decides to perform a timing adjustment since the timing offset ToffsetC is too far from the stipulated offset NTA+NTA offset×Tc.

  - Note that ToffsetC=ToffsetA+TdriftA,C is due to the drift between event A and event C, and possibly also to drifts occurring before event A as embodied by ToffsetA.

- The UE performs a timing adjustment of size TTAD.

  - The natural value for the timing adjustment is TTAD=NTA+NTA offset×Tc-ToffsetC bringing the transmission timing offset back to the stipulated value but the UE may also perform a smaller adjustment

- The UE transmits the UL SRS with transmission timing offset ToffsetE

We can write the timing offset difference between time instance B and E as

ToffsetE-ToffsetB=TdriftB,E+TTAD

Here we note that the UE has no knowledge on the transmission timing drift TdriftB,C or any parts of that. Only the timing adjustment TTAD is known to the UE. Thus we can write

ToffsetE-ToffsetA=TdriftB,E+TCOMP

where

TCOMP=TTAD

If the time between events A and B is much larger than the time between events B and E, we expect that the drifts that the timing adjustment compensates for to a large extent happened before the reception of the DL PRS (event A) and thus that
TCOMP=TTAD≫TdriftB,E

Thus, in this case the compensation term is useful.

If on the other hand, the time between events B and C is of the same order of magnitude as the time between events A and B as illustrated in Figure 7, then the timing adjustment D may compensate as much for the drift between B and C as for drifts before A. As a consequence the compensation term is useless and could even make the RTT  estimate worse.

Figure 7 Example 2b of events impacting a UE Rx-Tx time difference measurement and timing difference compensation

We note that the compensation term depending on the circumstance can consist of

- timing adjustment

- estimated transmission timing drift based on multiple TOA measurements of the serving cell

- sum of timing adjustment and estimated transmission timing drift

We note also that in some circumstances, the compensation term results in an improvement in RTT estimation accuracy while in other circumstances no improvement is achieved.

We conclude that to make transmission timing compensation useful it’s necessary that the transmission timing compensation term is reported together with a quality measure, giving a measure of the deviation of the transmission timing compensation term from the true difference in transmission timing.

To make transmission timing compensation useful, it’s necessary that the transmission timing compensation term is reported together with a quality measure, giving a measure of the deviation of the transmission timing compensation term from the true difference in transmission timing.

Based on the size of transmission timing compensation term and quality indication, the LMF can then use a number of alternative strategies such as e.g.

- Discard or downweigh measurements that are impacted by timing transmission timing compensation

  - Based e.g. on transmission timing compensation quality indication and transmission timing compensation size

- Compensate RTT estimates using reported transmission timing compensation

- Estimate RTT based without the use of reported transmission timing compensation

- Duplicate RTT estimates and compensate one of the duplicate RTT estimates with the reported timing adjustment, using an outlier rejection method like e.g. RANSAC to reject the faulty one.

- Estimate RTT, selecting the gNB Rx-Tx time difference measurement based on an SRS such that the corresponding transmission timing compensation has a good quality indication.

- Handling of timing error variations with time

Timing errors may vary with time. This could e.g. be due to variations in temperature with time and a dependence of filter group delays on temperature. It could also be due to a UE transmission timing adjustment.

One possibility to handle some types of variations with time could be to use the timestamps of the measurements. Two measurements that have been performed close in time and are in the same delay group could be assumed to have a small timing error difference while this would not be the case if they are either in different delay groups or if the timespan between the two measurements is large. UE requirements could be defined to specify how much timing errors would be allowed to vary over a certain time period.

One problem with this is that timing errors might also change due to more instantaneous processes such as UE timing adjustments. One possibility to handle such events would be for the UE to report them. The network would then know that if an event has occurred between measurements/ transmissions performed before and after the event then the timing error differences can’t be assumed to be small even if the same TEG was used for both measurements/transmissions. Such event reporting could be combined with time stamp based methods to handle more continuous processes.

Since filter group delays typically depend on temperature, temperature variations is one source of changes in timing errors. Fast variations of temperature are in turn due to changes in power consumption in the UE e.g. due to turning on and off UE transmission. Such state changes are known by the UE and could be used to understand if there is a risk for timing error changes or not.

One possibility to handle all these different types of changes in timing errors would be to let the UE maintain a temporal timing error index (TTEI). The current value of the TTEI would be reported together with each reported TEG association. The timing error difference between two measurements based on the same TEG would be smaller than the margin if the difference in reported TTEI is smaller than a fixed value of N index steps. However, if the TTEI difference is larger than or equal to N, then the timing error difference can’t be assumed to be smaller than the margin. The TTEI could be increased at a steady low pace (linearly with time) during periods when the UE state in terms of power consumption is stable. In a transition period after a UE power consumption state change the index could be increased at a higher pace. At the time instance when a UE performs an autonomous timing adjustment, the index could be incremented with a fixed numbers of steps. In this way, processes that are linear as well as nonlinear in time can be accounted for. The TTEI obviously needs to have a fixed size and when reaching it’s maximum it will restart from zero. TTEI differences will thus need to be calculated in a circular fashion. The size needs to be big enough to allow the LMF to distinguish when the TTEI index has restarted from zero. Note, however, that if the TTEI index has been incremented with N steps since the last report to the LMF, the UE can pause the index without impacting TEG validation in time. Doing so, the difference in TTEI between two subsequently reported TTEI will never be larger than N. The size of the TTEI would thus not have to be larger than N+1. Since mod(k+N,N+1)=k-1 a subsequent reports would then never go a full circle in index value, allowing the LMF to keep track of TTEI loops.

Note that for three subsequent measurements A, B and C, associated to the same TEG the timing error difference could be ensured to be below the margin for the pairs {A, B} and {B, C} but not for the pair {A, C} due to the larger accumulated timing error drifts between A and C. This fact can be captured by the TTEI, e.g. by stepping up the TTEI with N-1 steps between A and B as well as between B and C.

Support UE to maintain a UE RX temporal timing error index (TTEI). The state of the UE RX TTEI at the instance of DL PRS reception for an RSTD or UE Rx-Tx time difference measurement should be reported together with UE RX TEG association, timestamp and RSTD/UE Rx-Tx time difference measurement in the DL-TDOA/multi-RTT measurement report.

Support UE to maintain a UE TX temporal timing error index (TTEI). The state of the UE TX TTEI at the instance of UL SRS transmission should be reported together with UE TX TEG association and timestamp.

- Conclusion

In the previous sections we made the following observations:

Observation 1	Beam and UE TX TEG sweeping can be achieved by using multiple SRSs for sweeping or by using multiple ports of one SRS for sweeping. The latter alternative may be viewed as antenna switching. To achieve the multi TRP coverage needed for positioning, ports would need to be separated based on time division (TD). Using TD to separate ports the difference between antenna switching and beam and antenna panel sweeping would be very small.

Observation 2	To make transmission timing compensation useful, it’s necessary that the transmission timing compensation term is reported together with a quality measure, giving a measure of the deviation of the transmission timing compensation term from the true difference in transmission timing.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Downselect which among the following options apply to UE feature row 27-1-4:

(1)  UE performs multiple RSTD measurements towards the same TRP based on different repetitions of the same DL PRS resource,

(2)  UE performs multiple RSTD measurements towards the same TRP based on different symbols of the same DL PRS resource,

(3)  UE performs multiple RSTD measurements towards the same TRP based on different occasions of the same DL PRS resource.

Proposal 2	Inform RAN4 with an LS that RAN4 requirements should capture that, subject to UE capability, a UE configured to perform and report multiple RSTD measurements towards the same TRP, utilizing different UE RX TEGs, shall report one RSTD measurement for each UE RX TEG association for which the DL PRS is received with an appropriate configuration and with high enough SINR.

Proposal 3	The UE can be configured by the gNB to send UE TX TEG association reports for all SRS types including SRS for MIMO.

Proposal 4	The UE can be configured by the gNB with a list of SRS resource sets and SRS resources for which UE TX TEG association reporting should be performed. In case an SRS resource set is listed rather than an SRS resource then the UE should report the UE TX TEG association for all SRS resources in the SRS resource set.

Proposal 5	Support UE TX TEG sweeping over SRS resources for positioning in a SRS resource set configuration.

Proposal 6	The UE shall report the number of UE TX TEGs as part of UE capabilities.

Proposal 7	It shall be possible to configure a UE with an SRS resource with a restriction for the UE to utilize a certain UE TX TEG when transmitting the SRS

Proposal 8	The total number of UE beams needed to sweep all UE TX TEGs and all directions should be reported as part of UE capabilities

Proposal 9	To mitigate transmission timing changes for multi-RTT measurements:

Proposal 10	The transmission timing compensation should be reported by the UE to the LMF together with a quality estimate, giving a measure of the deviation of the transmission timing compensation from the true difference in transmission timing between UL subframe #k of UL SRS transmission and UL subframe #j closest in time to DL PRS reception.

Proposal 11	Support UE to maintain a UE RX temporal timing error index (TTEI). The state of the UE RX TTEI at the instance of DL PRS reception for an RSTD or UE Rx-Tx time difference measurement should be reported together with UE RX TEG association, timestamp and RSTD/UE Rx-Tx time difference measurement in the DL-TDOA/multi-RTT measurement report.

Proposal 12	Support UE to maintain a UE TX temporal timing error index (TTEI). The state of the UE TX TTEI at the instance of UL SRS transmission should be reported together with UE TX TEG association and timestamp.

# References

RP-202900, New WID on NR Positioning Enhancements, 3GPP TSG RAN Meeting #90e, December 7 - 11, 2020

3GPP TR 38.857 V1.0.0 (2020-12) Study on NR Positioning Enhancements; (Release 17)

R1-2008765, Potential positioning enhancements, Ericsson, 3GPP TSG-RAN WG1 Meeting #103-e, October 26th – November 13th, 2020

RP-210771, Terms of Reference (ToR) for 3GPP TSG RAN WG3 (RAN3), RAN#91-e, 16-26 March 2021

RP-210874, Terms of Reference (ToR) for 3GPP TSG RAN WG1 (RAN1), RAN#91-e, 16-26 March 2021

R1-2103170, Enhancements on Timing Error Mitigations for improved Accuracy, Qualcomm Incorporated, 3GPP TSG RAN WG1 #104b-e, April 12th – 20th, 2021

R1-2103771, Techniques mitigating Rx/Tx timing delays, Ericsson, RAN1#104bis-e, April 12th– 20th, 2021