3GPP TSG RAN WG1 Meeting #108-e	R1-2202423

e-Meeting, February 21 – March 3, 2022

Agenda Item:	8.1.5

Source:	Huawei, HiSilicon

Title:	Alignment of field names between specifications

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss the mis-alignment of field names between specifications, and provide the text proposals accordingly.

# Discussion

To support Rel-17 MIMO features, several DCI fields have been introduced [1]. However, different field names are used in different specifications when referring to them. In Table 1, we summarize such cases.

Table. 1 Summary of DCI field name among different specifications

| DCI field names in 38.212 | Terms used in 38.213 | Terms used in 38.214 |
| --- | --- | --- |
| TPC command for scheduled PUSCH | first TPC command |  |
| TPC command for scheduled PUCCH | first TPC command |  |
| Second TPC command for scheduled PUSCH | second TPC command |  |
| Second TPC command for scheduled PUCCH | second TPC command |  |
| SRS resource indicator |  | one SRS resource indicator |
| Second SRS resource indicator |  | two SRS resource indicator |
| Precoding information and number of layers |  | one Precoding information and number of layers |
| Second Precoding information |  | two Precoding information and number of layers |
| Second PTRS-DMRS association |  | DCI parameter ‘PTRS-DMRS association’ |

To align the field names and to avoid potential ambiguity, we have the following text proposal for TS 38.213 and TS 38.214.

Proposal 1: endorse the following text proposal to TS 38.213 and TS 38.214.

TS 38.213: < Start of the text proposal > 7.1.1 UE behavior < Unchanged parts are omitted > - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’, and is provided twoPUSCH-PC-AdjustmentStates - If the DCI format includes two TPC command values and the PUSCH transmissions are associated with l=0 and l=1, the UE applies the first TPC command value given by TPC command for scheduled PUSCH for l=0 and applies the second TPC command value given by Second TPC command for scheduled PUSCH for l=1 - If the DCI format includes two TPC command values and the PUSCH transmissions are associated with l=0, the UE applies the first TPC command value given by TPC command for scheduled PUSCH for l=0 and ignores the second TPC command value given by Second TPC command for scheduled PUSCH - If the DCI format includes two TPC command values and the PUSCH transmissions are associated with l=1, the UE applies the second TPC command value given by Second TPC command for scheduled PUSCH for l=1 and ignores the first TPC command value given by TPC command for scheduled PUSCH - If the DCI format includes one TPC command value, the UE applies the TPC command value given by TPC command for scheduled PUSCH for all l associated with the PUSCH transmission < Unchanged parts are omitted > 7.2.1 UE behavior < Unchanged parts are omitted > - If the DCI format includes two TPC command values and the PUCCH resource of the PUCCH transmission is associated with l=0 and l=1, the UE applies the first TPC command value given by TPC command for scheduled PUCCH for l=0 and applies the second TPC command value given by Second TPC command for scheduled PUCCH for l=1 - If the DCI format includes two TPC command values and the PUCCH resource of the PUCCH transmission is associated with l=0, the UE applies the first TPC command value given by TPC command for scheduled PUCCH for l=0 and ignores the second TPC command value given by Second TPC command for scheduled PUCCH - If the DCI format includes two TPC command values and the PUCCH resource of the PUCCH transmission is associated with l=1, the UE applies the second TPC command value given by Second TPC command for scheduled PUCCH for l=1 and ignores the first TPC command value given by TPC command for scheduled PUCCH - If the DCI format includes one TPC command value, the UE applies the TPC command value given by TPC command for scheduled PUCCH for all l associated with the PUCCH resource of the PUCCH transmission < End of the text proposal >

TS 38.214: < Start of the text proposal > 6.1.1.1 Codebook based UL transmission < Unchanged parts are omitted > When only one SRS resource set is configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook’, SRI and TPMI are given by the DCI fields of SRS resource indicator and Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2 or given by srs-ResourceIndicator and precodingAndNumberOfLayers according to clause 6.1.2.3. The TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource selected by the SRI when multiple SRS resources are configured, or if a single SRS resource is configured TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource. The transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to higher layer parameter nrofSRS-Ports in SRS-Config, as defined in Clause 6.3.1.5 of [4, TS 38.211]. When the UE is configured with the higher layer parameter txConfig set to 'codebook', the UE is configured with at least one SRS resource. The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook’, one or two SRI(s), and one or two TPMI(s) are given by the DCI fields of SRS resource indicator, Second SRS resource indicator, Precoding information and number of layers and Second Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2. The UE applies the indicated SRI(s) and TPMI(s) to one or more PUSCH repetitions according to the associated SRS resource set of a PUSCH repetition according to clause 6.1.2.1. Each TPMI, based on indicated codepoint of SRS resource set indicator or Second SRS resource set indicator, is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource selected by the corresponding SRI when multiple SRS resources are configured for the applicable SRS resource set, or if a single SRS resource is configured for the applicable SRS resource set TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource. For one or two TPMI(s), the transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to the higher layer parameter nrofSRS-Ports in SRS-Config for the indicated SRI(s), as defined in Clause 6.3.1.5 of [4, TS 38.211]. When two SRIs are indicated, the UE shall expect the nrofSRS-Ports for the two indicated SRS resources to be the same. When the UE is configured with the higher layer parameter txConfig set to 'codebook', the UE is configured with at least one SRS resource. Each of the indicated one or two SRI(s) in slot n is associated with the most recent transmission of SRS resource of associated SRS resource set identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook’, the UE is not expected to be configured with different number of SRS resources in the two SRS resource sets. < Unchanged parts are omitted > 6.1.1.2 Non-Codebook based UL transmission For non-codebook based transmission, PUSCH can be scheduled by DCI format 0_0, DCI format 0_1, DCI format 0_2 or semi-statically configured to operate according to Clause 6.1.2.3. If this PUSCH is scheduled by DCI format 0_1, DCI format 0_2, or semi-statically configured to operate according to Clause 6.1.2.3, the UE can determine its PUSCH precoder(s) and transmission rank based on the SRI(s) when multiple SRS resources are configured, where the one or two SRI(s) is given by the DCI fields of SRS resource indicator and second SRS resource indicator in DCI according to clause 7.3.1.1.2 and 7.3.1.1.3 of [5, 38.212] for DCI format 0_1 and DCI format 0_2, or the SRI is given by srs-ResourceIndicator according to clause 6.1.2.3, or two SRIs given by srs-ResourceIndicator and srs-ResourceIndicator2 according to clause 6.1.2.3. The SRS-ResourceSet(s) applicable for PUSCH scheduled by DCI format 0_1 and DCI format 0_2 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModList and srs-ResourceSetToAddModListDCI-0-2 in SRS-config, respectively. The UE shall use one or multiple SRS resources for SRS transmission, where, in a SRS resource set, the maximum number of SRS resources which can be configured to the UE for simultaneous transmission in the same symbol and the maximum number of SRS resources are UE capabilities. The SRS resources transmitted simultaneously occupy the same RBs. Only one SRS port for each SRS resource is configured. Only one or two SRS resource sets can be configured in srs-ResourceSetToAddModList with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', and only one or two SRS resource sets can be configured in srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook'. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook’, one or two SRIs are given by the DCI fields of SRS resource indicator and second SRS resource indicator in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2. The UE applies the indicated SRI(s) to one or more PUSCH repetitions according to the associated SRS resource set of a PUSCH repetition according to clause 6.1.2.1. The maximum number of SRS resources per SRS resource set that can be configured for non-codebook based uplink transmission is 4. Each of the indicated one or two SRIs in slot n is associated with the most recent transmission of SRS resource(s) of associated SRS resource set identified by the SRI, where the SRS transmission is prior to the PDCCH carrying the SRI. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook’, the UE is not expected to be configured with different number of SRS resources in the two SRS resource sets. < Unchanged parts are omitted > 6.2.3 UE PT-RS transmission procedure < Unchanged parts are omitted > For partial-coherent and non-coherent codebook-based UL transmission, the actual number of UL PT-RS port(s) is determined based on TPMI(s) and/or number of layers which are indicated by 'Precoding information and number of layers' field(s) in DCI format 0_1 and DCI format 0_2 or configured by higher layer parameter precodingAndNumberOfLayers: - if the UE is configured with the higher layer parameter maxNrofPorts in PTRS-UplinkConfig set to 'n2', the actual UL PT-RS port(s) and the associated transmission layer(s) are derived from indicated TPMI(s) as: - PUSCH antenna port 1000 and 1002 in indicated TPMI(s) share PT-RS port 0, and PUSCH antenna port 1001 and 1003 in indicated TPMI(s) share PT-RS port 1. - UL PT-RS port 0 is associated with the UL layer 'x' of layers which are transmitted with PUSCH antenna port 1000 and PUSCH antenna port 1002 in indicated TPMI(s), and UL PT-RS port 1 is associated with the UL layer 'y' of layers which are transmitted with PUSCH antenna port 1001 and PUSCH antenna port 1003 in indicated TPMI(s), where 'x' and/or 'y' are given by DCI field 'PTRS-DMRS association' and 'Second PTRS-DMRS association' as shown in DCI format 0_1 and DCI format 0_2 described in Clause 7.3.1 of [5, TS38.212]. < End of the text proposal >

# Conclusion

This contribution discusses the mis-alignment of field names between specifications, and made the following proposal:

Proposal 1: endorse the text proposal to TS 38.213 and TS 38.214 in section 2.

# References

- 3GPP TS 38.212 V17.0.0.