3GPP TSG-RAN WG1 Meeting #108-e	R1-2202427

e-Meeting, February 21 – March 3, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR |  | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Rel-16 UE dormancy adaptation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core, NR_UE_pow_sav-Core |  |  |  |  |  | Date: |  |  | 2022-02-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - RAN2 changed the configuration structure regarding dormancy cell groups from DormancySCellGroups, which explicitly includes a list of dormancy groups, in v16.0.0 to dormancyGroupWithinActiveTime in v16.7.0. TS 38.331 v16.7.0 specifies dormancyGroupWithinActiveTime as dormancy group ID for each configured SCell. Under this new configuration structure of dormancyGroupWithinActiveTime, it is not clear how to determine the number of bits for PDCCH dormancy indication field by the description of “according to higher layer parameter dormancyGroupWithinActiveTime”. - Similar issues exists for the determination of the number of bits for PDCCH dormancy indication field by the description of “according to higher layer parameter dormancyGroupOutsideActiveTime”. - The description of “with MSB to LSB of the bitmap corresponding to the first to last configured SCell group” is also not clear considering dormancyGroupWithinActiveTime is configured per configured SCell in TS 38.331 v16.7.0. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the description for dormancy indication field to capture that the field size is according to “the highest group index provided by higher layer parameter dormancyGroupWithinActiveTime”. - Update the description of “with MSB to LSB of the bitmap corresponding to the first to last configured SCell group” to “with MSB to LSB of the bitmap corresponding to the configured SCell group with lowest to highest group index”. - Similar changes are applied for the case when dormancyGroupOutsideActiveTime is configured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The specification is not clear. E.g. if gNB configures DormancyGroupID of 0 or 4 for the configured SCells but does not configure DormancyGroupID with value 1, 2 or 3 for any configured SCell, there can be two understanding on the field size, one is 2 bits and the other one is 5 bits. - The parameter dormancyGroupOutsideActiveTime and dormancyGroupWithinActiveTime in TS 38.331 v16.7.0 cannot support the description of “the first to last configured SCell group” in TS38.212. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.1.2, 7.3.1.2.2, 7.3.1.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.3.1.1.2	Format 0_1

========================= Unchanged parts =========================

-	SCell dormancy indication – 0 bit if higher layer parameter dormancyGroupWithinActiveTime is not configured; otherwise 1, 2, 3, 4 or 5 bits bitmap determined according to the highest group index provided by higher layer parameter dormancyGroupWithinActiveTime for SCells, where each bit corresponds to one of the SCell group(s) configured by higher layers parameter dormancyGroupWithinActiveTime, with MSB to LSB of the bitmap corresponding to the configured SCell group with lowest to highest group index. The field is only present when this format is carried by PDCCH on the primary cell within DRX Active Time and the UE is configured with at least two DL BWPs for an SCell.

========================= Unchanged parts =========================

##### 7.3.1.2.2	Format 1_1

========================= Unchanged parts =========================

-	SCell dormancy indication – 0 bit if higher layer parameter dormancyGroupWithinActiveTime is not configured; otherwise 1, 2, 3, 4 or 5 bits bitmap determined according to the highest group index provided by higher layer parameter dormancyGroupWithinActiveTime for SCells, where each bit corresponds to one of the SCell group(s) configured by higher layers parameter dormancyGroupWithinActiveTime, with MSB to LSB of the bitmap corresponding to the configured SCell group with lowest to highest group index. The field is only present when this format is carried by PDCCH on the primary cell within DRX Active Time and the UE is configured with at least two DL BWPs for an SCell.

If one-shot HARQ-ACK request is not present or set to '0', and all bits of frequency domain resource assignment are set to 0 for resource allocation type 0 or set to 1 for resource allocation type 1 or set to 0 or 1 for dynamic switch resource allocation type, this field is reserved and the following fields among the fields above are used for SCell dormancy indication, where each bit corresponds to one of the configured SCell(s), with MSB to LSB of the following fields concatenated in the order below corresponding to the SCell with lowest to highest SCell index

-	Modulation and coding scheme of transport block 1

-	New data indicator of transport block 1

-	Redundancy version of transport block 1

-	HARQ process number

-	Antenna port(s)

-	DMRS sequence initialization

========================= Unchanged parts =========================

##### 7.3.1.3.7	Format 2_6

DCI format 2_6 is used for notifying the power saving information outside DRX Active Time for one or more UEs.

The following information is transmitted by means of the DCI format 2_6 with CRC scrambled by PS-RNTI:

-	block number 1, block number 2,…, block number N

where the starting position of a block is determined by the parameter ps-PositionDCI-2-6 provided by higher layers for the UE configured with the block.

If the UE is configured with higher layer parameter ps-RNTI and dci-Format2-6, one block is configured for the UE by higher layers, with the following fields defined for the block:

-	Wake-up indication - 1 bit

-	SCell dormancy indication – 0 bit if higher layer parameter dormancyGroupOutsideActiveTime is not configured; otherwise 1, 2, 3, 4 or 5 bits bitmap determined according to the highest group index provided by higher layer parameter dormancyGroupOutsideActiveTime for SCells, where each bit corresponds to one of the SCell group(s) configured by higher layers parameter dormancyGroupOutsideActiveTime, with MSB to LSB of the bitmap corresponding to the configured SCell group with lowest to highest group index.

The size of DCI format 2_6 is indicated by the higher layer parameter sizeDCI-2-6, according to Clause 10.3 of [5, TS 38.213].