3GPP TSG RAN WG1 Meeting #108-e 	R1-2202431

E-meeting, February 21 –March 3, 2021

Agenda Item:	7.1

Source:	Huawei, HiSilicon

Title:	Discussion on HARQ-ACK multiplexing on PUSCH without PUCCH

Document for:	Discussion and Decision

Introduction

In RAN1#107-e, the handling of HARQ multiplexing on PUSCH without the indication of corresponding PUCCH and the following was agreed

Agreement For Rel-16 with multiple overlapping PUSCHs with no overlapping PUCCH with HARQ-ACK within a span of one PUCCH slot, if the UL-TDAI n.e. 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook) there is no consensus in RAN1 on Rel-16 UE behaviour Agreement For Rel-16 with one PUSCH and no overlapping PUCCH with HARQ-ACK within a span of one PUCCH slot (both single carrier and UL CA), if the UL-TDAI is not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook), the UE multiplexes HARQ-ACK following the UL-TDAI into the PUSCH.

It was also agreed that UE behavior for the following case in Rel-16 will be discussed further:

- More than one non-overlapping PUSCH and no overlapping PUCCH with HARQ-ACK within a span on one PUCCH slot (both single carrier and UL CA), if for at least one of the PUSCHs the UL-TDAI is not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook)

In this paper, we will discuss the UE behavior for the remaining case.

Discussion on UCI multiplexing on PUSCH without PUCCH

Case 1: Multiple non-overlapping mini-slot PUSCH(s) without HARQ-ACK within a span on one PUCCH slot for single CC

Figure 1 multiple mini-slot PUSCHs without PUCCH for single CC

For single CC, if mini-slot scheduling for PUSCH is supported and multiple PUSCHs are overlapped with PUCCH. The UE does not know whether the PUSCH is overlapped with PUCCH if the UE missed the DCI indicating the PUCCH resource. Thus only UL-TDAI can be used to determine whether the PUCCH is overlapped with PUCCH, this is the intention to introduce the UL-TDAI indication.

If only one UL-TDAI of the PUSCHs is not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook), the UE should multiplex the HARQ-ACK information on the PUSCH following the UL-TDAI. If there are more than one UL-TDAI of the PUSCHs not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook), the UE can multiplex the HARQ-ACK information on the selected PUSCH following the UL-TDAI. For the selection of PUSCH, the Rel-15 rules should be reused as much as possible to minimize specification change and implementation change. It is unnecessary to define a new rule. And during the time duration between two UL-TDAI, the HARQ information should not be updated following the restriction in the current specification.

Case 2: Multiple non-overlapping mini-slot PUSCH(s) without HARQ-ACK within a span on one PUCCH slot for single CC

Figure 2 multiple mini-slot PUSCHs, each PUSCH without PUCCH for single CC

For single CC, if mini-slot scheduling for PUSCH and multiple PUCCH transmission in a slot is supported. And each PUSCH is overlapped with one PUCCH, in this case, the UE should multiplex HARQ-ACK information on the PUSCH following the indication of UL-TDAI. But if the UE missed the indication of PUCCH, the UE cannot distinguish the above Case 1 and Case 2.

Case 3: Multiple non-overlapping PUSCH(s) without HARQ-ACK within a span on one PUCCH slot for UL CA

Case 3-1:

Figure 3 Multiple PUSCHs overlapping with one PUCCH for UL CA for mixed numerologies

For this mixed numerologies case, one PUCCH would overlap with multiple PUSCHs, from the UE side, the UE does not know whether PUSCH is overlapped and PUCCH without PUCCH. Thus only UL-TDAI can be used. The UE should multiplex HARQ-ACK information on PUSCH following the indication UL-TDAI. The solutions can be the same as Case 1.

Case 3-2:

Figure 4 Multiple PUSCHs overlapping with one PUCCH for UL CA

For this case, if only one the UL-TDAI of the PUSCHs is not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook), the UE should multiplex HARQ-ACK information on the PUSCH. But there is still a case when more than one UL-TDAI of the PUSCHs are not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook). For example, gNB cannot always know what traffic is coming in advance, after the gNB schedules a UCI multiplexing PUSCH, there may be another PUSCH overlapping with PUCCH and with higher priority. In this case, Rel-15 rules should be reused as much as possible to minimize specification change and implementation change.

Case 4: Multiple non-overlapping PUSCH(s) without HARQ-ACK within a span on one PUCCH slot for UL CA

Figure 5 Multiple mini-slot PUSCHs overlapping with multiple PUCCH for UL CA

If multiple PUCCH in a slot is supported, the UE has no idea there is one PUCCH overlapped two PUSCHs or the case in Figure 5, each PUSCH overlapped with one PUCCH. Thus this case should be an error case.

In conclusion, we find different cases need different solutions:

- For UL CA

  - Support of multiple PUCCHs in a slot (URLLC case)

    - This issue can leave to UE implementation.

  - Not support of multiple PUCCHs in a slot

    - Follow the indication of UL-TDAI

      - One UL-TDAI is not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook), multiple on PUSCH

      - More than one, reuse R15 mechanism and select one PUSCH to

- For single CC

  - Support of multiple PUCCHs in a slot (URLLC case)

    - This issue can leave to UE implementation.

  - Follow the indication of UL-TDAI

    - One UL-TDAI is not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook), multiplex on PUSCH

    - More than one, reuse R15 mechanism and select one PUSCH to multiplex

Proposal 1: In case of more than one non-overlapping PUSCH and no overlapping PUCCH with HARQ-ACK within a span on one PUCCH slot (both single carrier and UL CA), if for at least one of the PUSCHs the UL-TDAI is not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook), and if sub-slot feedback is not supported,

- The UE shall multiplex HARQ-ACK in PUSCH according to UL DAI if there is only one PUSCH with UL DAI not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook)

- The UE shall multiplex HARQ-ACK in PUSCH according to UL DAI if there are more than one PUSCH with UL DAI not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook) according to Rel-15 mechanism

- The DAI field value of multiple PUSCH(s) should be the same

Conclusions

In this contribution, we discuss the questions left for UCI multiplexing without PUCCH for Rel-16 in RAN1#107-e meeting. Based on above discussions, the following proposals are given.

Proposal 1: In case of more than one non-overlapping PUSCH and no overlapping PUCCH with HARQ-ACK within a span on one PUCCH slot (both single carrier and UL CA), if for at least one of the PUSCHs the UL-TDAI is not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook), and if sub-slot feedback is not supported,

- The UE shall multiplex HARQ-ACK in PUSCH according to UL DAI if there is only one PUSCH with UL DAI not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook)

- The UE shall multiplex HARQ-ACK in PUSCH according to UL DAI if there are more than one PUSCH with UL DAI not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook) according to Rel-15 mechanism

- The DAI field value of multiple PUSCH(s) should be the same

- References

- 3GPP; NR; Physical layer procedures for control(Release 16); 38.213(16.6.0)

- R1-2112859 summary of [107-e-NR-7.1CRs-6] in RNA1#107e meeting