3GPP TSG-RAN WG1 Meeting #108-e	R1-2202432

e-Meeting, February 21– March 3, 2022

Agenda Item:	8.12.4

Source:	Huawei, HiSilicon

Title:	Views on overall configurations for multicast

Document for:	Discussion and Decision

Introduction

Regarding the RRC parameters for multicast, the following conclusion was reached in RAN1#107-e meeting:

Conclusion For the RRC parameters that can be configured in PDSCH-Config / PDCCH-Config / SPS-Config in Rel-15/16, they can also be configured in PDSCH-Config-Multicast / PDCCH-Config-Multicast / SPS-Config-Multicast. - If some of these RRC parameters need changes for multicast reception (e.g., modify the default values, delete some useless parameters), RAN1 will list them explicitly in the RRC parameter list that will be sent to RAN2. - For other RRC parameters that do not need changes for multicast reception, RAN1 will not list them with postfix ‘-Multicast’ one by one in the RRC parameter list that will be sent to RAN2, and the default values of these parameters are the same as the default values of the corresponding parameters in dedicated unicast BWP.

However, as RAN2 has already implemented in the TS38.331 running CR [1] that some parameters are noted that they are not configured in the case of PDCCH configuration for MBS multicast, e.g., tpc-PUCCH, tpc-PUSCH, tpc-SRS, etc. However, such parameters are not included in the RRC parameters list sent to RAN2 [2].

Therefore, from RAN2 perspective, given they have identified some parameters are clearly not needed for multicast but not included in the RRC parameters list sent to RAN2, it is not clear to RAN2, among those parameters not included in the RRC parameters list, what parameters are not needed and what parameters are needed but without special default values.

This paper aims to analyze the parameters included in PDCCH-config/PDSCH-config and sorts out the parameters that are not needed so as to help RAN2 figure out the parameters that are needed without special default values.

Configurations of PDCCH-config for multicast

The following Table 1 starts from all the parameters currently included in PDCCH-Config for unicast in the first column and derives whether they are needed or not in the last column based on the background (if any) in the middle column:

Table 1: Analysis of parameters in PDCCH-config that are needed/not needed for multicast

| Parameters in PDCCH-config | Background | Whether needed for multicast? |
| --- | --- | --- |
| controlResourceSetToAddModList SEQUENCE(SIZE (1..3)) OF ControlResourceSet controlResourceSetToReleaseList SEQUENCE(SIZE (1..3)) OF ControlResourceSetId controlResourceSetToAddModListSizeExt-v1610 SEQUENCE (SIZE (1..2)) OF ControlResourceSet controlResourceSetToReleaseListSizeExt-r16 SEQUENCE (SIZE (1..5)) OF ControlResourceSetId-r16 | Agreements: For PTM transmission scheme 1, the CORESET for group-common PDCCH is configured within the common frequency resource for group-common PDSCH. - FFS: number of CORESET(s) for group-common PDCCH within the common frequency resource for group-common PDSCH | Needed. |
| searchSpacesToAddModList, searchSpacesToReleaseList, searchSpacesToAddModListExt2-r17 | List of UE specifically configured Search Spaces or MBS multicast Search Spaces. The network configures at most 10 Search Spaces per BWP per cell (including UE-specific and common Search Spaces). If the network includes searchSpaceToAddModListExt or searchSpacesToAddModListExt2, it includes the same number of entries, and listed in the same order, as in searchSpacesToAddModList in each of them. | Needed as RAN2 implemented in TS38.331 running CR. |
| downlinkPreemption SetupRelease { DownlinkPreemption } | Downlink preemption is indicated in DCI format 2_1. Agreement For multicast of RRC_CONNECTED UEs in Rel-17, - DCI format 2_x cannot be configured in the same CSS configuration with multicast DCI formats. | Not needed. |
| tpc-PUSCH SetupRelease { PUSCH-TPC-CommandConfig } | TPC command for PUCCH/PUSCH is indicated in DCI format 2_2. TPC command for SRS is indicated in DCI format 2_3. Agreement For multicast of RRC_CONNECTED UEs in Rel-17, - DCI format 2_x cannot be configured in the same CSS configuration with multicast DCI formats. | Not needed. |
| tpc-PUCCH SetupRelease { PUCCH-TPC-CommandConfig } |  | Not needed. |
| tpc-SRS SetupRelease { SRS-TPC-CommandConfig} |  | Not needed. |
| uplinkCancellation-r16 SetupRelease { UplinkCancellation-r16 } | Uplink cancellation is indicated in DCI format 2_4. Agreement For multicast of RRC_CONNECTED UEs in Rel-17, - DCI format 2_x cannot be configured in the same CSS configuration with multicast DCI formats. | Not needed |
| monitoringCapabilityConfig-r16 ENUMERATED { r15monitoringcapability,r16monitoringcapability } | Per span PDCCH monitoring is not agreed to be supported. | Not needed. The capability for PDCCH monitoring for multicast by default is the R15 monitoring capability. |
| searchSpaceSwitchConfig-r16 SearchSpaceSwitchConfig-r16 | Search space set group switching is indicated in DCI format 2_0. Agreement For multicast of RRC_CONNECTED UEs in Rel-17, - DCI format 2_x cannot be configured in the same CSS configuration with multicast DCI formats. | Not needed. |

Based on the above analysis, the parameters that are NOT needed for PDCCH-config for multicast consists of:

- downlinkPreemption

- tpc-PUCCH

- tpc-PUSCH

- tpc-SRS

- uplinkCancellation-r16

- monitoringCapabilityConfig-r16 (the default is R15monitoringcapablity)

- searchSpaceSwitchConfig-r16

Proposal 1: Inform RAN2 that the following parameters are NOT needed for PDCCH-Config for multicast:

  - downlinkPreemption

  - tpc-PUCCH

  - tpc-PUSCH

  - tpc-SRS

  - uplinkCancellation-r16

  - monitoringCapabilityConfig-r16 (the default is R15monitoringcapablity)

  - searchSpaceSwitchConfig-r16

Configurations of PDSCH-config for multicast

The following Table 2 starts from all the parameters currently included in PDSCH-Config for unicast in the first column and derives whether they are needed or not in the last column based on the background (if any) in the middle column:

Table 2: Analysis of parameters in PDSCH-config that are needed/not needed for multicast

| Parameters in PDSCH-config | Background | Whether needed for multicast? |
| --- | --- | --- |
| dataScramblingIdentityPDSCH INTEGER (0..1023) | Agreement: For initializing scrambling sequence generator for GC-PDSCH scheduled by the first DCI format for multicast received in Type-x CSS for RRC_CONNECTED UEs, - nID equals the higher layer parameter dataScramblingIdentityPDSCH if it is configured in PDSCH-Config in a CFR used for GC-PDSCH and the RNTI equals the G-RNTI or G-CS-RNTI; nID=NIDcell otherwise. - nRNTI corresponds to the RNTI associated with the GC-PDSCH transmission (i.e., the G-RNTI used by the scheduling GC-PDCCH, or the G-CS-RNTI used by the SPS GC-PDSCH activation PDCCH) | Needed |
| dmrs-DownlinkForPDSCH-MappingTypeA SetupRelease { DMRS-DownlinkConfig } | Agreement For DMRS of GC-PDSCH, - For GC-PDSCH scheduled by a DCI format 4_0/4_1, the UE assumes dmrs-AdditionalPosition = ‘pos2’, similar as that of DCI format 1_0. - For GC-PDSCH scheduled by a DCI format 4_2, the UE assumes dmrs-AdditionalPosition in DMRS-Config if configured in PDSCH-Config-Multicast, similar as that of DCI format 1_1. | Needed |
| dmrs-DownlinkForPDSCH-MappingTypeB SetupRelease { DMRS-DownlinkConfig } | Agreement For DMRS of GC-PDSCH, - For GC-PDSCH scheduled by a DCI format 4_0/4_1, the UE assumes dmrs-AdditionalPosition = ‘pos2’, similar as that of DCI format 1_0. - For GC-PDSCH scheduled by a DCI format 4_2, the UE assumes dmrs-AdditionalPosition in DMRS-Config if configured in PDSCH-Config-Multicast, similar as that of DCI format 1_1. | Needed |
| tci-StatesToAddModList | As analyzed in [3] | Not needed |
| vrb-ToPRB-Interleaver ENUMERATED {n2, n4} | Agreement The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by G-RNTI for multicast: - Frequency domain resource assignment - Time domain resource assignment – 4 bits as defined in Clause 5.1.2.1 of TS38.214 - VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5 in TS38.212 | Needed |
| resourceAllocation ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch} | From TS38.214: “setting a higher layer parameter resourceAllocation in PDSCH-Config-Multicast to 'dynamicSwitch' for DCI format 4_2” | Needed |
| pdsch-TimeDomainAllocationList SetupRelease { PDSCH-TimeDomainResourceAllocationList } pdsch-TimeDomainAllocationList-r16 SetupRelease { PDSCH-TimeDomainResourceAllocationList-r16 } | Agreement: For slot-level repetition for SPS GC-PDSCH for multicast RRC_CONNECTED UEs. - Config A or Config B can be configured to UE: - (Config A) UE can be optionally configured with pdsch-AggregationFactor per SPS-Config-Multicast. - (Config B) UE can be optionally configured with TDRA table with repetitionNumber as part of the TDRA table in PDSCH-Config-Multicast. - If UE is configured with Config B, UE does not expect to be configured with Config A for the same SPS group-common PDSCH. - For Config A, if pdsch-AggregationFactor in SPS-Config-Multicast is not configured, default value is - Alt1: equal to 1. | Needed |
| pdsch-AggregationFactor ENUMERATED { n2, n4, n8 } | Agreement: For slot-level repetition for SPS GC-PDSCH for multicast RRC_CONNECTED UEs. - Config A or Config B can be configured to UE: - (Config A) UE can be optionally configured with pdsch-AggregationFactor per SPS-Config-Multicast. - (Config B) UE can be optionally configured with TDRA table with repetitionNumber as part of the TDRA table in PDSCH-Config-Multicast. - If UE is configured with Config B, UE does not expect to be configured with Config A for the same SPS group-common PDSCH. - For Config A, if pdsch-AggregationFactor in SPS-Config-Multicast is not configured, default value is - Alt1: equal to 1. | Needed |
| rateMatchPatternToAddModList rateMatchPatternGroup1 rateMatchPatternGroup2 | Agreement For multicast RRC_CONNECTED UEs, rateMatchPatternToAddModList, rateMatchPatternGroup1 and rateMatchPatternGroup2 can be configured in PDSCH-Config-Multicast for GC-PDSCH rate matching, subject to UE capability. For PDSCH resource mapping with RB symbol level granularity, - The procedure for PDSCH scheduled by PDCCH with DCI format 4_1 is similar as that of DCI format 1_0 and the procedure for PDSCH scheduled by PDCCH with DCI format 4_2 is similar as that of DCI format 1_1, by applying the parameters of rateMatchPatternToAddModList, rateMatchPatternGroup1 and rateMatchPatternGroup2 configured in PDSCH-Config-Multicast. - rateMatchPatternToAddModList, rateMatchPatternGroup1 and rateMatchPatternGroup2 configured in PDSCH-Config for unicast do not apply for GC-PDSCHs. - rateMatchPatternToAddModList, rateMatchPatternGroup1 and rateMatchPatternGroup2 configured in PDSCH-Config-Multicast for multicast do not apply for unicast PDSCHs. | Needed |
| rbg-Size ENUMERATED {config1, config2}, | Agreement: RBG and PRG for multicast GC-PDSCH in CFR are defined using the same procedure as for unicast PDSCH in DL BWP. - For RBG, the size is defined based on the starting PRB of the CFR, size of the CFR and the higher layer parameter rbg-Size configured by PDSCH-Config for multicast in the CFR. - For PRG, the size is defined based on the starting PRB of the CFR, size of the CFR and precoding granularity for multicast which can be equal to one of the values among {2, 4, wideband}. - Note: Whether the RBG and PRG size for multicast (configured directly or indirectly) is the same as for unicast can be discussed separately. | Needed |
| mcs-Table ENUMERATED {qam256, qam64LowSE} | Agreement: For determination of maximum modulation order for LBRM and TBS determination for GC-PDSCH, - if mcs-Table in PDSCH-Config for MBS is not configured in CFR, Table 5.1.3.1-1 in TS38.214 is used (similar as the default value in R16). | Needed |
| maxNrofCodeWordsScheduledByDCI ENUMERATED {n1, n2} | Agreement DCI format 4_2 doesn’t include the following fields: - Scell dormancy indication - BWP indicator DCI format 4_2 includes the following field (configurable): - MCS/NDI/RV for TB2 - Support of this field is subject to UE capability Note: the fields of MCS/NDI/RV for TB2 are only present if maxNrofCodeWordsScheduledByDCI equals 2 | Needed |
| prb-BundlingType CHOICE { staticBundling SEQUENCE { bundleSize ENUMERATED { n4, wideband } OPTIONAL -- Need S }, dynamicBundling SEQUENCE { bundleSizeSet1 ENUMERATED { n4, wideband, n2-wideband, n4-wideband } OPTIONAL, -- Need S bundleSizeSet2 ENUMERATED { n4, wideband } OPTIONAL -- Need S } }, | From TS38.214: The PRB bundling procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_2, by applying the parameters of prb-BundlingType given by PDSCH-Config-Multicast as well as vrb-ToPRB-Interleaver given by PDSCH-Config-Multicast. | Needed. |
| aperiodic-ZP-CSI-RS-ResourceSetsToAddModList SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet aperiodic-ZP-CSI-RS-ResourceSetsToReleaseList SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId | Agreement DCI format 4_2 includes ‘ZP CSI-RS trigger’ field. The bitwidth for this field is determined by the number of aperiodic ZP CSI-RS resource sets configured by higher layer | Needed |
| zp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-Resources)) OF ZP-CSI-RS-Resource zp-CSI-RS-ResourceToReleaseList SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-Resources)) OF ZP-CSI-RS-ResourceId sp-ZP-CSI-RS-ResourceSetsToAddModList SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet sp-ZP-CSI-RS-ResourceSetsToReleaseList SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId p-ZP-CSI-RS-ResourceSet SetupRelease { ZP-CSI-RS-ResourceSet } | Except aperiodic ZP CSI-RS, periodic and semi-periodic ZP CSI-RS are not agreed to be supported. | Not needed. |
| maxMIMO-Layers-r16 SetupRelease { MaxMIMO-LayersDL-r16 } | Agreement: For LBRM and TBS determination for GC-PDSCH, the default value of the maximum number of layers is 1 if maxMIMO-Layers in PDSCH-Config for MBS in CFR is not configured. | Needed |
| minimumSchedulingOffsetK0-r16 SetupRelease { MinSchedulingOffsetK0-Values-r16 } | Agreement Multicast DCI format 1_1 includes all configurable fields of unicast DCI format 1_1 except - Identifier for DCI formats, TPC command for scheduled PUCCH, SRS request - FFS: Scell dormancy indication - One-shot HARQ-ACK request, PDSCH group index, New feedback indicator, Number of requested PDSCH group(s), ChannelAccess-Cpext - CBGTI, CBGFI - Minimum applicable scheduling offset indicator | Not needed |
| antennaPortsFieldPresenceDCI-1-2-r16 aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2-r16 SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet aperiodicZP-CSI-RS-ResourceSetsToReleaseListDCI-1-2-r16 SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2-r16 SetupRelease { DMRS-DownlinkConfig } dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2-r16 SetupRelease { DMRS-DownlinkConfig } dmrs-SequenceInitializationDCI-1-2-r16 harq-ProcessNumberSizeDCI-1-2-r16 mcs-TableDCI-1-2-r16 numberOfBitsForRV-DCI-1-2-r16 pdsch-TimeDomainAllocationListDCI-1-2-r16 SetupRelease { PDSCH-TimeDomainResourceAllocationList-r16 } prb-BundlingTypeDCI-1-2-r16 priorityIndicatorDCI-1-2-r16 rateMatchPatternGroup1DCI-1-2-r16 rateMatchPatternGroup2DCI-1-2-r16 resourceAllocationType1GranularityDCI-1-2-r16 vrb-ToPRB-InterleaverDCI-1-2-r16 referenceOfSLIVDCI-1-2-r16 resourceAllocationDCI-1-2-r16 ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch} | These parameters can be independently configured for “URLLC” from that for “eMBB” or for scheduling using DCI format 1_2 from that for using DCI format 1_1. | Not needed |
| priorityIndicatorDCI-1-1-r16 | priorityIndicatorDCI-4-2 should be used for multicast. | Needed, with name change to DCI format 4_2 |
| dataScramblingIdentityPDSCH2-r16 | The dataScramblingIdentityPDSCH2 is configured if coresetPoolIndex is configured with 1 for at least one CORESET in the same BWP (for the case mTRP). Standardized mTRP transmission is not assumed for MBS. | Not needed |
| repetitionSchemeConfig-r16 SetupRelease { RepetitionSchemeConfig-r16} repetitionSchemeConfig-v1630 SetupRelease { RepetitionSchemeConfig-v1630 | repetitionSchemeConfig configure the UE with repetition schemes. The network does not configure repetitionSchemeConfig-r16 and repetitionSchemeConfig-v1630 simultaneously to setup in the same PDSCH-Config. This is introduced by mTRP. Standardized mTRP transmission is not assumed for MBS. | Not needed |
| xOverhead | Agreement: For LBRM and TBS determination for GC-PDSCH: - xOverhead can be provided in PDSCH-Config for MBS in CFR; if not provided, a default value of zero is used. | Needed |

Based on the above analysis, the parameters that are NOT needed for PDSCH-config for multicast are sorted out in red in the last column in Table 2.

Proposal 2: Inform RAN2 that the following parameters are NOT needed for PDSCH-Config for multicast:

- tci-StatesToAddModList

- zp-CSI-RS-ResourceToAddModList, zp-CSI-RS-ResourceToReleaseList, sp-ZP-CSI-RS-ResourceSetsToAddModList, sp-ZP-CSI-RS-ResourceSetsToReleaseList, p-ZP-CSI-RS-ResourceSet

- minimumSchedulingOffsetK0-r16

- antennaPortsFieldPresenceDCI-1-2-r16, aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2-r16, aperiodicZP-CSI-RS-ResourceSetsToReleaseListDCI-1-2-r16, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2-r16, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2-r16, dmrs-SequenceInitializationDCI-1-2-r16, harq-ProcessNumberSizeDCI-1-2-r16, mcs-TableDCI-1-2-r16, numberOfBitsForRV-DCI-1-2-r16, pdsch-TimeDomainAllocationListDCI-1-2-r16, prb-BundlingTypeDCI-1-2-r16, priorityIndicatorDCI-1-2-r16, rateMatchPatternGroup1DCI-1-2-r16, rateMatchPatternGroup2DCI-1-2-r16, resourceAllocationType1GranularityDCI-1-2-r16, vrb-ToPRB-InterleaverDCI-1-2-r16, referenceOfSLIVDCI-1-2-r16, resourceAllocationDCI-1-2-r16,

- dataScramblingIdentityPDSCH2-r16

- repetitionSchemeConfig-r16, repetitionSchemeConfig-v1630

Conclusions

This contribution analyzes what parameters in PDCCH-config and PDSCH-config that are NOT needed for scheduling multicast based on the RAN1 agreements and the discussions in order to assist RAN2 to correctly formulate the RRC parameters. The analysis leads to the following proposals:

Proposal 1: Inform RAN2 that the following parameters are NOT needed for PDCCH-Config for multicast:

  - downlinkPreemption

  - tpc-PUCCH

  - tpc-PUSCH

  - tpc-SRS

  - uplinkCancellation-r16

  - monitoringCapabilityConfig-r16 (the default is R15monitoringcapablity)

  - searchSpaceSwitchConfig-r16

Proposal 2: Inform RAN2 that the following parameters are NOT needed for PDSCH-Config for multicast:

- tci-StatesToAddModList

- zp-CSI-RS-ResourceToAddModList, zp-CSI-RS-ResourceToReleaseList, sp-ZP-CSI-RS-ResourceSetsToAddModList, sp-ZP-CSI-RS-ResourceSetsToReleaseList, p-ZP-CSI-RS-ResourceSet

- minimumSchedulingOffsetK0-r16

- antennaPortsFieldPresenceDCI-1-2-r16, aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2-r16, aperiodicZP-CSI-RS-ResourceSetsToReleaseListDCI-1-2-r16, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2-r16, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2-r16, dmrs-SequenceInitializationDCI-1-2-r16, harq-ProcessNumberSizeDCI-1-2-r16, mcs-TableDCI-1-2-r16, numberOfBitsForRV-DCI-1-2-r16, pdsch-TimeDomainAllocationListDCI-1-2-r16, prb-BundlingTypeDCI-1-2-r16, priorityIndicatorDCI-1-2-r16, rateMatchPatternGroup1DCI-1-2-r16, rateMatchPatternGroup2DCI-1-2-r16, resourceAllocationType1GranularityDCI-1-2-r16, vrb-ToPRB-InterleaverDCI-1-2-r16, referenceOfSLIVDCI-1-2-r16, resourceAllocationDCI-1-2-r16,

- dataScramblingIdentityPDSCH2-r16

- repetitionSchemeConfig-r16, repetitionSchemeConfig-v1630

- References

R2-2201829, 38.331 running CR for NR MBS, Huawei, HiSilicon, RAN2#116bis-e, Jan 17-25, 2022.

R1-2200700, LS on updated Rel-17 NR higher-layers parameter list, RAN1, RAN1#107bis-e, Jan 17-25, 2022.

R1-2200948, Resource configuration and group scheduling for RRC_CONNECTED UEs reliability for RRC_CONNECTED UEs, Huawei, HiSilicon, CBN, RAN1#108-e, February 21-March 3, 2022.