3GPP TSG-RAN WG1 Meeting #108-e	R1-2202453

# e-Meeting, February 21 – March 3, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.215 | CR | xxxx | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to UL SRS-RSRP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN4 replied in R1-2200901 that the antenna reference point for UL SRS-RSRPP should depend on BS type. In the action point, RAN1 were requested by RAN4 to update the reference point of UL SRS-RSRP as well. 1. Overall Description: RAN4 thanks RAN1 for the LS. RAN4 has discussed the working assumption for reference point for UL SRS-RSRPP measurement. Based on the discussion RAN4 would like to provide following feedback. RAN4 feedback to RAN1: Working assumption: The reference point for UL SRS-RSRPP shall be: - for type 1-C base station TS 38.104 [1]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [1]: based on the combined signal from antenna elements corresponding to a given receiver branch, - for type 1-H base station TS 38.104 [1]: the Rx Transceiver Array Boundary connector. 2. Actions: To RAN WG1 group. ACTION: RAN4 kindly requests RAN1 to take the above feedback into consideration when formulating the working assumption for UL SRS-RSRPP measurement reference point. In addition, RAN4 finds that the definition of reference point for SRS-RSRP in 38.215 may need to be updated and based on RAN4 understanding it is same as above for UL SRS-RSRPP.. |
| Summary of change: |  | Change the reference point for SRS-RSRP to depend on BS types. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The reference point for FR1 BS type 1-H and 1-O is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: No inter-operatability issue is identified. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.2.5	UL SRS reference signal received power (UL SRS-RSRP)

| Definition | UL SRS reference signal received power (UL SRS-RSRP) is defined as linear average of the power contributions (in [W]) of the resource elements carrying sounding reference signals (SRS). UL SRS-RSRP shall be measured over the configured resource elements within the considered measurement frequency bandwidth in the configured measurement time occasions. The reference point for UL SRS-RSRPP shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: based on the combined signal from antenna elements corresponding to a given receiver branch - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. For frequency range 1 and 2, if receiver diversity is in use by the gNB, the reported UL SRS-RSRP value shall not be lower than the corresponding UL SRS-RSRP of any of the individual receiver branches. |
| --- | --- |