3GPP TSG RAN WG1 Meeting #108-e	R1-2202464

e-Meeting, February 21 – March 3, 2022

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on interruption for PUCCH SCell activation in invalid TA case

Document for:	Discussion and Decision

# Introduction

In the previous meeting, an LS [1] from RAN4 is sent to RAN1. RAN4 has been discussing the interruption requirements during PUCCH SCell activation. For the PUCCH SCell activation in invalid TA case, the PRACH transmission triggered by PDCCH order will be transmitted for obtaining valid timing advance (TA) information. The SCS of PUCCH SCell PRACH may be different from the SCS of the physical channels/signals on all activated serving cells.  Thus, some questions below on the UE capability of supporting parallel transmission of PRACH on PUCCH SCell and physical channels/signals with different SCS on other activated serving cells need to be clarified.

- Q1: Can UE capability diffNumerologyAcrossPUCCH-Group defined in TS 38.306 be used to allow indication of UE support of parallel transmission of PRACH on PUCCH SCell and physical channels/signals with different SCS on other activated serving cells? - If YES, RAN4 would like to respectfully ask RAN2 to clarify that diffNumerologyAcrossPUCCH-Group applies to PRACH in the corresponding “Definitions for parameters” in TS 38.306. - If NO, are there any other existing capabilities that can serve this purpose? If so, RAN4 would like RAN1 to indicate the capabilities. - Q2: If the answer in Q1 is NO, and there is no capabilities that can serve the purpose, what would be the expected UE behavior when a target PUCCH SCell PRACH has a different SCS from SpCell/SCell physical channels/signals according to RAN1/2 spec? e.g. Shall UE be able to simultaneously transmit PRACH on PUCCH SCell and physical channels/signals on other activated serving cells irrespective of SCS configurations?

In this contribution, some analysis on the questions and a draft LS reply from RAN1 perspective are provided.

# Discussion

Firstly, as defined in the 38.306 [2], the UE capability diffNumerologyAcrossPUCCH-Group is used to indicate whether UE support parallel transmission of control channel and data channel, this capability is irrelevant to the PRACH.

diffNumerologyAcrossPUCCH-Group Indicates whether different numerology across two NR PUCCH groups for data and control channel at a given time in NR CA and (NG)EN-DC/NE-DC is supported by the UE.

Secondly, as defined in the 38.211 [3], subcarrier spacing for random-access preambles ∆fRA depends on the PRACH format. ∆fRA is predefined to be 1.25kHz or 5 kHz for PRACH format 0/1/2/3. μ can be configured for PRACH format for LRA∈{139,571,1151}, only the values 15kHz or 30kHz for FR1 and 60 or 120kHz for FR2 are applicable, thus all the PRACH formats can be supported by UEs.

Thirdly, as defined in the 38.306 [2], UE capability parallelTxPRACH-SRS-PUCCH-PUSCH is defined to indicate UE support of parallel transmission of PRACH and SRS/PUCCH/PUSCH across CCs in an inter-band CA band combination irrespective of PRACH SCS and SRS/PUCCH/PUSCH SCS.

parallelTxPRACH-SRS-PUCCH-PUSCH Indicates whether the UE supports parallel transmission of PRACH and SRS/PUCCH/PUSCH across CCs in an inter-band CA band combination.

There is no additional UE capability for PRACH format or PRACH transmission in case of uplink CA. Therefore, from the perspective of RAN1, when a target PUCCH SCell PRACH has a different SCS from SpCell/SCell physical channels/signals, UE shall be able to simultaneously transmit PRACH on PUCCH SCell and physical channels/signals on other activated serving cells irrespective of SCS configurations if parallelTxPRACH-SRS-PUCCH-PUSCH is reported.

Observation 1: Whether the UE is capable to simultaneously transmit PRACH on PUCCH SCell and SRS/PUCCH/PUSCH on other activated serving cells is not subject to PRACH SCS.

Observation 2: Whether the UE is capable to simultaneously transmit PRACH on PUCCH SCell and PRACH on other activated serving cells is not subject to PRACH SCS.

Proposal 1: RAN1’s answer to the Q1 of RAN4 LS is ‘No, UE capability diffNumerologyAcrossPUCCH-Group is irrelevant to PRACH’.

Proposal 2: RAN1’s answer to the Q2 of RAN4 LS is:

- Whether the UE is capable to simultaneously transmit PRACH on PUCCH SCell and SRS/PUCCH/PUSCH on other activated serving cells is not subject to PRACH SCS.

- Whether the UE is capable to simultaneously transmit PRACH on PUCCH SCell and PRACH on other activated serving cells is not subject to PRACH SCS.

# Conclusion

According to the above discussions, we have the following observations and proposals:

Observation 1: Whether the UE is capable to simultaneously transmit PRACH on PUCCH SCell and SRS/PUCCH/PUSCH on other activated serving cells is not subject to PRACH SCS.

Observation 2: Whether the UE is capable to simultaneously transmit PRACH on PUCCH SCell and PRACH on other activated serving cells is not subject to PRACH SCS.

Proposal 1: RAN1’s answer to the Q1 of RAN4 LS is ‘No, UE capability diffNumerologyAcrossPUCCH-Group is irrelevant to PRACH’.

Proposal 2: RAN1’s answer to the Q2 of RAN4 LS is:

- Whether the UE is capable to simultaneously transmit PRACH on PUCCH SCell and SRS/PUCCH/PUSCH on other activated serving cells is not subject to PRACH SCS.

- Whether the UE is capable to simultaneously transmit PRACH on PUCCH SCell and PRACH on other activated serving cells is not subject to PRACH SCS.

# References

R1-2200871, “LS on interruption for PUCCH SCell activation in invalid TA case”

TS 38.306: “User Equipment (UE) radio access capabilities”.

TS 38.211: “Physical channels and modulation”.

# Appendix

# Title:	Draft reply LS on interruption for PUCCH SCell activation in invalid TA case

# Response to:	R1-2200871/ R4-2120420

# Release:	Rel-17

# Work Item:	NR_RRM_enh2-Core

Source:	Huawei [RAN WG1]

To:	RAN WG4

Cc:	RAN WG2

Contact Person:

Name:	Frank Long

E-mail Address:	frank.longyi{at }huawei.com

# Attachments: 	-

1. Overall Description:

RAN1 thanks RAN4 for the LS. Regarding the questions in the LS, RAN1 answers are provided below,

- Q1: Can UE capability diffNumerologyAcrossPUCCH-Group defined in TS 38.306 be used to allow indication of UE support of parallel transmission of PRACH on PUCCH SCell and physical channels/signals with different SCS on other activated serving cells?

  - If YES, RAN4 would like to respectfully ask RAN2 to clarify that diffNumerologyAcrossPUCCH-Group applies to PRACH in the corresponding “Definitions for parameters” in TS 38.306.

  - If NO, are there any other existing capabilities that can serve this purpose? If so, RAN4 would like RAN1 to indicate the capabilities.

Answer: No, UE capability diffNumerologyAcrossPUCCH-Group is irrelevant to PRACH.

- Q2: If the answer in Q1 is NO, and there is no capabilities that can serve the purpose, what would be the expected UE behavior when a target PUCCH SCell PRACH has a different SCS from SpCell/SCell physical channels/signals according to RAN1/2 spec? e.g. Shall UE be able to simultaneously transmit PRACH on PUCCH SCell and physical channels/signals on other activated serving cells irrespective of SCS configurations?

Answer:

  - Whether the UE is capable to simultaneously transmit PRACH on PUCCH SCell and SRS/PUCCH/PUSCH on other activated serving cells is not subject to PRACH SCS.

  - Whether the UE is capable to simultaneously transmit PRACH on PUCCH SCell and PRACH on other activated serving cells is not subject to PRACH SCS.

2. Actions:

To: RAN4

ACTION: RAN1 respectfully requests RAN4 to take the above responses into account in their future work.

3. Date of Next Meetings:

3GPP RAN1#109-e				16 - 27, May 2022	                Online

3GPP RAN1#110				22 - 26, August 2022	        Toulouse, FR