3GPP TSG RAN WG1 Meeting #108-e	R1-2202469

e-Meeting, February 21 – March 3, 2022

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Views on enhanced TCI state indication for UE-specific PDCCH MAC CE

Document for:	Discussion and Decision

Introduction

One RAN2 LS [1] was sent to RAN1, asking for feedback on the issue of enhanced TCI state indication for UE-specific PDCCH MAC CE. In this contribution, we discuss the questions of the LS and proposed answers.

Discussion

It has been agreed [2] that PDCCH candidates in CSS 3 can be associated with CORESET that is activated with two TCI states and configured with enhanced SFN scheme 1 or TRP based pre-compensation in last meeting. If enhanced TCI state indication for activating two TCI states isn’t applied to CORESET zero, the PDCCH candidates in CSS 3 will not be associated with CORESET zero, which introduces unnecessary restrictions on the gNB implementation. Therefore, we propose the following:

Proposal 1: Enhanced TCI state indication for UE specific PDCCH MAC CE can be applied to CORESET zero.

Conclusion

This contribution discussed the questions of the RAN2 LS, and provide the following proposals:

Proposal 1: Enhanced TCI state indication for UE specific PDCCH MAC CE can be applied to CORESET zero.

- References

- R1-2200886, LS on Enhanced TCI state indication for UE-specific PDCCH MAC CE, RAN2#116bis-e, Jan., 2022

- Chair’s Notes, RAN1#107-e, November 11th – 19th, 2021.