3GPP TSG RAN WG1 #108-e			R1-2202487

e-Meeting, February 21st – March 3rd, 2022

Source:	WILUS Inc.

Title:	Remaining issues for TB processing over multi-slot PUSCH

Agenda item:	8.8.1.2

Document for:	Discussion/Decision

Introduction

In the RAN1#106b-e meeting [1] and RAN#94-e meeting [2], following agreements were made on TB processing over multi-slot PUSCH.

Agreement at RAN1#106b-e: For the bit selection for each transmitted slot for TBoMS, one of the following is to be down selected in RAN1 #107-e for determining the index of the starting coded bit in the circular buffer: - Option B: the index of the starting coded bit in the circular buffer is the index continuous from the position of the last bit selected in the previous allocated slot. - Option C: the index of the starting coded bit in the circular buffer is the index continuous from the position of the last bit selected in the previous allocated slot, regardless of whether UCI multiplexing occurred in the previous allocated slot or not. FFS: whether the index of the starting coded bit for each transmitted slot is expressed as a multiple integer of the lifting size Zc Note: Dropping/cancellation rules are not considered for the starting bit position determination in both Option B and Option C. Agreement at RAN#94-e: For the determination of the index of the starting coded bit in a transmitted slot for TBoMS:​ - adopt option C at RAN#94e​ - task RAN1 to work on the corresponding CR(s) for RAN#95e

In this contribution, we provide our views on TB processing over multi-slot PUSCH.

Discussion on TB processing over multi-slot PUSCH

Rate-matching for TBoMS.

In the RAN#94-e meeting, it was agreed to adopt Option C for the determination of the index of the starting coded bit in a transmitted slot for TBoMS. We provide our views on remaining issues on the Option C.

Aspect 2: The value of G and E for TBoMS (or, alternatively, the value of H).

The value of G and E can have different interpretations based on whether to consider or not the number of UCI bits in bit selection and bit interleaving.

Aspect 2 – Interpretation 1. The starting index of circular buffer is determined assuming no UCI multiplexing, but the number of bits being selected in bit selection (value E) is determined considering UCI multiplexing. Aspect 2 – Interpretation 2. The starting index of circular buffer is determined assuming no UCI multiplexing, and the number of bits being selected in bit selection (value E) is determined assuming no UCI multiplexing.

Both interpretations can be illustrated with following examples:

- Example of Interpretation 1.

Figure 1. Example of Aspect 2 – Interpretation 1.

In Figure 1, a single TBoMS is allocated over 2 slots (N = 2) without repetition. Also, it is assumed that the size of bit sequence is 20 bits, 10 bits can be transmitted per slot, the size of UCI is 4 bits in Slot#0, and no filler bits. In this case, both bit selection and bit interleaving are performed based on punctured size of bit sequence in Slot#0. Accordingly, the value of G is defined after considering UCI per slot, and the value of rate-matching output bit sequence (i.e., E) is also changed. Thus, G = E = 6 bits for Slot#0 and G = E = 10 bits for Slot#1. Then, 4-bits UCI is multiplexed in Slot#0.

- Example of Interpretation 2.

Figure 2. Example of Aspect 2 – Interpretation 2.

In Figure 2, G = E = 10 bits for both Slot#0 and Slot#1 since UCI 4 bits in Slot#0 are not considered in bit selection and bit interleaving. The other assumptions are same with the Figure 1. In this case, both bit selection and bit interleaving are performed assuming no UCI multiplexing. Then, interleaved bit sequence in Slot#0 is punctured from 10 bits to 6 bits when 4-bits UCI is multiplexed.

Interpretation 1 has ambiguity in case of missed DCI for PUCCH, while Interpretation 2 is robust for missed DCI since the number of bits selected from circular buffer is determined regardless of the presence of UCI for multiplexing.

- Proposal 1: The starting index of circular buffer is determined assuming no UCI multiplexing, and the number of bits being selected in bit selection (value E) is determined assuming no UCI multiplexing (Interpretation 2).

Aspect 3: Handling of the filler bits in TBoMS.

It should be clarified whether to consider filler bits or not for determination of the starting bit for each allocated slot for TBoMS.

Aspect 3 – Direction 1. Filler bits are considered to pre-determine the index of the starting bit for each allocated slot for TBoMS, to ensure no overlap exists between bit sequences transmitted over consecutive slots. Aspect 3 – Direction 2. Filler bits are not considered to pre-determine the index of the starting bit for each allocated slot for TBoMS and overlap between bit sequences transmitted over consecutive slots is allowed.

According to FL’s summary [3] in the RAN1#107b-e meeting, both directions are illustrated as follows:

Figure 3. Aspect 3 – Direction 1.

Direction 1 is compatible with the previous agreement (the index of the starting coded bit in the circular buffer is the index continuous from the position of the last bit selected in the previous allocated slot). However, it requires offset τ that can be variable per slot, thus it may have an impact on ‘pre-determination’ of starting coded bit index at UE.

Figure 4. Aspect 3 – Direction 2.

Direction 2 is simple for UE implementation since there is no offset to pre-determine the index of the starting bit for each allocated slot for TBoMS. However, it reverts the previous agreements due to overlapping between different slots.

In our view, a UE can be aware of the presence/number of filler bits in advance since filler bits are inserted before LDPC encoding. Thus, the index of the starting bit for each allocated slot for TBoMS can be pre-determined even considering filler bits (i.e., Direction 1). Also, it might be more compatible with the previous agreement that the index of the starting coded bit in the circular buffer is the index continuous from the position of the last bit selected in the previous allocated slot.

- Proposal 2: Filler bits are considered to pre-determine the index of the starting bit for each allocated slot for TBoMS, to ensure no overlap exists between bit sequences transmitted over consecutive slots (Direction 1).

Conclusion

In this contribution, we provided our views on TBoMS and the followings were proposed:

- Proposal 1: The starting index of circular buffer is determined assuming no UCI multiplexing, and the number of bits being selected in bit selection (value E) is determined assuming no UCI multiplexing (Interpretation 2).

- Proposal 2: Filler bits are considered to pre-determine the index of the starting bit for each allocated slot for TBoMS, to ensure no overlap exists between bit sequences transmitted over consecutive slots (Direction 1).

References

- Final Report of 3GPP TSG RAN WG1 #106b-e v1.0.0

- RAN#94-e draft meeting report (draft_MeetingReport_RAN_94e_211217_eom)

- R1-2200752, Final FL summary of TB processing over multi-slot PUSCH (AI 8.8.1.2)