3GPP TSG RAN WG1 #108                                                               R1-2202519

e-Meeting, February 21st – March 3rd, 2022

Source: 	Moderator (Intel Corporation)

Title: 	Summary of the Preparation Phase for Rel.16 NR Positioning Maintenance

Agenda item:	7.2.8

Document for: 	Discussion and Decision

# Introduction

This document provides overview of contributions submitted for R16 NR Positioning maintenance [1]-[4]. In addition, it provides feature lead recommendation and summary of the following RAN1 e-mail discussion [108-e-Prep-AI7.2.8]:

- [108-e-Prep-AI7.2.8] Preparation phase for Rel-16 NR Positioning maintenance

# Overview of Contributions

## Aspect #1: Descriptions of dl-PRS-ID

In [1], it is proposed to make a correction to reflect that dl-PRS-ID (or a TRP) is not always associated with multiple DL PRS resource sets, as the number of DL PRS resource sets of a TRP can be 1 in some cases. It is proposed to resolve this in the same way as it was done in TS37.355 [2] by using the following descriptions marked in yellow.

dl-PRS-ID This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resources ID to uniquely identify a DL-PRS Resource. This ID can be associated with multiple DL-PRS Resource Sets associated with a single TRP. Each TRP should only be associated with one such ID.

The following TP is provided to resolve minor inconsistency discussed in aspect #1

Text Proposal

TS 38.214, section 5.1.6.5 < Unchanged parts are omitted > The UE expects that it will be configured with dl-PRS-ID each of which is defined such that it is can be associated with multiple DL PRS resource sets. The UE expects that one of these dl-PRS-ID along with a nr-DL-PRS-ResourceSetID and a nr-DL-PRS-ResourceID-r16 can be used to uniquely identify a DL PRS resource. < Unchanged parts are omitted >

## Aspect #2: Descriptions of dl-PRS-CombSizeN-AndReOffset

In [1] it is proposed to make a correction to align a description related to dl-PRS-CombSizeN-AndReOffset in TS38.214 and 37.355.

A DL PRS resource is defined by: … - dl-PRS-CombSizeN-AndReOffset defines the starting RE offset of the first symbol within a DL PRS resource in frequency. The relative RE offsets of the remaining symbols within a DL PRS resource are defined based on the initial offset and the rule described in Clause 7.4.1.7.3 of [4, TS 38.211]. …

It is found that the description of this parameter in TS38.214 is not aligned with the description in 37.355 and incomplete, since this parameter should not only specifie REOffset, but also CombSizeN.

NR-DL-PRS-Resource-r16 ::= SEQUENCE { nr-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16, dl-PRS-SequenceID-r16 INTEGER (0.. 4095), dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { n2-r16 INTEGER (0..1), n4-r16 INTEGER (0..3), n6-r16 INTEGER (0..5), n12-r16 INTEGER (0..11), ... }, dl-PRS-ResourceSlotOffset-r16 INTEGER (0..nrMaxResourceOffsetValue-1-r16), dl-PRS-ResourceSymbolOffset-r16 INTEGER (0..12), dl-PRS-QCL-Info-r16 DL-PRS-QCL-Info-r16 OPTIONAL, --Need ON ... }

dl-PRS-CombSizeN-AndReOffset This field specifies the Resource Element spacing in each symbol of the DL-PRS Resource and the Resource Element (RE) offset in the frequency domain for the first symbol in a DL-PRS Resource. All DL-PRS Resource Sets belonging to the same Positioning Frequency Layer have the same value of comb size. The relative RE offsets of following symbols are defined relative to the RE Offset in the frequency domain of the first symbol in the DL-PRS Resource according to TS 38.211 [41]. The comb size configuration should be aligned with the comb size configuration for the frequency layer.

The following TP is proposed to align description of parameter in TS 38.214 and TS 37.355

Text Proposal

TS 38.214, section 5.1.6.5 < Unchanged parts are omitted > A DL PRS resource is defined by: … - dl-PRS-CombSizeN-AndReOffset defines the comb size of a DL-PRS Resource and the starting RE offset of the first symbol within athe DL PRS resource in frequency. The relative RE offsets of the remaining symbols within a DL PRS resource are defined based on the initial offset and the rule described in Clause 7.4.1.7.3 of [4, TS 38.211]. … < Unchanged parts are omitted >

## Aspect #3: Reference point for UL SRS-RSRP

In [2] and [3], it is noticed that RAN4 had discussed the reference point for UL SRS-RSRP triggered by RAN1 LS (R1-2112744) and had sent Response LS to RAN1 (R4-2202682). However, the current description of the reference point for UL SRS-RSRP in section 5.2.5 in 38.215 is not matched with the Response LS from RAN4 (R4-2202682).

The same TP is proposed for aspect #3 in [2] and [3].

Text proposal

TS 38.215 < Unchanged parts are omitted > ### 5.2.5 UL SRS reference signal received power (UL SRS-RSRP) Definition; UL SRS reference signal received power (UL SRS-RSRP) is defined as linear average of the power contributions (in [W]) of the resource elements carrying sounding reference signals (SRS). UL SRS-RSRP shall be measured over the configured resource elements within the considered measurement frequency bandwidth in the configured measurement time occasions. The reference point for UL SRS-RSRPP shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: based on the combined signal from antenna elements corresponding to a given receiver branch, - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. For frequency range 1 and 2, if receiver diversity is in use by the gNB, the reported UL SRS-RSRP value shall not be lower than the corresponding UL SRS-RSRP of any of the individual receiver branches.. < Unchanged parts are omitted >

FL note:

In definition, the abbreviation of UL SRS-RSRPP seems should be changed to UL SRS-RSRP, as there are different definitions for SRS-RSRP and SRS-RSRPP and reference points are applicable for both.

## Aspect #4: Correction to expected RSTD

In [3], it is observed that specification (TS 38.214) is not clear how the difference is defined for parameters “expected RSTD” in the assistance data. It is proposed to clarify that parameter in the assistance data is defined between the target DL PRS and the assistance data reference.

The following TP is proposed to address described isssue.

Text proposal

#### TS 38.214 section 5.1.6.5 #### 5.1.6.5 PRS reception procedure ========================= Unchanged parts ========================= The UE may be configured to report quality metrics NR-TimingQuality corresponding to the DL RSTD and UE Rx-Tx time difference measurements which include the following fields: - timingQualityValue which provides the best estimate of the uncertainty of the measurement - timingQualityResolution which specifies the resolution levels used in the timingQualityValue field. The UE expects to be configured with higher layer parameter nr-DL-PRS-ExpectedRSTD, which defines the time difference for the target DL PRS with respect to the received DL subframe timing of the reference indicated by the higher layer parameter nr-DL-PRS-ReferenceInfo, and nr-DL-PRS-ExpectedRSTD-Uncertainty, which defines a search window around the nr-DL-PRS-ExpectedRSTD. For DL UE positioning measurement reporting in higher layer parameters NR-DL-TDOA-SignalMeasurementInformation or NR-Multi-RTT-SignalMeasurementInformation the UE can be configured to report the DL PRS resource ID(s) or the DL PRS resource set ID(s) associated with the DL PRS resource(s) or the DL PRS resource set(s) which are used in determining the UE measurements DL RSTD, or UE Rx-Tx time difference, respectively. ========================= Unchanged parts =========================

Preparation Phase Discussion

Round #1

From FL perspective, all aspects provided in submitted contributions deserve clarifications and can be recommended for subsequent RAN1 e-mail discussion(s).

Companies are invited to express their views on maintenance aspects to be further discussed by RAN1.

Comments from companies:

| Company Name | Comments |
| --- | --- |
| CATT | We support to discuss and clarify all the four Aspects in subsequent RAN1 e-mail discussions. For the TP in Aspect#3, we share the same view with FL that the abbreviation of UL SRS-RSRPP should be changed to UL SRS-RSRP. |
| vivo | Support to discuss all four aspects. |
| Nokia/NSB | We support only discussing Aspect #3. The other aspects are clear when reading the spec together with LPP and TS 38.211. It is not necessary that 214 has word for word the same things as the other specs. If companies insist to discuss such non-essential issues that is fine, but we don’t support any of the changes. |
| Ericsson | OK with aspect #3. The other topics seem over-correction of the spec which is already very stable. |
| ZTE | We share the same view as Nokia and Ericsson. Per chair’s guidance, only essential corrections should be discussed. |
| OPPO | Aspect #3 does need change and typo of SRS-RSRPP should be corrected. Aspect#1, 2 and 4: they are not essential but they are good for improving the specification text. So we are ok to discuss them |
| Huawei, HiSilicon | We would be fine to discuss all the aspects, although from our side, we do not think that the change proposed in Aspect #2 is needed, because we already have “dl-PRS-CombSizeN” serving the purpose of indicating the comb size, while dl-PRS-CombSizeN-AndReOffset mainly serves the indication of comb offset. In addition, we think that reference [3] and [4] are somehow fixed up by the FL. |

Summary

7 companies agree to discuss Aspect #3

4 out of 7 companies expressed support to discuss all aspects

3 companies expressed the view that aspects 1, 2, 4 are not essential corrections of the spec

Considering that slight majority of companies prefer to discuss all aspects and that some of the proposed corrections (1, 2, 4) seems to be straightforward clarifications in nature, it seems worthwhile to have short discussion. The recommendation from FL is to organize short discussion for raised issues.

Conclusions

In this document, we have provided overview of the contributions submitted to RAN1#108e for R16 NR positioning maintenance. The following e-mail discussions were approved based on responses received in this document:

[108-e-R16-Pos-01] Email discussion/approval on the proposals in the draft CRs in R1-2202267 and R1-2202453 by February 25 – Xiaotao (CATT)

[108-e-R16-Pos-02] Email discussion/approval on editorial changes to be recommended to the specification editors on aspects 1, 2 and 4 in R1-2202519 by February 25 – Alexey (Intel)

References

- R1-2201077	Maintenance on Rel-16 NR positioning	vivo

- R1-2202267	Draft CR on reference point for UL SRS-RSRP	CATT

- R1-2202453	Correction to UL SRS-RSRP	Huawei, HiSilicon

- R1-2202420	Correction to expected RSTD	Huawei, HiSilicon