3GPP TSG-RAN WG1 #108-e 																	R1-2202567

e-Meeting, February 21st – March 3rd, 2022

Agenda item:			8.8.1.1

Source:	Moderator (Sharp)

Title:	FL Summary #2 on Enhancements on PUSCH repetition type A

Document for:	Discussion and Decision

Introduction

For PUSCH enahancements the following objectives are described in the Coverage Enhancement WID.

- Specification of PUSCH enhancements [RAN1, RAN4]

  - Specify the following mechanisms for enhancements on PUSCH repetition type A [RAN1]

    - Increasing the maximum number of repetitions up to a number to be determined during the course of the work.

    - The number of repetitions counted on the basis of available UL slots.

This document is intended to facilitate view exchange and discussions on the enhancements on PUSCH repetition type A, for the following assigned email discussion.

[108-e-R17-CovEnh-01] Email discussion regarding enhancements for PUSCH repetition type A – Toshi (Sharp)

- 1st check point: February 25

- Final check point: March 3

Discussion

Remaining issues

For this meeting, the following remaining issues have been raised.

- Issue#1-1: Available slot counting for DG-PUSCH scheduled by DCI format 0_0 with other RNTI than TC-RNTI

- Issue#1-2: Available slot counting for DG-PUSCH repetition Type A with K=1

- Issue#1-3: Available slot counting for CG-PUSCH repetition Type A with K=1

- Issue#1-4: The slot indicated by K2 offset for the available slot counting (DG-PUSCH with K=1)

- Issue#1-5: The slot indicated by K2 offset for the available slot counting (DG-PUSCH with K>1)

- Issue#1-6: The slot determined in 38.321 Section 5.8.2 for the available slot counting (CG-PUSCH with K=1)

- Issue#1-7: The slot determined in 38.321 Section 5.8.2 for the available slot counting (CG-PUSCH with K>1)

- Issue#1-8: Use of SSBs across multiple TRPs for the available slot determination

- Issue#1-9: Use of SSBs of other serving cells in half duplex CA operation for the available slot determination

[Close] Issue#1-1: Available slot counting for DG-PUSCH scheduled by DCI format 0_0 with other RNTI than TC-RNTI

So far, we have not made any agreement on whether available slot counting is applied to DG-PUSCH scheduled by DCI format 0_0 with other RNTIs than TC-RNTI. In the current TS38.214 [5], the behaviors related to the available slot counting is specified for DCI format 0_1 and 0_2 but not for DCI format 0_0. In RAN1#107bis-e, FL made the following proposal for a possible conclusion.

FL proposal: - A slot for PUSCH scheduled by DCI format 0_0 with C-RNTI / CS-RNTI with NDI=1 / MCS-C-RNTI is determined by physical slots (not available slots), irrespective of whether AvailableSlotCounting is enabled or not. - No specification impact is expected.

Although most of the companies accepted to make this clarification, a few companies raised the concern on the relationship between this proposed conclusion and the K2 timeline.

According to the contributions for RAN1#108-e, companies views are summarized as follows.

- A slot for PUSCH scheduled by DCI format 0_0 with C-RNTI / CS-RNTI with NDI=1 / MCS-C-RNTI is determined by physical slots (not available slots), irrespective of whether AvailableSlotCounting is enabled or not.

  - Nokia/ Nokia Shanghai Bell [7], Apple [18], CMCC [19]

To address the concern on the K2 timeline raised in the last meeting, it is clarified that Issues #1-4 and #1-5 are about PUSCH scheduled by DCI format 0_1 or 0_2, and also a sub-bullet saying “Note: the same K2 timeline as Rel-15/16 is applied” is added in the following proposal.

Proposed conclusion on Issue#1-1:

- A slot for PUSCH scheduled by DCI format 0_0 with C-RNTI / CS-RNTI with NDI=1 / MCS-C-RNTI is determined by physical slots (not available slots), irrespective of whether AvailableSlotCounting is enabled or not.

  - Note: No specification impact is expected.

  - Note: The same K2 timeline as in Rel-15/16 is applied.

1st round (Issue#1-1)

Do you agree on the proposed conclusion on Issue#1-1?

| Company | Comments |
| --- | --- |
| QC | Agree |
| ZTE | Ok with the proposed conclusion. |
| Panasonic | Agree |
| Apple | OK with the conclusion. So, this proposal implicitly means K=1 supported for available slot counting, as DCI format 0_0 doesn’t repetition, i.e., only supports K=1. It’s a bit overlapped with issue#1-2. |
| InterDigital | Yes. It is also fine to not make any new conclusion since this is the current status. |
| Intel | Although we do not think this is needed as AvailableSlotCounting is only applied for PUSCH repetitions, we are fine with the proposed conclusion. |
| NTT DOCOMO | We are fine with the conclusion. |
| LG | Agree |
| vivo | Fine. |
| Samsung | It is not clear this conclusion is needed. Available slot counting is for counting repetitions. |
| CMCC | Support. DCI 0_0 with other RNTI than TC-RNTI cannot schedule PUSCH repetitions. And the available slot counting is used for the PUSCH repetitions. Then the DCI format 0_0 scheduled slot should be determined by physical slots. |
| TCL | Agree |
| OPPO | Agree |
| CATT | Agree. |
| Ericsson | Agree |
| Spreadtrum | Agree |
| Huawei, HiSilicon | It was a RAN1 conclusion that PUSCH repetition type A cannot be scheduled by DCI format 0_0. RRC AvailableSlotCounting is only applicable to PUSCH repetition type A. Therefore, we don’t feel the proposal is necessary. For Msg3 repetitions, its retransmission may be scheduled by DCI 0_0, but it is a different UE feature from Rel-17 PUSCH repetition type A. We cannot assume a UE capable of Rel-17 PUSCH repetition type A can be scheduled by DCI 0_0 for PUSCH repetition type A even the UE is capable of Msg3 repetitions. |
| Nokia/NSB | Agree with Huawei/HiSilicon |

1st round summary (Issue#1-1)

The 1st round inputs are summarized as follows:

- Agree on taking the proposed conclusion on Issue#1-1

  - QC, ZTE, Panasonic, Apple, InterDigital, Intel, NTT DOCOMO, LG, vivo, CMCC, TCL, OPPO, CATT, Ericsson, Spreadtrum

- No need to make the proposed conclusion

  - Samsung, Huawei/HiSilicon, Nokia/NSB

15 companies agree on the proposals, while 5 companies do not think the proposed conclusion is necessary. It should be noted that all the companies (including the 5 companies that do not support the proposal) have the understanding that the available slot counting is not applicable to the PUSCH scheduled by DCI format 0_0 with C-RNTI / CS-RNTI with NDI=1 / MCS-C-RNTI. Based on the above, even without making the explicit conclusion, it is observed that all the companies have the common understanding on this point. Therefore, with this observation, FL would like to close this issue without making a formal conclusion.

[Pending] Issue#1-2: Available slot counting for DG-PUSCH repetition Type A with K=1 scheduled by DCI format 0_1 or 0_2

So far, we have not explicitly made any agreement on whether available slot counting is applied to DG-PUSCH with K=1 scheduled by DCI format 0_1 or 0_2. In RAN1#107bis-e, it was discussed whether the physical slot counting or the available slot counting is used for the case of  K=1 when AvailableSlotCounting is enabled

According to the contributions for RAN1#108-e, companies’ preferences are summarized as follows:

- The available slot counting is applied for PUSCH repetition type A with K=1 scheduled by DCI format 0_1 or 0_2, when AvailableSlotCounting is enabled.

  - Nokia/ Nokia Shanghai Bell [7], OPPO [10], CMCC [19], Ericsson (for DG-PUSCH) [20], Sharp [23]

- The physical slot counting is applied for PUSCH repetition type A with K=1 scheduled by DCI format 0_1 or 0_2, when AvailableSlotCounting is enabled.

  - CATT [11], Spreadtrum [15], InterDigital [16], Ericsson (for CG-PUSCH) [20], Qualcomm [22]

Even for the behaviour captured in the current version of RAN1 specifications, different companies have different interpretations. Nokia/Nokia Shanghai Bell [7] is saying that current specification allows available slot counting for PUSCH repetition type A with K=1 because the current specification does not specify any exception in terms of available slot counting. Meanwhile, argues that the available slot transmission procedure does not apply to K=1 case, but can only be used to K>1 case, because of the following descriptions in the current specification.

6.1.2.1 Resource allocation in time domain … For PUSCH repetition Type A, in case K>1: - If the PUSCH is scheduled by DCI format 0_1 or 0_2 - if AvailableSlotCounting is enabled, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot.

One thing that should be noted is that, for Msg3, the following TP has been agreed in AI 8.8.3 in RAN1#107bis-e. During the discussion on the TP, it was clarified that the value range of NPUSCHrepeat covers 1. This is the reason why “transmits” was replaced by “repeats”. Here, the following sentence says “the UE determines the NPUSCHrepeat slots as the first NPUSCHrepeat slots starting from slot n+k2+∆ where a repetition of the PUSCH transmission does not include a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or indicated as a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst” and this statement covers the case of NPUSCHrepeat=1. Therefore, according to the agreed TP, once the UE requests repetition of Msg3 PUSCH, the available slot counting is applied to Msg3 PUSCH even if K=1 is signaled by RAR UL grant. Note that Qualcomm []expresses their view that, for Msg3 transmission and retransmission, the use of available slot-based counting should be restricted only to the case when number of repetitions are greater than 1.

- 8.3 PUSCH scheduled by RAR UL grant <Unchanged parts are omitted> A UE can be provided in RACH-ConfigCommon a set of numbers of repetitions for a PUSCH transmission with PUSCH repetition Type A that is scheduled by a RAR UL grant or by a DCI format 0_0 with CRC scrambled by a TC-RNTI. If the UE requests repetition of PUSCH scheduled by RAR UL grant [11, TS 38.321], Tthe UE transmits repeats the PUSCH transmission over NPUSCHrepeat slots, where NPUSCHrepeat is indicated by the 2 MSBs of the MCS field in the RAR UL grant or in the DCI format 0_0 with CRC scrambled by the TC-RNTI, and determines a redundancy version and RBs for each repetition as described in [6, TS 38.214]. For unpaired spectrum operation, the UE determines the NPUSCHrepeat slots as the first NPUSCHrepeat slots starting from slot n+k2+∆ where a repetition of the PUSCH transmission does not include a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or indicated as a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. <Unchanged parts are omitted>

1st round (Issue#1-2)

Q1: Do you agree on the following FL’s observation?

- As there are two interpretations on the current specification in terms of the slot counting method for DG-PUSCH repetition Type A with K=1 scheduled by DCI format 0_1 or 0_2, RAN1 has to make a clear agreement/conclusion on whether the physical slot counting or the available slot counting is applied in this case. Otherwise, mis-aligned assumptions between the UE and the network may happen in the real field.

Q2: Do you think a unified slot counting method should be applied to the following two cases?

- PUSCH repetition type A with K=1 scheduled by RAR UL grant or by DCI format 0_0 with TC-RNTI when the UE requested repetition of the PUSCH scheduled by RAR UL grant, and

- PUSCH repetition type A with K=1 scheduled by DCI format 0_1 or 0_2 when AvailableSlotCounting is enabled.

| Company | Comments |
| --- | --- |
| QC | Q1: Yes, we don’t think K=1 was ever clearly discussed or clarified. It doesn’t even seem to be within the scope of this sub-agenda. A clarification/conclusion on the desired behavior is necessary. Q2: We prefer to retain legacy behavior for the case when K=1 for DCI 0_1 and DCI 0_2 irrespective of availableSlotCounting being enabled or not. We think this helps UE and gNB to not have to re-implement various checks for prioritization, out-of-order scheduling, overlap resolution, A-CSI multiplexing, etc. These procedures are currently tied to the value of K2 and if this does not hold anymore, this feature has a huge impact on existing implementations for both UE and gNB. It will significantly add to implementation and IODT overhead, while bringing no gains to coverage. Here is an example of what could happen with OoO scheduling: Here is an example of what could happen with checks for overlapped PUSCH transmissions Each of these checks will need to be rewritten if we don’t preserve legacy behavior for K=1. The implementation burden does not seem justified. |
| ZTE | Q1: Agree clarification is needed. Q2: Prefer to align the two cases. As it has concluded for Msg3 repetition to apply available slot counting also for K=1, the same can be applied to PUSCH repetition type A with K=1 scheduled by DCI format 0_1 or 0_2. In our view, this has no additional specification impact. |
| Panasonic | Q1: Agree Q2: Yes |
| Apple | Q1: agree Q2: Yes |
| InterDigital | Q1: In our understanding, the current specification applies “available slot counting” even for K=1. The part conditional to K>1 is only to specify that symbol allocation is the same between the repetitions. Nevertheless, RAN1 should make agreement/conclusion on this case. Q2: Yes. Our preference is that legacy behaviour continues to apply in all cases when K=1 and TBoMS is not enabled (N=1). |
| Intel | Q1: agree Q2: Yes |
| NTT DOCOMO | Q1: agree Q2: Yes |
| LG | Q1: We agree on the FL’s observation, and clarification on the UE behavior seems necessary. Q2: We prefer to apply the legacy behavior as a unified solution. |
| vivo | Q1: Agree. But for DG PUSCH, the first slot can be controlled by the network so that it can be always available in our view. And this should be up to implementation, there should be no issue in real field. Q2: Yes. |
| Samsung | Q1: OK to clarify, but it is up to the network to choose a proper K2. Q2: Preferable |
| CMCC | Q1: yes and no. From our understanding, the previous agreement is clear that K=1 is included in the available slot counting. But the current TP is not fully aligned with the agreement without decriptions of K=1. We support the clarify this issue at this meeting. Q2: No. We think the Msg 3 repetition is clear. And the issue here is to solve the K=1 in the available slot counting method. As we mentioned in Q1, the agreement of the available slot counting is clear. It is not necessary to introduce other elements for reference or considerations. |
| TCL | Q1: Agree Q2: Yes |
| OPPO | Q1: Agree. Q2: Yes. |
| FL | @vivo and Samsung: Some network vendor thinks such restriction is not necessary from not only specification perspective but also the real operation perspective. If no clarification is made, there is a possibility that the network sets K2 offset that indicates an invalid slot for UL, while the UE assumes that never happens. |
| CATT | (1) Yes and better to clarify. Although to us, it is clearly that current Rel-17 spec does not support K=1 case (only allow K>1 transmission with available slot counting). If no further agreements achieved, by default, K=1 with available slot counting is not supported in current spec, at least for DCI format 0_1 or 0_2. (2) Prefer to follow legacy behavior as Qualcomm mentioned, i.e. unified method and K=1 can only use physical slot counting. Even in AI 8.8.3, companies have very few discussion on K=1 case, since it is no much difference than legacy behavior. |
| Ericsson | Q1: The current specification on determination of available slots for DG-PUSCH can be applied to K=1 and K>1. The quoted clause for K>1 for DG-PUSCH is only about the same symbol allocated across multiple slots. Therefore, K=1 for DG-PUSCH with available slot counting is already supported in 38.214 v17.0.0. Anyway, we are fine with an agreement/conclusion if it helps the interpretation. Q2: Yes. |
| Spreadtrum | Q1: Agree. Q2: Yes. |
| Huawei, HiSilicon | Q1: agree Q2: for both cases, legacy UE behavior is preferred for K=1. |
| Nokia/NSB | Q1: agree with Samsung Q2: Not necessarily. |

1st round summary (Issue#1-2)

Q1: Do you agree on the following FL’s observation? - As there are two interpretations on the current specification in terms of the slot counting method for DG-PUSCH repetition Type A with K=1 scheduled by DCI format 0_1 or 0_2, RAN1 has to make a clear agreement/conclusion on whether the physical slot counting or the available slot counting is applied in this case. Otherwise, mis-aligned assumptions between the UE and the network may happen in the real field. Q2: Do you think a unified slot counting method should be applied to the following two cases? - PUSCH repetition type A with K=1 scheduled by RAR UL grant or by DCI format 0_0 with TC-RNTI when the UE requested repetition of the PUSCH scheduled by RAR UL grant, and - PUSCH repetition type A with K=1 scheduled by DCI format 0_1 or 0_2 when AvailableSlotCounting is enabled.

The 1st round inputs are summarized as follows:

- For Q1, all the companies agree to make a clear agreement/conclusion on whether the physical slot counting or the available slot counting is applied in the case of DG-PUSCH repetition Type A with K=1 scheduled by DCI format 0_1 or 0_2 when AvailableSlotCounting is enabled.

- For Q2,

  - A unified solution is preferred

    - (16 companies) QC, ZTE, Panasonic, Apple, InterDigital, Intel, NTT DOCOMO, LG, vivo, TCL, OPPO, CATT, Ericsson, Spreadtrum, Huawei/HiSilicon

  - Not necessarily

    - (3 companies) CMCC, Nokia/NSB

As all the companies agree to make a clear agreement/conclusion between the physical slot counting or the available slot counting, FL suggest trying down-select between them again under Issue#1-4.

[Pending] Issue#1-3: Available slot counting for CG-PUSCH repetition Type A with K=1

Similar to DG-PUSCH with K=1, in RAN1#107bis-e, it was discussed whether the physical slot counting or the available slot counting is used for the case of CG-PUSCH with K=1 when AvailableSlotCounting is enabled.

According to the contributions for RAN1#108-e, companies’ views are summarized as follows:

- The available slot counting is applied for PUSCH repetition type A with K=1, when AvailableSlotCounting is enabled.

  - Nokia/ Nokia Shanghai Bell [7], OPPO [10], CMCC [19], Sharp [23]

- The physical slot counting is applied for PUSCH repetition type A with K=1, when AvailableSlotCounting is enabled.

  - CATT [11], Spreadtrum [15], InterDigital [16], Ericsson [20], Qualcomm [22]

Unlike DG-PUSCH, in FL’s understanding the interpretation of behaviours for CG-PUSCH with K=1 in the current specification is quite clear. The available slot counting for CG-PUSCH is defined only for K>1 and there is no available slot counting procedure for CG-PUSCH with K=1 in the current specification. Therefore, if no specification change is agreed, then that means the physical slot counting is applied for PUSCH repetition type A with K=1, no matter AvailableSlotCounting is enabled.

For both Type 1 and Type 2 PUSCH transmissions with a configured grant, when K > 1, - For unpaired spectrum: - If AvailableSlotCounting is enabled, the UE shall repeat the TB across the N∙K slots determined for the PUSCH transmission applying the same symbol allocation in each slot. - A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - Otherwise, the UE shall repeat the TB across the N∙K consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. - For paired spectrum: - Irrespective of whether AvailableSlotCounting is enabled or not, the UE shall repeat the TB across the N∙K consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. - If AvailableSlotCounting is enabled, and in case of reduced capability half-duplex UE, a slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

1st round (Issue#1-3)

Q1: Do you agree on the following FL’s observation?

- The available slot counting for CG-PUSCH is defined only for K>1 and there is no available slot counting procedure for CG-PUSCH with K=1 in the current specification. Therefore, if no specification change is agreed, then that means the physical slot counting is applied for PUSCH repetition type A with K=1, no matter AvailableSlotCounting is enabled.

Q2: Do you think a unified slot counting method should be applied to the following two cases?

- DG-PUSCH repetition type A with K=1 scheduled by DCI format 0_1 or 0_2 when AvailableSlotCounting is enabled.

- CG-PUSCH repetition type A with K=1 when AvailableSlotCounting is enabled.

| Company | Comments |
| --- | --- |
| QC | Q1: Agree. Q2: Yes, a uniform behavior is desirable. It makes it a lot easier for the gNB scheduler and also for UE implementation. We prefer to retain legacy behavior when K=1 for all cases. |
| ZTE | Q1: Agree clarification is needed. Q2: Prefer to align the two cases. |
| Panasonic | Q1: Agree Q2: Yes |
| Apple | Q1: we think the UE behaviour is not defined for K=1 with available slot-based counting. We need conclusion, such as physical slot counting is applied for for K=1 with available slot-based counting. Q2: Yes |
| InterDigital | Q1: Agree. However, it also means that physical slot counting would be applied to TBoMS with K=1 and N>1, which may be against TBoMS agreement? Q2: Yes, we prefer unified slot counting method between DG-PUSCH and CG-PUSCH. |
| Intel | Q1: Agree Q2: it is good to have a unified solution for these two cases. |
| Panasonic | Q1: Agree Q2: Yes |
| LG | Q1: We agree on the FL’s observation. Q2: We prefer to apply a unified solution. |
| vivo | Q1: Yes Q2: In our understanding, for DG PUSCH with K=1, it should be always available in a normal dynamic scheduling, meaning that one can say it’s based on either available slot or physical slot. There’s no need to align CG with DG with respect to available slot determination. |
| Samsung | Q1: Yes Q2: ok to align, but not strictly necessary. |
| CMCC | Q1: We are not sure the understanding is correct. We have following agreements in #107 meeting. Clearly, repK-r17 is defined for Type 1 CG-PUSCH and Type 2 CG-PUSCH including repetition factor K=1. And numberOfRepetitions should also work for Type 2 CG-PUSCH. based on those agreements, the specification should be updated. If there is any misunderstanding, please correct me. Agreement - Rel-17 does not support numberOfRepetitions-r17 for DG-PUSCH scheduled by DCI format 0_0 and for Type 2 CG-PUSCH activated by DCI format 0_0. - repK-r17 supporting up-to-32 repetitions is introduced and is applicable to Type 1 CG-PUSCH and Type 2 CG-PUSCH (irrespective of the activating DCI format). - Note: No RAN1 spec impact is expected. - The possible values of repK-r17 includes 16 and 32. FFS: other values. - numberOfRepetitions-r17 is not applicable to Type 1 CG-PUSCH repetition type A. Agreement - All the following combinations support the counting based on available slots. - DG-PUSCH with Rel-15 repetition factor - Type-1 CG-PUSCH with Rel-15 repetition factor - Type-2 CG-PUSCH with Rel-15 repetition factor - DG-PUSCH with Rel-16 repetition factor - Type-2 CG-PUSCH with Rel-16 repetition factor - DG-PUSCH with Rel-17 repetition factor - Type-1 CG-PUSCH with Rel-17 repetition factor, if supported in Issue#1-1 - Type-2 CG-PUSCH with Rel-17 repetition factor Agreement - For repK-r17, - The value range of repK-17 is {1, 2, 4, 8, 12, 16, 24, 32}. - repK-r17 is included in ConfiguredGrantConfig. - When repK-r17 is provided, the legacy repK is not provided.. Q2: Yes, a unified behavior is desired. |
| TCL | Q1: Agree Q2: Yes |
| OPPO | Q1: Agree. Q2: Yes. |
| CATT | Q1: Agree. Q2: Open to discuss. Slightly prefer unified design to make the situation simpler. |
| Ericsson | Q1: Agree Q2: We prefer to keep the current specification. No need to align the counting method between DG-PUSCH and CG-PUSCH as they are configured/scheduled in different ways. |
| Spreadtrum | Q1: Agree. Q2: Yes. |
| Huawei, HiSilicon | Q1: agree Q2: for both cases, legacy UE behavior is preferred for K=1. |
| Nokia/NSB | Q1: Agree clarification is needed. Q2: Ok to align but not strictly necessary. |
| Xiaomi | Q1: Agree Q2: Yes |

1st round summary (Issue#1-3)

Q1: Do you agree on the following FL’s observation? - The available slot counting for CG-PUSCH is defined only for K>1 and there is no available slot counting procedure for CG-PUSCH with K=1 in the current specification. Therefore, if no specification change is agreed, then that means the physical slot counting is applied for PUSCH repetition type A with K=1, no matter AvailableSlotCounting is enabled. Q2: Do you think a unified slot counting method should be applied to the following two cases? - DG-PUSCH repetition type A with K=1 scheduled by DCI format 0_1 or 0_2 when AvailableSlotCounting is enabled. - CG-PUSCH repetition type A with K=1 when AvailableSlotCounting is enabled.

The 1st round inputs are summarized as follows:

- For Q1, almost all the companies agree on the FL’s observation.

- For Q2,

  - A unified solution is preferred

    - (15 companies) QC, ZTE, Panasonic, Apple, InterDigital, Intel, NTT DOCOMO, LG, CMCC, TCL, OPPO, CATT, Spreadtrum, Huawei/HiSilicon

  - Not necessarily

    - (5 companies) vivo, Samsung, Ericsson, Nokia/NSB

Similar to DG-PUSCH, FL suggest trying down-select for CG-PUSCH between the physical slot counting vs the available slot counting again under Issue#1-6.

[Open] Issue#1-4: The slot indicated by K2 offset for the available slot counting (DG-PUSCH with K=1 scheduled by DCI format 0_1 or 0_2)

In RAN1#107bis-e, it was discussed whether the slot indicated by K2 offset is by default an available slot or not. Companies have different views on whether the slot indicated by K2 offset shall be an available slot or not. Besides, several companies mentioned that whether the slot indicated by K2 offset shall be an available slot or not depends on whether K=1 or K>1. Furthermore, companies have different views on what behavior is captured in the current spec, e.g., some companies think it is precluded by the current spec language, some others think the current spec allows the slot to be an unavailable slot. The clarification on this point seems important, because different understanding on this point would cause mis-aligned assumptions between the UE and the network.  In the last meeting, FL provided the several options for each of the combinations of DG-PUSCH/CG-PUSCH and K=1/K>1, to collect companies’ views.

In this meeting, FL assigns different issue indices for different combination of DG-PUSCH/CG-PUSCH and K=1/K>1. Under this section, we discuss the case of DG-PUSCH, when AvailableSlotCounting is enables, and for K=1.

According to the RAN1#108-e contributions, companies views are summarized as follow:

- For DG-PUSCH scheduled by DCI format 0_1 or 0_2, when AvailableSlotCounting is enables, and for K=1,

  - Option 1: A UE does not assume that PUSCH symbols overlap with DL symbol or SSB symbol in the slot indicated by K2 offset is the slot which is not counted in K available slot(s).

      - CATT [11], China Telecom [13], Spreadtrum [15], Apple [18], TCL [24]

  - Option 2: A UE assumes that PUSCH symbols can overlap with DL symbol or SSB symbol in the slot indicated by K2 offset.

    - Option 2-1: When overlapping, the UE drops the PUSCH transmission in the slot indicated by K2 offset and does not perform the PUSCH transmission in later slots, either.

      - LG [25]

    - Option 2-2: When overlapping, the UE does not perform the PUSCH transmission in the slot indicated by K2 offset and performs the PUSCH transmission in the next available slot subject to PUSCH dropping rules.

      - Nokia/ Nokia Shanghai Bell [7], ZTE [9], OPPO [10], Panasonic [12], NTT DOCOMO [14], CMCC [19], Ericsson [20]

Related to this issue, Qualcomm [22] is raising an issue of Out-of-Order scheduling due to available slot counting. Considering a scenario where PDCCH received in Slot 0 schedules PUSCH in Slot 3 (using K2 offset of 3) and another PDCCH schedules a second PUSCH in Slot 4 (K2 offset of 3), this is a valid scheduling for Rel-15/16. For Rel-17, on the other hand, if Slot 3 is not an available slot for the first PUSCH transmission and the first PUSCH transmission gets deferred to Slot 4 with starting symbol later than that of the second PUSCH, then this leads to Out-of-Order.

Figure 1 of R1-2202151 [22]

In the FL’s understanding, this Out-of-Order scheduling is prohibited by the following descriptions in TS38.214 [5], where in this context, the “symbol j” for the first PUSCH transmission is the starting symbol of PUSCH after the deferral due to the available slot counting.

A UE shall upon detection of a PDCCH with a configured DCI format 0_0, 0_1 or 0_2 transmit the corresponding PUSCH as indicated by that DCI unless the UE does not generate a transport block as described in [10, TS38.321]. Upon detection of a DCI format 0_1 or 0_2 with 'UL-SCH indicator' set to '0' and with a non-zero 'CSI request' where the associated reportQuantity in CSI-ReportConfig set to 'none' for all CSI report(s) triggered by 'CSI request' in this DCI format 0_1 or 0_2, the UE ignores all fields in this DCI except the 'CSI request' and the UE shall not transmit the corresponding PUSCH as indicated by this DCI format 0_1 or 0_2. When the UE is scheduled with multiple PUSCHs by a DCI, HARQ process ID indicated by this DCI applies to the first PUSCH not overlapping with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, HARQ process ID is then incremented by 1 for each subsequent PUSCH(s) in the scheduled order, with modulo operation of nrofHARQ-ProcessesForPUSCH applied. HARQ process ID is not incremented for PUSCH(s) not transmitted if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. For any HARQ process ID(s) in a given scheduled cell, the UE is not expected to transmit a PUSCH that overlaps in time with another PUSCH. Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet for the active BWP of a serving cell and PDCCHs that schedule two non-overlapping in time domain PUSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex, for any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol i. When the PDCCH candidates are associated with a search space set configured with searchSpaceLinking, for the purpose of determining the PDCCH ending in symbol i, the PDCCH candidate that ends later in time among the two configured PDCCH candidates is used. The UE is not expected to be scheduled to transmit another PUSCH by a DCI format 0_0 with CRC scrambled by TC-RNTI, for a given HARQ process with the DCI received before the end of the expected transmission of the last PUSCH for that HARQ process if the latter is scheduled by a DCI format 0_0 with CRC scrambled by TC-RNTI or by an UL grant in RA Response. The UE is not expected to be scheduled to transmit another PUSCH by DCI format 0_0, 0_1 or 0_2 scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI for a given HARQ process with the DCI received before the end of the expected transmission of the last PUSCH for that HARQ process if the latter is scheduled by a DCI with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI.

Spreadtrum [15] raised another interesting point that whether overlapping with DL is allowed when DCI format 2_0 is monitored. In Rel-16, 38.213 clause 11.1.1 has the following restriction in terms of collision between DG-PUSCH and DL symbols indicated by SFI. For Rel-17, it may need to be clarified whether this restriction applies after the available slot determination or before the available slot determination. Normally, the DL symbols configured by tdd-UL-DL-Configuration are also indicated as DL symbols by SFI in DCI format 2_0. Therefore, if the below restriction applies before the available slot determination, the UE does not expect the PUSCH in the slot indicated by K2 offset overlaps with DL symbols configured by tdd-UL-DL-Configuration when DCI format 2_0 monitoring is configured. In contrast, if the restriction applies after the available slot determination, the UE assumes the PUSCH in the slot indicated by K2 offset can overlap with DL symbols configured by tdd-UL-DL-Configuration.

For a set of symbols of a slot, a UE does not expect to detect a DCI format 2_0 with an SFI-index field value indicating the set of symbols in the slot as downlink and to detect a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR indicating to the UE to transmit PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot.

1st round (Issue#1-4)

In the 1st round discussion, FL suggest firstly discussing the new aspects that we have not discussed yet, rather than discussing the listed options again.

Q1: Please provide your views on the above-described Out-of-Order scheduling issue.

Q2: Please provide your views on the collision handling between DG-PUSCH and DL symbols indicated by dynamic SFI.

| Company | Comments |
| --- | --- |
| QC | Q1: The above was just one example. Resolving overlapping PUSCH transmissions is another case. We believe more issues will surface as the features go through the rigors of implementation. We feel these ancillary aspects could significantly hamper this feature’s commercial deployment. Q2: We don’t think a UE should be given such a conflicting configuration. This should remain as an error case with or without available slot counting. Any other option would require very detailed timeline discussions. We prefer to avoid these issues by restricting the slot pointed to by K2 offset to be an available slot. We believe a vast majority of commercial BSs already adher to this principle and we prefer to formalize it. |
| ZTE | Q1: The order of order issue could be regarded as an error case. Q2: Our understanding is the cited restriction for dynamic SFI is applied after the slot determination which is only based on semi-static configurations. |
| Panasonic | Q1: We agree to the above FL’s understanding. Second PUSCH transmission can only be scheduled after the end of the first PUSCH which includes the deferral due to the available slot counting. Q2: The restriction should be applied after the available slot determination. |
| Apple | Q1: for the example case, we think it’s an error case, the slot for first transmission should be always available slot, which can be used for UL transmission. There is no reason that gNB schedules the first transmission on an invalid slot. Q2: We believe this is an error case. The first dynamic grant transmission is always controllable by gNB, it doesn’t make sense to schedule the UE on invalid resources, then UE delay the transmission to next available sources which is still managed by gNB. |
| InterDigital | Q1: The signaling does not prevent potential out-of-order issue in R15/R16 either. It is up to the network to avoid this case. Q2: Prefer to keep the same as legacy behaviour for K=1 and N=1. In our understanding this corresponds to Option 2-1. There is no need to identify this as an error case in the specs. |
| Intel | Q1: We think it depends on whether to consider the first slot indicated by K2 is available slot. Q2: Our view is that dynamic SFI is applied after the available slot determination in the 2-step approach. |
| NTT DOCOMO | Q1: We agree to FL understanding. On the other hand if these issues (e.g. Q1/Q2) bring difficulty for UE implementation, we may understand they can be error cases. |
| LG | Q1: In our view, out-of-order issue may be avoided by the network, since available slots are determined based on semi-static configurations. However, it seems make the issue clear not to postpone the PUSCH transmission slot for K=1. Q2: The restriction should be applied after the available slot determination. |
| vivo | The transmission of different PUSCH transmissions are up to network implementation to make sure the scheduling is in order as specified in NR Rel-15/16 specification in our understanding. It seems not necessary to discuss the legacy collision rules to be applied for an actual PUSCH transmission here either. |
| Samsung | Q1: Scheduling issues can be avoided by the network. Q2: This case is also a scheduling issue that the network can handle. |
| CMCC | Q1: the out of order scheduling issue should be an error case. As gNB has the full information of the available slot and the scheduling behind, it should not schedule any transmission which may induce an out of order scheduling. |
| TCL | Q1: We think the out-of-order issue can be avoided by gNB, anyway, the network could ensure the slot for the first transmission is available slot. Q2: We think dynamic SFI is applied after the available slot determination. |
| OPPO | Q1: We agree to FL’s understanding, out-of-order issue may be avoided by the network. Q2: The restriction should be applied after the available slot determination. |
| CATT | Q1: Considering that Rel-15/16 rules are based on physical slot counting, some clarification will help. We are OK if we can conclude that ‘the “symbol j” for the first PUSCH transmission is the starting symbol of PUSCH after the deferral due to the available slot counting’, so the out-of-order issue is resolved. Q2: We think the current spec prohibits conflicted configuration between two dynamic signals (i.e. SFI and scheduling DCI). We are OK if we can conclude that the legacy rule is applied after available slot counting. So such collision is avoided. |
| Ericsson | Q1: The non-out-of-order rule applies to PUSCH repetition based on available slots. The actual PUSCH transmission is used to determine 'the end of the first PUSCH' if it is postponed. Q2: FL provides interpretations regarding the restriction is applied before and after the first step. We would like to clarify the impact of the restriction on step 2. For DG-PUSCH with K=1, if the restriction applies after the available slot determination, does it apply before the second step? If so, it prevents the collision between a DL slot indicated by dynamic SFI and the determined available slot. In other words, the flexible symbols which are considered as an available slot in the first step should not be indicated as DL by dynamic SFI, if any. To answer the question if the slot indicated by K2 offset is always an available slot, we also need to consider no dynamic SFI case. |
| Spreadtrum | Q1: We think it’s an error case and can be avoided by gNB. Q2: We share the view with QC and Apple. We think it is an error case if PUSCH symbols overlapping with DL symbol or SSB. For DL symbol configured by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, and these DL symbols cannot be scheduled to transmit DG-PUSCH. The gNB performs the first transmission on the time slot indicated by K2. |
| Nokia/NSB | Q1: In our view, PUSCH transmissions can be handled by the network to make sure the scheduling is in order and that the slot for first transmission is always an available slot. Q2: This also seems a scheduling issue that network can handle. |

1st round summary (Issue#1-4)

Q1: Please provide your views on the above-described Out-of-Order scheduling issue. Q2: Please provide your views on the collision handling between DG-PUSCH and DL symbols indicated by dynamic SFI.

- For Q1, the large majority thinks that it should be assumed that the gNB schedules DG-PUSCHs such that Out-of-Order does not happen even with the available slot counting.

- For Q2, the large majority thinks that it should be assumed that the gNB schedules DG-PUSCHs such that it does not collide with DL symbols indicated by SFI even with the available slot counting. In addition, 7 companies (ZTE, Panasonic, Intel, LG, TCL, OPPO, CATT) agrees on that the restriction that no collision between DG-PUSCH and DL indicated by SFI is expected should be applied after the available slot determination. In fact, this aspect is not much related to K2 timeline discussion. FL suggests looking into a bit more details later for the case that DCI 2_0 is monitored.

Based on the above, FL would like to make the following proposal.

FL proposal on Issue#1-4:

- It should be assumed that the gNB schedules DG-PUSCHs such that Out-of-Order does not happen even with the available slot counting.

2nd round (Issue#1-4)

Q1: Provide your comment only if you have a strong concern on the above FL proposal on Issue#1-4.

Q2: Do you agree on that, when checking A-CSI reference resource identification, the slot indicated by K2 offset needs to be valid for UL.

| Company | Comments |
| --- | --- |
| QC | Q1: While its good that companies assume that OoO issues should be handled by the gNBs, this doesn’t address the underlying concern. Not mandating that the slot indicated by K2 offset to be an available slot means that K2 can no longer be used to determine the start of a PUSCH transmission. Legacy UE (and gNB, we assume) implementations rely on K2 parameter to carry out checks for cancellation, prioritization, multiplexing, etc. This entire logic will now be impacted. The feature has a very large footprint on UE uplink operations as a consequence. We feel this will hamper commercializing this feature especially in TDD bands where it might be most useful. While we are okay with the proposal, what we want is for the slot pointed to by K2 offset to be an available slot. Q2: The point we were trying to make here is that CSI multiplexing requires looking back from the slot where PUSCH transmission occurs to figure out which CSI resource is to be used as a reference. K2 is used to determine this. If K2 points to an invalid slot, then once again this procedure too is impacted. Also, as an aside, we see that for the case where we have A-CSI multiplexing on PUSCH without UL-SCH payload, the slot for transmission continues to be specified using K2 (See Section 6.1.2.1 in 38.214). Do we then assume that available slot counting does not apply to this case? This then once again leads to divergent behavior depending on whether there is a transport block to transmit or not. Not guaranteeing the slot pointed to by the K2 offset to be an available slot leads to unnecessary additional UE complexity, with no benefit to the network. We think such a configuration should be barred. |
| vivo2 | Q2: A-CSI multiplexing on PUSCH is performed first and collision handling is performed in a second step in our understanding, so A-CSI reference resource identification does not have to consider the validation. |
| CMCC | Share similar view as vivo. |
| LG | Q2: If the slot indicated by K2 cannot be available and the first slot of PUSCH transmission is postponed in that case, the slot for A-CSI reporting also should be postponed in our view. If this behavior causes other problems, it seems better to assume the slot indicated by K2 is available always. |
| Nokia/NSB | Q2: aligned with Vivo. |
| Intel | Q1: for FL proposal on Issue#1-4, our understanding is that even if the slot indicated by K2 is not available slot for whatever reason gNB decides to do so, we should still need to ensure that OOO handling is determined based on the slot indicated by K2 value. This would help ensure the same timeline checking at UE side without changing the implementation. Q2: we share similar view as Vivo. |
| InterDigital | Q2: what Vivo suggests seems reasonable. However, there may specification impact in section 5.2.2.5 of 38.214 because the CSI reference resource there is referred to the uplink slot in which PUSCH transmission takes place. To avoid this impact, it may be ok to require that first slot is available for the DG case. |
| Ericsson | Q2: Agree with vivo. A-CSI reference resource identification doesn’t affect if the slot indicated by K2 offset is available or not. |
| Samsung | Q2: same understanding as vivo and others above. |
| Spreadtrum | Q1: We support the proposal. While if K =1, K2 can only be an available slot would be no problem for OoO. Q2: Yes, it should be a valid slot. We do not agree with vivo. Because the determination of CSI reference resource is based on “a CSI reporting in uplink slot n'”, not the slot indicated by K2. So A-CSI multiplexing on PUSCH is performed later than determine the PUSCH slot. There is no problem if the slot indicated by K2 is an available slot. 38214 clause 5.2.2.5 CSI reference resource definition In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref,. |

2nd round summary (Issue#1-4)

Q1: Provide your comment only if you have a strong concern on the above FL proposal on Issue#1-4. Q2: Do you agree on that, when checking A-CSI reference resource identification, the slot indicated by K2 offset needs to be valid for UL.

The 2nd round inputs are summarized as follow:

- For Q1, there was no objection to the FL proposal on Issue#1-4. On the other hand, Intel raised another point that we should still need to ensure that OoO handling is determined based on the slot indicated by K2 value. If this is required and if the slot indicated by K2 offset can be unavailable for UL, then the scheduler has to take case of two different OoO handling – one is OoO handling before deferral and the other is OoO handling after deferral, and the UE also needs two different OoO checks.

- For Q2, 3 companies (QC, LG, Spreadtrum) see the issue for A-CSI reference resource identification from the UE complexity perspective, and 1 company (InterDigital) see some specification impact on CSI reference resource definition. Meanwhile, 7 companies (vivo, CMCC, Nokia/NSB, Intel, Ericsson, Samsung) do not see any issue for A-CSI reference resource identification.

- 1 company (QC) raised another point that which counting method is applied to PUSCH without UL-SCH when AvailableSlotCounting is enabled, and whether a unified counting method is applied to the PUSCH with UL-SCH with K=1 and the PUSCH without UL-SCH.

As several companies identified some UE complexity increase if the UE needs to handle the deferral of PUSCH transmission from the slot indicated by the K2 offset, FL would like to suggest taking the following proposal.

Updated FL proposal on Issue#1-4:

- For DG-PUSCH scheduled by DCI format 0_1 or 0_2, when AvailableSlotCounting is enabled, and for K=1,

  - The UE does not expect that the PUSCH symbols in the slot indicated by K2 offset overlap with DL symbol or SSB symbol.

    - FFS: Whether this restriction is captured in the specification.

    - Note: With this restriction, there is no difference between the physical slot counting and the available slot counting for DG-PUSCH with K=1.

[Pending]  Issue#1-5: The slot indicated by K2 offset for the available slot counting (DG-PUSCH with K>1 scheduled by DCI format 0_1 or 0_2)

Under this section, we discuss the case of DG-PUSCH, when AvailableSlotCounting is enables, and for K>1.

According to the RAN1#108-e contributions, companies views are summarized as follow:

- For DG-PUSCH scheduled by DCI format 0_1 or 0_2, when AvailableSlotCounting is enables, and for K>1,

  - Option 1: A UE does not assume that the slot indicated by K2 offset is the slot which is not counted in K available slot(s).

      - Spreadtrum [15], Apple [18], Qualcomm [22]

  - Option 2: A UE assumes that the slot indicated by K2 offset can be the slot which is not counted in K available slot(s).

      - Nokia/ Nokia Shanghai Bell [7], ZTE [9], OPPO [10], Panasonic [12], China Telecom [13], NTT DOCOMO [14], CMCC [19], Ericsson [20], LG [25]

Samsung [21] expresses their view that the slot indicated by K2 is subject to the same conditions as any other slot when available slot counting is enabled, however a gNB would indicate a value of K2 so that the indicated slot is an available slot and the PUSCH transmission can be actually transmitted in that slot, similar to Rel-15/16.

Panasonic [12], Ericsson [20] and Sharp [23] have the understanding that, in Rel-15/16 PUSCH repetitions, there is no restriction that the slot indicated by K2 offset needs to be valid for the PUSCH transmission. On the contrary, TS38.213 Clause 11.1 [4] defines the collision handling rule between DG-PUSCH and DL/SSB symbols as the following.

For a set of symbols of a slot that are indicated to a UE as downlink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit PUSCH, PUCCH, PRACH, or SRS when the PUSCH, PUCCH, PRACH, or SRS overlaps, even partially, with the set of symbols of the slot. For a set of symbols of a slot that are indicated to a UE as flexible by tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated if provided, the UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE in the set of symbols of the slot and dedicated higher layer parameters configuring reception by the UE in the set of symbols of the slot. For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE.

1st round (Issue#1-5)

Q: Please provide your views on Rel-15/16 UE’s assumptions for Rel-15/16 DG-PUSCH with K>1.

- Assumption #1: Rel-15/16 UEs may consider the case of overlapping between PUSCH symbols in the slot indicated by K2 offset and DL symbols as an unspecified error case.

- Assumption #2: Rel-15/16 UEs need to assume that overlapping between PUSCH symbols in the slot indicated by K2 offset and DL symbols may occur.

| Company | Comments |
| --- | --- |
| QC | Not sure what “unspecified error case” means, but we would like the slot indicated by K2 to be an available slot. Without this, checks for OoO scheduling A-CSI reference resource identification, etc, will need to be discussed and re-implemented. A question for companies suggesting that K2 offset could point to an unavailable slot, what is the benefit of this? Why is the base station not able to ensure such a thing does not occur? |
| ZTE | Assumption #2. Agree that the collision is allowed and dropping behavior is specified according to the cited spec texts. Regarding QC’s question, our understanding is it depends on the available K2 values for scheduling. It is possible that gNB cannot find a proper K2 value to avoid collision for the first repetition in TDD, with also considering the available PDCCH transmission occasions. In addition, even gNB can avoid the collision for the first repetition in some cases with a proper K2, it may not be able to avoid collisions for the rest of repetitions. Possibly, it may cause more collisions due to ensure the first repetition is available. |
| Panasonic | Assumption #2 |
| Apple | Assumption#1. We are not so sure what is gNB intention to do so. The UL transmission is not delayed to next slot, only result is UE drop the scheduled UL transmission and the UL grant is wasted. |
| InterDigital | Assumption #2. For DG there is no reason why the network would do this, but for the CG case it may be a bit restrictive for the network to disallow it. We prefer same behaviour for CG and DG. |
| Intel | Assumption #2. This seems similar to the discussion for the first PUCCH repetition. |
| NTT DOCOMO | Assumption #2. |
| LG | Assumption #2. |
| vivo | Assumption #2. |
| Samsung | The network would indicate a K2 value that schedules a transmission in a slot where the transmission can actually be possible. |
| CMCC | Assumption #2. |
| TCL | Assumption #2. |
| OPPO | Assumption #2. |
| CATT | Although we agree that the spec does not limit K2 slot must be a valid one, we do feel it is natural for a gNB to indicate a valid slot to the UE as the starting position. For PUCCH repetitions with K>1, as far as we know, Rel-16 clarifies that the 1st PUCCH can be in an unavailable slot (mainly due to it is already counted by available slot), but the group does not achieve common understanding whether it is also applied to Rel-15. So literately, Assumption#2 is interpreted. Nevertheless, even if we cannot agree the legacy behavior in Rel-15/16, we still have a chance to take Assumption#2 for Rel-17 PUSCH available slot counting, right? |
| Ericsson | Assumption #2. |
| Spreadtrum | Assumption#1.There is no reason for gNB to schedule an unavailable slot for the slot indicated by K2 offset. Because, K2 range is large enough e.g. from 0 to 32, so it can find a suitable slot for the PUSCH transmission. Therefore, the slot indicated by K2 offset is an available slot on which DG-PUSCH symbols cannot overlap with DL symbols. |
| Nokia/NSB | Assumption #2. In general, it is reasonable to assume that network would indicate a K2 value that schedules a transmission in a slot where the transmission is possible. |

1st round summary (Issue#1-5)

The 1st round inputs on Rel-15/16 UE’s assumptions for Rel-15/16 DG-PUSCH with K>1 are summarized as follows:

- Assumption #1: Rel-15/16 UEs may consider the case of overlapping between PUSCH symbols in the slot indicated by K2 offset and DL symbols as an unspecified error case.

  - QC, Samsung, Spreadtrum

- Assumption #2: Rel-15/16 UEs need to assume that overlapping between PUSCH symbols in the slot indicated by K2 offset and DL symbols may occur.

  - ZTE, Panasonic, InterDigital, Intel, NTT DOCOMO, LG, vivo, CMCC, TCL, OPPO, CATT, Ericsson, Nokia/NSB

In terms of Rel-15/16 UE’s assumptions for Rel-15/16 DG-PUSCH with K>1, it seems the majority of the companies consider Assumption#2 while 3 companies have different understanding. QC raised the concern that, in order to check A-CSI reference resource identification, the slot indicated by K2 offset needs to be a valid slot for UL.

It also seems that the discussion point is the same as for Issue#1-4. Therefore, it is suggested having technical discussions (including A-CSI reference resource identification aspect) under Issue#1-4 to avoid duplicated discussions.

[Open] Issue#1-6: The slot determined in 38.321 Section 5.8.2 for the available slot counting (CG-PUSCH with K=1)

Under this section, we discuss the case of CG-PUSCH, when AvailableSlotCounting is enables, and for K=1.

According to the RAN1#108-e contributions, companies views are summarized as follow:

- For CG-PUSCH, when AvailableSlotCounting is enables, and for K=1,

  - Option 1: A UE does not assume that PUSCH symbols overlap with DL symbol or SSB symbol in the slot determined in 38.321 Section 5.8.2.

  - Option 2: A UE assumes that PUSCH symbols can overlap with DL symbol or SSB symbol in the slot determined in 38.321 Section 5.8.2.

    - Option 2-1: When overlapping, the UE drops the PUSCH transmission in the slot determined in 38.321 Section 5.8.2 and does not perform the PUSCH transmission in later slots, either.

      - China Telecom [13], Ericsson [20], TCL [24], LG [25]

    - Option 2-2: When overlapping, the UE does not perform the PUSCH transmission in the slot determined in 38.321 Section 5.8.2 and performs the PUSCH transmission in the next available slot subject to PUSCH dropping rules.

      - Nokia/ Nokia Shanghai Bell [7], ZTE [9], OPPO [10], Panasonic [12], NTT DOCOMO [14], CMCC [19], Sharp [23]

It seems all the companies that express their views on this issue in their contributions prefer taking Option 2. Therefore, FL made the following proposal. If this proposal is agreeable, then the remaining issue for CG-PUSCH with K=1 is the decision between Options 2-1 and 2-2, which is highly correlated with Issue#1-3.

FL proposal on Issue#1-6:

- For CG-PUSCH, when AvailableSlotCounting is enables, and for K=1, a UE assumes that PUSCH symbols can overlap with DL symbol or SSB symbol in the slot determined in 38.321 Section 5.8.2.

1st round (Issue#1-6)

Do you agree on the above FL proposal on Issue#1-6?

| Company | Comments |
| --- | --- |
| ZTE | Support |
| Panasonic | Agree |
| InterDigital | Agree. For K=1 and CG-PUSCH (and N=1) we prefer Option 2-1. |
| Intel | Just clarify: is there any spec impact for this? |
| NTT DOCOMO | Agree. |
| LG | Agree |
| vivo | Same comment as for issue 1-4. |
| QC | What is the benefit of such a configuration? Why cant the gNB point to an available slot in Section 5.8.2? If we are relaxing this, there needs to be a strong justification. |
| CMCC | To FL should this proposal is for K=1 not K>1 ? FL proposal on Issue#1-6: - For CG-PUSCH, when AvailableSlotCounting is enables, and for K>1, a UE assumes that PUSCH symbols can overlap with DL symbol or SSB symbol in the slot determined in 38.321 Section 5.8.2. |
| TCL | Agree. |
| OPPO | Agree. |
| FL | @CMCC and All: I noticed a typo “K>1” in the proposal, which is corrected to be “K=1”. Thanks, CMCC, for spotting the typo! @QC: At least my understanding is that Rel-15/16 allows the network to configure CG-PUSCH with K=1 with CG period equal to 14 symbols (i.e., 1 slot) for any TDD configuration such as “DDSUU”. In this case, the slot determined in Section 5.8.2 is any slot including “D” slot, but the UE does not transmit CG-PUSCH in “D” slot, because of the rule defined in 38.213 Section 11.1. The consequence of this configuration is that the UE can start the CG-PUSCH transmission at any UL slot. |
| CATT | OK with K=1 CG-PUSCH case (with the typo corrected). |
| Ericsson | Agree the revised FL proposal |
| Spreadtrum | Agree |
| Huawei, HiSilicon | If a gNB would have to ensure all PUSCH symbols in CG configurations not to overlap with any DL symbol or SSB symbol, then it would have been not necessary to introduce the feature enabled by AvailableSlotCounting in the first place. A clarification may be needed, whether the PUSCH symbols in the proposal refer to the symbols in the first configured slot of a CG periodicity. |
| Nokia/NSB | Agree. Option 2-2 as per above list is the best course of action in our view. |

1st round summary (Issue#1-6)

Almost all the companies accepted the following proposal. To move one-step forward (i.e., precluding Option 1), FL suggests taking the following proposal. As for the comment raised by Intel and vivo, spec impact can be discussed as part of down selection between Options 2-1 and 2-2. As for the clarification question by Huawei, “the slot determined in 38.321 Section 5.8.2” is the first configured slot in a CG period. More specifically, it means the slot which contains “the symbol” determined in 38.321 Section 5.8.2 (see the excerpt below).

After an uplink grant is configured for a configured grant Type 1, the MAC entity shall consider sequentially that the Nth (N >= 0) uplink grant occurs in the symbol for which: [(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] = (timeReferenceSFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). After an uplink grant is configured for a configured grant Type 2, the MAC entity shall consider sequentially that the Nth (N >= 0) uplink grant occurs in the symbol for which: [(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] = [(SFNstart time × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slotstart time × numberOfSymbolsPerSlot + symbolstart time) + N × periodicity] modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). where SFNstart time, slotstart time, and symbolstart time are the SFN, slot, and symbol, respectively, of the first transmission opportunity of PUSCH where the configured uplink grant was (re-)initialised.

FL proposal on Issue#1-6:

- For CG-PUSCH, when AvailableSlotCounting is enables, and for K=1, a UE assumes that PUSCH symbols can overlap with DL symbol or SSB symbol in the slot determined in 38.321 Section 5.8.2.

Note that the selection from Options 2-1 (i.e. the physical slot counting) and 2-2 (i.e. the available slot counting) is equivalent to Issue#1-3, therefore further down-selection will be discussed under Issue#1-3.

2nd round (Issue#1-6)

Q1: Provide your comment only if you have a strong concern on the above FL proposal on Issue#1-6.

Q2: For CG-PUSCH, when AvailableSlotCounting is enables, and for K=1, select one from the following:

- Option 2-1 (physical slot counting with PUSCH dropping): When PUSCH symbols overlap with DL symbol or SSB symbol in the slot determined in 38.321 Section 5.8.2, the UE drops the PUSCH transmission in the slot and does not perform the PUSCH transmission in later slots of the same CG period, either.

- Option 2-2 (available slot counting): When PUSCH symbols overlap with DL symbol or SSB symbol in the slot determined in 38.321 Section 5.8.2, the UE does not perform the PUSCH transmission in the slot and performs the PUSCH transmission in the next available slot of the same CG period subject to PUSCH dropping rules.

| Company | Comments |
| --- | --- |
| Panasonic | Q2: Our preference is Option 2-2 and a unified slot counting method should be applied to “DG-PUSCH with K=1 when AvailableSlotCounting is enabled” and “CG-PUSCH with K=1 when AvailableSlotCounting is enabled”. |
| Xiaomi | Q1:agree FL proposal Q2: prefer option 2-2. Because AvailableSlotCounting is enabled, so slot counting based on available slot should be performed when the original slot is unavailable. |
| QC | Q1: Why does a UE have to deal with such configurations? What is the benefit? This seems like a careless configuration by the gNB. We don’t see any benefit to such configurations. Q2: We don’t think K=1 is within the scope of the WI. We prefer to go with Option 2-1. |
| vivo2 | Fine. |
| FL | @QC, Let me echo my previous comment. Let’s assume DDSUU configuration. How can the R15 gNB fully utilize UL slots for CG-PUSCH with K=1? In order to achieve this, the R15 gNB can configure CG periodicity set to 1-slot length. This is not a careless configuration but an appropriate configuration which enables full use of the UL slots. Therefore, R15 UE capable of CG has to be able to deal with a collision between CG-PUSCH with K=1 and DL symbols. |
| CMCC | Q1: support Q2: option 2-2. Following the agreements of the available slot counting, option 2-2 is correct understanding. |
| LG | Q2: We prefer Option 2-1.We’d like to apply the legacy rule for DG/CG-PUSCH for K=1. |
| CATT | Q1: OK. Q2: Option 2-2 is slightly prefer. Can live with Option 2-1. |
| Nokia/NSB | Q1: agree with FL proposal. Please note that there is a typo similar to FL proposal on Issue#1-7, i.e., enables enabled. Q2: Option 2-2 is our preference. |
| Intel | Q1: we do not support the proposal. Our understanding is that counting based on available slot is only applied for the case for PUSCH repetitions, i.e., K > 1. For single slot CG-PUSCH or DG-PUSCH transmission, we do not apply available slot counting, which is not aligned with the scope in the WID. |
| InterDigital | Q1: support proposal in principle. However, does the condition “K=1” include TBoMS with N>1? If this is intended to non-TBoMS case (N=1) this should be clarified. Q2: prefer 2-1 to keep consistent behaviour with R15/R16 when K=1 and N=1. |
| FL | @Intel, Thanks for the feedback on Q1. The proposal is not saying that the available slot counting is used for CG-PUSCH with K=1. Q1 is to ask whether to introduce the configuration restriction that the gNB has to configure CG resources and TDD configuration such that the CG resouce in any CG period does not overlap with DL symbols. On the contrary, when the overlapping happens to CG-PUSCH with K=1, whether the physical slot counting (i.e., option 2-1) or the available slot counting (i.e., option 2-1) is applied would be the next discussion point. Hope the above clarifies the intention. @InterDigital, Thanks. Yes, your understanding is correct. This discussion is not intended to cover TBoMS. The clarifications on TBoMS should be discussed under AI 8.8.1.2. |
| Ericsson | Q1: support FL proposal. Q2: Option 2-1 is preferred. It is aligned with current specification. |
| Samsung | Q2: Option 2-1. FL proposal on Issue#1-6 +Option 2-1 seems to be equivalent to not supporting available slot counting with K=1. Would it be better to simply agree whether or not to support available slot counting with no repetition (for DG-PUSCH/CG-PUSCH)? |
| Spreadtrum | Q1: Agree Q2: Option 2-1. The Rel-17 specification doesn’t include available slot counting procedure for CG-PUSCH with K=1. We prefer Option 2-1 to reuse Rel-15/16 counting method based on physical slots. |
| OPPO | Q1: Support. Q2: We prefer option 2-2. |

2nd round summary (Issue#1-6)

The 2nd round inputs are summarized as follows:

- For the FL proposal on Issue#1-6, only 2 companies (QC and Intel) expressed strong concerns.

FL proposal on Issue#1-6:

  - For CG-PUSCH, when AvailableSlotCounting is enables, and for K=1, a UE assumes that PUSCH symbols can overlap with DL symbol or SSB symbol in the slot determined in 38.321 Section 5.8.2.

- As for further down-selection, the slight majority prefers Option 2-1, i.e., the physical slot counting, although the number of supporting companies are close to even.

  - Option 2-1 (physical slot counting with PUSCH dropping): When PUSCH symbols overlap with DL symbol or SSB symbol in the slot determined in 38.321 Section 5.8.2, the UE drops the PUSCH transmission in the slot and does not perform the PUSCH transmission in later slots of the same CG period, either.

    - (8 companies) QC, LG, CATT (can live with), Intel(?), InterDigital, Ericsson, Samsung, Spreadtrum

  - Option 2-2 (available slot counting): When PUSCH symbols overlap with DL symbol or SSB symbol in the slot determined in 38.321 Section 5.8.2, the UE does not perform the PUSCH transmission in the slot and performs the PUSCH transmission in the next available slot of the same CG period subject to PUSCH dropping rules.

    - (7 companies) Panasonic, Xiaomi, CMCC, CATT, Nokia/NSB, OPPO

For CG-PUSHC with K=1, considering the slight majority prefer the physical slot counting and also it is more aligned with what the current specification captures, FL’s recommendation is to take the physical slot counting. In addition, the physical slot counting is the legacy behavior, FL suggests reusing the legacy UE assumption and not having further discussions on whether or not the UE assumes that PUSCH symbols can overlap with DL symbol or SSB symbol in the slot determined in 38.321 Section 5.8.2.

Updated FL proposal on Issue#1-6:

- For CG-PUSCH with K=1, when AvailableSlotCounting is enabled, the legacy counting method (i.e., the physical slot counting) is applied.

  - Note: No RAN1 spec impact is expected.

  - Note: No additional restriction on CG periodicity configurations, compared to Rel-16, is considered for CG-PUSCH with K=1.

[Open] Issue#1-7: The slot determined in 38.321 Section 5.8.2 for the available slot counting (CG-PUSCH with K>1)

Under this section, we discuss the case of CG-PUSCH, when AvailableSlotCounting is enables, and for K=1.

According to the RAN1#108-e contributions, companies views are summarized as follow:

- For CG-PUSCH, when AvailableSlotCounting is enables, and for K>1,

  - Option 1: A UE does not assume that the slot determined in 38.321 Section 5.8.2 is the slot which is not counted in K available slot(s).

  - Option 2: A UE assumes that the slot determined in 38.321 Section 5.8.2 can be the slot which is not counted in K available slot(s).

      - Nokia/ Nokia Shanghai Bell [7], ZTE [9], OPPO [10], Panasonic [12], China Telecom [13], NTT DOCOMO [14], CMCC [19], Ericsson [20], Sharp [23], LG [25]

It seems all the companies that express their views on this issue in their contributions prefer taking Option 2. Therefore, FL made the following proposal.

FL proposal on Issue#1-7:

- For CG-PUSCH, when AvailableSlotCounting is enables, and for K>1, a UE assumes that the slot determined in 38.321 Section 5.8.2 can be the slot which is not counted in K available slot(s).

1st round (Issue#1-7)

Do you agree on the above FL proposal on Issue#1-7?

| Company | Comments |
| --- | --- |
| ZTE | Support |
| Panasonic | Agree |
| InterDigital | Agree |
| Intel | Just clarify: what is the difference between Issue 1-6 and 1-7? And what is the spec impact for this? |
| NTT DOCOMO | Agree. |
| LG | Agree |
| vivo | Fine. |
| QC | What is the benefit of such a configuration? Why can’t the gNB point to an available slot in Section 5.8.2? If we are relaxing this, there needs to be a strong justification. |
| CMCC | Support. |
| TCL | Agree. |
| OPPO | Agree. |
| FL | @Intel: Once we reach the consensus on the intended behavior, the next step would be the discussion on the spec impact. @QC: Let me raise another example. In Rel-15/16 38.214 Section 6.1.2.3.1 says that, for CG-PUSCH, the initial transmission of a transport block may start at any of the transmission occasions of the K repetitions if the configured RV sequence is {0,0,0,0}. Here the first transmission occasion corresponds to the slot determined in 38321 Section 5.8.2. In other words, even in Rel-15/16, the CG-PUSCH repetitions do not always start at the slot determined in 38321 Section 5.8.2, for example in the case when the collision occurs at the first slot. |
| CATT | OK. |
| Ericsson | Agree |
| Spreadtrum | Agree |
| Huawei, HiSilicon | Similar comment as to issue#1-6, the proposal may need some refinement. |
| Nokia/NSB | Agree. |

1st round summary (Issue#1-7)

Almost all the companies accepted the following proposal. Therefore, FL suggests taking the following proposal.

FL proposal on Issue#1-7:

- For CG-PUSCH, when AvailableSlotCounting is enabled, and for K>1, a UE assumes that the slot determined in 38.321 Section 5.8.2 can be the slot which is not counted in K available slot(s).

2nd round (Issue#1-7)

Provide your comment only if you have a strong concern on the above FL proposal on Issue#1-7.

| Company | Comments |
| --- | --- |
| QC | What is the benefit to such a relaxation? Why is the gNB not able to configure the UE appropriately? @FL to your comment directed at us: R15/R16 didn’t have the concept of deferral. A late start is not the issue here. I am trying to understand why the gNB is configuring the UE is such a way in the first place. Cant this be resolved at the gNB itself? |
| Vivo2 | Just one editorial comment: FL proposal on Issue#1-7: - For CG-PUSCH, when AvailableSlotCounting is enablesd, and for K>1, a UE assumes that the slot determined in 38.321 Section 5.8.2 can be the slot which is not counted in K available slot(s). |
| FL | @QC, Thanks for the response. I’m glad to know that we have the same understanding that in R15/16 the late start was not the problem for CG-PUSCH. I guess that also means R15/16 gNB is allowed to configure in such a way the first configured slot in a CG period overlaps with a DL slot, and R15/16 UEs assume such configurations may happen. In fact, I understand your concern on K2 timeline for DG-PUSCH, because R15/16 gNB normally set a suitable K2 value so that the scheduled PUSCH does not collide with DL. However, CG-PUSCH is a different story. R15/16 gNB does not have to always avoid collision between CG-PUSCH and DL by configurations. I’m wondering why we need to force R17 gNB to ensure no collision? We didn’t have such configuration restrictions in R15/16. @vivo, Thanks for spotting the typo! Now fixed. |
| Nokia/NSB | Agree |
| InterDigital | Agree in principle. One question is this should also apply to K=1, N>1 (TBoMS case without repetition)? Agree with FL that it would be constraining to guarantee availability for CG-PUSCH. |
| Ericsson | Agree. One editorial comment is that since K>1, the brackets of ‘slot(s)’ can be removed. |
| Spreadtrum | We are fine to support the proposal. |

2nd round summary (Issue#1-7)

The 2nd round inputs are summarized as follows:

- For the FL proposal on Issue#1-7, only 1 company (QC) expressed a strong concern.

FL proposal on Issue#1-7:

  - For CG-PUSCH, when AvailableSlotCounting is enabled, and for K>1, a UE assumes that the slot determined in 38.321 Section 5.8.2 can be the slot which is not counted in K available slots.

Given the large majority supports the FL proposal on Issue#1-7, FL would like to ask QC to be flexible to make a progress.

FL proposal on Issue#1-7:

- For CG-PUSCH, when AvailableSlotCounting is enabled, and for K>1, a UE assumes that the slot (i.e., the first configured slot in a CG period) determined in 38.321 Section 5.8.2 can be the slot which is not counted in K available slots.

[Pending] Issue#1-8: Use of SSBs across multiple TRPs for the available slot determination

In NR Rel-17 feMIMO WI, it was agreed to support different SSB sets with different PCIs transmitted from mTRPs in a same serving cell. In RAN1#107-e and RAN1#107bis-e, we discussed whether to consider more than one SSB sets for available slot counting for CovEnh. After several rounds of discussions during RAN1#107bis-e, the following conclusion was made, because most of the companies had the same understanding that the MIMO group did not have much discussions on the collision handling between UL channels/signals and multiple SSBs for inter-cell mTRPs and duplicated discussions in two different WIs should be avoided.

Conclusion - The CovEnh discussion on the available slot counting for inter-cell mTRPs is deferred until further progress on the collision handling between UL channels/signals and multiple SSBs for inter-cell mTRPs is made in feMIMO session.

For RAN1#108-e, vivo [8] is proposing having discussions to downselect from the following three options.

- Option 1: SSBs with PCI different from the serving cell PCI are only used for actual PUSCH repetition determination after the available slots are determined

- Option 2: Both SSBs with serving cell PCI and SSBs with PCI different from the serving cell PCI are used for the available slot determination

- Option 3: SSBs with the same PCI as the SSB used to determine the spatial relation of the PUSCH, are used to determine the available slots. Other set of SSBs are used for actual PUSCH repetition determination after the available slots are determined.

Also, Ericsson [20] and Samsung [21] are proposing taking Option 2 above.

However, considering the conclusion that we made in the last meeting and feMIMO discussion status, FL would like to not discuss this issue in this meeting, unless any progress is made in feMIMO WI.

[Close] Issue#1-9: Use of SSBs of other serving cells in half duplex CA operation for the available slot determination

For CA case, the collision handling is performed according to following text in the same section of 38.213 v17.0.0.

If a UE - is configured with multiple serving cells and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the multiple serving cells, and - indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and - is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells, for a set of symbols of a slot that are indicated to the UE for reception of SS/PBCH blocks in a first cell of the multiple serving cells by ssb-PositionsInBurst in SystemInformationBlockType1 or by ssb-PositionsInBurst in ServingCellConfigCommon, when provided to the UE, the UE does not transmit PUSCH, PUCCH, or PRACH in the slot if a transmission would overlap with any symbol from the set of symbols, and the UE does not transmit SRS in the set of symbols of the slot in - any of the multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and - any one of the cells corresponding to the same band as the first cell, irrespective of any capability indicated by simultaneousRxTxInterBandCA.

In RAN1#107bis-e, it was discussed whether SSBs in the other serving cells than the one where the PUSCH is transmitted is considered or not for available slot determination for the Type A PUSCH repetition transmissions. Since there was no company which propose changing the currently captured the available slot counting procedure, FL provided the following proposal for a possible conclusion.

Proposed conclusion: - For half duplex CA operation, no additional spec impact is necessary in terms of consideration of SSB(s) for the available slot determination.

The above conclusion was not taken in the last meeting, because one company did not think the conclusion is necessary. However, the large majority supported the proposed conclusion. Moreover, even the company who did not think making this conclusion is necessary also think that no additional spec impact is necessary.

In RAN1#108-e, vivo [8] is proposing making a conclusion that only the SSBs transmitted within the serving cell are considered for available slot determination for the Type A PUSCH transmissions if CA is configured.

However, based on the situation in the last meeting, without any conclusion, it seems to be commonly understood that the current specification already teaches that, for half duplex CA operation, only the SSBs transmitted within the serving cell are considered for available slot determination for the Type A PUSCH transmissions. Therefore, FL would like to suggest keeping this issue closed.

Correction proposals

For this meeting, the following proposals for corrections on Rel-17 RAN1 specifications have been raised.

- Issue#2-1: Corrections on frequency hopping

- Issue#2-2: Corrections on available slot counting for FDD

- Issue#2-3: Corrections on PUSCH repetition type A for multi-TRP operation

[Open] Issue#2-1: Corrections on frequency hopping

In RAN1#107bis-e, the issue for TS38.214v17.0.0 [5] was raised, which was that, for PUSCH repetition type A frequency hopping, the current descriptions limit the PUSCH is scheduled by DCI format 0_1 or 0_2, RAR UL grant or DCI format 0_0 with TC-RNTI. However, intra-slot frequency hopping is also applicable to a single slot PUSCH transmission, for example, the case when PUSCH is scheduled by DCI format 0_0 with C-RNTI. After several rounds of discissions, the following three options were provided by FL [1].

Option 1: Adopt TP#1

TP#1 - 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots For PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, DCI format 0_1 or 0_2 and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in onfiguredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.

Option 2: Adopt TP#2

TP#2 - 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots For PUSCH repetition Type A other than the PUSCH scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in onfiguredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.

Option 3: Adopt TP#3

TP#3 - 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots For PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH) and for PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in onfiguredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission. For operation with shared spectrum channel access, the UE does not expect that two hops of a PUSCH transmission are in different RB sets. In case of resource allocation type 2, the UE transmits PUSCH without frequency hopping. In case of resource allocation type 1, whether or not transform precoding is enabled for PUSCH transmission, the UE may perform PUSCH frequency hopping, if the frequency hopping field in a corresponding detected DCI format or in a random access response UL grant is set to 1, or if for a Type 1 PUSCH transmission with a configured grant the higher layer parameter frequencyHoppingOffset is provided, otherwise no PUSCH frequency hopping is performed. When frequency hopping is enabled for PUSCH, the RE mapping is defined in clause 6.3.1.6 of [4, TS 38.211]. For a PUSCH scheduled by RAR UL grant, fallbackRAR UL grant, or by DCI format 0_0 with CRC scrambled by TC-RNTI, frequency offsets are obtained as described in clause 8.3 of [6, TS 38.213]. Otherwise, for a PUSCH scheduled by DCI format 0_0/0_1 or a PUSCH based on a Type2 configured UL grant activated by DCI format 0_0/0_1 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetLists in pusch-Config. For a PUSCH scheduled by DCI format 0_2 or a PUSCH based on a Type2 configured UL grant activated by DCI format 0_2 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetListsDCI-0-2 in pusch-Config. - When the size of the active BWP is less than 50 PRBs, one of two higher layer configured offsets is indicated in the UL grant. - When the size of the active BWP is equal to or greater than 50 PRBs, one of four higher layer configured offsets is indicated in the UL grant.

According to the contributions for RAN1#108-e, companies’ views are summarized as follows:

- No TP is necessary, which means that the “Frequency hopping flag” field in DCI format 0_0 with C-RNTI is used to indicate whether 'Intra-slot frequency hopping' is enabled irrespective of the RRC parameter frequencyHopping in pusch-Config.

  - Huawei/HiSilicon [6]

- Discuss whether the RRC parameter frequencyHopping in pusch-Config is applied to the PUSCH scheduled by DCI format 0_0 with C-RNTI

  - CMCC [19]

- Adopt TP#1

  - ZTE [9], CATT [11]

- Adopt TP#2

  - Vivo [8], Sharp [23]

- Adopt TP#3

  - Nokia/ Nokia Shanghai Bell [7], Intel [17], Apple [18], Samsung [21]

1st round (Issue#2-1)

Q: Please provide your views on which is Rel-15/16 FH behavior for the PUSCH scheduled by DCI format 0_0 with C-RNTI.

- Behavior #1: When Rel-15/16 UEs determine whether to perform intra-slot frequency hopping, the UEs refer to the frequency hopping flag field value in the DCI format 0_0 and ignore the RRC parameter frequencyHopping provided in pusch-Config.

- Behavior #2: When Rel-15/16 UEs determine whether to perform intra-slot frequency hopping, the UEs refer to both the RRC parameter frequencyHopping provided in pusch-Config and the frequency hopping flag field value in the DCI format 0_0.

- Any other behaviors?

Note that, in any case, for the frequency hopping of the PUSCH scheduled by the DCI format 0_0 with C-RNTI, the Rel-15/16 UEs refer to both the RRC parameter frequencyHoppingOffsetLists in pusch-Config and frequency domain resource assignment field value in the DCI format 0_0 for the determination of frequency offsets.

| Company | Comments |
| --- | --- |
| ZTE | Behavior #2 In Rel-16, the RRC parameter frequencyHopping also applies to DCI format 0_0. Therefore, if it is configured by inter-slot FH, the UE would ignore the frequency hopping flag field in DCI. Otherwise, whether to perform intra-slot FH is based on the indication of frequency hopping flag field in DCI. |
| Panasonic | In RAN1#101e, there is the following conclusion. R1-2003594 was rejected for Rel-15. Conclusion Draft CR in R1-2003594 is rejected for Rel-15 with the following understanding - For a PUSCH scheduled by a DCI format 0_0 with CRC scrambled by C-RNTI, MCS-C-RNTI or CS-RNTI, the frequency hopping field in the DCI format 0_0 is not expected to be set to 1 if frequencyHopping in PUSCH-Config is not provided or set to interSlot. - For Type 2 CG PUSCH activated by DCI format 0_0 with CRC scrambled by CS-RNTI, the frequency hopping field in the DCI format 0_0 is not expected to be set to 1 if frequencyHopping in configuredGrantConfig is not provided or set to interSlot. It seems the Rel.15/16 FH behaviour for the PUSCH scheduled by DCI format 0-0 with C-RNTI is neither Behavior #1 nor Behavior #2. |
| Intel | Behavior #1. For PUSCH scheduled by DCI format 0_0 with C-RNTI, only intra-slot frequency hopping is supported. In this case, frequencyHopping is not applied. UE only needs to check frequency hopping flag field value in the DCI format 0_0 |
| vivo | Since there’s no repetition supported for PUSCH scheduled by DCI0-0 with C-RNTI which means only intra-slot FH can be applied in such case, it’s not necessary for UE to read FH type indication in pusch-Config. So, behavior #1 seems enough. |
| CMCC | Behavior #2. Our understanding is when UE initial access to the NW without RRC configurations, FH in DCI format 0_0 should also work. When the RRC connection is established, the RRC parameter frequencyHopping in pusch-Config should not conflict with DCI 0_0 scheduling. DCI 0_0 can only scheduling PUSCH without repetition, then the inter-slot frequency hopping should not be configured. TP#3 is our preference since it is more clear that PUSCH repetition cannot be scheduled by DCI 0_0. |
| FL | Thanks, Panasonic, for digging out the past CR discussion! Looking at the conclusion in RAN1#101-e, indeed picking one option from Behaviors #1 and #2 does not seem a very good approach. I will bring another discussion point in the next round. |
| CATT | According to the information from Panasonic, it seems Behavior#1 is incorrect. But Behavior#2 is also incomplete. At least it should be conditional, i.e. the UE does not expect frequency hopping field in the DCI format 0_0 indicates ‘on’ if frequencyHopping in PUSCH-Config is not provided. |
| Ericsson | Given the conclusion provided by Panasonic, we support TP#3 with an editorial change. The newly added words are better to be moved ahead of the brackets. For PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 and for TB processing over multiple slots and for PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH) and for PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, a UE … |
| Huawei, HiSilicon | DCI format 0_0 is a fallback DCI, which means its UE behavior is not supposed to depend on any UE dedicated RRC parameter. However, RRC parameter frequencyHopping is a UE dedicated parameter which is configured via BWP-UplinkDedicated->PUSCH-Config-> frequencyHopping. Additionally, during initial access, a UE may be scheduled by DCI 0_0 but no UE dedicated RRC including frequencyHopping has been configured to the UE. In this case, it is quite clear that the UE has to do intra-slot hopping if the DCI 0_0 indicates so. Behavior#2 is not in line with this UE behavior. Thirdly, in TS 38.212, there is always the “Frequency hopping flag” field in DCI format 0_0. Frequency hopping flag – 1 bit according to Table 7.3.1.1.1-3, as defined in Clause 6.3 of [6, TS 38.214]. Table 7.3.1.1.1-3: Frequency hopping indication Bit field mapped to index; PUSCH frequency hopping. 0; Disabled. 1; Enabled.. Therefore, Behavior#2 is incorrect at least for initial access. |

1st round summary (Issue#2-1)

Panasonic pointed out that there was the similar discussion in RAN1#101-e and the conclusion form the past discussion was as follows.

R1-2003594 was rejected for Rel-15. Conclusion Draft CR in R1-2003594 is rejected for Rel-15 with the following understanding - For a PUSCH scheduled by a DCI format 0_0 with CRC scrambled by C-RNTI, MCS-C-RNTI or CS-RNTI, the frequency hopping field in the DCI format 0_0 is not expected to be set to 1 if frequencyHopping in PUSCH-Config is not provided or set to interSlot. - For Type 2 CG PUSCH activated by DCI format 0_0 with CRC scrambled by CS-RNTI, the frequency hopping field in the DCI format 0_0 is not expected to be set to 1 if frequencyHopping in configuredGrantConfig is not provided or set to interSlot.

Looking at the conclusion, for a PUSCH scheduled by a DCI format 0_0 with CRC scrambled by C-RNTI, the conclusion does not require the UE to refer to frequencyHopping in PUSCH-Config and at the same time it does not preclude to refer to frequencyHopping in PUSCH-Config. Neither putting only DCI 0_1 and 0_2 nor putting all the DCI 0_0, 0_1 and 0_2 in the concerned paragraph of 38.214 Section 6.3.1 may not resolve the issue, unfortunately.

A possible way forward is to go back to Rel-15/16 expressions (i.e., removing “DCI format 0_1 or 0_2”) and just adding “for TB processing over multiple slots” as the following TP.

- TP#4 - 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots For PUSCH repetition Type A and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot PUSCH transmission and and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission. For operation with shared spectrum channel access, the UE does not expect that two hops of a PUSCH transmission are in different RB sets. In case of resource allocation type 2, the UE transmits PUSCH without frequency hopping. In case of resource allocation type 1, whether or not transform precoding is enabled for PUSCH transmission, the UE may perform PUSCH frequency hopping, if the frequency hopping field in a corresponding detected DCI format or in a random access response UL grant is set to 1, or if for a Type 1 PUSCH transmission with a configured grant the higher layer parameter frequencyHoppingOffset is provided, otherwise no PUSCH frequency hopping is performed. When frequency hopping is enabled for PUSCH, the RE mapping is defined in clause 6.3.1.6 of [4, TS 38.211]. For a PUSCH scheduled by RAR UL grant, fallbackRAR UL grant, or by DCI format 0_0 with CRC scrambled by TC-RNTI, frequency offsets are obtained as described in clause 8.3 of [6, TS 38.213]. Otherwise, for a PUSCH scheduled by DCI format 0_0/0_1 or a PUSCH based on a Type2 configured UL grant activated by DCI format 0_0/0_1 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetLists in pusch-Config. For a PUSCH scheduled by DCI format 0_2 or a PUSCH based on a Type2 configured UL grant activated by DCI format 0_2 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetListsDCI-0-2 in pusch-Config. - When the size of the active BWP is less than 50 PRBs, one of two higher layer configured offsets is indicated in the UL grant. - When the size of the active BWP is equal to or greater than 50 PRBs, one of four higher layer configured offsets is indicated in the UL grant.

For your reference, Rel-16 descriptions are copied below.

- 6.3.1 Frequency hopping for PUSCH repetition Type A For PUSCH repetition Type A (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot PUSCH transmission and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission. For operation with shared spectrum channel access, the UE does not expect that two hops of a PUSCH transmission are in different RB sets. In case of resource allocation type 2, the UE transmits PUSCH without frequency hopping. In case of resource allocation type 1, whether or not transform precoding is enabled for PUSCH transmission, the UE may perform PUSCH frequency hopping, if the frequency hopping field in a corresponding detected DCI format or in a random access response UL grant is set to 1, or if for a Type 1 PUSCH transmission with a configured grant the higher layer parameter frequencyHoppingOffset is provided, otherwise no PUSCH frequency hopping is performed. When frequency hopping is enabled for PUSCH, the RE mapping is defined in clause 6.3.1.6 of [4, TS 38.211]. For a PUSCH scheduled by RAR UL grant, fallbackRAR UL grant, or by DCI format 0_0 with CRC scrambled by TC-RNTI, frequency offsets are obtained as described in clause 8.3 of [6, TS 38.213]. Otherwise, for a PUSCH scheduled by DCI format 0_0/0_1 or a PUSCH based on a Type2 configured UL grant activated by DCI format 0_0/0_1 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetLists in pusch-Config. For a PUSCH scheduled by DCI format 0_2 or a PUSCH based on a Type2 configured UL grant activated by DCI format 0_2 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetListsDCI-0-2 in pusch-Config. - When the size of the active BWP is less than 50 PRBs, one of two higher layer configured offsets is indicated in the UL grant. - When the size of the active BWP is equal to or greater than 50 PRBs, one of four higher layer configured offsets is indicated in the UL grant.

2nd round (Issue#2-1)

Please provide your views on the above TP#4.

| Company | Comments |
| --- | --- |
| Panasonic | We are fine with the above TP#4 with the following minor editorial correction. Intra-slot frequency hopping, applicable to single slot and multi-slot PUSCH transmission and and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. |
| Xiaomi | Fine with above TP#4 and Panasonic correction |
| vivo2 | TP #4 does not solve issue, as the original intention of the changes compared to Rel-16 spec. is to capture that Msg3 repetition is only allowed to perform inter-slot FH. And in this case, UE does not need to read the dedicated signaling for FH of Msg3 when Msg3 is repeated. So, TP#2 (simply exclude Type A Msg3 repetition case) is preferred from our side. |
| FL | @vivo, Thanks for the feedback! My understanding is that UE does not need to read the dedicated signaling for FH of Msg3, irrespective of whether Msg3 is repeated or not. In this sense, Rel-17 UE behaviors related to reading of the dedicated signal (i.e., not read the the dedicated signal) for Msg3 is exactly the same as Rel-16 Msg3. That’s why I suggest going back to R16 descriptions. Do I miss something? |
| CMCC | General fine with the TP#4. Removing the specific DCI format could solve some of the issues, such as single slot or multiple slot PUSCH is scheduled by certain DCI format or not. |
| CATT | Fine with it. In our understanding, in the sub-bullet of TP#4, TBoMS is included in ‘multi-slot PUSCH transmission’. |
| Nokia/NSB | We do not support TP#4 for the same reason explained by. If we remove the adjective “configured” before PUSCH in the first bullet, we include Msg3 repetitions which is wrong. Maybe we could consider a variant of TP#3 as follows: For PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in onfiguredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, and for PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.. |
| Intel | We are fine with TP#4 with the update from Panasonic. |
| FL | @all, The update provided by Panasonic is incorporated in the TP. @vivo, The conclusion in RAN1#101-e did not cover Msg3 PUSCH. For the initial access, a UE transmits Msg3 PUSCH before acquiring the dedicated RRC signaling. There is no way to refer to it. For the Msg3 transmission after RRC establishment, the UE can refer to it, but the gNB cannot be aware of whether the UE which the RAR UL grant is intended for has the dedicated RRC signaling already or not. Therefore, even after RRC establishment, the UE does not refer to the dedicated RRC signaling for Msg3 PUSCH transmission. This is clarified by TS38.213 Sections 8.2 and 8.3. See Issue#17 of “[108-e-Prep-AI7.1] Rel-15 NR maintenance preparation phase” email discussion. @Nokia, Thanks for pointing out the issue due to removal of “configured”. Then, we can keep the “Intra-slot frequency hopping,…” bullet as is. - Updated TP#4 - 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots For PUSCH repetition Type A and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission. For operation with shared spectrum channel access, the UE does not expect that two hops of a PUSCH transmission are in different RB sets. In case of resource allocation type 2, the UE transmits PUSCH without frequency hopping. In case of resource allocation type 1, whether or not transform precoding is enabled for PUSCH transmission, the UE may perform PUSCH frequency hopping, if the frequency hopping field in a corresponding detected DCI format or in a random access response UL grant is set to 1, or if for a Type 1 PUSCH transmission with a configured grant the higher layer parameter frequencyHoppingOffset is provided, otherwise no PUSCH frequency hopping is performed. When frequency hopping is enabled for PUSCH, the RE mapping is defined in clause 6.3.1.6 of [4, TS 38.211]. For a PUSCH scheduled by RAR UL grant, fallbackRAR UL grant, or by DCI format 0_0 with CRC scrambled by TC-RNTI, frequency offsets are obtained as described in clause 8.3 of [6, TS 38.213]. Otherwise, for a PUSCH scheduled by DCI format 0_0/0_1 or a PUSCH based on a Type2 configured UL grant activated by DCI format 0_0/0_1 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetLists in pusch-Config. For a PUSCH scheduled by DCI format 0_2 or a PUSCH based on a Type2 configured UL grant activated by DCI format 0_2 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetListsDCI-0-2 in pusch-Config. - When the size of the active BWP is less than 50 PRBs, one of two higher layer configured offsets is indicated in the UL grant. - When the size of the active BWP is equal to or greater than 50 PRBs, one of four higher layer configured offsets is indicated in the UL grant.. |
| Spreadtrum | We are fine to support the updated TP#4. |
| vivo3 | Thanks for FL’s clarification. It seems the actual concern is that if we preclude Msg3, we should preclude not only Msg3 when repeated (Rel-17), but also for the Msg3 not repeated (Rel-16), while people are hesitant to change Rel-16 spec. Actually, we proposed to preclude both repetition and non-repetition case for Msg3 in TP2 only for Rel-17. I try to put a table here to make sure that our understandings are aligned regarding whether reading dedicated FH type configuration signaling is needed for all possible transmissions. Is this correct understanding? If yes, it seems with TP2, there will be no other exceptions not precluded. Transmissions; Included by “PUSCH repetition Type A” in the first sentence of 6.3.1 of 38.213? i.e. reading the dedicated signaling is required?. Msg3 PUSCH transmission/retransmission (repetition or not repetition); No.. Normal PUSCH (not Msg3) scheduled by fallback DCI or CG; Yes. (note that dedicated signaling may be not provide before RRC). TBoMS; Yes.. Normal PUSCH scheduled by non-fallback DCI; Yes.. Hiding these facts in conclusions or 3GPP email discussions may trigger repeated discussions in future, be more misleading and worse for those working on products and not working in 3GPP. To save time and move forward, we will let feature leader to decide whether to capture these in a TP for the first sentence in Rel-17. We do not support following changes in TP4: - Intra-slot frequency hopping, applicable to single slot and multi-slot PUSCH transmission and and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured.. Is the intention to support intra-slot FH for Msg3 repetitions, which reverts RAN1 agreement? |
| OPPO | Fine to support the updated TP#4. |

2nd round summary (Issue#2-1)

The 2nd round inputs are summarized as follows:

- Fine with the updated TP#4

  - Panasonic, Xiaomi, CMCC, CATT, Intel

- Prefer TP#2

  - vivo

- Prefer a variant of TP#3

  - Nokia

Companies still have different views on which TP we should take. On the other hand, there was the very good discussions via RAN1 reflector about the 1st sentence of Clause 6.3.1 in TS 38.214, including what should be precluded from the sentence and what should not be touched in the sentence.

Some companies pointed out that the following changes should not be adopted. Otherwise, this bullet includes Msg3 repetitions, which is wrong. FL agrees on this argument.

- Intra-slot frequency hopping, applicable to single slot and multi-slot PUSCH transmission and and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured.

As for the conclusion made in maintenance session in RAN1#101-e, FL’s understanding is that, if the frequency hopping field in the DCI format 0_0 is set to 1, the UE can consider that frequencyHopping in PUSCH-Config is provided and set to intraSlot. This means that, for DCI format 0_0 with C-RNTI, the UE is allowed to determine that intra-slot FH is enabled, by checking the frequency hopping field value only (without reading frequencyHopping in PUSCH-Config). At the same time, the conclusion does not preclude UE’s additional reading of frequencyHopping in PUSCH-Config. In other words, for DCI format 0_0 with C-RNTI, whether to read frequencyHopping in PUSCH-Config is left to UE implementation. The descriptions in Section 6.3 of Rel-15 TS38.214 does not conflict with this conclusion.

R1-2003594 was rejected for Rel-15. Conclusion Draft CR in R1-2003594 is rejected for Rel-15 with the following understanding - For a PUSCH scheduled by a DCI format 0_0 with CRC scrambled by C-RNTI, MCS-C-RNTI or CS-RNTI, the frequency hopping field in the DCI format 0_0 is not expected to be set to 1 if frequencyHopping in PUSCH-Config is not provided or set to interSlot. - For Type 2 CG PUSCH activated by DCI format 0_0 with CRC scrambled by CS-RNTI, the frequency hopping field in the DCI format 0_0 is not expected to be set to 1 if frequencyHopping in configuredGrantConfig is not provided or set to interSlot.

- Rel-15 TS38.214 - 6.3 UE PUSCH frequency hopping procedure A UE is configured for frequency hopping of scheduled or configured PUSCH transmission by the higher layer parameter frequencyHopping provided respectively in pusch-Config or in configuredGrantConfig. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot PUSCH transmission. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission. In case of resource allocation type 1, whether or not transform precoding is enabled for PUSCH transmission, the UE may perform PUSCH frequency hopping, if the frequency hopping field in a corresponding detected DCI format or in a random access response UL grant is set to 1, or if for a Type 1 PUSCH transmission with a configured grant the higher layer parameter frequencyHoppingOffset is provided, otherwise no PUSCH frequency hopping is performed. When frequency hopping is enabled for PUSCH, the RE mapping is defined in clause 6.3.1.6 of [4, TS 38.211].

Furthermore, The Spec editor already added the red part in Rel-17 spec, which was not in Rel-16 spec. FL believes the editor’s intention was to clarify that frequency hopping for Msg3 (with / without repetitions) is configured by “the frequency hopping flag information field” but not by “the higher layer parameter frequencyHopping”. FL thinks this would address vivo’s concern that, for Msg3, reading of frequencyHopping provided in pusch-Config should be precluded.

For PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in onfiguredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.

Lastly, in Rel-16 maintenance discussion [108-e-R16-URLLC-06], it has been discussed what the general “PUSCH repetition Type A” concept in Rel-16 covers, and it is observed that the common understanding is that PUSCH repetition Type A covers all cases that are not PUSCH repetition Type B, including a PUSCH scheduled by RAR UL grant, a PUSCH scheduled by fallbackRAR UL grant, a PUSCH scheduled by DCI format 0_0 with TC-RNTI, a PUSCH scheduled by DCI format 0_0 with C-RNTI.

Companies are again encouraged to check if the updated TP#4 is acceptable, taking all the aforementioned background information into account.

FL proposal on Issue#2-1:

Adopt the updated TP#4 to TS38.214.

- Updated TP#4 - 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots For PUSCH repetition Type A and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission. For operation with shared spectrum channel access, the UE does not expect that two hops of a PUSCH transmission are in different RB sets. In case of resource allocation type 2, the UE transmits PUSCH without frequency hopping. In case of resource allocation type 1, whether or not transform precoding is enabled for PUSCH transmission, the UE may perform PUSCH frequency hopping, if the frequency hopping field in a corresponding detected DCI format or in a random access response UL grant is set to 1, or if for a Type 1 PUSCH transmission with a configured grant the higher layer parameter frequencyHoppingOffset is provided, otherwise no PUSCH frequency hopping is performed. When frequency hopping is enabled for PUSCH, the RE mapping is defined in clause 6.3.1.6 of [4, TS 38.211]. For a PUSCH scheduled by RAR UL grant, fallbackRAR UL grant, or by DCI format 0_0 with CRC scrambled by TC-RNTI, frequency offsets are obtained as described in clause 8.3 of [6, TS 38.213]. Otherwise, for a PUSCH scheduled by DCI format 0_0/0_1 or a PUSCH based on a Type2 configured UL grant activated by DCI format 0_0/0_1 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetLists in pusch-Config. For a PUSCH scheduled by DCI format 0_2 or a PUSCH based on a Type2 configured UL grant activated by DCI format 0_2 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetListsDCI-0-2 in pusch-Config. - When the size of the active BWP is less than 50 PRBs, one of two higher layer configured offsets is indicated in the UL grant. - When the size of the active BWP is equal to or greater than 50 PRBs, one of four higher layer configured offsets is indicated in the UL grant.

[Open] Issue#2-2: Corrections on available slot counting for FDD

In RAN1#107bis-e, the issue for TS38.214v17.0.0 [5] was raised, which was an inconsistency between the main bullet and its sub-bullet, because main bullet says “N∙K consecutive slots” but the sub-bullet implies “N∙K slots” may be non-consecutive. After several rounds of discussions, FL provided two possible TPs as follows [1].

- Option 1: Adopt the updated TP#A

- TP#A - 6.1.2.1 Resource allocation in time domain [Omitted] For paired spectrum and SUL band: - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2, irrespective of whether AvailableSlotCounting is enabled or not. - Except for the case of reduced capability half-duplex UEwith AvailableSlotCounting enabled, in which case a slot is not counted in the number of N∙K slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, based on the TDRA information field value in the RAR UL grant. - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, based on the TDRA information field value in the DCI scheduling the PUSCH. [Omitted] - 6.1.2.3.1 Transport Block repetition for uplink transmissions of PUSCH repetition Type A with a configured grant [Omitted] For both Type 1 and Type 2 PUSCH transmissions with a configured grant, when K > 1, - For unpaired spectrum: - If AvailableSlotCounting is enabled, the UE shall repeat the TB across the N∙K slots determined for the PUSCH transmission applying the same symbol allocation in each slot. - A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - Otherwise, the UE shall repeat the TB across the N∙K consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. - For paired spectrum: - Irrespective of whether AvailableSlotCounting is enabled or not, the UE shall repeat the TB across the N∙K consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. - Except for the case of reduced capability half-duplex UE with AvailableSlotCounting enabled, in which case a slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

- Option 2: Adopt the updated TP#B’

- TP#B’ - 6.1.2.1 Resource allocation in time domain [Omitted] For paired spectrum and SUL band: - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. - A reduced capability half-duplex UE with AvailableSlotCounting enabled determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. N∙K - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, based on the TDRA information field value in the RAR UL grant. - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, based on the TDRA information field value in the DCI scheduling the PUSCH. [Omitted] - 6.1.2.3.1 Transport Block repetition for uplink transmissions of PUSCH repetition Type A with a configured grant [Omitted] For both Type 1 and Type 2 PUSCH transmissions with a configured grant, when K > 1, - For unpaired spectrum: - If AvailableSlotCounting is enabled, the UE shall repeat the TB across the N∙K slots determined for the PUSCH transmission applying the same symbol allocation in each slot. - A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - Otherwise, the UE shall repeat the TB across the N∙K consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. - For paired spectrum: - The UE shall repeat the TB across the N∙K consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. - If AvailableSlotCounting is enabled, and in case of reduced capability half-duplex UE, the UE shall repeat the TB across the N∙K slots applying the same symbol allocation in each slot. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. N∙K

According to the contributions for RAN1#108-e, companies’ preferences are summarized as follow:

- Adopt TP#B’

  - OPPO [10], Intel (with a slight update on the wording) [17], Samsung [21], Qualcomm [22], Sharp [23]

1st round (Issue#2-2)

Do you agree to adopt TP#B’?

| Company | Comments |
| --- | --- |
| ZTE | Yes |
| Intel | Agree. Suggest to update the first part starting with the following, which is aligned with the CG-PUSCH. For the case of a reduced capability half-duplex UE, and when AvailableSlotCounting is enabled, the UE determines … |
| LG | Agree |
| Vivo | Fine. |
| QC | Yes |
| OPPO | Agree. |
| CATT | Fine. |
| Ericsson | Yes. |
| Nokia/NSB | Ok |

1st round summary (Issue#2-2)

All the companies accepted the TP#B’. Therefore, FL suggests taking the following proposal. Note that it reflects small wording modification suggested by Intel.

FL proposal on Issue#2-2:

Adopt the following TP#B’’ to TS38.214.

| - TP#B’’ - 6.1.2.1 Resource allocation in time domain [Omitted] For paired spectrum and SUL band: - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. - For the case of a reduced capability half-duplex UE, and when AvailableSlotCounting is enabled, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. N∙K - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, based on the TDRA information field value in the RAR UL grant. - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, based on the TDRA information field value in the DCI scheduling the PUSCH. [Omitted] - 6.1.2.3.1 Transport Block repetition for uplink transmissions of PUSCH repetition Type A with a configured grant [Omitted] For both Type 1 and Type 2 PUSCH transmissions with a configured grant, when K > 1, - For unpaired spectrum: - If AvailableSlotCounting is enabled, the UE shall repeat the TB across the N∙K slots determined for the PUSCH transmission applying the same symbol allocation in each slot. - A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - Otherwise, the UE shall repeat the TB across the N∙K consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. - For paired spectrum: - The UE shall repeat the TB across the N∙K consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. - If AvailableSlotCounting is enabled, and in case of reduced capability half-duplex UE, the UE shall repeat the TB across the N∙K slots applying the same symbol allocation in each slot. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. N∙K |  |
| --- | --- |

2nd round (Issue#2-2)

Provide your comment only if you have a strong concern on the above FL proposal on Issue#2-2.

| Company | Comments |
| --- | --- |

2nd round summary (Issue#2-2)

The above FL proposal on Issue#2-2 has been stable for more than 40 hours. Therefore, FL would like to suggest it for an email approval.

[Pending] Issue#2-3: Corrections on PUSCH repetition type A for multi-TRP operation

For TS38.214v17.0.0 [5], Intel [17] is pointing out that the current description for PUSCH repetition type A for multi-TRP operation is not accurately captured in Section 6.1.2.1, because, for Rel-17 PUSCH repetition type A enhancement with counting based on available slots, same symbol allocation can be applied across non-consecutive slots. To address this issue, Intel [17] is proposing the following TP.

TP in R1-2201708 [17]

------------------------------ TP for TS 38.214----------------------------------- - 6.1.2.1 Resource allocation in time domain < Unchanged text omitted > When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or 'noncodebook', for PUSCH repetition Type A, when AvailableSlotCounting is disabled, in case K>1, the same symbol allocation is applied across the K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the K consecutive slots applying the same symbol allocation in each slot, and the association of the first and second SRS resource set in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 to each slot is determined as follows: < Unchanged text omitted >

1st round (Issue#2-3)

Do you agree to adopt the above TP in R1-2201708?

| Company | Comments |
| --- | --- |
| ZTE | Fine with the TP. |
| Intel | Agree. |
| LG | Fine with the TP. |
| vivo | We can understand the concern raised in the TP, but it seems also fine to consider the consecutive “slot” as consecutive “available slot” when available slot counting is enabled. Another question is: Is the intention of the TP to preclude the support of mTRP when PUSCH repetition is based on available slot? |
| QC | Similar question as Vivo. What about the case when available slot counting is enabled? |
| CATT | Understand the motivation, but we have similar confusion as vivo and QC. The case of ‘available slot counting’ + ‘mTRP’ is missing. |
| Ericsson | Same question with vivo. It would be better to refer to determination of available slot in the same section. |
| Nokia/NSB | We are not sure about the need for this TP. This aspect was never discussed in AI 8.8.1.1 and we should not introduce new functionalities at such at late stage. |

1st round summary (Issue#2-3)

The 1st round inputs are summarized as follows:

- Fine with the TP in R1-2201708

  - ZTE, Intel, LG

- Need clarification for the case with mTRPs when available slot counting is enabled

  - Vivo, QC, CATT, Ericsson

- No need to adopt the TP

  - Nokia/NSB

We have made the following conclusion in the last meeting. As stated in the conclusion, after feMIMO discussion, the CovEnh discussion on the available slot counting for inter-cell mTRPs would take place, if necessary. Until then, we cannot specify the behaviors for the case with mTRPs and with available slot counting.

Conclusion - The CovEnh discussion on the available slot counting for inter-cell mTRPs is deferred until further progress on the collision handling between UL channels/signals and multiple SSBs for inter-cell mTRPs is made in feMIMO session.

Intel’s intention is understandable, since feMIMO people have been assuming the inter-cell mTRPs procedure is applicable to the physical slot counting for sure. Having said that, it is also true that at this moment it is not clear if the condition “when AvailableSlotCounting is disabled” is necessary, because no one knows the behaviors for the case with mTRPs and with available slot counting. Therefore, FL suggests discussing this issue as part of “the CovEnh discussion on the available slot counting for inter-cell mTRPs” in the previous conclusion.

References

- R1-2200789	FL Summary #4 on Enhancements on PUSCH repetition type A	Moderator (Sharp)

- 3GPP TS38.211V17.0.0, December 2021

- 3GPP TS38.212V17.0.0, December 2021

- 3GPP TS38.213V17.0.0, December 2021

- 3GPP TS38.214V17.0.0, December 2021

- R1-2200966	Discussion on coverage enhancements for PUSCH repetition type A	Huawei, HiSilicon

- R1-2201012	Enhancements on PUSCH repetition type A	Nokia, Nokia Shanghai Bell

- R1-2201104	Remaining issues on enhancement for PUSCH repetition type A	vivo

- R1-2201164	Discussion on remaining issues for enhanced PUSCH repetition type A	ZTE

- R1-2201283	Enhancements on PUSCH repetition type A	OPPO

- R1-2201373	Remaining issues on enhancements on PUSCH repetition type A	CATT

- R1-2201380	Discussion on enhancements on PUSCH repetition Type A	Panasonic Corporation

- R1-2201442	Remaining issues on PUSCH repetition type A enhancements	China Telecom

- R1-2201487	Remaining issues on enhancements on PUSCH repetition type A	NTT DOCOMO, INC.

- R1-2201554	Discussion on enhancements for PUSCH repetition Type A	Spreadtrum Communications

- R1-2201657	Type-A PUSCH repetition for coverage enhancement	InterDigital, Inc.

- R1-2201708	Remaining details of enhancements on PUSCH repetition type A	Intel Corporation

- R1-2201780	Remaining issues on PUSCH repetition type A enhancement	Apple

- R1-2201868	Remaining issues on PUSCH repetition type A enhancement	CMCC

- R1-2201961	Remaining Issues for PUSCH Repetition Type A Enhancement	Ericsson

- R1-2202026	Enhancements on PUSCH repetition type A	Samsung

- R1-2202151	Enhancements on PUSCH Repetition Type A	Qualcomm Incorporated

- R1-2202196	Enhancements on PUSCH repetition type A	Sharp

- R1-2202239	Discussion on PUSCH repetition type A enhancement	TCL Communication Ltd.

- R1-2202299	Discussions on PUSCH repetition type A enhancements	LG Electronics

List of agreements

Agreements in RAN1#104-e

Agreements:

Select one of the following alternatives, considering the aspect whether or not the determination of all the available slots should be done prior to the first actual transmission of the repetitions (other alternatives are not precluded)

-        Alt1: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and does not depend on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

-        Alt2: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and also depends on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

Agreements:

The maximum number of repetitions for DG-PUSCH is also applicable to CG-PUSCH.

Agreements:

For defining available slots: a slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the symbol not intended for UL transmissions

- FFS details

Agreements:

Rel-17 PUSCH repetition Type A supports the increase of maximum number of repetitions with repetition factors configured in a TDRA list with a row index indicated either by the configured grant configuration or by TDRA field in a DCI.

- FFS: increasing the maximum number of repetitions with repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig.

Conclusion:

Discuss further to select one of the following alternatives:

- Alt-a: The determination of all the available slots has to be done prior to the first actual transmission of the repetitions.

- Alt-b: The determination of all the available slots does not have to be done prior to the first actual transmission of the repetitions. The timeline requirement is per repetition basis.

Agreements in RAN1#105-e

Agreement:

- RV cycling is based on available slot for the Type A PUSCH repetition enhancement with repetitions counted based on available slot in Rel-17

Agreement:

- Down-selection in RAN1#106-e:

- Alt 1: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is 32, irrespective of counting method,

- Alt 2: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is: 32 for the counting based on physical slots; and 16 (i.e. no change from Rel-16) for the counting based on available slots.

Conclusion:

- The following agreement in RAN1#104-e is applied to all slots including special slots.

Agreements: For defining available slots: a slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the symbol not intended for UL transmissions. - FFS details

Agreement:

In addition to {1, 2, 3, 4, 7, 8, 12, 16} and {32}, the following additional value set for repetition factor is supported in Rel-17.

- {20, 24, 28}

Agreement:

- Each available slot identified by the UE is considered as a transmission occasion for PUSCH repetition.

  - RV is cycled across transmission occasions, irrespective of whether PUSCH transmission in the transmission occasion is further omitted or not.

Agreement:

- If PUSCH symbol in a slot overlaps with flexible symbol(s) with SSB transmission, the slot is determined as not available during the counting of repetitions. As there is no PUSCH in the slot, no PUSCH omission applies to the slot.

Agreement:

Select one from the following (further refinement of the alternatives can be further discussed), for the procedure of Rel-17 PUSCH repetition Type A (other alternatives are not precluded)

- Alt 1-B consisting of two steps

  - Step 1: Determine available slots for K repetitions based on RRC configuration(s) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI

  - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

- Alt 1-B’ consisting of two steps

  - Step 1: Determine K repetitions based on available slots, where the available slot is the UL slot and flexible slot indicated by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated.

  - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

  - FFS: handling of dynamic signaling (e.g. UL CI, DCI for high priority channel), e.g., UE without CI capability

- Alt 2-A consisting of a single step

  - Step 1: Determine available slots for K repetitions based on RRC configuration(s) and dynamic signaling (e.g. SFI, UL CI, DCI for high priority channel) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI

- Alt 2-B consisting of two steps

  - Step 1: Determine available slots for K repetitions based on RRC configuration(s) and dynamic SFI in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI

    - FFS timeline for the dynamic signalling

  - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

Agreements in RAN1#106-e

Agreement:

- For Rel-17 PUSCH repetition Type A without joint channel estimation, no new inter-slot frequency hopping mechanism is introduced.

Agreement

Take Option 1-B as an agreement for the procedure of Rel-17 PUSCH repetitions counted on the basis of available slots.

- Alt 1-B consisting of two steps

- Step 1: Determine available slots for K repetitions based on RRC configuration(s) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI

- Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

- FFS: Rel-17 PUSCH dropping rules are also applied if introduced in other WI(s)

Agreement

For PUSCH repetition Type A for Rel-17 CG-PUSCH, semi-static flexible symbol is considered as available.

Agreement

For PUSCH repetition Type A for Rel-17 DG-PUSCH, semi-static flexible symbol is considered as available.

Note: The applicability for Msg 3 is to be discussed in 8.8.3

Agreement

- DCI format 0_1 and DCI format 0_2 support Rel-17 PUSCH repetition Type A with the increased maximum repetition numbers configured in TDRA lists.

Agreement

- For DG-PUSCH with counting based on the available slots, count of available slots continues until satisfying the conditions defined for DG-PUSCH repetition Type A in Rel-16.

Working Assumption

The maximum number of repetitions accounted for available slots supported by Rel-17 PUSCH repetition Type A is 32

Agreements in RAN1#106bis-e

Working Assumption is confirmed

Working Assumption

The maximum number of repetitions accounted for available slots supported by Rel-17 PUSCH repetition Type A is 32

Conclusion:

For CG-PUSCH repetitions counted on the basis of available slots, all the K transmission occasions including the 1st transmission occasion are determined on the basis of available slots.

Agreement

For CG-PUSCH repetition Type A with the counting based on available slots, the R16 existing restrictions as defined in Clause 6.1.2.3.1 of TS38.214 at least on the initial transmission of a transport block are applied, assuming the K repetitions of R17 determined based the rule of counting available slots.

Observation

- Whether or not the counting based on available slots is applicable only to unpaired spectrum is not discussed under AI 8.8.1.1 in RAN1#106bis-e. Discussions on how HD-FDD RedCap UEs support the available slot counting may take place in AI 8.8.1.1 in RAN1#107-e, depending on the progress of RedCap WI discussions.

Agreement

- For the K repetitions of DG-PUSCH, Step 1 of the previously agreed two-step procedure (i.e., Alt 1-B) determines the K earliest available slots no earlier than the slot which is determined by the slot offset K2.

  - No RAN1 spec impact is expected in terms of the relation with the slot which is determined by the slot offset K2.

  - Note: The available slot determination is to be specified.

- For the K repetitions of CG-PUSCH, Step 1 of the previously agreed two-step procedure (i.e., Alt 1-B) determines the K earliest available slots no earlier than the first slot which is determined by at least ConfiguredGrantConfig.

  - No RAN1 spec impact is expected in terms of the relation with the first slot which is determined by at least ConfiguredGrantConfig.

  - Note: The available slot determination is to be specified.

Agreement

- Only tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst are considered for the determination of available slots.

  - Any other RRC configuration is not considered for the determination of available slots.

Agreement

- The existing restriction “The UE is not expected to be configured with the time duration for the transmission of K repetitions larger than the time duration derived by the periodicity P” applies to both the counting based on physical slots and the counting based on available slots.

- The above “the time duration for the transmission of K repetitions” means the time duration between the start of the 1st slot of the K repetitions and the end of the last slot of the K repetitions for any instance of a CG period.

Agreements in RAN1#107-e

Agreement

- The counting based on available slots is applicable to unpaired spectrum, paired spectrum and SUL

  - For paired spectrum and SUL except HD-FDD, all slots are considered as available slots in the first step of determining the available slots.

Agreement

- For HD-FDD RedCap Ues supporting the counting based on available slots.

  - For CG-PUSCH, ssb-PositionsInBurst is used in the first step of determining of available slots.

    - A slot is not counted in the number of available slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionInBurst.

  - FFS: For DG-PUSCH

  - Note: Neither tdd-UL-DL-ConfigurationCommon nor tdd-UL-DL-ConfigurationDedicated is configured for FDD.

Agreement

- Rel-17 does not support numberOfRepetitions-r17 for DG-PUSCH scheduled by DCI format 0_0 and for Type 2 CG-PUSCH activated by DCI format 0_0.

- repK-r17 supporting up-to-32 repetitions is introduced and is applicable to Type 1 CG-PUSCH and Type 2 CG-PUSCH (irrespective of the activating DCI format).

  - Note: No RAN1 spec impact is expected.

  - The possible values of repK-r17 includes 16 and 32. FFS: other values.

- numberOfRepetitions-r17 is not applicable to Type 1 CG-PUSCH repetition type A.

Agreement

- All the following combinations support the counting based on available slots.

  - DG-PUSCH with Rel-15 repetition factor

  - Type-1 CG-PUSCH with Rel-15 repetition factor

  - Type-2 CG-PUSCH with Rel-15 repetition factor

  - DG-PUSCH with Rel-16 repetition factor

  - Type-2 CG-PUSCH with Rel-16 repetition factor

  - DG-PUSCH with Rel-17 repetition factor

  - Type-1 CG-PUSCH with Rel-17 repetition factor, if supported in Issue#1-1

  - Type-2 CG-PUSCH with Rel-17 repetition factor

Conclusion

  - Rel-17 PUSCH repetition Type A with K>1 does not support PUSCH transmission without UL-SCH.

Agreement

  For repK-r17,

  The value range of repK-17 is {1, 2, 4, 8, 12, 16, 24, 32}.

  repK-r17 is included in ConfiguredGrantConfig.

  When repK-r17 is provided, the legacy repK is not provided.

Agreement

  For HD-FDD RedCap Ues supporting the counting based on available slots.

  For DG-PUSCH, ssb-PositionsInBurst is used in the first step of determining of available slots.

  A slot is not counted in the number of available slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionInBurst.

  Note: Neither tdd-UL-DL-ConfigurationCommon nor tdd-UL-DL-ConfigurationDedicated is configured for FDD.

Agreements in RAN1#107bis-e

Conclusion:

No consensus to introduce pusch-AggregationFactor-r17.

Agreement

- Remove the notes from “Per (UE, cell, TRP, …)” and “Comment” columns of the existing AvailableSlotCounting in the consolidated RRC parameter list.

- If separate FGs are defined for DG-PUSCH and CG-PUSCH, add another AvailableSlotCounting to the consolidated RRC parameter list, with the following contents.

| WI code | Sub-feature group | RAN1 specification | Section | RAN2 Parant IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_cov_enh-Core | Enhancement on PUSCH repetition Type A |  |  |  |  | AvailableSlotCounting | new |  | Enabling PUSCH repetitions counted on the basis of available slots | ENUMERATED {enabled, disable } |  | in ConfiguredGrantConf | UE-specific | 38.331 | Agreement: • Each available slot identified by the UE is considered as a transmission occasion for PUSCH repetition. o RV is cycled across transmission occasions, irrespective of whether PUSCH transmission in the transmission occasion is further omitted or not. |

Conclusion:

- The cancellation of LP PUSCH (introduced in Rel-17 eIIoT/URLLC WI) is applied in Step 2 of the previously agreed 2-step procedure of Rel-17 PUSCH repetitions counted on the basis of available slots (i.e., Option 1-B).

  - No specification impact is expected.

Conclusion

- The CovEnh discussion on the available slot counting for inter-cell mTRPs is deferred until further progress on the collision handling between UL channels/signals and multiple SSBs for inter-cell mTRPs is made in feMIMO session.