3GPP TSG RAN WG1 #108-e	R1-2202690

e-Meeting, Feb 21st – Mar 3rd, 2022

Title:	Draft reply LS on UE capability for supporting single DCI transmission schemes for multi-TRP

Response to:	R1-2200872/R4-2120652

Release:	Rel-16

Work Item:	NR_eMIMO-Core

Source:	Apple, [RAN1]

To:	RAN4

Cc:	RAN2

Contact Person:

Name:	Haitong Sun

E-mail Address:	haitong_sun@apple.com

# 1. Overall Description

RAN1 would like to thank RAN4 for the LS on UE capability for supporting single DCI transmission schemes for multi-TRP operation. RAN1 would like to provide the answer to the following question.

Q: RAN4 requests RAN1 to clarify if explicit signaling of maxNumberActiveTCI-PerBWP other than n1 is required for single DCI FDM and SDM transmission schemes or the UE capability of supportFDM-SchemeA-r16 or singleDCI-SDM-scheme-r16 is sufficient.

A: In a CC/band, if a UE supports any of

- Single-DCI Multi-TRP SDM, i.e.,  singleDCI-SDM-scheme-r16

- Single-DCI Multi-TRP fdmSchemeA, i.e., supportFDM-SchemeA-r16

- Single-DCI Multi-TRP fdmSchemeB, i.e., supportFDM-SchemeB-r16

- Single-DCI Multi-TRP tdmSchemeA, i.e., supportTDM-SchemeA-r16

It is RAN1’s understanding that

- The UE is required to support at least 2 active TCI states in the corresponding CC/band

- The UE is not required to explicitly signal maxNumberActiveTCI-PerBWP greater than n1 in the corresponding band

# 3. Actions:

To: RAN4

RAN1 respectfully asks RAN4 to take the above information into account for future work.

# 4. Date of Next RAN1 Meetings:

TSG RAN WG1   Meeting #109-e            	   May. 16 – 27, 2021          	Online