3GPP TSG RAN WG1 #108-e		                                   					       												                 R1-2202860

e-Meeting, February 21st – March 3rd, 2022

Source:	Ad-Hoc Chair (AT&T)

Title:	Session Notes of AI 8.16.2

Agenda Item:	8.16.2

Document for: 	Endorsement

#### 8.16.2	UE features for supporting NR from 52.6 GHz to 71 GHz

[108-e-R17-UE-features-52-71GHz-01] Email discussion on UE features for supporting NR from 52.6 GHz to 71 GHz – Ralf (AT&T)

- 1st check point: February 25

- Final check point: March 3

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-1d | Multiple PDSCH scheduling by single DCI for 120kHz in FR2-2 | 1. Multi-PDSCH scheduling by single DCI for the operation with 120 kHz SCS 2. HARQ enhancements [for both type 1 and type 2 HARQ codebook] for supporting multi-PDSCH scheduling with singe DCI | 24-1 | Yes | N/A | Multiple PDSCH scheduling by single DCI for 120kHz is not supported in FR2-2 | Per band | N/A | N/A | N/A | FFS: to extend this FG to other frequency ranges | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1f | Multiple PDSCH scheduling by single DCI for 120kHz in FR2-1 | 1. Multi-PDSCH scheduling by single DCI for the operation with 120 kHz SCS 2. HARQ enhancements [for both type 1 and type 2 HARQ codebook] for supporting multi-PDSCH scheduling with singe DCI |  | Yes | N/A | Multiple PDSCH scheduling by single DCI for 120kHz is not supported in FR2-1 | Per band | N/A | N/A | N/A |  | Optional with capability signalling |

- Continue discussion on extending 24-1f to other SCSs

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-1e | Multiple PUSCH scheduling by single DCI for 120kHz in FR2-2 | 1. Multi-PUSCH scheduling by single DCI for the operation with 120 kHz SCS | 24-1a | Yes | N/A | Multiple PUSCH scheduling by single DCI for 120kHz is not supported in FR2-2 | Per band | N/A | N/A | N/A | FFS: to extend this FG to other frequency ranges | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1g | Multiple PUSCH scheduling by single DCI for 120kHz in FR2-1 | 1. Multi-PUSCH scheduling by single DCI for the operation with 120 kHz SCS with non-contiguous allocation |  | Yes | N/A | Multiple PUSCH scheduling by single DCI for 120kHz is not supported in FR2-1 with non-contiguous allocation | Per band | N/A | N/A | N/A |  | Optional with capability signalling |

- Continue discussion on extending 24-1g to other SCSs

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-8 | 32 DL HARQ processes for FR 2-2 | Support 32 HARQ processes in DL for 120/480/960 kHz | 24-1 | Yes | N/A | 32 DL HARQ processes for FR 2-2 is not supported | FFS | N/A | N/A | N/A | FFS: 120 kHz A UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL in FR2-2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - Continue discussion whether a new, separate FG for FR2-1 is introduced

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-9 | 32 UL HARQ processes for FR 2-2 | Support 32 HARQ processes in UL for 120/480/960 kHz | 24-1a | Yes | N/A | 32 UL HARQ processes for FR 2-2 is not supported | FFS | N/A | N/A | N/A | FFS: 120 kHz A UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for UL shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for UL in FR2-2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - Continue discussion whether a new, separate FG for FR2-1 is introduced

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-4 | 480KHz SCS support for DL | 1. 480KHz SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 480KHz with (Xs,Ys) = (4,1) FFS: 3. Multi-PDSCH scheduling by single DCI for the operation with 480 kHz SCS and corresponding HARQ enhancements 4. Within the Ys = 1 slot (with Xs=4), monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS with a maximum of two monitoring spans per slot with a span duration of Y symbols and a minimum gap of X symbols between the start of two spans, where set2 (X,Y) = (4, 3) and (7, 3) are supported symbols where set2 is defined in FG3-5b (FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS) 5. Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for FDD (This supersedes corresponding component of FG 3-5b) 6. Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for TDD (This supersedes corresponding component of FG 3-5b) [7. For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot of the slot group, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group.] | 24-1 | Yes | N/A | 480KHz SCS for DL is not supported | Per band | N/A | N/A | N/A | FFS: component description without a reference to other R15 FGs | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-5 | 960KHz SCS support for DL | 1. 960KHz SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys)=(8,1) FFS: 3. Multi-PDSCH scheduling by single DCI for the operation with 960 kHz SCS and corresponding HARQ enhancements 43. Within the Ys = 1 slot (with Xs=8), monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS with a span duration of Y symbols and a minimum gap of X symbols between the start of two spans, where (X,Y) set1 = (7, 3) is supported symbols where set1 is defined in FG3-5b (FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS) 54. Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for FDD (This supersedes corresponding component of FG 3-5b) 65. Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for TDD (This supersedes corresponding component of FG 3-5b) [7. For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot of the slot group, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group.] | 24-1 | Yes | N/A | 960KHz SCS support for DL is not supported | Perband | N/A | N/A | N/A | FFS: component description without a reference to other R15 FGs | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-1d | Multiple PDSCH scheduling by single DCI for 120kHz in FR2-2 | 1. Multi-PDSCH scheduling by single DCI for the operation with 120 kHz SCS 2. HARQ enhancements [for both type 1 and type 2 HARQ codebook] for supporting multi-PDSCH scheduling with singe DCI | 24-1 | Yes | N/A | Multiple PDSCH scheduling by single DCI for 120kHz is not supported in FR2-2 | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1f | Multiple PDSCH scheduling by single DCI for 120kHz in FR2-1 | 1. Multi-PDSCH scheduling by single DCI for the operation with 120 kHz SCS 2. HARQ enhancements [for both type 1 and type 2 HARQ codebook] for supporting multi-PDSCH scheduling with singe DCI |  | Yes | N/A | Multiple PDSCH scheduling by single DCI for 120kHz is not supported in FR2-1 | Per band | N/A | N/A | N/A |  | Optional with capability signalling |

R1-2200958	Rel-17 UE features for extension to 71 GHz	Huawei, HiSilicon

R1-2201121	Discussions on UE features for NR operation from 52.6GHz to 71GHz	vivo

R1-2201302	Discussion on UE features for FR2-2	OPPO

R1-2201395	Discussion on UE features for 52.6 to 71GHz	ZTE, Sanechips

R1-2201409	On UE features for supporting NR from 52.6 GHz to 71 GHz	Nokia, Nokia Shanghai Bell

R1-2201502	Views on Rel-17 UE features for supporting NR in FR2-2	NTT DOCOMO, INC.

R1-2201727	Discussion on UE capability for extending NR up to 71 GHz	Intel Corporation

R1-2201741	UE features for extending current NR operation to 71 GHz	Ericsson

R1-2201792	Views on Rel-17 Beyond 52.6 GHz UE features	Apple

R1-2202039	On UE features for supporting NR from 52.6 GHz to 71 GHz	Samsung

R1-2202075	Views on UE features for supporting NR from 52.6 GHz to 71 GHz	MediaTek Inc.

R1-2202166	UE features for NR from 52.6 Ghz to 71 Ghz	Qualcomm Incorporated

Withdrawn

R1-2202355	Discussion on UE features for NR above 52.6 GHz	LG Electronics