3GPP TSG RAN WG1 #108-e	R1-2202885

e-Meeting, 21st February – 3rd March 2022

Title:	[Draft] LS on operation with and without SSB for RedCap UE

Response to:

Release:	Rel-17

Work Item:	Support of reduced capability NR devices (NR_redcap)

Source:	Ericsson [To be RAN1]

To:	RAN2, RAN4

Cc:

Contact person:	Johan Bergman, johan (dot) bergman (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN1#108-e has made the following agreements and conclusion:

Agreement: - The following working assumptions from RAN1#107-e are NOT confirmed for idle/inactive mode and furthermore they are replaced by the agreements further down for connected mode. - For FR1, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective - For FR2, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective - For BWP#0 configuration option 1, - For FR1, - For a separate initial DL BWP, for a RedCap UE in connected mode, paging can only be configured if it contains CD-SSB and the entire CORESET#0. - For FR2, - For a separate initial DL BWP, for a RedCap UE in connected mode, paging can only be configured if it contains CD-SSB. - Note: For BWP#0 configuration option 2, - For FR1, - For a separate initial DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0), if it is configured for paging, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE supporting FG 6-1a does not expect it to contain SSB/CORESET#0/SIB - For FR2, - For a separate initial DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0), if it is configured for paging, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE supporting FG 6-1a does not expect it to contain SSB/CORESET#0/SIB Agreement: Replace the working assumption from RAN1#107e “Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on for CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities” with the following agreement: - For FR1, - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities [FG 6-1a] with supporting CSI-RS, or [FG 6-1a] without supporting CSI-RS. - For FR2, - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities [FG 6-1a] with supporting CSI-RS, or [FG 6-1a] without supporting CSI-RS. Note: The cases that CSI-RS in this agreement can support are left to RAN4. Agreement: - A RedCap UE supports existing applicable mandatory feature(s) that are based on SSB using NCD-SSB (including NCD-SSB based measurements) as mandatory feature(s) in an RRC-configured DL BWP that does not include CD-SSB. - NCD-SSB is ‘QCL’-ed with CD-SSB when the NCD-SSB and CD-SSB share the same SSB index. - Note: RAN1 assumes that NCD-SSB is configured by higher layer Conclusion: - From RAN1 perspective, whether and under what conditions a RedCap UE requires to be configured with existing measurement gaps to support operation without SSB in an RRC-configured active BWP, and its related UE feature discussion (including measurement gaps) is up to RAN4. - Send an LS to RAN4 to inform them about the conclusion.

# 2	Actions

To RAN2 and RAN4:

ACTION: 	RAN1 respectfully requests RAN2 and RAN4 to take the above into account in their future work.

# 3	Dates of next TSG-RAN WG1 meetings

TSG-RAN WG1 Meeting #109-e	16th – 27th May 2022			Online

TSG-RAN WG1 Meeting #110	22nd – 26th August 2022		Toulouse, FR