3GPP TSG RAN WG1 Meeting #108-e                                                                      R1-2202904

e-Meeting, February 21st - March 3rd, 2022

Source:              Moderator (ZTE)

Title:                   FL summary #4 on SRS enhancements

Agenda Item:     8.1.3

Document for:   Discussion and Decision

- Updated TP 3-2 after the final round of email discussion

TP 3-2: Endorse the following text proposal for TS 38.214 V17.0.0, to be included in Editor’s CR.

< Start of the text proposal > #### 6.2.1.2 UE sounding procedure for DL CSI acquisition < Unchanged parts are omitted > - For 1T=1R, or 2T=2R, or 4T=4R, if the UE is not indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], up to two SRS resource sets each with one SRS resource can be configured, where the number of SRS ports for each resource is equal to 1, 2, or 4. If the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets],two SRS resource sets configured with resourceType in SRS-ResourceSet set to ‘semi-persistent’ and one SRS resource set configured with resourceType in SRS-ResourceSet set to ‘periodic’ can be configured, and the two SRS resource sets configured with ‘semi-persistent’ are not activated at the same time, or up to two SRS resource sets can be configured, where each SRS resource set has one SRS resource, the number of SRS ports for each resource is equal to 1, 2, or 4, or < End of the text proposal >