3GPP TSG-RAN WG1 Meeting #108-e	R1-2202961

# Electronic Meeting, February 21st – March 3rd, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0255 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-Core |  |  |  |  |  | Date: |  |  | 2022-03-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Corrections on enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 6.2.1, the support of PRS for PDC as a spatial reference for an SRS resource for RTT-based PDC is introduced. - In clause 9, the configuration of SRS for PDC is corrected - A new subclause 9.1 is added to define the PRS for PDC reception. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support of enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 9, 9.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

### 6.2.1	UE sounding procedure

The UE may be configured with one or more Sounding Reference Signal (SRS) resource sets as configured by the higher layer parameter SRS-ResourceSet or SRS-PosResourceSet. For each SRS resource set configured by SRS-ResourceSet, a UE may be configured with SRS resources (higher layer parameter SRS-Resource), where the maximum value of K is indicated by UE capability [13, 38.306]. When SRS resource set is configured with the higher layer parameter SRS-PosResourceSet, a UE may be configured with K ≥1 SRS resources (higher layer parameter SRS-PosResource), where the maximum value of K is 16. The SRS resource set applicability is configured by the higher layer parameter usage in SRS-ResourceSet. When the higher layer parameter usage is set to 'beamManagement', only one SRS resource in each of multiple SRS resource sets may be transmitted at a given time instant, but the SRS resources in different SRS resource sets with the same time domain behaviour in the same BWP may be transmitted simultaneously.

For aperiodic SRS at least one state of the DCI field is used to select at least one out of the configured SRS resource set(s).

The following SRS parameters are semi-statically configurable by higher layer parameter SRS-Resource or SRS-PosResource.

-	srs-ResourceId or SRS-PosResourceId determines SRS resource configuration identity.

-	Number of SRS ports, as defined by the higher layer parameter nrofSRS-Ports and described in clause 6.4.1.4 of [4, TS 38.211]. If not configured, nrofSRS-Ports is 1.

-	Time domain behaviour of SRS resource configuration as indicated by the higher layer parameter resourceType, which may be periodic, semi-persistent, aperiodic SRS transmission as defined in clause 6.4.1.4 of [4, TS 38.211].

-	Slot level periodicity and slot level offset as defined by the higher layer parameters periodicityAndOffset-p or periodicityAndOffset-sp for an SRS resource of type periodic or semi-persistent. The UE is not expected to be configured with SRS resources in the same SRS resource set SRS-ResourceSet or SRS-PosResourceSet with different slot level periodicities. For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a slot level offset is defined by the higher layer parameter slotOffset. For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a list of zero up to four different available slot offset values from the reference slot n + k to the slot where the aperiodic SRS resource set is transmitted where n is the slot with triggering DCI and k is SlotOffset is defined by the higher layer parameter AvailableSlotOffset. The parameter AvailableSlotOffset can be configured up to 4 different values. For an SRS-PosResourceSet configured with higher layer parameter resourceType set to 'aperiodic', the slot level offset is defined by the higher layer parameter slotOffset for each SRS resource.

-	Number of OFDM symbols in the SRS resource, starting OFDM symbol of the SRS resource within a slot including repetition factor R as defined by the higher layer parameter resourceMapping and described in clause 6.4.1.4 of [4, TS 38.211]. If R is not configured, then R is equal to the number of OFDM symbols in the SRS resource.

-	SRS bandwidth and , as defined by the higher layer parameter freqHopping and described in clause 6.4.1.4 of [4, TS 38.211]. If not configured, then= 0.

-	Frequency hopping bandwidth , as defined by the higher layer parameter freqHopping and described in clause 6.4.1.4 of [4, TS 38.211]. If not configured, then = 0.

-	Defining partial frequency sounding factor and start RB index for partial frequency sounding as defined by the higher layer parameters FreqScalingFactor PF and StartRBIndex kF, respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, then PF = 1 and kF,= 0.

-	Defining start RB index hopping for partial frequency sounding in different SRS frequency hopping periods for aperiodic/periodic/semi-persistent SRS based on the hopping pattern khopping as described in clause 6.4.1.4.3 in [4, TS 38.211. If not configured, then start RB hopping is not enabled and khopping is fixed to be 0 for all SRS symbols.

-	Defining frequency domain position and configurable shift, as defined by the higher layer parameters freqDomainPosition and freqDomainShift, respectively, and described in clause 6.4.1.4 of [4, TS 38.211]. If freqDomainPosition is not configured, freqDomainPosition is zero.

-	Cyclic shift, as defined by the higher layer parameter cyclicShift-n2, cyclicShift-n4, or cyclicShift-n8 for transmission comb value 2, 4 or 8, and described in clause 6.4.1.4 of [4, TS 38.211].

-	Transmission comb value, as defined by the higher layer parameter transmissionComb described in clause 6.4.1.4 of [4, TS 38.211].

-	Transmission comb offset, as defined by the higher layer parameter combOffset-n2, combOffset-n4, and combOffset-n8 for transmission comb value 2, 4, or 8, and described in clause 6.4.1.4 of [4, TS 38.211].

-	SRS sequence ID, as defined by the higher layer parameter sequenceId in clause 6.4.1.4 of [4, TS 38.211].

-	The configuration of the spatial relation between a reference RS and the target SRS, where the higher layer parameter spatialRelationInfo or spatialRelationInfoPos, if configured, contains the ID of the reference RS. The reference RS may be an SS/PBCH block, CSI-RS configured on serving cell indicated by higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise, or an SRS configured on uplink BWP indicated by the higher layer parameter uplinkBWP, and serving cell indicated by the higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise. When the target SRS is configured by the higher layer parameter SRS-PosResourceSet, the reference RS may also be a DL PRS configured on a serving cell or a non-serving cell indicated by the higher layer parameter dl-PRS, or an SS/PBCH block of a non-serving cell indicated by the higher layer parameter ssb-Ncell. If the UE is configured with [TCI-State]s with [tci-StateId_r17], the reference RS may additionally be an SS/PBCH block associated with a PCI different from the PCI of the serving cell.

The UE may be configured by the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying  adjacent OFDM symbols within the last 6 symbols of the slot, or at any symbol location within the slot if resourceMapping-r16 is provided subject to UE capability, where all antenna ports of the SRS resources are mapped to each symbol of the resource. When the SRS is configured with the higher layer parameter SRS-PosResourceSet the higher layer parameter resourceMapping-r16 in SRS-PosResource indicates an SRS resource occupying NS∈1,2,4,8,12 adjacent symbols anywhere within the slot. When the SRS is configured with the higher layer parameter SRS-ResourceSet, the higher layer parameter resourceMapping-r17 in SRS-Resource indicates an SRS resource occupying NS∈1,2,4,8,10,12,14 adjacent symbols anywhere within the slot.

If a PUSCH with a priority index 0 and SRS configured by SRS-Resource are transmitted in the same slot on a serving cell, the UE may only be configured to transmit SRS after the transmission of the PUSCH and the corresponding DM-RS.

If a PUSCH transmission with a priority index 1 or a PUCCH transmission with a priority index 1 would overlap in time with an SRS transmission on a serving cell, the UE does not transmit the SRS in the overlapping symbol(s).

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource or SRS-PosResource is set to 'periodic':

-	if the UE is configured with the higher layer parameter spatialRelationInfo, spatialRelationInfo-PDC or spatialRelationInfoPos containing the ID of a reference 'ssb-Index', 'ssb-IndexServing', or 'ssb-IndexNcell', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference SS/PBCH block, if the higher layer parameter spatialRelationInfo, spatialRelationInfo-PDC or spatialRelationInfoPos contains the ID of a reference 'csi-RS-Index' or 'csi-RS-IndexServing', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference periodic CSI-RS or of the reference semi-persistent CSI-RS, if the higher layer parameter spatialRelationInfo, spatialRelationInfo-PDC or spatialRelationInfoPos containing the ID of a reference 'srs' or 'srs-spatialRelation', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the transmission of the reference periodic SRS. When the SRS is configured by the higher layer parameter SRS-PosResource and if the higher layer parameter spatialRelationInfoPos contains the ID of a reference 'dl-PRS', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference DL PRS. When the SRS is configured by the higher layer parameter SRS-Resource and if the higher layer parameter spatialRelationInfo-PDC contains the ID of a reference 'dl-PRS-PDC', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference DL PRS for RTT-based propagation delay compensation according to clause 9.

<omitted text>

# 9	UE procedures for transmitting and receiving for RTT-based propagation delay compensation

For operation with RTT-based propagation delay compensation, the UE may be configured with either:

-	one CSI-RS for tracking with higher layer parameter pdc-Info for Rx – Tx time difference estimation at UE side and one SRS resource set with usage-pdc, or

-	one PRS configuration of higher layer parameter NR-DL-PRS-PDC-ResourceSet [12, TS 38.331] for Rx – Tx time difference estimation at UE side and one SRS resource set with usage-pdc.

The related UE procedures for transmitting uplink reference signals and receiving downlink reference signals for RTT-based propagation delay compensation are defined as follows:

-	for reception of CSI-RS for tracking with higher layer parameter pdc-Info, the UE follows the procedures for reception of CSI-RS for tracking defined in Clause 5.1.6.1.1.

-	for reception of the one PRS configuration provided by RRC [12, TS 38.331] for RTT-based propagation delay compensation, the UE follows the procedure for PRS reception for RTT-based propagation delay compensation defined in Clause 9.1.

-	for transmission of an SRS resource set configured with usage-pdc, the UE follows the procedures for SRS transmission defined in Clause 6.2.1.

## 9.1	PRS reception procedure for RTT-based propagation delay compensation

The DL PRS resource set for RTT-based propagation delay compensation consists of K≥1 DL PRS resource(s) where each has an associated spatial transmission filter. Each DL PRS resource is configured via higher layer parameters NR-DL-PRS-Resource and is uniquely defined by nr-DL-PRS-ResourceID. The subcarrier spacing and the cyclic prefix of the DL PRS resources are defined by the subcarrier spacing and the cyclic prefix of the DL active bandwidth part of the serving cell.

The DL PRS resource set for RTT-based propagation delay compensation is configured by NR-DL-PRS-PDC-ResourceSet, consists of one or more DL PRS resource(s) and it is defined by:

-	dl-PRS-Periodicity-and-ResourceSetSlotOffset defines the DL PRS resource periodicity and takes values TperPRS∈2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 slots, where μ=0, 1, 2, 3 for 15, 30, 60 and 120 kHz subcarrier spacing respectively and the slot offset for DL PRS resource set with respect to SFN0 slot 0. All the DL PRS resources are configured with the same DL PRS resource periodicity.

-	dl-PRS-ResourceRepetitionFactor defines how many times each DL-PRS resource is repeated for a single instance of the DL-PRS resource set and takes values TrepPRS∈1,2,4,6,8,16,32. All the DL PRS resources within the resource set have the same resource repetition factor.

-	dl-PRS-ResourceTimeGap defines the offset in number of slots between two repeated instances of a DL PRS resource with the same nr-DL-PRS-ResourceID within a single instance of the DL PRS resource set. The UE only expects to be configured with dl-PRS-ResourceTimeGap if dl-PRS-ResourceRepetitionFactor is configured with value greater than 1. The time duration spanned by one instance of a nr-DL-PRS-ResourceSet is not expected to exceed the configured value of DL PRS periodicity. All the DL PRS resources within the resource set have the same value of dl-PRS-ResourceTimeGap.

-	dl-PRS-ResourceList determines the DL PRS resources that are contained within the DL PRS resource set.

-	dl-PRS-ResourceBandwidth defines the number of resource blocks configured for DL PRS transmission. The parameter has a granularity of 4 PRBs with a minimum of 24 PRBs and a maximum of 272 PRBs. All the DL PRS resources within the resource set have the same value of dl-PRS-ResourceBandwidth.

-	dl-PRS-StartPRB defines the starting PRB index of the DL PRS resource with respect to subcarrier 0 in common resource block 0. The starting PRB index has a granularity of one PRB with a minimum value of 0 and a maximum value of 2176 PRBs. All the DL PRS resources within the resource set have the same value of dl-PRS-StartPRB.

-	dl-PRS-NumSymbols defines the number of symbols of the DL PRS resource within a slot where the allowable values are given in Clause 7.4.1.7.3 of [4, TS38.211]. All the DL PRS resources within the resource set have the same value of dl-PRS-NumSymbols.

A DL PRS resource is defined by:

-	nr-DL-PRS-ResourceID determines the DL PRS resource configuration identity. All DL PRS resource IDs are locally defined within the DL PRS resource set.

-	dl-PRS-SequenceID is used to initialize cinit value used in pseudo random generator as described in Clause 7.4.1.7.2 of [4, TS 38.211] for generation of DL PRS sequence for a given DL PRS resource.

-	dl-PRS-CombSizeN-AndReOffset defines the comb size of a DL PRS resource, where the allowable values are given in Clause 7.4.1.7.3 of [TS38.211], and the starting RE offset of the first symbol within a DL PRS resource in frequency. The UE may expect the same comb size to be configured for all DL PRS resources of the PRS resource set. The relative RE offsets of the remaining symbols within a DL PRS resource are defined based on the initial offset and the rule described in Clause 7.4.1.7.3 of [4, TS 38.211].

-	dl-PRS-ResourceSlotOffset determines the starting slot of the DL PRS resource with respect to corresponding DL PRS resource set slot offset.

-	dl-PRS-ResourceSymbolOffset determines the starting symbol of a slot configured with the DL PRS resource.

The UE assumes constant EPRE is used for all REs of a given DL PRS resource.

The UE assumes that the DL PRS from the serving cell is not mapped to any symbol that contains SS/PBCH block from the serving cell.

The UE does not expect to be scheduled or configured for reception of any downlink channel or any other downlink signal(s) in the OFDM symbol(s) and PRBs of the DL PRS resource for RTT-based propagation delay compensation to be received.

The UE is expected to receive the DL PRS for RTT-based propagation delay compensation only in RRC_CONNECTED state and within the DL active bandwidth part of the serving cell.

<omitted text>