3GPP TSG-WG1 Meeting #108	R1-2202971

e-meeting, February 21 – March 3, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | 0567 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to additional enhancements for NB-IoT and eMTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh4_LTE_eMTC6 |  |  |  |  |  | Date: |  |  | 2022-03-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is an editorial error in the higher layer parameter named “CE-PDSCH-14HARQ-Config” in TS 36.211 clause 5.4.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The higher layer parameter named “CE-PDSCH-14HARQ-Config” in TS 36.211 clause 5.4.3 has been corrected as “ce-PDSCH-14HARQ-Config”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An incorrect higher layer parameter name would cause that the configuration of the 14 HARQ process feature cannot be obtained correctly by the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.4.3	Mapping to physical resources

The block of complex-valued symbols  shall be multiplied with the amplitude scaling factor  in order to conform to the transmit power  specified in Clause 5.1.2.1 in 3GPP TS 36.213 [4], and mapped in sequence starting with  to resource elements. PUCCH uses one or more resource block in each of the two slots in a subframe. Within the physical resource block(s) used for transmission, the mapping of  to resource elements  on antenna port  and not used for transmission of reference signals shall be in increasing order of first , then  and finally the slot number, starting with the first slot in the subframe. The relation between the index  and the antenna port number  is given by Table 5.2.1-1.

For non-BL/CE UEs, except for PUCCH format 4, the physical resource blocks to be used for transmission of PUCCH in slot  are given by

For BL/CE UEs, PUCCH is transmitted with  repetitions.

-	The BL/CE UE is not expected to transmit with NrepPUCCH>1 when ce-PDSCH-14HARQ-Config is configured.

The PUCCH transmission spans  consecutive subframes, including subframes that are not BL/CE UL subframes where the UE postpones the PUCCH transmission if . If the BL/CE UE is configured with ce-HARQ-AckDelay-r17 indicating Alt-2e, the UE does not postpone the PUCCH transmission.

-	The quantity  is given

-	by the higher layer parameter pucch-NumRepetitionCE-Format1 for PUCCH format 1/1a and pucch-NumRepetitionCE-Format2 for PUCCH format 2/2a/2b, if configured. Otherwise

-	by the higher-layer parameter pucch-NumRepetitionCE-Msg4-Level0-r13, pucch-NumRepetitionCE-Msg4-Level1-r13, pucch-NumRepetitionCE-Msg4-Level2-r13 or pucch-NumRepetitionCE-Msg4-Level3-r13.

-	If uplink resource reservation is enabled for the UE as specified in [9], then in case of PUCCH transmission with  associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space including PUCCH transmission without a corresponding MPDCCH,

-	In a subframe that is fully reserved as defined in clause 8.0 in [4], the PUCCH transmission is postponed until the next BL/CE uplink subframe that is not fully reserved.

-	In a subframe that is partially reserved, the reserved SC-FDMA symbols shall be counted in the PUCCH mapping but not used for transmission of the PUCCH.

The physical resource blocks to be used for transmission of PUCCH in subframe  within the  consecutive subframes are given by

where  is the absolute subframe number of the first uplink subframe intended for PUCCH.

The variable  depends on the PUCCH format.

-	Formats 1, 1a and 1b:

-	Formats 2, 2a and 2b:

-	Format 3:

-	Format 5 (non-BL/CE UEs only):

For non-BL/CE UEs, for PUCCH format 4, the physical resource blocks to be used for transmission of PUCCH in slot  are given by

where  is obtained from [4].

Mapping of modulation symbols for the physical uplink control channel for PUCCH formats 1 – 3 is illustrated in Figure 5.4.3-1.

In case of simultaneous transmission of sounding reference signal and PUCCH format 1, 1a, 1b, 3, 4 or 5 when there is one serving cell configured, the shortened PUCCH format shall be used where the last SC-FDMA symbol in the second slot of a subframe shall be left empty.

In case of guard period for narrowband or wideband retuning for BL/CE UEs, if an SC-FDMA symbol is left empty due to guard period, the SC-FDMA symbol shall be counted in the PUCCH mapping but not used for transmission of the PUCCH. The SC-FDMA symbol affected by the guard period can be the first SC-FDMA symbol in the first slot of a subframe and/or the last SC-FDMA symbol in the second slot of a subframe.

Figure 5.4.3-1: Mapping to physical resource blocks for PUCCH formats 1 – 3 for non-BL/CE UEs.