3GPP TSG-WG1 Meeting #108	R1-2202977

e-meeting, February 21 – March 3, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | 0092 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to IIoT and URLLC enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-Core |  |  |  |  |  | Date: |  |  | 2022-03-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Higher-layer parameters for PRS configuration and the reference point for resource-element mapping is unclear in case of RTT-based propagation delay estimation missing. Scrambling when multiplexing high-priority and low-priority ACK on PUCCH is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added higher-layer parameters and clarification of the reference point. Updated scrambling description for PUCCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous specifications and incorrect operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.5.1, 6.3.26.1, 7.4.1.7.3, 7.4.1.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 6.3.2.5.1	Scrambling

The block of bits b0,…,b(Mbit-1), where Mbit is the number of bits transmitted on the physical channel, shall be scrambled prior to modulation, resulting in a block of scrambled bits b0,…,b(Mbit-1) according to the following pseudo code

bi=bi+c(i) mod 2

Set i = 0

while i<Mbit

if bi=y	// UCI placeholder bits

bi= bi-1

else

bi=bi+c(i) mod 2

end if

i = i + 1

end while

where y is the tag defined in [4, TS38.212] and the scrambling sequence c(i)cq(i) is given by clause 5.2.1. The scrambling sequence generator shall be initialized with

cinit=nRNTI∙215+nID

where

-	nID∈0,1,…,1023 equals the higher-layer parameter dataScramblingIdentityPUSCH if configured,

-	nID=NIDcell otherwise

and nRNTI is given by the C-RNTI.

##### 6.3.2.6.1	Scrambling

The block of bits b0,…,b(Mbit-1), where Mbit is the number of bits transmitted on the physical channel, shall be scrambled prior to modulation, resulting in a block of scrambled bits b0,…,b(Mbit-1) according to the following pseudo code

bi=bi+c(i) mod 2

Set i = 0

while i<Mbit

if bi=y	// UCI placeholder bits

bi= bi-1

else

bi=bi+c(i) mod 2

end if

i = i + 1

end while

where y is the tag defined in [4, TS38.212] and the scrambling sequence  c(i) is given by clause 5.2.1. The scrambling sequence generator shall be initialized with

cinit=nRNTI∙215+nID

where

-	nID∈0,1,…,1023 equals the higher-layer parameter dataScramblingIdentityPUSCH if configured,

-	nID=NIDcell otherwise

and nRNTI is given by the C-RNTI.

##### 7.4.1.7.3	Mapping to physical resources in a downlink PRS resource

For each downlink PRS resource configured, the UE shall assume the sequence rm is scaled with a factor βPRS and mapped to resources elements k,lp,μ according to

ak,lp,μ=βPRS rm
m=0, 1, …
k=mKcombPRS+koffsetPRS+k' mod KcombPRS
l=lstartPRS, lstartPRS+1, …, lstartPRS+LPRS-1

when the following conditions are fulfilled:

-	the resource element k,lp,μ is within the resource blocks occupied by the downlink PRS resource for which the UE is configured;

-	the symbol l is not used by any SS/PBCH block used by a serving cell for downlink PRS transmitted from the same serving cell or any SS/PBCH block from a non-serving cell whose time frequency location is provided to the UE by higher layers for downlink PRS transmitted from the same non-serving cell;

-	the slot number satisfies the conditions in clause 7.4.1.7.4.

and where

-	the antenna port p=5000

-	lstartPRS is the first symbol of the downlink PRS within a slot and given by the higher-layer parameter dl-PRS-ResourceSymbolOffset;

-	the size of the downlink PRS resource in the time domain LPRS∈2,4,6,12 is given by the higher-layer parameter dl-PRS-NumSymbols;

-	the comb size KcombPRS∈2, 4, 6,12 is given by the higher-layer parameter dl-PRS-CombSizeN-AndReOffset for a downlink PRS resource configured for RTT-based propagation delay compensation, otherwise by the higher-layer parameter dl-PRS-CombSizeN such that the combination LPRS,KcombPRS is one of {2, 2},{4, 2}, {6, 2}, {12, 2}, {4, 4}, {12, 4}, {6, 6}, {12, 6} and {12, 12};

-	the resource-element offset koffsetPRS∈0,1,…,KcombPRS-1 is obtained from the higher-layer parameter dl-PRS-CombSizeN-AndReOffset;

-	the quantity k' is given by Table 7.4.1.7.3-1.

If the downlink PRS resource is configured for RTT based propagation delay compensation as described in clause 9 of [6, TS 38.214], the reference point for k=0 is subcarrier 0 in common resource block 0; Otherwise, the reference point for k=0 is the location of the point A of the positioning frequency layer, in which the downlink PRS resource is configured where point A is given by the higher-layer parameter dl-PRS-PointA.

Table 7.4.1.7.3-1: The frequency offset k' as a function of l-lstartPRS.

| KcombPRS | Symbol number within the downlink PRS resource l-lstartPRS |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| 2 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 4 | 0 | 2 | 1 | 3 | 0 | 2 | 1 | 3 | 0 | 2 | 1 | 3 |
| 6 | 0 | 3 | 1 | 4 | 2 | 5 | 0 | 3 | 1 | 4 | 2 | 5 |
| 12 | 0 | 6 | 3 | 9 | 1 | 7 | 4 | 10 | 2 | 8 | 5 | 11 |

##### 7.4.1.7.4	Mapping to slots in a downlink PRS resource set

For a downlink PRS resource in a downlink PRS resource set, the UE shall assume the downlink PRS resource being transmitted when the slot and frame numbers fulfil

Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS mod TperPRS∈iTgapPRSi=0TrepPRS-1

and one of the following conditions are fulfilled:

-	the higher-layer parameters dl-PRS-MutingOption1 and dl-PRS-MutingOption2 are not provided;

-	the higher-layer parameter dl-PRS-MutingOption1 is provided with bitmap b1 but dl-PRS-MutingOption2 with bitmap b2 is not provided, and bit bi1 is set;

-	the higher-layer parameter dl-PRS-MutingOption2 is provided with bitmap b2 but dl-PRS-MutingOption1 with bitmap b1 is not provided, and bit bi2 is set;

-	the higher-layer parameters dl-PRS-MutingOption1 with bitmap b1 and dl-PRS-MutingOption2 with b2 are both provided, and both bit bi1 and bi2 are set.

where

-	bi1 is bit i=Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS TmutingPRSTperPRSmod L in the bitmap given by the higher-layer parameter dl-PRS-MutingOption1 where L∈2, 4, 6, 8, 16, 32 is the size of the bitmap;

-	bi2 is bit i=Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS mod TperPRS TgapPRS mod TrepPRS in the bitmap given by the higher-layer parameter dl-PRS-MutingOption2;

-	the periodicity TperPRS∈2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 and the slot offset ToffsetPRS∈0,1,…,TperPRS-1 are given by the higher-layer parameter dl-PRS-Periodicity-and-ResourceSetSlotOffset;

-	the downlink PRS resource slot offset Toffset,resPRS is given by the higher-layer parameter dl-PRS-ResourceSlotOffset;

-	the repetition factor TrepPRS∈1,2,4,6,8,16,32 is given by the higher-layer parameter dl-PRS-ResourceRepetitionFactor;

-	the muting repetition factor TmutingPRS is given by the higher-layer parameter dl-PRS-MutingBitRepetitionFactor;

-	the time gap TgapPRS∈1,2,4,8,16,32 is given by the higher-layer parameter dl-PRS-ResourceTimeGap;

For a downlink PRS resource in a downlink PRS resource set configured for RTT-based propagation delay compensation, the UE shall assume the downlink PRS resource being transmitted as described in clause 9 of [6, TS 38.214]; otherwise, the UE shall assume the downlink PRS resource being transmitted as described in clause 5.1.6.5 of [6, TS 38.214].