3GPP TSG RAN WG1#110		R1-22xxxxx

Toulouse, France, August 22nd – 26th, 2022

Agenda Item:	8.2

Source:	Moderator (Lenovo)

Title:	Draft FL Summary for B52.6 GHz PDCCH monitoring enhancements

Document for:	Discussion, Decision

# Issues for PDCCH monitoring enhancements

Table 1 – Identified issues for PDCCH monitoring enhancements

| Issue# | Issue | References | FL initial assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| 8.2.2-1 | multi-slot PDCCH monitoring for for Group (2) SSs | [1] [11]+[12] | H | H [QC] L – We don’t think the changes are critical. The agreement on the Group (2) SS sets in RAN1 #109-e was already captured in the UE feature 24-4 and 24-5. Thus, we think the text in the CR is redundant. Note that, for Rel-15, the restriction of MO for Type1 CSS w/o dedicated RRC and Type0/0A/2 CSS is also captured in FG 3-1, not in TS 38.213. |
| 8.2.2-2 | Correction of PDCCH monitoring as given by monitoringSlotsWithinSlotGroup | [13] | L (may not be necessary) | Agree with moderator assessment – doesn't seem necessary as the previous sentence already seems to capture the case when monitoringSlotsWithinSlotGroup is not provided. [QC] Agree with the moderator’s assessment. |
| 8.2.2-3 | default value for duration-r17 for 480/960 kHz SCS | [6] | H | H [QC] Agree with the moderator’s assessment. |
| 8.2.2-4 | PDCCH multi-slot monitoring restriction for DCI format 2_1 | [2] | H | L As per previous agreement the UE monitors in at most Ys consecutive slots per group of slots, hence the proposed TP seems to place further restriction and we're not sure why. [QC] We generally agree with moderator’s assessment, but the issue may involve many other related issues, which we think should be discussed jointly. For example. - Currently, DL pre-emption indication is per-UE capability. But it is questionable wether pre-emption indication is needed for 480/960 kHz SCSs, considering that the slot length is already very short. - How to define the 14 resources for slot-group based PDCCH monitoring? |
| 8.2.2-5 | multi-slot PDCCH monitoring in CA or NR-DC scenarios | [3] [4] [5] [9]+[10] | H | H Question on the TP in [4]: We're not clear on the reason for the restriction to same Xs for all serving cells. While this seems okay if all cells have the same SCS, what happens in a mixed SCS case? [QC] Agree with moderator’s assessment except [4]. We share the same view as Ericsson. |
| 8.2.2-6 | SSSG switching with multiple cells and different Xs | [7]+[8] | L | L Not clear on the motivation. [QC] H – the issue was discussed in RAN1 #108-e. |

# Details

## Issue 8.2.2-1: multi-slot PDCCH monitoring for for Group (2) SSs

Documents [1] [11] [12] identify a lack of specification text for Group (2) SSs, since prior agreements have not been fully captured. Additionally a fix of typo and additional description of monitoringSlotsWithinSlotGroup are suggested.

38.213 TP according to [1]:

## 10.1 UE procedure for determining physical downlink control channel assignment <Unrelated part omitted> For each DL BWP configured to a UE in a serving cell, the UE is provided by higher layers with S≤10 search space sets where, for each search space set from the S search space sets, the UE is provided the following by SearchSpace: - a search space set index s, 0<s<40 , by searchSpaceId - an association between the search space set s and a CORESET p by controlResourceSetId or by controlResourceSetId-v1610 - a PDCCH monitoring periodicity of ks slots and a PDCCH monitoring offset of os slots, by monitoringSlotPeriodicityAndOffset or by monitoringSlotPeriodicityAndOffset-r17 - a PDCCH monitoring pattern within a slot, indicating first symbol(s) of the CORESET for PDCCH monitoring within each slot where the UE monitors PDCCH, by monitoringSymbolsWithinSlot - a duration of Ts<ks indicating a number of slots that the search space set s exists by duration, or a number of slots in consecutive groups of slots where the search space set s can exist by duration-r17 - a bitmap, by monitoringSlotsWithinSlotGroup, that applies per group of slots and provides a PDCCH monitoring pattern indicating slots in a group of slots for PDCCH monitoring - a size of the group of slots is same as a size of monitoringSlotsWithinSlotGroup - for a Type1-PDCCH CSS set provided by ra-SearchSpace in dedicated RRC signaling, or for a Type3-PDCCH CSS set, or for a USS set, the PDCCH monitoring pattern indicates only consecutive slots in the group of slots for PDCCH monitoring and, at least for one combination Xs,Ys indicated by the UE as a capability, a number of the consecutive slots is not larger than Ys - for a Type1-PDCCH CSS set provided by ra-SearchSpace in SIB1, the PDCCH monitoring pattern indicates only up to 1 slot in the group of slots for PDCCH monitoring - for a Type0-PDCCH CSS set or for a Type0A-PDCCH CSS set, or for a Type2-PDCCH CSS set, the PDCCH monitoring pattern indicates slots in the group of slots for PDCCH monitoring, and the slots are not restricted to be consecutive, a number of those slots is not larger than the size of monitoringSlotsWithinSlotGroup - a number of PDCCH candidates Ms(L) per CCE aggregation level L by aggregationLevel1, aggregationLevel2, aggregationLevel4, aggregationLevel8, and aggregationLevel16, for CCE aggregation level 1, CCE aggregation level 2, CCE aggregation level 4, CCE aggregation level 8, and CCE aggregation level 16, respectively - an indication that search space set s is either a CSS set or a USS set by searchSpaceType <Unrelated part omitted> Denote by MScss(i)(L), 0≤i<Icss, the number of counted PDCCH candidates for monitoring for CSS set Scss(i) and by MSuss(j)(L), 0≤j<Juss, the number of counted PDCCH candidates for monitoring for search space set Suss(j). If a UE indicates three-BDforSSsetLinking and is provided for search space set sj, by searchSpaceLinking, a search space set si with si<sj, set MScss(j)(L)=2∙MScss(j)(L) if si and sj are CSS sets or set MSuss(j)(L)=2∙MSuss(j)(L) if si and sj are USS sets. For the CSS sets in Scss, a UE monitors MPDCCHcss=i=0Icss-1LMScss(i)(L) PDCCH candidates requiring a total of CPDCCHcss non-overlapping CCEs in a slot, or in group of Xs slots for a corresponding combination Xs,Ys, or in a span. Denote by VCCESuss(j) the set of non-overlapping CCEs for search space set Suss(j) and by CVCCESuss(j); the cardinality of VCCESuss(j) where a UE determines the non-overlapping CCEs for search space set Suss(j) considering the allocated PDCCH candidates for monitoring for the CSS sets in Scss and the allocated PDCCH candidates for monitoring for all search space sets Suss(k), 0≤k≤j. Set MPDCCHuss=minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ-MPDCCHcss Set CPDCCHuss=minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ-CPDCCHcss Set j=0 <Unrelated part omitted>

38.213 TP according to [11]:

## 10.1 UE procedure for determining physical downlink control channel assignment *** Unchanged text omitted *** For each DL BWP configured to a UE in a serving cell, the UE is provided by higher layers with S≤10 search space sets where, for each search space set from the S search space sets, the UE is provided the following by SearchSpace: - a search space set index s, 0<s<40 , by searchSpaceId - an association between the search space set s and a CORESET p by controlResourceSetId or by controlResourceSetId-v1610 - a PDCCH monitoring periodicity of ks slots and a PDCCH monitoring offset of os slots, by monitoringSlotPeriodicityAndOffset or by monitoringSlotPeriodicityAndOffset-r17 - a PDCCH monitoring pattern within a slot, indicating first symbol(s) of the CORESET for PDCCH monitoring within each slot where the UE monitors PDCCH, by monitoringSymbolsWithinSlot - a duration of Ts<ks indicating a number of slots that the search space set s exists by duration, or a number of slots in consecutive groups of slots where the search space set s can exist by duration-r17 - a bitmap of length L, by monitoringSlotsWithinSlotGroup, that applies per group of slots and provides a PDCCH monitoring pattern indicating slots in a group of slots for PDCCH monitoring - a size of the group of slots is same as a size of monitoringSlotsWithinSlotGroup - for a Type1-PDCCH CSS set provided by ra-SearchSpace in dedicated RRC signaling, or for a Type3-PDCCH CSS set, or for a USS set, the PDCCH monitoring pattern indicates only consecutive slots in the group of slots for PDCCH monitoring and, at least for one combination Xs,Ys indicated by the UE as a capability, a number of the consecutive slots is not larger than Ys - for a Type0/0A/2-PDCCH CSS set, the PDCCH monitoring pattern can indicate consecutive or non-consecutive slots in the group of slots for PDCCH monitoring, and the number of slots can be up to L - for a Type1-PDCCH CSS set provided by ra-SearchSpace in PDCCH-ConfigCommon, the PDCCH monitoring pattern can indicate one slot in the group of slots for PDCCH monitoring *** Unchanged text omitted ***

## Issue 8.2.2-2: Correction of PDCCH monitoring as given by monitoringSlotsWithinSlotGroup

Document [13] identies that a description on the determination of PDCCH monitoring occasion by the parameter of monitoringSlotsWithinSlotGroup for 480/960 kHz SCS is missing from clause 10.1 in TS38.213.

FL Note: This aspect is somewhat covered by the following existing text in 38.213, but it would be good to check opinions if that is sufficient.

For each DL BWP configured to a UE in a serving cell, the UE is provided by higher layers with S≤10 search space sets where, for each search space set from the S search space sets, the UE is provided the following by SearchSpace: […] a bitmap, by monitoringSlotsWithinSlotGroup, that applies per group of slots and provides a PDCCH monitoring pattern indicating slots in a group of slots for PDCCH monitoring - a size of the group of slots is same as a size of monitoringSlotsWithinSlotGroup - for a Type1-PDCCH CSS set provided by ra-SearchSpace in dedicated RRC signaling, or for a Type3-PDCCH CSS set, or for a USS set, the PDCCH monitoring pattern indicates only consecutive slots in the group of slots for PDCCH monitoring and, at least for one combination Xs,Ys indicated by the UE as a capability, a number of the consecutive slots is not larger than Ys […]

38.213 TP according to [13]:

10.1 UE procedure for determining physical downlink control channel assignment >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< A UE determines a PDCCH monitoring occasion on an active DL BWP from the PDCCH monitoring periodicity, the PDCCH monitoring offset, and the PDCCH monitoring pattern within a slot. For search space set s, the UE determines that PDCCH monitoring occasions exist in a slot with number ns,fμ [4, TS 38.211] in a frame with number nf if (nf Nslot frame, μ+ ns,f μ- os) modks=0nfNslotframe,μ+ns,fμ-op,smod kp,s=0. If monitoringSlotsWithinSlotGroup is provided, the slot is the first slot in a group of slots and PDCCH monitoring occasions exist in the group of slots. The UE monitors PDCCH candidates for search space set s for Ts consecutive slots, starting from slot ns,fμ, and does not monitor PDCCH candidates for search space set s for the next ks-Ts consecutive slots. The slot(s) to be monitored within the Ts consecutive slots are configured by monitoringSlotsWithinSlotGroup if monitoringSlotsWithinSlotGroup is provided. >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

## Issue 8.2.2-3: default value for duration-r17 for 480/960 kHz SCS

Document [6] identies that no default value for duration-r17  has been implemented in any of the RAN1 or RAN2 specs, although RAN1#108e agreed that If duration-r17 is absent, the UE assumes the duration in slots is equal to L.

Proposal: Send an LS to RAN2 to request that the default value of duration-r17 for 480/960 kHz SCS when it is absent be specified in 38.331.

## Issue 8.2.2-4: default value for duration-r17 for 480/960 kHz SCS

Document [2] identies that the existing monitoring occasion restriction for DCI format 2_1 in a slot needs to be extended to PDCCH multi-slot monitoring.

38.213 TP according to [2]:

## 11.2 Interrupted transmission indication If a UE is provided DownlinkPreemption, the UE is configured with an INT-RNTI provided by int-RNTI for monitoring PDCCH conveying DCI format 2_1 [5, TS 38.212]. The UE is additionally configured with - a set of serving cells by int-ConfigurationPerServingCell that includes a set of serving cell indexes provided by corresponding servingCellId and a corresponding set of locations for fields in DCI format 2_1 by positionInDCI - an information payload size for DCI format 2_1 by dci-PayloadSize - an indication granularity for time-frequency resources by timeFrequencySet If a UE detects a DCI format 2_1 for a serving cell from the configured set of serving cells, the UE may assume that no transmission to the UE is present in PRBs and in symbols that are indicated by the DCI format 2_1, from a set of PRBs and a set of symbols of the last monitoring period. The indication by the DCI format 2_1 is not applicable to receptions of SS/PBCH blocks. The set of PRBs is equal to the active DL BWP as defined in clause 12 and includes PRBs. If a UE detects a DCI format 2_1 in a PDCCH reception in a slot, the set of symbols is the last symbols prior to the first symbol of the PDCCH reception in the slot where is the PDCCH monitoring periodicity provided by the value of monitoringSlotPeriodicityAndOffset, as described in clause 10.1, is the number of symbols per slot, is the SCS configuration for a serving cell with mapping to a respective field in the DCI format 2_1, is the SCS configuration of the DL BWP where the UE receives the PDCCH with the DCI format 2_1. If the UE is provided tdd-UL-DL-ConfigurationCommon, symbols indicated as uplink by tdd-UL-DL-ConfigurationCommon are excluded from the last symbols prior to the first symbol of the PDCCH reception in the slot. The resulting set of symbols includes a number of symbols that is denoted as . The UE does not expect to be provided values of , , and resulting to a value of that is not an integer. The UE does not expect to be configured by monitoringSymbolsWithinSlot with more than one PDCCH monitoring occasion for DCI format 2_1 in a slot or by monitoringSlotsWithinSlotGroup with more than one PDCCH monitoring occasion for DCI format 2_1 in a group of Xs slots. A UE is provided the indication granularity for the set of PRBs and for the set of symbols by timeFrequencySet. If the value of timeFrequencySet is 'set0', 14 bits from MSB of a field in DCI format 2_1 have a one-to-one mapping with 14 groups of consecutive symbols from the set of symbols where each of the first symbol groups includes symbols, each of the last symbol groups includes symbols, a bit value of 0 indicates transmission to the UE in the corresponding symbol group and a bit value of 1 indicates no transmission to the UE in the corresponding symbol group.

## Issue 8.2.2-5: multi-slot PDCCH monitoring in CA or NR-DC scenarios

Documents [3] [9] [10] suggest naming alignment of capability parameters.

38.213 TP according to [3]:

# 10 UE procedure for receiving control information *** Unchanged text omitted *** If a UE indicates in UE-NR-Capability a carrier aggregation capability larger than one downlink cell with monitoringCapabilityConfig = r15monitoringcapability, or larger than one downlink cell with monitoringCapabilityConfig = r16monitoringcapability, or larger than one downlink cell with monitoringCapabilityConfig = r17monitoringcapability, the UE includes in UE-NR-Capability an indication for a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs the UE can monitor for downlink cells with monitoringCapabilityConfig = r15monitoringcapability, or for downlink cells with monitoringCapabilityConfig = r16monitoringcapability, or for downlink cells with monitoringCapabilityConfig = r17monitoringcapability when the UE is configured for carrier aggregation operation over more than three downlink cells with at least one downlink cell with monitoringCapabilityConfig = r15monitoringcapability, at least one downlink cell with monitoringCapabilityConfig = r16monitoringcapability and at least one downlink cell with monitoringCapabilityConfig = r17monitoringcapability. When a UE is not configured for NR-DC operation, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per slot or per span or per group of Xs slots that corresponds to Ncells,r15/{r16,r17}cap-r17 downlink cells or to Ncells,r16/{r15,r17}cap-r17 downlink cells or to Ncells,r17/{r15,r16}cap-r17 downlink cells, respectively, where - Ncells,r15/{r16,r17}cap-r17 is the number of configured downlink cells if the UE does not provide pdcch-BlindDetectionCA1 - otherwise, - if the UE reports only one combination of (pdcch-BlindDetectionCA1, pdcch-BlindDetectionCA2, pdcch-BlindDetectionCA3), Ncells,r15/{r16,r17}cap-r17 is the value of pdcch-BlindDetectionCA1 - else, Ncells,r15/{r16,r17}cap-r17 is the value of pdcch-BlindDetectionCA1 from a combination of (pdcch-BlindDetectionCA1, pdcch-BlindDetectionCA2, pdcch-BlindDetectionCA3) that is provided by pdcch-BlindDetectionCA-CombIndicator-r17 - Ncells,r16/{r15,r17}cap-r17 is the number of configured downlink cells if the UE does not provide pdcch-BlindDetectionCA2 - otherwise, - if the UE reports only one combination of (pdcch-BlindDetectionCA1, pdcch-BlindDetectionCA2, pdcch-BlindDetectionCA3), Ncells,r16/{r15,r17}cap-r17 is the value of pdcch-BlindDetectionCA2 - else, Ncells,r16/{r15,r17}cap-r17 is the value of pdcch-BlindDetectionCA2 from a combination of (pdcch-BlindDetectionCA1, pdcch-BlindDetectionCA2, pdcch-BlindDetectionCA3) that is provided by pdcch-BlindDetectionCA-CombIndicator-r17 *** Unchanged text omitted ***

38.213 TP according to [10]:

# 10 UE procedure for receiving control information *** Unchanged text omitted *** When a UE is configured for NR-DC operation with a total of NNR-DCDL,cells downlink cells on both the MCG and the SCG and the UE is provided monitoringCapabilityConfig = r17monitoringcapability for all downlink cells where the UE monitors PDCCH, the UE expects to be provided pdcch-BlindDetection4 for the MCG and pdcch-BlindDetection4 for the SCG with values that satisfy - pdcch-BlindDetection4 for the MCG + pdcch-BlindDetection4 for the SCG <= pdcch-BlindDetectionCA3, if the UE reports pdcch-BlindDetectionCA3, or - pdcch-BlindDetection4 for the MCG + pdcch-BlindDetection4 for the SCG <= NNR-DCDL,cells, if the UE does not report pdcch-BlindDetectionCA3 When a UE is configured for NR-DC operation and the UE is provided monitoringCapabilityConfig = r17monitoringcapability for all downlink cells where the UE monitors PDCCH, the UE may indicate, through pdcch-BlindDetectionMCG-UE3 and pdcch-BlindDetectionSCG-UE3, respective maximum values for pdcch-BlindDetection for the MCG and pdcch-BlindDetection for the SCG. If the UE reports pdcch-BlindDetectionCA3, - the value range of pdcch-BlindDetectionMCG-UE3 or of pdcch-BlindDetectionSCG-UE3 is [1, …, pdcch-BlindDetectionCA3-1], and - pdcch-BlindDetectionMCG-UE3 + pdcch-BlindDetectionSCG-UE3 >= pdcch-BlindDetectionCA3. Otherwise, if NNR-DC, max, r17DL,cells is a maximum total number of downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability and the UE is configured on both the MCG and the SCG for NR-DC as indicated in UE-NR-Capability - the value range of pdcch-BlindDetectionMCG-UE3 or of pdcch-BlindDetectionSCG-UE3 is [1, 2, 3], and - pdcch-BlindDetectionMCG-UE3 + pdcch-BlindDetectionSCG-UE3 >= NNR-DC, max, r17DL,cells. *** Unchanged text omitted ***

Document [4] proposes to expect the same Xs value across multiple carriers in multi-slot based PDCCH monitoring.

38.213 TP according to [4]:

10 UE procedure for receiving control information =============================== Unchanged Text Omitted =================================== For μ=6, if the UE indicates a capability to monitor PDCCH according to multiple combinations Xs,Ys and a configuration of search space sets to the UE for PDCCH monitoring on a serving cell results to a separation of every two consecutive groups of Ys slots that is not smaller than Xs for more than one combinations Xs,Ys, of the multiple combinations Xs,Ys, the UE monitors PDCCH on the cell according to the combination Xs,Ys, from the more than one combinations Xs,Ys, that is associated with the largest maximum number of MPDCCHmax,Xs,μ and CPDCCHmax,Xs,μ defined in Table 10.1-2B and Table 10.1-3B. The UE expects to monitor PDCCH on the cells according to the same Xs, if the UE is configured for carrier aggregation operation or NR-DC operation. =============================== Unchanged Text Omitted ===================================

Document [5] proposes to resolve minimum CA capability values:

  - Case 4: Capability on the number of CCs with Rel-17 monitoring capability only

    - Range of pdcch-BlindDetectionCA-R17: {[2 or 4], …, 16}

  - Case 5: Capability on the number of CCs with Rel-15 monitoring capability and Rel-17 monitoring capability on different serving cells

      - Range of pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R17: {[3 or 4], …, 16}

  - Case 6: Capability on the number of CCs with Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells

      - Range of pdcch-BlindDetectionCA-R16 + pdcch-BlindDetectionCA-R17: {[2 or 3], …, 16}

  - Case 7: Capability on the number of CCs with Rel-15 monitoring capability, Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells

      - Range of pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16 + pdcch-BlindDetectionCA-R17: {[3 or 4], …, 16}

## Issue 8.2.2-6: SSSG switching with multiple cells and different Xs

Documents [7] 8] suggests naming alignment (please refer to individual documents for details).

38.213 TP according to [7]:

## 10.4 Search space set group switching and skipping of PDCCH monitoring *** Unchanged text is omitted *** A UE determines a slot and a symbol in the slot to start or stop PDCCH monitoring according to search space sets for a serving cell that the UE is provided searchSpaceGroupIdList or, if cellGroupsForSwitchList is provided, for a set of serving cells, based on the largest Xs if the SCS configuration μ among all configured DL BWPs in the set of serving cells equals to 6, otherwise, based on the smallest SCS configuration μ among all configured DL BWPs in the serving cell or in the set of serving cells and, if any, in the serving cell where the UE receives a PDCCH and detects a corresponding DCI format 2_0 triggering the start or stop of PDCCH monitoring according to search space sets. *** Unchanged text is omitted ***

# References

Contributions related to PDCCH monitoring enhancements

[1]	R1-2206081	Draft CR on multi-slot PDCCH monitoring for TS 38.213	ZTE, Sanechips

[2]	R1-2206363	Corrections for PDCCH monitoring occasion for DCI format 2_1 for the features extending NR operation to 71 GHz	CATT

[3]	R1-2206732	Correction on multi-slot PDCCH monitoring in CA scenario with mixed capability types	vivo

[4]	R1-2206791	Draft CR for multi-slot PDCCH monitoring in FR2-2	Samsung

[5]	R1-2207021	On UE capability parameters for CA with per-slot group monitoring	Nokia, Nokia Shanghai Bell

[6]	R1-2207023	Discussion on default value of duration-r17 in FR2-2	LG Electronics

[7]	R1-2207024	Draft CR for SSSG switching with multiple cells in FR2-2	LG Electronics

[8]	R1-2207025	Discussion on SSSG switching with multiple cells in FR2-2	LG Electronics

[9]	R1-2207464	Draft CR on UE capability name alignment	Ericsson

[10]	R1-2207465	Discussion on UE capability name alignment	Ericsson

[11]	R1-2207466	Draft CR on Group 2 search space configuration	Ericsson

[12]	R1-2207467	Discussion on Group 2 search space configuration	Ericsson

[13]	R1-2207519	Corrections on PDCCH monitoring enhancement for 52-71GHz spectrum	Huawei, HiSilicon