3GPP TSG RAN meeting #97-e									RP-22xxxx

Electronic Meeting, September 12-16, 2022

## Status Report to TSG

Agenda item:			9.3.1.1

| WI / SI Name | Enhancement of NR Dynamic spectrum sharing (DSS) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_DSS_enh |  |  |  |  |
| Unique ID | 940093 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-221622 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: mm/yyyy | Core part: 12/2023 | Performance part: 06/2024 | Testing part: mm/yyyy |  |
| Overall Completion level | Study Item: xx % | Core part: 85% | Performance Part: 0% | Testing part: xx% |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Ravikiran Nory |
|  | Company | Ericsson |
|  | Email | ravikiran.nory@ericsson.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

##### NR PDCCH reception in symbols with LTE CRS REs

###### RAN1#109-e (May 2022)

Agreement

To evaluate the following options:

  - Option-1-1: No NR-PDCCH-DMRS is transmitted for only the REs overlapping with LTE-CRS of the OFDM symbol, NR-PDCCH is punctured on REs colliding with LTE-CRS, NR-PDCCH must span at least 2 consecutive symbols with at least 1 symbol not overlapping with LTE-CRS

  - Option-1-2: No NR-PDCCH-DMRS is transmitted in any RE of the OFDM symbol, NR-PDCCH is transmitted on REs not colliding with LTE-CRS including the original DMRS, NR-PDCCH is punctured on REs colliding with LTE-CRS, NR-PDCCH must span at least 2 consecutive symbols with at least 1 symbol not overlapping with LTE-CRS

  - Option-2: NR-PDCCH or NR-PDCCH-DMRS is transmitted on REs not colliding with LTE-CRS, NR-PDCCH and NR-PDCCH-DMRS may or may not be punctured on REs colliding with LTE-CRS

    - No puncture is baseline (UE side)

Observation

For evaluations consider the following options:

Option 1-1:

- PDCCH and PDCCH DMRS mapping to REs: Legacy

- PDCCH REs overlapping with LTE CRS: Receiver punctures

- PDCCH DMRS REs overlapping with LTE CRS: All DMRS REs on overlapping symbol Not used for CE, or legacy pattern is assumed

- gNB transmits: Irrelevant what the gNB transmits on REs overlapping with the LTE CRS REs as indicated in the CRS RM pattern.

- Channel estimator: operate on clean symbol DMRS only, Legacy

Option 1-2:

- PDCCH and PDCCH DMRS mapping to REs: New PDCCH rate-matching

  - No PDCCH DMRS on the symbol overlapping with LTE CRS

- PDCCH REs overlapping with LTE CRS: Receiver punctures

- PDCCH DMRS REs overlapping with LTE CRS: Not expected

- Channel estimator (UE assumption): Operate on clean symbol DMRS only

- gNB transmits: Irrelevant what the gNB transmits on REs overlapping with the LTE CRS REs as indicated in the CRS RM pattern

Option 2:

- PDCCH and PDCCH DMRS mapping to REs: Legacy

- PDCCH REs overlapping with LTE CRS: Baseline: Process as legacy

- PDCCH DMRS REs overlapping with LTE CRS: Aware or unaware

- Channel estimator: Baseline: Process as legacy (Receiver does not puncture DMRS), Optional: Advanced receiver (Use the DMRS other than legacy behavior)

- gNB transmits:

  - Baseline: May puncture the PDCCH/PDCCH DMRS, or may superposition the two.

  - Optional: may puncture LTE CRS of Port#2&3.

  - Impact to LTE UEs should be considered if superposition is used.

Agreement

For evaluations consider the following list of scenarios:

Scenario#1A: 1 symbol CORESET, overlapped with CRS – Option 2 only

Scenario#2: 2 symbols CORESET, including 1 overlapping symbol and 1 clean symbol – Option 1-1/1-2/2

Scenario#3: 3 symbols CORESET, including 1 overlapping symbol and 2 clean symbols – Option 1-1/1-2/2

Agreement

LLS simulations assumptions, [] are optional:

| Parameters | Values |
| --- | --- |
| Carrier frequency | 2 GHz |
| SCS | 15 kHz |
| Bandwidth | 20 MHz [5, 10 MHz], LTE bandwidth equal or smaller than NR |
| Channel model | TDL-C 300, [TDL-A 300] |
| Correlation | Low |
| Number of BS antennas | 4 Tx, (M,N,P,Mg,Ng;Mp,Np)= (1,2,2,1,1;1,1), [2 Tx, (M,N,P,Mg,Ng;Mp,Np)= (1,1,2,1,1;1,1).] |
| Number of UE antennas | 2 Rx (M,N,P,Mg,Ng;Mp,Np)= (1,1,2,1,1;1,1) |
| DCI payload (excluding CRC) | 60 bits [50bits] |
| Interleaving | Non-Interleaved, [Interleaved] |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 PRBs |
| CRS | single 4 port CRS pattern, [additional 4 port CRS pattern] |
| Channel estimation | practical – companies to report details |
| UE speed | 30 kmph [3kmph, 120 kmph, 350 kmph] |
| Power ratio of LTE-CRS RE/NR PDCCH RE, Power ratio of LTE-CRS RE/NR PDCCH-DMRS RE | Companies to report (if applicable) |

Agreement

SLS simulations assumptions, [] are optional:

| Parameters | Values |
| --- | --- |
| Carrier frequency | 2.1 GHz |
| SCS | 15 kHz |
| Simulation bandwidth | 20 MHz [5, 10 MHz]，same as that in LLS |
| BS antenna height | 25 m |
| UE height | 1.5m |
| TRP transmit power | 49 dBm 20 MHz |
| Scenario | Urban Macro (500m ISD), [Rma (1732m ISD)] |
| Device deployment | 80% indoor, 20% outdoor (Uma) [50% indoor,50% in-car (Rma)] |
| UE speeds | Indoor users: 3km/h |
|  | Outdoor users (in-car): 30 km/h |
| BS noise figure | 5 dB |
| BS antenna element gain | 8 dBi |
| UE noise figure | 9 dB |
| Thermal noise level | -174 dBm/Hz |
| Traffic geometry | Full Buffer |
| Macro sites | 19 |
| Downtilt | 102° or according to Scenario |
| Minimum BS to UE distance | 35m |
| KPI | Companies to report (e.g., total PDCCH capacity, PDCCH coverage/outage, Potential degradation of LTE, whether and how to achieve coexistence with legacy UEs) |
| Others | Companies to report (e.g., fraction of LTE UEs, fraction of Rel-18 DSS NR UEs, etc.). Companies to report considered baseline(s). Baseline(s) aim(s) to be comparable to the evaluated option(s) |

###### RAN1#110 (August 2022)

Agreement

If the feature “Reception of NR PDCCH candidates that overlap with LTE CRS REs “is supported, it is RAN1 understanding that the feature is supported by UE performing channel estimation with a regular legacy DMRS pattern in frequency dimension, i.e., no change to UE assumption on PDCCH DMRS RE positions/pattern in a symbol that are used for the purpose of channel estimation.

Agreement

Reception of NR PDCCH candidates that overlap with LTE CRS REs is supported by Rel18 UEs

PDCCH candidates and PDCCH-DMRS RE mapping are based on that of R15 from UE side.

Note: depends on UE capability

Following options can be used for PDCCH-DMRS channel estimation

- legacy CE assumption

  - RAN1 consider support this, if no RAN4 performance requirements are defined

- CE on clean symbol(s) only (this channel estimation option does not apply for 1 symbol CORESET)

Note: Restriction on the symbols and/or LTE CRS patterns applicable for above agreements can be considered during UE capability session.

Conclusion

RAN1 understands above agreements applies to 15kHz SCS only.

##### UE support for two overlapping CRS rate matching patterns

###### RAN1#109-e (May 2022)

Working Assumption

Introduce two new RRC parameters (i.e., list3 and list4). gNB cannot configure legacy list1/2 and new list3/4 simultaneously. If new list3/4 are configured, UE applies list3/4 regardless of whether CORESETPoolIndex= 1 is configured for any of DL BWPs of the serving cell.

Working Assumption

  - Introduce two new RRC parameters lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig around which the UE shall do rate matching for PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS.

    - The network does not configure lte-CRS-PatternList3-r18 and any of lte-CRS-ToMatchAround, lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 simultaneously. Lte-CRS-PatternList4-r18 is configured if lte-CRS-PatternList3-r18 is configured in ServingCellConfig.

    - For case when UE is not configured with two CORESET pools two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied

    - For case when UE is configured with two CORESET pools two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18

      - If UE is configured with crs-RateMatch-PerCoresetPoolIndex, lte-CRS-PatternList3-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘0’, and lte-CRS-PatternList4-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘1’

      - Otherwise, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied.

    - The legacy configuration rule in TS 38.331 is applied in Rel-18 DSS, i.e.,

      - “The first LTE CRS pattern in lte-CRS-PatternList4 shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList3, The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList31, and so on.”

Agreement

- Introduce new UE capability(ies) for support of two overlapping LTE CRS patterns in Rel-18 DSS if the UE is NOT configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet.

  - NW can configure two LTE-CRS overlapping rate matching patterns without coresetPoolIndex only if the UE indicates support of the new capability(ies).

- Clarify that the Rel-16 UE capability overlapRateMatchingEUTRA-CRS-r16 is subject to support of multiDCI-Multi-TRP-r16.

- Maximum number of LTE-CRS rate matching patterns for PDSCH supported by a UE (i.e., maxNumberPatterns-r16 and maxNumberNon-OverlapPatterns-r16) is kept unchanged.

###### RAN1#110 (August 2022)

Agreement

  - Introduce two new RRC parameters lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig around which the UE shall do rate matching for PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS.

    - The network does not configure lte-CRS-PatternList3-r18 and any of lte-CRS-ToMatchAround, lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 simultaneously. Lte-CRS-PatternList4-r18 is configured only if lte-CRS-PatternList3-r18 is configured in ServingCellConfig.

    - For case when UE is not configured with two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied

    - For case when UE is configured with two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18

      - If UE is configured with crs-RateMatch-PerCoresetPoolIndex, lte-CRS-PatternList3-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘0’, and lte-CRS-PatternList4-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘1’

      - Otherwise, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied.

    - The legacy configuration rule in TS 38.331 is applied in Rel-18 DSS, i.e.,

      - “The first LTE CRS pattern in lte-CRS-PatternList4 shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList3, The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList3, and so on.”

The working Assumption made in RAN1#109e does not need to be confirmed.

Working Assumption(RAN1#109e)

  - Introduce two new RRC parameters lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig around which the UE shall do rate matching for PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS.

    - The network does not configure lte-CRS-PatternList3-r18 and any of lte-CRS-ToMatchAround, lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 simultaneously. Lte-CRS-PatternList4-r18 is configured if lte-CRS-PatternList3-r18 is configured in ServingCellConfig.

    - For case when UE is not configured with two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied

    - For case when UE is configured with two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18

      - If UE is configured with crs-RateMatch-PerCoresetPoolIndex, lte-CRS-PatternList3-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘0’, and lte-CRS-PatternList4-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘1’

      - Otherwise, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied.

    - The legacy configuration rule in TS 38.331 is applied in Rel-18 DSS, i.e.,

      - “The first LTE CRS pattern in lte-CRS-PatternList4 shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList3, The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList3, and so on.”

Agreement

lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applicable to 15 kHz SCS PDSCH only.

Agreement

The following TP for sub-clause 5.1.4.2 of TS 38.214 h20 is endorsed in principle.

## 5.1.4.2 PDSCH resource mapping with RE level granularity <omitted text> A UE may be configured with any of the following higher layer parameters: - REs indicated by the 'RateMatchPatternLTE-CRS' in lte-CRS-ToMatchAround in ServingCellConfig or ServingCellConfigCommon configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH. - REs indicated by 'RateMatchPatternLTE-CRS' in lte-CRS-PatternList1-r16 or lte-CRS-PatternList3-r18 in ServingCellConfig configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH. - For the UE for broadcast reception, REs indicated by 'RateMatchPatternLTE-CRS' in PDSCH-Config-MCCH or PDSCH-Config-MCCH configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH. - Each RateMatchPatternLTE-CRS configuration contains v-Shift consisting of LTE-CRS-vshift(s), nrofCRS-Ports consisting of LTE-CRS antenna ports 1, 2 or 4 ports, carrierFreqDL representing the offset in units of 15 kHz subcarriers from (reference) point A to the LTE carrier centre subcarrier location, carrierBandwidthDL representing the LTE carrier bandwidth, and may also configure mbsfn-SubframeConfigList representing MBSFN subframe configuration. A UE determines the CRS position within the slot according to Clause 6.10.1.2 in [15, TS 36.211], where slot corresponds to LTE subframe. - If the UE is configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet and is also configured by the higher layer parameter lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 in ServingCellConfig, the following REs are declared as not available for PDSCH: - if the UE is configured with crs-RateMatch-PerCoresetPoolIndex, REs indicated by the CRS pattern(s) in lte-CRS-PatternList1-r16 if the PDSCH is associated with coresetPoolIndex set to ‘0’, or the CRS pattern(s) in lte-CRS-PatternList2-r16 if PDSCH is associated with coresetPoolIndex set to ‘1’; - otherwise, REs indicated by lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16, in ServingCellConfig. - If the UE is not configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet, and if the UE is configured by higher layer parameter lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig, REs indicated by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are declared as not available for PDSCH. - If the UE is configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet and is also configured by the higher layer parameter lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig, the following REs are declared as not available for PDSCH: - if the UE is configured with crs-RateMatch-PerCoresetPoolIndex, REs indicated by the CRS pattern(s) in lte-CRS-PatternList3-r18 if the PDSCH is associated with coresetPoolIndex set to ‘0’, or the CRS pattern(s) in lte-CRS-PatternList4-r18 if PDSCH is associated with coresetPoolIndex set to ‘1’; - otherwise, REs indicated by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18, in ServingCellConfig. <omitted text>

Agreement

- Agree the following RRC parameters for support of two overlapping LTE-CRS patterns.

- Send an LS to RAN2 about the agreed RRC parameters together with all RAN1 agreements in AI 9.9.2.

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_DSS_enh | Support for two overlapping CRS rate matching patterns | lte-CRS-PatternList3 | New | A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH. The LTE CRS patterns in this list shall be non-overlapping in frequency. This list can be configured if [Rel-18 DSS capability(ies)] specified in TS 38.306 [26] is indicated. The network does not configure this field and lte-CRS-ToMatchAround, or this field and lte-CRS-PatternList1, or this field and lte-CRS-PatternList2 simultaneously. | LTE-CRS-PatternList-r16 | ServingCellConfig | UE-specific | 38.331 |
| NR_DSS_enh | Support for two overlapping CRS rate matching patterns | lte-CRS-PatternList4 | New | A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH. The LTE CRS patterns in this list shall be non-overlapping in frequency. This list can be configured if [Rel-18 DSS capability(ies)] specified in TS 38.306 [26] is indicated. The first LTE CRS pattern in this list shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList3. The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList3, and so on. Network configures this field only if the field lte-CRS-ToMatchAround is not configured and the field lte-CRS-PatternList3 is configured. | LTE-CRS-PatternList-r16 | ServingCellConfig | UE-specific | 38.331 |

Agreement

The draft LS R1-2208193 is endorsed in principle by replacing

- Clarify that the Rel-16 UE capability overlapRateMatchingEUTRA-CRS-r16 is subject to support of multiDCI-Multi-TRP-r16.

With

- Clarify that the Rel-16 UE capability overlapRateMatchingEUTRA-CRS-r16 is subject to support of multiDCI-Multi-TRP-r16 in Rel-18 ASN.1.

Agreement

Final LS R1-2208194 is endorsed.

#### 2.1.2	Remaining Open issues

- Finalize RAN1 specification updates

  - to introduce support for reception of NR PDCCH candidates that overlap with LTE CRS REs

  - to introduce support for two overlapping CRS rate matching patterns

- Finalize UE capabilities

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

- Finalize specification of RRC signaling (if any) including UE capability signaling

  - to introduce support for reception of NR PDCCH candidates that overlap with LTE CRS REs

  - to introduce support for two overlapping CRS rate matching patterns

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

RAN4 work is scheduled to start in Q4, 2023 according to agreed time plan in [2],[3].

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

- Specify necessary UE demodulation performance requirements for the objectives in the Core part of the WI.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

#### General

[1] RP-221622 – “Revised WID on Enhancement of NR Dynamic spectrum sharing (DSS)”, Ericsson, RAN#96, Budapest, Jun 2022.

[2] RP-213469 – “Summary for RAN Rel-18 Package”, RAN Chair (Qualcomm), RAN1 Chair (Samsung), RAN2 Chair (MediaTek), RAN3 Chair (ZTE), RAN4 Chair (Huawei), RAN#94e, Dec 2021.

[3] RP-213697 – “Overview of time unit allocation for all RAN REL-18 projects”,	RAN1 Chair (Samsung), RAN2 Chair (MediaTek), RAN3 Chair (ZTE), RAN4 Chair (Huawei), RAN#94e, Dec 2021.

#### RAN1#109-e (May 2022)

45 contributions (for details, please see A.I. 9.9, 9.9.1, 9.9.2 and 9.9.3 in RAN1#109-e Tdoc list - link)

#### RAN1#110 (August 2022)

40 contributions (for details, please see A.I. 9.9, 9.9.1, 9.9.2 in RAN1#109-e Tdoc list - link)

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template