3GPP TSG RAN WG1 Meeting #110  	          	     		  R1-2205851

Toulouse, France, August 22nd – 26th, 2022

Agenda Item:	9.4.1.2

Source: 	LG Electronics

Title: 	Discussion on physical channel design framework for sidelink on unlicensed spectrum

Document for:	Discussion and decision

- Introduction

In RAN1#109-e meeting [1], followings are agreed for channel access mechanism for SL-U:

Agreement SL BWP, SL resource pool in R16/R17 NR SL and RB set in R16 NR-U are reused for SL-U as baseline - Only one SL BWP is (pre-)configured within a carrier - The SL BWP is (pre-)configured to include one or multiple SL resource pools - At least support that one SL resource pool can be (pre-)configured to include integer number of RB sets - FFS: whether/how to support one SL resource pool can include sub-set of PRBs of one RB set - FFS: the applicable resource pool - FFS: the impact on sub-channel size and number of sub-channels in a resource pool if sub-channel is supported - PRBs within intra-cell guard band of two adjacent RB sets belong to a resource pool if the resource pool includes the two adjacent RB sets - FFS details, e.g., how such PRBs are used, the applicable resource pool, etc. - FFS: whether R16/R17 NR SL S-SSB slots and/or new S-SSB slots (if supported) are excluded from resource pool - FFS: which slots belong to resource pool, e.g., how to set the value of bitmap, whether to consider SL-U/NR-U operating in the same carrier and whether TDD configuration are considered, etc. - FFS: the impact of PSCCH/PSSCH mapping to frequency resources on resource pool configuration, on sub-channel definition if sub-channel is supported, etc. Agreement For PSCCH and PSSCH in SL-U: - Both R16/R17 NR SL contiguous RB-based and R16 NR-U interlace RB-based transmissions are considered as starting point - RAN1 strives to have unified design for both contiguous RB-based and interlace RB-based transmissions - FFS: whether/how to address IBE (In Band Emission) impact Agreement For PSCCH and PSSCH in SL-U: - For interlace RB-based transmission (if supported), at least the following candidates can be discussed: - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission - FFS: Other resource allocation granularity, e.g., RB-level - 1 sub-channel equals K interlaces if sub-channel is supported - FFS details - Other candidates are not precluded - FFS: mapping of PSCCH to frequency resources - FFS: resource indication in time/frequency domain, e.g., how to handle using one RB set or multiple RB sets, etc. Agreement For slot structure in SL-U: - At least R16/R17 NR SL slot-based PSCCH/PSSCH transmission is supported - FFS: whether/how to support additional starting symbol(s) within a slot for the PSCCH/PSSCH transmission Agreement For PSFCH and SL-HARQ in SL-U: - At least R16 NR SL PSFCH format 0 is supported - FFS whether to introduce new PSFCH format - FFS: how to meet OCB and PSD requirement for PSFCH transmission, e.g., using interlaced RB transmission, whether/how to avoid too small PSFCH capacity, etc. - FFS: the locations of PSFCH resources, e.g., (pre-)configured, dynamically indicated, etc. - FFS: whether/how to address PSFCH transmission dropping due to LBT failure, e.g., whether to have multiple PSFCH occasions for a PSSCH and the related PSSCH-PSFCH mapping relationship, impact on SL HARQ-ACK reporting to the gNB for Mode 1, etc. - FFS: whether/how to address PSFCH and related PSSCH in different COTs Agreement For S-SSB and synchronization in SL-U: - FFS the time domain locations of S-SSB resources, e.g., whether/how to introduce more candidate occasions compared with R16/R17 NR SL design, etc. - Down-selection at least one of the following solutions to meet OCB and PSD requirement for S-SSB transmission - Option 1: Using interlaced RB transmission - Option 2: S-SSB multiplexing with other SL transmissions in the same slot - Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain - Option 4: S-PSS/S-SSS/PSBCH with wider bandwidth - FFS: whether to support 4 symbols S-SSB - Note: 4 symbols S-SSB can be considered with options 1/2/3/4 above - FFS whether the temporary exemption of OCB requirement is applicable for S-SSB transmission - FFS whether any changes to R16/R17 NR SL synchronization procedure

In this contribution, we discuss issues on the physical channel design for NR sidelink transmission on unlicensed band.

- Discussion

  - SL BWP and SL resource pool configuration

In NR-U, BWP is configured to be aligned with RB set(s) in boundaries. Moreover, according to the agreement made in RAN1#109-e meeting [1], a SL resource pool can be (pre)configured to include integer number of RB sets. In those of points of view, it would be necessary that the SL BWP is also (pre)configured to be aligned with RB set(s) in boundaries.

Considering that the BWP in NR-U is always aligned with RB set(s) in boundaries, it is unclear benefit of supporting that a SL resource pool includes sub-set of PRBs of one RB set. Moreover, considering that the granularity of the channel sensing operation is a RB set, it would be better in terms of resource utilization not to restrict available resource into a certain subset of resources in the RB set. In Rel-16, since the granularity of the number of frequency resources belonging to a SL resource pool is a PRB, there is no problem to align the boundary of the SL resource pool with the boundary of RB set(s).

Depending on the regulation, the occupied bandwidth would need to be larger than 80% over the total bandwidth. Moreover, power spectral density for any 1MHz could be limited. In NR-U, to meet these regulations, interlaced RB-based transmission is supported. In RAN1#109-e meeting [1], it is agreed that at least for PSCCH and PSSCH, both NR SL contiguous RB-based and interlaced RB-based transmissions are considered. In this case, it is necessary to decide the granularity of (pre)configuration to enable or disable the interlaced RB-based transmission. In NR-U, the interlaced RB-based transmission is a part of UE capability, and enabling/disabling interlaced RB-based transmission is cell-specific. Meanwhile, if the interlaced RB-based transmission is (pre)configured per SL BWP or SL carrier, then UE(s) without the interlaced RB-based TX capability would not be supported on the SL carrier. In our view, since S-SSB transmission still be a form of interlaced structure, it would be unclear the benefit of allowing the coexistence between contiguous RB-based transmission and interlaced RB-based transmission in a SL carrier. To be specific, even if the contiguous RB-based transmission is allowed in a SL carrier with OCB requirement, the UEs without interlaced RB-based transmission capability cannot transmit S-SSB transmission.

Observation 1: If S-SSB transmission is a form of interlace structure, the benefit of the coexistence between contiguous RB-based transmission and interlaced RB-based transmission in a SL carrier is unclear.

Proposal 1: For SL BWP configuration on shared spectrum,

- Starting PRB of SL BWP is aligned with the lowest PRB of the lowest RB set within SL BWP

- Ending PRB of SL BWP is aligned with the highest PRB of the highest RB set within SL BWP

- Either contiguous RB-based transmission or interlaced RB-based transmission is (pre)configured

Proposal 2: For SL resource pool configuration in frequency domain on shared spectrum,

- Starting PRB (i.e., sl-StartRB-Subchannel) is aligned with the lowest PRB of the lowest RB set within the resource pool.

- The number of PRBs (i.e., sl-RB-Number) is set so that the ending PRB is aligned with the highest PRB of the highest RB set within the resource pool.

In case of SL communication on licensed spectrum, for TDD, cell-specific UL slots can be belonging to a resource pool. For flexible slots, if the SL symbol group in a slot is set to cell-specific UL, then the flexible slots can be belonging to a resource pool as well. For simplicity, even for unlicensed spectrum operation, the same principle could be adopted for candidate slots that can be belonging to a resource pool. If it is allowed that other flexible slots or DL slots to belong to a resource pool, it would be further discuss how to define prioritization rule between DL reception (e.g., DL discovery burst, PDCCH monitoring, CSI measurement) and SL transmission/reception. If the UE is configured with tdd-UL-DL-ConfigurationCommon for the unlicensed carrier, it seems straightforward to use it as in Rel-16/17 NR SL.

Proposal 3: For SL communication on shared spectrum, a resource pool excludes following slots:

- Slots whose symbols from sl-StartSymbol to sl-StartSymbol + sl-LengthSymbols – 1 are not semi-statically configured as UL as per the higher layer parameter tdd-UL-DL-ConfigurationCommon of the serving cell if provided or sl-TDD-Configuration if provided or sl-TDD-Config of the received PSBCH if provided

- S-SSB slots

- Reserved slot as specified in section 8 of TS 38.214

    - Contiguous RB-based transmission

According to NR-U, the granularity of channel sensing operation is RB set, and the size of the RB set is between 100 and 110 for 15kHz SCS or between 50 and 55 except for at most one RB set which may contain 56 RBs for 30kHz SCS. Moreover, depending on the carrier, there could exist guard-band between two adjacent RB sets. If UE accesses two RB sets, then the UE can transmit UL channel/signal on resources belonging to guard bands as well to ensure contiguous UL transmission in frequency domain.

For contiguous RB-based PSCCH/PSSCH transmission, since the granularity of PSCCH/PSSCH transmission is sub-channel, it would be necessary to investigate the relationship between RB sets and sub-channels. If Rel-16 NR SL sub-channelization is applied to the multiple RB sets, sub-channels would not be aligned with RB sets in boundaries. In this case, it would be possible that a subset of PRBs belonging to a sub-channel is outside the RB set, and these PRBs would not be used for PSCCH/PSSCH transmissions. For instance, in Figure 1, only two sub-channels are fully confined within a RB set#1, and the parts of the other sub-channels are outside the RB set. In this case, when UE transmits PSCCH/PSSCH on RB set#1, then the only two sub-channels can be used. In this case, a number of resources within the RB set cannot be utilized.

Observation 2: If sub-channels are not aligned with RB sets in boundaries, a number of resources would not be utilized especially when small number of RB set(s) are allocated for PSCCH/PSSCH transmission.

Figure 1: Examples of sub-channelization for contiguous RB-based PSCCH/PSSCH transmission.

Alternatively, sub-channels could be defined to be aligned with RB set(s) in boundaries. To be specific, for each RB set belonging to a SL resource pool, sub-channels can be allocated to be fully confined within the RB sets as shown in Figure 2. Meanwhile, the remaining PRBs after allocating the sub-channel of each RB set and PRBs belonging to a guard band can be automatically used for PSCCH/PSSCH transmission only if the adjacent RB sets are allocated for PSCCH/PSSCH transmission. Moreover, to ensure enabling the same TB size regardless of whether or not to use these remaining PRBs, it can be considered that the remaining PRBs are not counted for TBS determination.

Figure 2: Examples of sub-channelization for contiguous RB-based PSCCH/PSSCH transmission.

Proposal 4: For contiguous RB-based transmission,

- Sub-channels are defined to be aligned with RB set(s) in boundaries.

- PRBs between sub-channels belonging to different RB sets can be used automatically for PSCCH/PSSCH transmission when the sub-channels belonging to different RB sets are used for the PSCCH/PSSCH transmission.

- PRBs not belonging to a sub-channel is not counted for TBS determination.

    - Interlaced RB-based transmission

In RAN1#109-e meeting [1], it is agreed that at least for PSCCH and PSSCH, interlaced RB-based transmissions are considered. Since the granularity of PSCCH/PSSCH scheduling is a sub-channel, it would be necessary to discuss how to define sub-channel for the interlaced structure. Or, it would be necessary how to modify FRIV indication to indicate RB set and/or interlaced index for the current resource and the reserved resources.

First of all, sub-channel can consists of PRBs belonging to one or multiple interlaces within a RB set. In this case, it can be considered that sub-channel indexing is done in increasing order of first the interlace index, and then the RB set index as shown in Figure 3. In this case, the UE can use multiple interlaces within a RB set by using the existing contiguous sub-channel allocation indicator, and it would be beneficial in terms of channel availability. However, if the UE uses more than one RB sets for PSCCH/PSSCH transmission, it would not guarantee that the same set of interlaces are used. In this case, PAPR for PSCCH/PSSCH transmission over multiple RB sets would be increased further.

Observation 3: If a subchannel consists of PRBs corresponding to an interlace index within a RB set, and if the subchannel indexing is done in increasing order of first the interlace index, and then the RB set index, and if more than one RB sets are scheduled for PSSCH transmission, the same set of interlaces across different RB sets would not be guaranteed. It can cause high PAPR.

Figure 3: Examples of sub-channelization for interlaced RB-based PSCCH/PSSCH transmission.

Alternatively, sub-channel indexing can be done in increasing order of first the RB set index, and then the interlace index as shown in Figure 4. In this case, when the UE uses more than one RB sets for PSCCH/PSSCH transmission, the same set of interlaces could be guaranteed. However, in this approach, even though the number of sub-channels is small, multiple RB sets will be used, and it would not be good in terms of channel availability.

Observation 4: If a subchannel consists of PRBs corresponding to an interlace index within a RB set, and if the subchannel indexing is done in increasing order of first the RB set index, and then the interlace index, and if more than one interlaces are scheduled for PSSCH transmission, it may need to use multiple RB sets for PSSCH transmission. In this case, the UE needs to access both RB sets for a PSSCH transmission.

Figure 4: Examples of sub-channelization for interlaced RB-based PSCCH/PSSCH transmission.

Meanwhile, considering PSD requirement, it would be necessary to support the distributed mapping of PSSCH transmission as shown in Figure 4. For instance, when a UE uses sub-channel#0 and sub-channel#1 for a PSSCH transmission, according to PSD requirement, 10 dBm will be distributed into a PRB of subchannel#0 and a PRB of subchannel#1 in Figure 3 while 10 dBm will be allocated to both a PRB of subchannel#0 and a PRB of sub-channel#1. In the above example, the mapping of Figure 4 can use TX power twice more than the mapping of Figure 3.

Observation 5: Due to PSD requirement (i.e., 8 or 10 dBm/MHz), depending on the number of interlaces, it would be useful using multiple RB sets rather than using single RB set for PSSCH transmission for the same number of PRBs or interlaces.

In RAN1#109-e meeting [1], some discussion points for interlaced RB-based transmission were listed up. One of them is that 1 sub-channel equals K interlaces. However, in case of 30 kHz SCS, the total number of interlaces is 5, so if the value of K is 2 or 3, it would not be possible that the all the sub-channels consist of K interlaces. Instead, it might be needed to allow the case where different sub-channels have different number of interlaces. It would not be beneficial to ensure enabling the same TBS between initial transmission and retransmission with different set of allocated subhcannels.

Observation 6: For 30kHz SCS, since the number of interlaces is 5, the number of interlaces belonging to a subchannel can be different.

Observation 7: If contiguous sub-channel allocation indicator is reused for interlaced RB-based PSSCH transmission, depending on the sub-channelization method, the resource allocation could be inefficient in terms of channel accessibility, PAPR, and/or TX power restriction.

Since the sub-channelization without a separate indication of allocated RB set will lose scheduling flexibility, it would be better to consider indicating RB set separately from indicating interlaces or sub-channels. In this case, a single sub-channel can consists of PRBs belonging to a single or multiple interlaces across RB set(s). The final PRBs allocated for PSSCH transmission will be determined by intersection between the indicated sub-channel(s) and the indicated RB set(s) like NR-U resource allocation type 2. Since it would be necessary to consider how to indicate reserved resources in NR SL, FRIV can be used to indicate RB set(s) instead of RIV. In this approach, we can maximizes the scheduling flexibility and can guarantee the same set of interlaces across different RB sets when more than one RB sets are used for PSSCH transmission.

Proposal 5: For interlaced RB based PSSCH transmission, one of followings is supported:

- Option 1:

  - A subchannel consists of PRBs belonging to K interlaces within a RB set.

    - Subchannel indexing is done in increasing order of first the interlace index, and then the RB set index.

    - K is at least 1.

  - SCI indicates FRIV for subchannel allocation.

  - PSSCH transmission resource(s) are determined by the indicated sub-channel(s)

  - When more than one RB sets are used, UE expects that the same set of interlaces are used.

- Option 2:

  - A subchannel consists of PRBs belonging to K interlaces within a RB set.

    - Subchannel indexing is done in increasing order of first the RB set index, and then the interlace index.

    - K is at least 1.

  - SCI indicates FRIV for subchannel allocation.

  - PSSCH transmission resource(s) are determined by the indicated sub-channel(s)

- Option 3:

  - A subchannel consists of PRBs belonging to K interlaces.

    - Subchannel indexing is done in increasing order of the interlace index.

    - K is at least 1.

  - SCI indicates FRIV for subchannel allocation and FRIV for RB set allocation.

  - PSSCH transmission resource(s) are determined by the intersection of indicated sub-channel(s) and indicated RB set(s).

To save bit field size for FRIV indicating RB set(s), it can be further considered to combine Option 1 and Option 2. For instance, additional one bit indicator can be used to indicate the sub-channel mapping rule between option 1 and option 2.

Next, for PSCCH mapping, it can be considered that all or a subset of PRBs belonging to the lowest subchannel within the lowest RB set allocated for PSSCH transmission is used.

Proposal 6: PSCCH is mapped on the lowest subchannel within the lowest RB set allocated for PSSCH transmission.

  - Time domain resource assignment for PSCCH/PSSCH on shared spectrum

    - Additional starting symbol(s) within a slot

In RAN1#109-e meeting [1], it was discussed that the possibility of introducing additional starting symbol(s) within a slot for PSCCH/PSSCH transmission to handle LBT failure. One possible approach is to support sub-slot-based PSCCH/PSSCH transmission. In other words, two PSSCH transmissions with 7 or 6 symbol duration including AGC symbol and TX-RX switching symbol can be allowed in a slot for normal CP or extended CP, respectively. However, in this case, PSFCH resource(s) cannot be (pre)configured in the SL resource pool since PSSCH DMRS pattern is not defined for the PSSCH symbol duration including AGC symbol less than 5.

Observation 8: For sub-slot-based PSCCH/PSSCH transmission, if two PSSCH transmissions with 7-symbol duration including AGC symbol and TX-RX switching period are allowed in a slot (for normal CP case), PSFCH resource(s) cannot be allocated since the proper PSSCH DMRS pattern is not defined.

Another approach is to support multiple starting positions of slot-based PSCCH/PSSCH transmission. In this case, when UE fails to access channel before the earlier starting position, then the UE attempt to access the channel for the next starting position for a PSCCH/PSSCH transmission. When UE prepares the PSCCH/PSSCH transmission with later starting position after determining LBT failure for the earlier starting position, the processing time budget would not be sufficient. Alternatively, it can be considered that the UE prepares multiple OFDM waveforms for multiple starting positions in advance. However, in this case, the UE needs to have sufficient storage to store multiple waveforms with multiple starting positions. Even for this case, it is necessary to have sufficient processing time budget for switching proper waveforms. Moreover, it is necessary to check whether or how to ensure enabling the same TB size among PSSCH transmission candidates with different starting positions. To alleviate this problems, it can be considered that the UE punctures some earlier OFDM symbols after determining LBT failure, but this approach also needs to consider whether or how to avoid puncturing 1st SCI and/or 2nd SCI.

Observation 9: It is necessary to carefully investigate the feasibility on that UE prepares and performs the next PSSCH transmission with the next starting symbol after UE decide LBT failure for the 1st starting symbol in terms of processing time budget and UE complexity. Moreover, it is necessary to ensure the same TB size among PSSCH transmission(s) with different starting symbol(s).

Since PSCCH transmission will be confined within the PSSCH transmission, if more than one starting positions are allowed, the PSCCH transmission candidates would be also increased. In this case, the PSSCH RX UE needs to perform blind decoding for multiple starting positions. Moreover, the PSSCH RX UE will perform AGC procedure for each starting position. Since these additional AGC symbol would be not be used for decoding, the throughput performance would be also degraded.

Observation 10: If additional starting symbol(s) within a slot for slot-based PSCCH/PSSCH transmission is allowed, the PSSCH RX UE needs to perform AGC procedure for each starting symbol, and it will cause throughput degradation.

In those points of views, our preference is to deprioritize supporting additional starting symbol(s) within a slot for the PSCCH/PSSCH transmission in Rel-17 NR SL. It seems necessary to finalize the essential parts for SL-U operation first.

    - SL transmission burst structure

To minimize overhead for channel sensing operation, it can be considered to introduce SL burst transmission for the same TB and/or different TBs. According to TS 37.213 [2], for burst transmission, gNB or UE can skip LBT operation for consecutive DL or UL transmissions without gaps after the gNB or UE accesses the channel according to channel access procedure, respectively.

Meanwhile, according to SL slot structure, the last SL symbol is used for TX-RX switching period, and any SL channel/signals is not mapped on the last SL symbol. In this case, at least for SL burst transmission, it would be necessary how to reduce or remove TX-RX switching period in SL slot. One simply way is that whether or not to use the last SL symbol for SL transmission is implicitly or explicitly indicated by PSCCH/PSSCH TX UE. In this case, the TX UE will perform rate-matching for the last SL symbol as shown in Figure 5-(a). Another approach is to perform CP extension to use all or a subset of TX-RX switching symbol of the earlier SL transmission of the TX UE as shown in Figure 5-(b).

(a)

(b)

Figure 5: Example of SL burst transmission.

Proposal 7: For SL transmission burst, time gap between adjacent PSCCH/PSSCH transmissions is not greater than 16usec.

- To fill out all or a subset of TX-RX switching symbol, down-select one of followings:

  - Alt 1: Extended CP of the later SL transmission can occupy part of TX-RX switching symbol of the earlier SL transmission.

  - Alt 2: Parts of a SL transmission can occupy part of TX-RX switching symbol of the SL transmission.

Moreover, depending on the SL BWP (pre)configuration, it is possible that SL transmission resource presents in the middle of a slot, and this slot structure will be applied to all the SL slots. In this case, even though multiple SL channels/signals are mapped on consecutive slots, there time gap will exists between two consecutive SL transmissions. To support SL burst transmission, it would be necessary to fix the value of sl-StartSymbol to 0 and to fix the value of sl-LenthSymbols to 14 for normal CP or 12 for extended CP. Similarly, consecutive physical slots also need to be belonging to a resource pool for SL burst transmission.

In SL slot structure, PSFCH resource is TDMed with PSCCH/PSSCH resource within a slot, and the maximum PSFCH resource period is 4 logical slots. In this case, the maximum duration of SL burst transmission would be also limited to 4 slots. To extend the time duration of the SL burst transmission, one way is to increase the maximum PSFCH resource period. Alternatively, it can be considered that the TX UE transmit any signal in a PSFCH occasion to ensure the time gap between transmission(s) is no greater than 16usec.

Proposal 8: For SL transmission burst, down-select one or more of followings:

- Option 1: Maximum size of SL transmission burst is determined by PSFCH resource period.

- Option 2: PSSCH TX UE can transmit any signal in a PSFCH occasion to ensure the time gaps between transmissions within a SL transmission burst is no greater than 16usec.

  - SL HARQ procedure

In RAN1#109-e meeting [1], it was discussed that the possibility of introducing new PSFCH format and/or introducing multiple chances of PSFCH transmission in response of a PSSCH transmission.

Unlike Uu link, a TX UE can transmit PSSCH transmissions to different RX UEs, and then the TX UE can receive a number of SL HARQ-ACK feedbacks from different RX UEs. In this case, each RX UE will transmit one SL HARQ-ACK information to the same TX UE. In this case, rather than new PSFCH format with large payload size, the existing PSFCH format 0 is sufficient for the container of the SL HARQ-ACK feedback. Moreover, since new PSFCH format with large payload size would have no or small multiplexing capacity, the overall PSFCH resource overhead would be too large.

Observation 11: Since PSSCH TX UE can receive a number of SL HARQ-ACK feedbacks from different PSSCH RX UEs, R16 NR SL PSFCH format 0 is sufficient for the container of the SL HARQ-ACK feedback.

In Rel-16 PSSCH-to-PSFCH mapping, the implicit mapping rule is adopted rather than dynamic indication of PSFCH resources. It is motivated from the fact that if the dynamic indication is adopted, the PSSCH TX UE needs to perform sensing operation for PSFCH further. Since the SL HARQ-ACK enabling/disabling and SL-HARQ-ACK feedback option indicators are present in 2nd SCI, the TX UE would need to success to decode 2nd SCI of other UEs as well for sensing operation. If the TX UE does not perform such sensing operation for PSFCH, even though PSSCH resource collision does not occur, PSFCH resource can be collided.

Observation 12: If the locations of PSFCH resources are dynamically indicated, UE needs to perform sensing operation for PSFCH further. Otherwise, even though PSSCH resources are not overlapping each other, PSFCH resource collision can occur.

Regarding LBT failure handling for PSFCH transmission, it would be necessary to check the progress in channel access mechanism for PSFCH transmission. To be specific, if UE-to-UE COT sharing is applied to PSFCH transmission in response of the received PSSCH transmission, the PSFCH transmission dropping due to LBT failure would be alleviated.

Observation 13: If UE can share COT initiated by UE transmitting PSCCH/PSSCH for PSFCH in response of the PSCCH/PSSCH, the PSFCH TX dropping problem due to LBT failure would be mitigated.

If we consider multiple PSFCH transmission chances to handle PSFCH transmission dropping due to LBT failure, there could be two domains to be considered. To be specific, since the sensing results could be different across different RB sets, it can be considered that the UE attempts to access multiple RB sets to transmit a single PSFCH transmission. In this case, once the UE successes to access at least one RB set, then the UE can use the PSFCH resource within the RB set for PSFCH transmission. To avoid sensing operation for PSFCH, the implicit mapping rule can be used. For instance, PRB groups for PSFCH transmission is (pre)configured in each RB set, and Rel-16 PSSCH-to-PSFCH resource determination rule is applied to each RB set. Figure 6 shows an example of PSSCH-to-PSFCH mapping for each RB set. In this example, the PSSCH transmission with index n is associated with PSFCH resource with index n.

Figure 6: Example of PSSCH-to-PSFCH mapping for handling PSFCH TX dropping due to LBT failure.

Another domain is to increase the PSFCH occasions associated with the same PSSCH slot. To do this, it can be considered that UE is (pre)configured with more than one sl-MinTimeGapPSFCH and the difference between minimum timing values will be larger than or equal to the value of PSFCH resource period. In this case, for a given PSSCH slot, there could be more than one PSFCH occasions to be associated with the PSSCH slot. To avoid PSFCH resource collisions between different PSSCH-to-PSFCH timeline, PRB groups for PSFCH transmission is (pre)configured in each timeline, and Rel-16 PSSCH-to-PSFCH resource determination rule is applied to each timeline. Figure 7 shows an example of PSSCH-to-PSFCH mapping for each timeline. In this example, the PSSCH transmission with index n is associated with PSFCH resource with index n. This approach may not be suitable for the case when the PDB is limited. Alternatively, instead of multiple PSFCH slots, it can be considered to allow multiple PSFCH occasions in a slot. However, it would require the huge specification work for multiplexing PSCCH/PSSCH/PSFCH in a slot.

Figure 7: Example of PSSCH-to-PSFCH mapping for handling PSFCH TX dropping due to LBT failure.

In the above approaches, it requires additional frequency resources for PSFCH transmissions. Therefore, this approach would be applicable when a single PRB is used for PSFCH transmission. On the other hand, if a number of PRBs are used for PSFCH transmission as in interlaced RB-based PSFCH transmission, it would not be possible to reserve frequency domain resources for PSFCH transmission further for this purposes. In other words, mechanisms for handling PSFCH TX dropping due to LBT failure would not be applicable to all the type of PSFCH transmissions.

Proposal 9: For handling PSFCH format 0 transmission dropping due to LBT failure, followings can be discussed:

- Option 1: UE tries to transmit SL HARQ-ACK feedback on PSFCH resource in the next RB set.

  - R16 NR SL PSSCH-to-PSFCH resource determination rule is applied to each RB set.

  - UE can use PSFCH resource in the RB set which UE successes to access.

- Option 2: UE tries to transmit SL HARQ-ACK feedback on PSFCH resource in the next PSFCH occasion.

  - For each min-PSSCH-to-PSFCH timing, UE is (pre)configured with different PRB groups for PSFCH resources.

  - R16 NR SL PSSCH-to-PSFCH resource determination rule is applied to each PSFCH occasion.

    - FFS: PSFCH occasions are present in the same slot or different slots.

  - UE can use PSFCH resource in the earliest PSFCH occasion which UE successes to access the channel.

Meanwhile, in Rel-16/17 NR SL, PSFCH TX or RX could be dropped according to PSFCH TX/RX prioritization rule based on SL priority value. In this case, it would be necessary to support that UE can perform PSFCH reception(s) if the UE fails to access the channel for prioritized PSFCH TX(s). Even for PSFCH TX/TX collisions, it would be beneficial to transmit deprioritized PSFCH transmission instead of prioritized PSFCH transmission when the UE drops the prioritized PSFCH transmission due to LBT failure.

Proposal 10: If a UE fails to access channel for PSFCH transmission, then the UE can perform deprioritized PSFCH reception(s) instead.

Proposal 11: If a UE tries to access channel for each PSFCH transmission, and if the UE fails to access channel for PSFCH transmission with smaller SL priority value, then the UE can transmit PSFCH transmission with larger SL priority value.

Alternatively, it can be considered that a UE handles PSFCH TX/RX collisions for the PSFCH transmission(s) after passing the LBT procedure.

In case of PSFCH transmission, considering multiplexing capacity, the interlaced transmission could be considered rather than wideband transmission. Since the PSFCH format 0 is designed based on PUCCH format 0, and the PUCCH format 0 have interlaced transmission form, it can be considered that interlaced transmission form of PUCCH format 0 is considered as a baseline for interlaced PSFCH transmission. Since the number of PRBs for a single PSFCH transmission would be highly increased, it would be necessary to check whether the number of PSFCH resources are sufficient. To increase the number of PSFCH resources, it can be considered to apply time-domain and/or frequency-domain OCC to PSFCH transmission. In case of time-domain OCC, it would be necessary to carefully investigate the impact of power imbalance across different symbols due to OCC. The power imbalance in time domain would have negative impact on the AGC performance. Moreover, since first symbol will be used for AGC, time-domain OCC would not be always feasible for all the UEs. In case of frequency-domain OCC, PAPR needs to be checked. Alternatively, it can be considered to introduce comb structure for the interlaced PSFCH transmission. However, in this case, the number of cyclic shift pairs could be reduced, so total number of PSFCH resources would not be large enough.

When the PSFCH resource is associated with RB set index and interlaced index, it would be necessary to modify the implicit PSSCH-to-PSFCH resource determination rule as well. According to Rel-16 NR SL, subchannel index and slot index of PSSCH are used to determine RB sub-group of PSFCH, and source ID of PSSCH is used to determine RB index within the RB sub-group and cyclic shift pair index. For interlaced PSFCH transmission, it would be necessary to define how RB set index and interlace index of PSFCH are derived from PSSCH resource and source ID. Due to limited number of frequency resources of the interlaced RB-based PSFCH transmission, subgrouping would need to be extended to code-domain resources. For instance, the UE can determines the subset of interlaces and RB sets and cyclic shifts pairs based on subchannel, RB set, and slot associated with PSSCH transmission. Then, the UE finally selects PSFCH resource within the subset of PSFCH resource candidates based on source ID and/or M_ID.

Proposal 12: For PSFCH transmission, to meet the OCB and PSD requirements,

- Support interlaced RB-based PSFCH transmission

  - Interlaced PUCCH format 0 is considered as a baseline.

    - FFS: PSFCH structure for 60kHz SCS.

  - For PSSCH-to-PSFCH determination rule,

    - Among (pre)configured set of interlaces, RB sets, and cyclic shift pairs for PSFCH transmission, PSFCH resource indexing is done in increasing order of first interlace index, and then RB set index, and then cyclic shift pair index.

    - Set of PSFCH resources is partitioned by the RB set of PSSCH, and then is partitioned by the interlace of PSSCH, and then is partitioned by PSSCH slot.

    - Within the subset of PSFCH resources, the UE selects final PSFCH resource index based on source ID and/or M_ID.

In this approach, unlike Rel-16 NR SL, it will allows CDM among PSFCH resources associated with PSSCH transmission of which resources are TDMed and/or FDMed.

  - Synchronization procedure and channels

Depending on the regulation, S-SSB transmission also needs to meet the OCB requirement and PSD requirement. For this purpose, a number of options are listed up in RAN1#109-e meeting [1].

On the time domain resources for S-SSB, unlike NR-U, S-SSB transmission is performed by a UE of which TX power would be limited. In Rel-16 NR SL, 2-symbol duration of S-PSS and S-SSS could achieve energy combining gain. In that point of view, at least for S-PSS and S-SSS needs to have 2 symbol duration as in Rel-16 NR SL. Moreover, considering that the first symbol of S-SSB is used for AGC, even if 1 symbol duration is used for both S-PSS and S-SSS, only one symbol will be effectively used to decode PSBCH for the 4 symbol duration of S-SSB transmission. It will restrict SL communication coverage and make SL transmissions asynchronous.

Observation 14: Considering coverage of SL communication the S-SSB structure in time domain (i.e., 2-symbol S-PSS and 2-symbol S-SSS) needs to be kept to achieve energy combining gain.

Proposal 13: For S-SSB in SL-U,

- R16 S-SSB structure in time domain is reused (i.e., 14-symbol duration of S-SSB for normal CP, and 12-symbol duration of S-SSB for extended CP).

In our companion contribution [3], Type 2A channel access procedure would be applicable for the S-SSB transmission when the total duration of the S-SSB transmission in UE perspective or in system perspective meets the duty-cycle requirements as in NR-U DL. To increase channel availability, it would be better to (pre)configure the time domain locations of S-SSB resources so that the transmission(s) duration is at most 1ms, and the duty cycle is at most 1/20 to use Type 2A channel access procedure.

Observation 15: Time domain locations of S-SSB resources needs to be determined based on the condition to use Type 2A DL channel access procedure for DL discovery burst transmissions, which is that the transmission(s) duration is at most 1ms, and the duty cycle is at most 1/20 to use Type 2A channel access procedure.

According to EN 301 893 [4], within a COT duration, the UE may not need to fulfil the OCB requirement temporarily as follows:

- During a Channel Occupancy Time (COT), equipment may operate temporarily with an occupied bandwidth of less than 80 % of its nominal channel bandwidth. The occupied bandwidth shall not be less than 2 MHz.

Since S-PSS and S-SSS start from the second symbol of S-SSB, it is understood that both S-PSS and S-SSS would be within the COT duration. In this case, it would be possible to reuse the existing S-PSS and S-SSS sequences without a consideration of the OCB requirement. However, in this case, due to the PSD requirement, the total power of the S-PSS and S-SSS would be too small, and it will cause SL communication coverage reduction.

Observation 16: According to EN 301 893, the temporary exemption of OCB requirement can be applicable at least for S-PSS and S-SSS. However, due to PSD requirement, the communication coverage would be too restrictive.

In case of interlaced RB-based S-SSB transmission, it would be possible to reuse the existing sequences if the number of PRBs belonging to one interlace within a RB set is 11. Otherwise, it would be necessary to use two interlaces for S-SSB transmission. If two interlaces within a RB set is used for S-SSB transmission, due to PSD requirement, the TX power would be limited compared to the case when one interlace is used.

Observation 17: For interlaced RB-based transmission for S-SSB, depending on the number of PRBs associated with a single interlace, it would be necessary to use two interlaces for S-SSB transmission to ensure bandwidth of 11 PRBs.

Observation 18: For interlaced RB-based transmission for S-SSB, if two interlaces are used, the PSD would be halved compared to the case when single interlace is used due to PSD requirement.

In NR-U, the interlaced structure is defined for 15 kHz SCS and 30 kHz SCS, but not for 60 kHz SCS. Meanwhile, in Rel-16 NR SL, for FR1, S-SSB transmission is supported for 60 kHz. For 60 kHz SCS, it would be necessary to define interlaced structure or to adopt another approach for the OCB requirement. The PRB gap between interlaces is 10 or 5 for 15 kHz or 30 kHz, respectively. Since 5/2 is not an integer, it seem not straightforward to design interlace structure for 60 kHz.

Observation 19: For 60 kHz SCS, since interlace structure is not defined in NR-U, interlaced RB-based S-SSB transmission would not be straightforward.

Moreover, in NR-U, since the interlaced RB-based transmission is a part of the UE capability, it would be possible that UEs without the capability cannot relay the received S-SSB transmission. In this case, the system performance of the synchronization procedure and the subsequent Mode 2 operation would be degraded further due to the synchronization error.

Observation 20: For interlaced RB-based S-SSB transmission, UEs without interlaced RB-based transmission cannot relay the received S-SSB transmission, and it will cause system performance degradation due to synchronization error.

For the OCB and PSD requirement, repetition of S-SSB in frequency domain also discussed in RAN1#109-e meeting [1]. However, in this case, PARP could be highly increased. To alleviate this high PAPR problem, at least S-PSS and S-SSS could be repeated with phase adjustment while a PSBCH is transmitted in wideband transmission manner.

Observation 21: Repetition of S-SSB in frequency domain would increase PAPR, and it will further reduce coverage.

In NR-U, wideband PRACH transmission is introduced for OCB and PSD requirements. To design wideband S-SSB transmission, it can be considered to reuse target bandwidth of wideband PRACH transmission in NR-U. However, for different length of the M-sequence or Gold sequence, it would be necessary to find new polynomials and input polynomials.

Observation 22: For S-SSB with wider bandwidth, it would be necessary to find new polynomials and input polynomials for each SCS. It would highly increase specification work load.

Proposal 14: For S-SSB in SL-U, support Option 1 to meet OCB and PSD requirement for S-SSB transmission:

- Option 1: Using interlaced RB transmission. For details, down-select one of followings:

  - Option 1-1:

    - R16 S-PSS/S-SSS/PSBCH are mapped on 11 PRBs associated with one or two interlaces within a RB set.

    - A (pre)configuration provides the RB set(s) and interlace(s) for S-SSB mapping.

  - Option 1-2:

    - R16 S-PSS/S-SSS/PSBCH are mapped on PRBs associated with one interlace within a RB set.

      - If the number of PRBs associated with the interlace within the RB set is smaller than 11 PRBs, parts of sequences can be truncated.

    - A (pre)configuration provides the RB set(s) and interlace(s) for S-SSB mapping.

  - Option 1-3:

    - R16 S-PSS/S-SSS/PSBCH are mapped on 11 PRBs equally distributed over a RB set.

      - PRB gap is implicitly or explicitly determined.

    - A (pre)configuration provides the RB set(s) and/or PRB gap for S-SSB mapping.

- Conclusions

In this contribution, we discussed physical channel design framework for sidelink on unlicensed spectrum. Based on the above discussion, our observations and proposals are given as follows:

Observation 1: If S-SSB transmission is a form of interlace structure, the benefit of the coexistence between contiguous RB-based transmission and interlaced RB-based transmission in a SL carrier is unclear.

Observation 2: If sub-channels are not aligned with RB sets in boundaries, a number of resources would not be utilized especially when small number of RB set(s) are allocated for PSCCH/PSSCH transmission.

Observation 3: If a subchannel consists of PRBs corresponding to an interlace index within a RB set, and if the subchannel indexing is done in increasing order of first the interlace index, and then the RB set index, and if more than one RB sets are scheduled for PSSCH transmission, the same set of interlaces across different RB sets would not be guaranteed. It can cause high PAPR.

Observation 4: If a subchannel consists of PRBs corresponding to an interlace index within a RB set, and if the subchannel indexing is done in increasing order of first the RB set index, and then the interlace index, and if more than one interlaces are scheduled for PSSCH transmission, it may need to use multiple RB sets for PSSCH transmission. In this case, the UE needs to access both RB sets for a PSSCH transmission.

Observation 5: Due to PSD requirement (i.e., 8 or 10 dBm/MHz), depending on the number of interlaces, it would be useful using multiple RB sets rather than using single RB set for PSSCH transmission for the same number of PRBs or interlaces.

Observation 6: For 30kHz SCS, since the number of interlaces is 5, the number of interlaces belonging to a subchannel can be different.

Observation 7: If contiguous sub-channel allocation indicator is reused for interlaced RB-based PSSCH transmission, depending on the sub-channelization method, the resource allocation could be inefficient in terms of channel accessibility, PAPR, and/or TX power restriction.

Observation 8: For sub-slot-based PSCCH/PSSCH transmission, if two PSSCH transmissions with 7-symbol duration including AGC symbol and TX-RX switching period are allowed in a slot (for normal CP case), PSFCH resource(s) cannot be allocated since the proper PSSCH DMRS pattern is not defined.

Observation 9: It is necessary to carefully investigate the feasibility on that UE prepares and performs the next PSSCH transmission with the next starting symbol after UE decide LBT failure for the 1st starting symbol in terms of processing time budget and UE complexity. Moreover, it is necessary to ensure the same TB size among PSSCH transmission(s) with different starting symbol(s).

Observation 10: If additional starting symbol(s) within a slot for slot-based PSCCH/PSSCH transmission is allowed, the PSSCH RX UE needs to perform AGC procedure for each starting symbol, and it will cause throughput degradation.

Observation 11: Since PSSCH TX UE can receive a number of SL HARQ-ACK feedbacks from different PSSCH RX UEs, R16 NR SL PSFCH format 0 is sufficient for the container of the SL HARQ-ACK feedback.

Observation 12: If the locations of PSFCH resources are dynamically indicated, UE needs to perform sensing operation for PSFCH further. Otherwise, even though PSSCH resources are not overlapping each other, PSFCH resource collision can occur.

Observation 13: If UE can share COT initiated by UE transmitting PSCCH/PSSCH for PSFCH in response of the PSCCH/PSSCH, the PSFCH TX dropping problem due to LBT failure would be mitigated.

Observation 14: Considering coverage of SL communication the S-SSB structure in time domain (i.e., 2-symbol S-PSS and 2-symbol S-SSS) needs to be kept to achieve energy combining gain.

Observation 15: Time domain locations of S-SSB resources needs to be determined based on the condition to use Type 2A DL channel access procedure for DL discovery burst transmissions, which is that the transmission(s) duration is at most 1ms, and the duty cycle is at most 1/20 to use Type 2A channel access procedure.

Observation 16: According to EN 301 893, the temporary exemption of OCB requirement can be applicable at least for S-PSS and S-SSS. However, due to PSD requirement, the communication coverage would be too restrictive.

Observation 17: For interlaced RB-based transmission for S-SSB, depending on the number of PRBs associated with a single interlace, it would be necessary to use two interlaces for S-SSB transmission to ensure bandwidth of 11 PRBs.

Observation 18: For interlaced RB-based transmission for S-SSB, if two interlaces are used, the PSD would be halved compared to the case when single interlace is used due to PSD requirement.

Observation 19: For 60 kHz SCS, since interlace structure is not defined in NR-U, interlaced RB-based S-SSB transmission would not be straightforward.

Observation 20: For interlaced RB-based S-SSB transmission, UEs without interlaced RB-based transmission cannot relay the received S-SSB transmission, and it will cause system performance degradation due to synchronization error.

Observation 21: Repetition of S-SSB in frequency domain would increase PAPR, and it will further reduce coverage.

Observation 22: For S-SSB with wider bandwidth, it would be necessary to find new polynomials and input polynomials for each SCS. It would highly increase specification work load.

Proposal 1: For SL BWP configuration on shared spectrum,

- Starting PRB of SL BWP is aligned with the lowest PRB of the lowest RB set within SL BWP

- Ending PRB of SL BWP is aligned with the highest PRB of the highest RB set within SL BWP

- Either contiguous RB-based transmission or interlaced RB-based transmission is (pre)configured

Proposal 2: For SL resource pool configuration in frequency domain on shared spectrum,

- Starting PRB (i.e., sl-StartRB-Subchannel) is aligned with the lowest PRB of the lowest RB set within the resource pool

- The number of PRBs (i.e., sl-RB-Number) is set so that the ending PRB is aligned with the highest PRB of the highest RB set within the resource pool

Proposal 3: For SL communication on shared spectrum, a resource pool excludes following slots:

- Slots whose symbols from sl-StartSymbol to sl-StartSymbol + sl-LengthSymbols – 1 are not semi-statically configured as UL as per the higher layer parameter tdd-UL-DL-ConfigurationCommon of the serving cell if provided or sl-TDD-Configuration if provided or sl-TDD-Config of the received PSBCH if provided

- S-SSB slots

- Reserved slot as specified in section 8 of TS 38.214

Proposal 4: For contiguous RB-based transmission,

- Sub-channels are defined to be aligned with RB set(s) in boundaries.

- PRBs between sub-channels belonging to different RB sets can be used automatically for PSCCH/PSSCH transmission when the sub-channels belonging to different RB sets are used for the PSCCH/PSSCH transmission.

- PRBs not belonging to a sub-channel is not counted for TBS determination.

Proposal 5: For interlaced RB based PSSCH transmission, one of followings is supported:

- Option 1:

  - A subchannel consists of PRBs belonging to K interlaces within a RB set.

    - Subchannel indexing is done in increasing order of first the interlace index, and then the RB set index.

    - K is at least 1.

  - SCI indicates FRIV for subchannel allocation.

  - PSSCH transmission resource(s) are determined by the indicated sub-channel(s)

  - When more than one RB sets are used, UE expects that the same set of interlaces are used.

- Option 2:

  - A subchannel consists of PRBs belonging to K interlaces within a RB set.

    - Subchannel indexing is done in increasing order of first the RB set index, and then the interlace index.

    - K is at least 1.

  - SCI indicates FRIV for subchannel allocation.

  - PSSCH transmission resource(s) are determined by the indicated sub-channel(s)

- Option 3:

  - A subchannel consists of PRBs belonging to K interlaces.

    - Subchannel indexing is done in increasing order of the interlace index.

    - K is at least 1.

  - SCI indicates FRIV for subchannel allocation and FRIV for RB set allocation.

  - PSSCH transmission resource(s) are determined by the intersection of indicated sub-channel(s) and indicated RB set(s).

Proposal 6: PSCCH is mapped on the lowest subchannel within the lowest RB set allocated for PSSCH transmission.

Proposal 7: For SL transmission burst, time gap between adjacent PSCCH/PSSCH transmissions is not greater than 16usec.

- To fill out all or a subset of TX-RX switching symbol, down-select one of followings:

  - Alt 1: Extended CP of the later SL transmission can occupy part of TX-RX switching symbol of the earlier SL transmission.

  - Alt 2: Parts of a SL transmission can occupy part of TX-RX switching symbol of the SL transmission.

Proposal 8: For SL transmission burst, down-select one or more of followings:

- Option 1: Maximum size of SL transmission burst is determined by PSFCH resource period.

- Option 2: PSSCH TX UE can transmit any signal in a PSFCH occasion to ensure the time gaps between transmissions within a SL transmission burst is no greater than 16usec.

Proposal 9: For handling PSFCH format 0 transmission dropping due to LBT failure, followings can be discussed:

- Option 1: UE tries to transmit SL HARQ-ACK feedback on PSFCH resource in the next RB set.

  - R16 NR SL PSSCH-to-PSFCH resource determination rule is applied to each RB set.

  - UE can use PSFCH resource in the RB set which UE successes to access.

- Option 2: UE tries to transmit SL HARQ-ACK feedback on PSFCH resource in the next PSFCH occasion.

  - For each min-PSSCH-to-PSFCH timing, UE is (pre)configured with different PRB groups for PSFCH resources.

  - R16 NR SL PSSCH-to-PSFCH resource determination rule is applied to each PSFCH occasion.

    - FFS: PSFCH occasions are present in the same slot or different slots.

  - UE can use PSFCH resource in the earliest PSFCH occasion which UE successes to access the channel.

Proposal 10: If a UE fails to access channel for PSFCH transmission, then the UE can perform deprioritized PSFCH reception(s) instead.

Proposal 11: If a UE tries to access channel for each PSFCH transmission, and if the UE fails to access channel for PSFCH transmission with smaller SL priority value, then the UE can transmit PSFCH transmission with larger SL priority value.

Proposal 12: For PSFCH transmission, to meet the OCB and PSD requirements,

- Support interlaced RB-based PSFCH transmission

  - Interlaced PUCCH format 0 is considered as a baseline.

    - FFS: PSFCH structure for 60kHz SCS.

  - For PSSCH-to-PSFCH determination rule,

    - Among (pre)configured set of interlaces, RB sets, and cyclic shift pairs for PSFCH transmission, PSFCH resource indexing is done in increasing order of first interlace index, and then RB set index, and then cyclic shift pair index.

    - Set of PSFCH resources is partitioned by the RB set of PSSCH, and then is partitioned by the interlace of PSSCH, and then is partitioned by PSSCH slot.

    - Within the subset of PSFCH resources, the UE selects final PSFCH resource index based on source ID and/or M_ID.

Proposal 13: For S-SSB in SL-U,

- R16 S-SSB structure in time domain is reused (i.e., 14-symbol duration of S-SSB for normal CP, and 12-symbol duration of S-SSB for extended CP).

Proposal 14: For S-SSB in SL-U, support Option 1 to meet OCB and PSD requirement for S-SSB transmission:

- Option 1: Using interlaced RB transmission. For details, down-select one of followings:

  - Option 1-1:

    - R16 S-PSS/S-SSS/PSBCH are mapped on 11 PRBs associated with one or two interlaces within a RB set.

    - A (pre)configuration provides the RB set(s) and interlace(s) for S-SSB mapping.

  - Option 1-2:

    - R16 S-PSS/S-SSS/PSBCH are mapped on PRBs associated with one interlace within a RB set.

      - If the number of PRBs associated with the interlace within the RB set is smaller than 11 PRBs, parts of sequences can be truncated.

    - A (pre)configuration provides the RB set(s) and interlace(s) for S-SSB mapping.

  - Option 1-3:

    - R16 S-PSS/S-SSS/PSBCH are mapped on 11 PRBs equally distributed over a RB set.

      - PRB gap is implicitly or explicitly determined.

    - A (pre)configuration provides the RB set(s) and/or PRB gap for S-SSB mapping.

Reference

[1] RAN1 Chair’s Notes, RAN1#109-e.

[2] 3GPP TS 37.213, “Physical layer procedures for shared spectrum channel access”.

[3] R1-2205850, “Discussion on channel access mechanism for sidelink on unlicensed spectrum,” LG Electronics.

[4] ETSI EN 301 893, “5 GHz WAS/RLAN; Harmonised Standard for access to radio spectrum”.