3GPP TSG RAN WG1 #110		                                                      R1-2206397

Toulouse, France, Aug 22nd – 26th, 2022

Source:	CATT

Title:	Discussion on evaluation on NR duplex evolution

Agenda Item:	9.3.1

Document for:	Discussion and Decision

- Introduction

During RAN1#109-e meeting, evaluation methodologies and assumptions on NR duplex evolution were discussed and the following consensuses were reached on this topic [1].

Agreement For discussion purpose for evaluation, define the following deployment cases for SBFD: - Deployment Case 1 (Non-coexistence case with single SBFD subband configuration): One single operator using one single carrier is considered. All the cells belonging to the operator use SBFD operation with the same SBFD subband configuration. - Deployment Case 2 (Non-coexistence case with multiple SBFD subband configurations): One single operator using one single carrier is considered. All the cells belonging to the operator use SBFD operation, but different cells may use different SBFD subband configurations. - Deployment Case 3 (Co-channel co-existence case): One single operator using one single carrier is considered. Among the cells belonging to the operator, some of them use legacy TDD operation (static TDD operation) while the others use SBFD operation with the same SBFD subband configuration. - Deployment Case 3-1: Only 1-layer is considered - Deployment Case 3-2: 2-layer is considered - Deployment Case 4 (Adjacent-channel co-existence case): Two operators each using one carrier are considered and the two carriers are adjacent carriers. One operator uses legacy TDD operation (static TDD operation) while the other operator uses SBFD operation with the same SBFD subband configuration. Note: This definition has no intention to preclude any potential solutions for SBFD in AI9.3.2 Note: SBFD subband configuration is from gNB perspective. Agreement For SBFD Deployment Case 1, at least consider the following scenarios for evaluation: - For FR1, - Indoor office (use Indoor office defined in TR38.802/TR38.901 as starting point) - Urban macro (use Urban macro defined in TR38.802/TR38.901 as starting point) - FFS: UE outdoor/indoor proportion, clustering, etc - Optional: Dense Urban with 1-layer or 2-layer (use Dense Urban defined in TR38.802/TR38.901 as starting point) - FFS: Rural - For FR2-1, - Indoor office (use Indoor office defined in TR38.802/TR38.901 as starting point) - Dense Urban Macro layer (use Dense Urban defined in TR38.802 as starting point) - FFS: UE outdoor/indoor proportion, clustering, etc - Optional: Dense Urban micro (use Dense Urban micro defined in TR38.802/TR38.901 as starting point) - FFS: Whether FR2-2 is considered or not in Rel-18. Note: For optional scenarios, they can be captured in TR and it is up to each company to provide the results. The results can be used to draw conclusion/recommendation depending on the number of companies providing the results. Agreement: Regarding gNB self-interference modelling for system level simulation purpose, consider introducing ratio of self-interference (RSI) to represent the overall self-interference suppression capability of gNB by means of spatial isolation, subband frequency isolation, digital interference cancellation and beamform nulling/isolation, etc. RSI also takes into account the impact of Tx/Rx antenna element gain on self-interference. The RSI, denoted as αSIm,n, can be defined as the ratio of the total power transmitted by gNB across all transmit chains on a frequency unit m (e.g., subband/RB/subcarrier m) in a SBFD carrier to the residual self-interference received by the same gNB on a single receiver chain on a different frequency unit n (e.g., another subband/RB/subcarrier n) in the same SBFD carrier. - FFS: Model for link level simulations and relevant questions to ask RAN4 - FFS: details of gNB self-interference modelling using RSI in SLS. As one example based on per-RB-RSI, the gNB self-interference on a single receiver chain at UL RB n can be modelled as - ISIn=m≠nm∈DL subbandISIm,n, wherein, - dBmISIm,n=Ptxm-dBαSIm,n - ISIm,nis the gNB self-interference on a single receiver chain at UL RB n caused by DL transmission on DL RB m. - m is the DL RB index in DL subbands. - Ptxm is gNB’s DL transmission power across all transmit chains at RB m (in dBm). - αSIm,n is the per-RB-RSI. - FFS: consider a statistical clutter model based on statistics of clutter strength and AoA. - The following should be asked to RAN4: - What is the value range of RSI αSI(n,m) for each frequency range, and under what assumptions on the self-interference suppression means the value range of RSI is provided? - RAN1 understands the RSI can be described per subband, per RB, or per subcarrier depending on the granularity of the frequency unit, and it is up to RAN4 to provide the RSI in which granularity. - Whether it is possible for RAN4 to provide RAN1 the respective capabilities of different self-interference suppression means? e.g., is it possible to provide the separate estimates for spatial isolation, subband frequency isolation, beamform nulling/isolation, and digital cancellation, etc., as below? - dB(αSI(m,n))= dB(αSI-spatial)+dB(αSI-frequency (m,n))+dB(αSI-beam)+dB(αSI-digtial)+… - αSI-spatial denotes the spatial isolation. - αSI-frequency (n,m) denotes the suband frequency isolation between the Tx frequency unit m and the Rx frequency unit n. - αSI-beam denotes the beamform nulling or beam isolation. - αSI-digtial denotes the digital cancellation capability. - Whether it is possible to simplify the RSI as frequency flat model, and under which condition(s) the dependency of the RSI on frequency can be ignored? - The feasibility of provided value range of RSI regarding factors such as blocking, AGC, etc. - Does RSI have any dependency with the following factors or any other factors? What are the dependencies? - gNB’s antenna aspects, e.g., the assumed antenna architecture, the number of transmit chains and receive chains, etc. - Frequency aspects, e.g., the frequency distance between the Tx frequency unit m and the Rx frequency unit n, the number of RBs allocated for DL transmission, etc. - Beam aspects, e.g., Tx/Rx beam-pair for FR1/FR2 especially for clutter echo, etc. - Note: RAN1’s consideration on the frequency locations and sizes of SBFD DL subband and SBFD UL subband assumed in SBFD operation can be provided to RAN4. Agreement For discussion of gNB-gNB and UE-UE co-channel inter-subband CLI modelling in system level simulation, RAN1 understands at least the following two aspects need to be considered: - Aspect 1: The unwanted emissions due to Tx non-linearity at the transmitter of the aggressor from the allocated RBs to the non-allocated RBs in the same carrier. - Aspect 2: The receiver selectivity at the victim to receive the desired signal in the allocated RBs in the presence of the unwanted signals at the non-allocated RBs. (e.g. receiver blocking at the victim, overload of the receiver dynamic range, etc) - The following questions should be asked to RAN4: - Whether it is feasible to consider the above two aspects for gNB-gNB and UE-UE co-channel inter-subband CLI modelling in system level simulation? Are there any other aspects should also be taken into account? - For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of gNB-gNB link, where the DL frequency unit m and UL frequency unit n are in the same carrier and non-overlapping in frequency, and assuming the aggressor gNB transmits on the DL frequency unit m and the victim gNB receives on the UL frequency unit n, - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 1 (defined above) at the gNB transmitter? - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 2 (defined above) at the gNB receiver? - How to model the above interferences for the following two cases: - inter-site gNB-gNB co-channel inter-subband CLI - co-site inter-sector co-channel inter-subband CLI - For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of UE-UE link, where the DL frequency unit m and UL frequency unit n are in the same carrier and non-overlapping in frequency, and assuming the aggressor UE transmits on the UL frequency unit n and the victim UE receives on the DL frequency unit m, - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 1 (defined above) at the UE transmitter? - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 2 at the UE receiver? FFS: Usage of the above model provided by RAN4 in the evaluation Agreement At least the following metrics are considered for SBFD and dynamic/flexible TDD evaluation. - DL/UL UPT or user throughput (CDF or {mean, 5%, 50%, 95%}) using SLS - Latency (CDF or {mean, 5%, 50%, 95%}) using SLS - Resource utilization using SLS - DL/UL received SINR using SLS - Coverage metric - FFS: MPL to achieve a certain bit rate in UL and DL - FFS: definitions of the above metrics - FFS: other metrics Agreement Regarding traffic model for SBFD and dynamic/flexible TDD evaluation, at least FTP3 is considered. Performance evaluation comparison between different duplex modes (e.g., legacy static TDD vs. SBFD) should be performed based on the same amount of input traffic. - FFS: other traffic models, e.g., XR, VoIP - FFS: Packet size, traffic load, ratio of DL/UL traffic - FFS: additionally consider different amount of input traffic at least for adjacent-channel/co-channel coexistence studies Agreement For discussion for duplex evolution study (all agenda items), consider the following as RAN1’s common understanding: - Co-channel interference: The interference is from the aggressor to the victim in the same carrier. - Co-channel intra-subband interference: The interference is caused by transmission of the aggressor on a set of contiguous RBs in a carrier to reception of the victim on the same set of contiguous RBs in the same carrier. - Co-channel inter-subband interference: The interference is caused by transmission of the aggressor in a first set of contiguous RBs in a carrier to reception of the victim in a second set of contiguous RBs in the same carrier, where the two contiguous RB sets are non-overlapping in frequency. - Adjacent channel interference: The interference is from the aggressor in carrier#1 to the victim in carrier#2, where the carrier#1 and carrier#2 are adjacent carriers. Note 1: ‘Co-channel’ here means ‘co-carrier’. ‘Adjacent-channel’ here means ‘adjacent-carrier’. Agreement For discussion for duplex evolution study (all agenda items), consider the following as the common understanding in RAN1 on the definition of interference types for SBFD operation: - gNB self-interference (SI): Interference caused by DL transmission on a set of DL RBs in a carrier to UL reception on a set of UL RBs in the same carrier at the gNB side, where the two RB sets are non-overlapping in frequency. - gNB-UE co-channel intra-subband interference: This is the same as the legacy DL interference type in legacy TDD network with static TDD UL/DL configuration. - UE-gNB co-channel intra-subband interference: This is the same as the legacy UL interference type in legacy TDD network with static TDD UL/DL configuration. - (inter-cell) inter-site gNB-gNB co-channel intra-subband CLI: CLI caused by DL transmission of the aggressor gNB on a set of RBs in one carrier to UL reception of the victim gNB in a different site on the same set of RBs in the same carrier. - (inter-cell) co-site inter-sector co-channel intra-subband CLI: CLI caused by DL transmission of the aggressor gNB on a set of RBs in one carrier to UL reception of the victim gNB in another sector of the same site on the same set of RBs in the same carrier. - (inter-cell) UE-UE co-channel intra-subband CLI: CLI caused by UL transmission of the aggressor UE on a set of RBs in one carrier to DL reception of the victim UE on the same set of RBs in the same carrier. - (inter-cell) inter-site gNB-gNB co-channel inter-subband CLI: CLI caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to UL reception of the victim gNB in a different site on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency. - (inter-cell) co-site inter-sector co-channel inter-subband CLI: CLI caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to UL reception of the victim gNB in another sector of the same site on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency. - (intra-cell/inter-cell) UE-UE co-channel inter-subband CLI: CLI caused by UL transmission of the aggressor UE on a first set of RBs in a carrier to DL reception of the victim UE on a second set of RBs in the same cell or neighboring cell in the same carrier, where the two RB sets are non-overlapping in frequency. - gNB-gNB adjacent-channel CLI: CLI caused by DL transmission of the aggressor gNB in a carrier to UL reception of the victim gNB in another adjacent carrier. - This includes adjacent-channel CLI between gNBs in the same and different sectors of the same site, i.e., co-site intra and inter-sector adjacent-channel CLI. - UE-UE adjacent-channel CLI: CLI caused by UL transmission of the aggressor UE in a carrier to DL reception of the victim UE in another adjacent carrier. Note: Some of the interferences may not be used according to the deployment scenarios, e.g, whether the SBFD subband configurations are the same or different across gNBs. Note: This does not imply we need to consider all the above interference types in evaluation for SBFD. Agreement Regarding gNB-gNB and UE-UE adjacent-channel CLI modelling for system level simulation, RAN1 understands at least the following aspects need to be considered: - Aspect 1: The unwanted emissions due to Tx non-linearity at the transmitter of the aggressor from the allocated RBs in one carrier to the non-allocated RBs in the adjacent carrier. - Aspect 2: The receiver selectivity at the victim to receive the desired signal in the allocated RBs in one carrier in the presence of the unwanted signals at the non-allocated RBs in the adjacent carrier. (e.g. receiver blocking at the victim, overload of the receiver dynamic range, etc) The following questions should be asked to RAN4: - Whether it is feasible to consider the above two aspects for gNB-gNB and UE-UE adjacent-channel CLI modelling in system level simulation? Are there any other aspects should also be taken into account? - For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of gNB-gNB link, where the DL frequency unit m and UL frequency unit n are in adjacent carriers and non-overlapping in frequency, and assuming the aggressor gNB transmits on the DL frequency unit m and the victim gNB receives on the UL frequency unit n, - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 1 (defined above) at the gNB transmitter? - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 2 (defined above) at the gNB receiver? - How to model the above interferences for the following cases: - the two gNBs are from the same sector of the same site in adjacent carriers, i.e., co-site co-sector gNB-gNB adjacent-channel CLI - the two gNBs are from different sectors of the same site in adjacent carriers, i.e., co-site inter-sector gNB-gNB adjacent-channel CLI - the two gNBs are from different sites in adjacent carriers, i.e., inter-site gNB-gNB adjacent-channel CLI - Whether it is feasible to define a similar interference ratio as BS-BS ACIR in TR38.828 but in the subband of the adjacent carrier, with finer granularity (e.g., per subband or per RB), to represent the overall effect of the Aspect 1 and Aspect 2 described above? - For example, whether it is feasible to define gNB-gNB-adjacent-channel-per-RB/subband interference ratio as the ratio of the power transmitted by the aggressor gNB on DL frequency unit m to the interference received by the victim gNB on UL frequency unit n? If it is feasible, then what is the value range of the gNB-gNB-adjacent-channel-per-RB/subband interference ratio for each frequency range? - For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of UE-UE link, where the DL frequency unit m and UL frequency unit n are in adjacent carriers and non-overlapping in frequency, and assuming the aggressor UE transmits on the UL frequency unit n and the victim UE receives on the DL frequency unit m, - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 1 (defined above) at the UE transmitter? - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 2 at the UE receiver? - Whether it is feasible to define a similar interference ratio as UE-UE ACIR in TR38.828 but in the subband of the adjacent carrier, with finer granularity (e.g., per subband or per RB), to represent the overall effect of the Aspect 1 and Aspect 2 described above? - For example, whether it is feasible to define UE-UE-adjacent-channel-per-RB/subband interference ratio as the ratio of the power transmitted by the aggressor UE on UL frequency unit n to the interference received by the victim UE on DL frequency unit m? If it is feasible, then what is the value range of the UE-UE-adjacent-channel-per-RB/subband interference ratio for each frequency range? FFS: How to make use of the interference model in RAN1 Agreement For SBFD evaluation, consider the following for SBFD subband configurations: - SBFD Subband configuration#1 with {DUD} pattern, which means one SBFD slot consists of one UL subband at the center of the channel bandwidth and two DL subbands at two sides of the channel bandwidth. - SBFD Subband configuration#2 with {DU} pattern, which means one SBFD slot consists of one UL subband at one side of the channel bandwidth and one DL subband at the other side of the channel bandwidth. - Use the following parameters for description of SBFD subband configuration in evaluation assumptions: - ND: the number of RBs in one DL subband - NU: the number of RBs in one UL subband - NG: the number of RBs in one guard band between one UL subband and one DL subband Agreement For performance evaluation and comparison between baseline legacy TDD operation and SBFD operation under SBFD Deployment Case 1 (Non-coexistence case with single SBFD subband configuration), consider the following alternatives: - Alt 2 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD): - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. - Alt 4 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD): - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] - SBFD: Frame structure#3 (XXXXX), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. - Alt 1 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD): - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] - SBFD: Frame structure#1 (DXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. - Alt 3 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD): - Legacy TDD: Static TDD UL/DL configuration with {DDSUU}, where S=[12D:2G:0U] - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. FFS: whether dynamic TDD can optionally be used for legacy TDD for comparison. Agreement For gNB-gNB co-channel/adjacent-channel channel model and UE-UE co-channel/adjacent-channel channel model in RAN1 SLS, - Large scale fading (e.g., path loss, penetration loss, shadowing) should be modelled, and companies report whether small scale fading (e.g., fast fading including antenna gain) is also modelled in their simulation. - Note: Antenna gain is calculated based on the gNB-gNB or UE-UE LOS direction instead on the multi-path directions if fast fading is not modelled. - FFS: how to model realistic LOS probability for gNB-gNB and UE-UE channel model. - FFS: How to set aligned channel model amongst companies for SLS calibration (if needed). Agreement For gNB-gNB channel model, reuse gNB-to-UE channel model in TR 38.901 with necessary modification - Replacing the UE’s antenna height with gNB’s antenna height, updating the angular spread - FFS: whether/how to update LOS probability. - FFS: Other details and necessary modifications Agreement For SBFD simulation, consider 4GHz for FR1 and 30GHz for FR2-1. Agreement For evaluation of SBFD operation, BS uses separate panels for simultaneous downlink transmission and uplink reception, we can call it separate-Tx/Rx antenna array for description of evaluation assumption. - Companies can report the separation of the Tx panel and Rx panel assumed in their simulation. - Companies can report how the antenna elements are used for transmission or reception in a slot if BS does not perform simultaneous downlink transmission and uplink reception. Agreement For evaluation of legacy TDD operation, BS uses the same antenna array for downlink transmission and uplink reception, we can call it shared-Tx/Rx antenna array for description of evaluation assumption. Agreement For evaluation and comparison between SBFD and legacy TDD, assume the total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD. Regarding antenna elements, both of the two options can be used. - Opt 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. - Opt 2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. - Companies report which option is assumed in their simulation. Agreement For SBFD Deployment Case 4, at least consider the following scenarios for evaluation from RAN1 perspective: - FR1: Urban Macro - FR2-1: Dense Urban Macro layer - FFS: UE outdoor/indoor proportion, clustering, etc - FFS: the grid shift between two networks, e.g., 0%, 100% - FFS: Indoor hotspot, Dense Urban Micro layer

In this contribution, we provide our analyses and views on the remaining issues on deployment scenarios, evaluation methodologies and evaluation assumptions.

- Discussion

  - Deployment scenarios

    - SBFD

In last meeting, InH for FR 2-2 was proposed as optional deployment scenario in [2] and [3] for SBFD deployment Case 1. We think FR2-2 is not a target scenario in terms of latency, coverage and capacity as analyzed below. So we propose to not consider FR 2-2 in Rel-18 duplex evolution.

- Latency: The slot/symbol duration reduces with the increasing SCS. Due to the large SCS in FR 2-2, latency is not an issue for FR 2-2 deployment.

- Coverage: For FR 2-2, constrained by high path loss, the typical scenario is small cell in indoor scenario where coverage is not a key issue.

- Capacity: Due to limited cell radius, the number of UEs served is limited. Thus capacity enhancement is not urgent.

Proposal 1: FR 2-2 is deprioritized in Rel-18 duplex evolution SI.

    - Dynamic/flexible TDD

In last meeting, deployment scenario for dynamic/flexible TDD was extensively discussed [3]. However, no consensus was achieved. Further analyses and views are provided in this section.

Figure 1: Layout for indoor office

Considering the fact that lower transmission power at gNB side will alleviate gNB-to-gNB CLI impact, indoor office scenario as depicted in Figure 1 is proposed to be included for flexible/dynamic TDD evaluation. In addition, a heterogeneous deployment with macro layer and indoor in the same carrier as depicted in Figure 2 is another attractive scenario for dynamic/flexible TDD. With respect to the TDD configuration of indoor gNB, the following two options were proposed in last meeting [3],

- Option 1: Indoor gNBs use UL dominant static TDD UL/DL configuration

- Option 2: Indoor gNBs use dynamic TDD UL/DL assignment

Considering the intention of the evaluation is to evaluate the performance enhancement via enhanced CLI schemes compared to that of existing CLI solutions, simpler scenario, i.e. Option 1, is preferred.

Proposal 2: The deployment scenarios for dynamic/flexible TDD evaluation at least include indoor office and heterogeneous deployment with Urban Macro and Indoor office.

  - Indoor office with dynamic TDD UL/DL assignment

  - HetNet with Urban Macro and Indoor office deployed in the same carrier

    - Macro layer use DL dominant static TDD UL/DL configuration: {DDDSU}

    - Indoor layer use UL dominant static TDD UL/DL configuration: {DSUUU}

Figure 2: Heterogeneous deployment with macro and indoor

  - Evaluation methodology

SLS metrics were agreed in last meeting. In this section, we share our views on the agreed metrics. The following are the definitions based on [6], it is proposed to use these definitions in the evaluation methodology .

- UPT (User perceived throughput): defined as the size of a packet divided by the time between the arrival of the packet and the reception of the last bit of the packet.

- Latency: defined as the time between the arrival of the packet and the reception of the last bit of the packet.

- Resource utilization: defined as the number of RBs per cell used by UL/DL traffic during observation time divided by total number of RB per cell available for UL/DL traffic over observation time; or, defined as the number of RB per cell used by UL/DL traffic during observation time divided by total number of RB per cell available for UL&DL traffic over observation time.

- DL/UL received SINR: (BS/UE Tx power - coupling loss) / (noise + legacy inter-cell interference from N interference sources + CLI from M interference sources), where FFS whether dominant interference sources or all the interference sources are taken into account, and coupling loss includes all kinds of losses (e.g. path loss and penetration loss) minus gains (e.g. Tx antenna gain, Rx antenna gain).

Proposal 3: Adopt the following definitions for SLS metrics for duplex enhancement evaluation.

- UPT: defined as the size of a packet divided by the time between the arrival of the packet and the reception of the last bit of the packet.

- Latency: defined as the time between the arrival of the packet and the reception of the last bit of the packet.

- Resource utilization: defined as the number of RBs per cell used by UL/DL traffic during observation time divided by total number of RB per cell available for UL/DL traffic over observation time; or, defined as the number of RB per cell used by UL/DL traffic during observation time divided by total number of RB per cell available for UL&DL traffic over observation time.

- DL/UL received SINR: (BS/UE Tx power - coupling loss) / (noise + legacy inter-cell interference from N interference sources + CLI from M interference sources)

  - FFS whether dominant interference sources or all the interference sources are taken into account

  - Coupling loss includes all kinds of losses (e.g. path loss and penetration loss) minus gains (e.g. Tx antenna gain, Rx antenna gain).

  - Evaluation assumptions

    - SBFD

For SBFD evaluation, various traffic loads which are represented by different resource utilization (RU) ratios should be considered to show the benefit of SBFD in different loads. For low RU ratio with a lot of non-occupied resources, configuring UL subband in DL symbols will enhance UL coverage and improve UL throughout via increased UL transmission duration. At the same time, DL throughput could also be improved as a result of shortened DL HARQ ACK feedback latency. For medium/high RU ratios, SBFD still can be expected to enhance UL coverage and improve UL throughout via increased UL transmission duration. However, DL throughput might be negatively affected due to less DL resources. Therefore different RU ratios should be considered during the evaluation to get a comprehensive analysis. It is proposed that 20%, 50% and 80% can be used as the reference values for low, medium and high RU ratios respectively.

Proposal 4: Evaluate low, medium and high RU ratios (20%, 50% and 80%) via different packet arrival rates in SBFD system evaluation.

The gNB-gNB and UE-UE channel model were discussed in last meeting [3]. And companies achieved consensus on gNB-gNB channel model while no agreement was achieved for UE-UE channel model. With respect to UE-UE channel model, reusing the UE-UE channel model for flexible duplex evaluation in TR 38.802 for both FR1 and FR2 is preferred.

In addition, there is an FFS on whether/how to update LOS probability [3]. Considering that updating LOS probability should base on the practical testing data which will cost significant workload, reusing existing LOS probability in TR 38.901 and TR 38.802 is preferred, i.e.reuse LOS probability of gNB-to-UE channel model in TR 38.901 as that of gNB-to-gNB channel model and reuse the LOS probability of UE-UE channel model for flexible duplex evaluation in TR 38.802 as that of UE-UE channel model.

Proposal 5: Reuse the UE-UE channel model for flexible duplex evaluation in TR 38.802 for both FR1 and FR2-1 SBFD evaluation.

Proposal 6: Reuse LOS probability in TR 38.901 and TR 38.802 for gNB-gNB and UE-UE channel model respectively.

With respect to UE distribution, UE cluster was proposed in [4] and [5] to reflect CLI impact on system performance in urban macro and dense urban scenarios. While the details of UE clustering can be further discussed, we think uniform UE distribution should be considered as a baseline.

Proposal 7: Uniform UE distribution for urban macro and dense urban scenarios is the baseline and FFS details of UE clustering.

Regard to grid shift for Case 4, co-location (0% grid shift) and maximum distance (100% grid shift) between neighbor operators can be considered as the worst and best co-existence case respectively.

Proposal 8: Adopt 0% grid shift and 100% grid shift for deployment Case 4 for SBFD.

Coverage metric was agreed for SBFD and dynamic/flexible TDD evaluation with details FFS. SLS based solution was proposed in [4]. Considering UE throughput does not only depend on path loss, but also depend on CLI and transmission schemes, UE throughput around one specific pathloss can be quite scattering. Thus it would be difficult to get a UE throughput vs. pathloss curve. From this point of view, SLS based solution is not preferred for coverage evaluation. And the method (LLS+link buget analysis) used for R17 coverage enhancement can be adopted.

Proposal 9: Use the methodology (LLS+link buget analysis) for R17 coverage enhancement for SBFD coverage evaluation.

Detailed simulation assumptions for SBFD evaluation are provided in Table 1 and Table 2.

Proposal 10: Adopt simulation assumptions in Table 1 and Table 2 for SBFD system evaluation.

Table 1: Simulation assumptions for SBFD FR1 system evaluation

| Parameters | Indoor office | Urban macro | Dense urban (Optional) |
| --- | --- | --- | --- |
| Layout | Single layer 12BSs per 120m x 50m | Single layer Macro layer: Hex. Grid | Single layer Macro layer: Hex. Grid |
| Carrier frequency | 4GHz (30kHz SCS) |  |  |
| Simulation bandwidth | 100MHz (20MHz for UL subband) |  |  |
| Inter-BS distance | 20m | 500m | 200m |
| Minimum BS-UE (2D) distance | 0m | 35m |  |
| Minimum UE-UE (2D) distance | 1~3m (38.828) | 3m (38.802) |  |
| BS antenna configuration | (Mg,Ng,M,N,P)=(1,1,4,4,2) (dH,dV)=(0.5,0.5) λ | (Mg,Ng,M,N,P)=(1,1,4,4,2) (dH,dV)=(0.5,0.8) λ |  |
| BS antenna height | 3m | 25 m | 25 m |
| BS Tx power | 24dBm (38.901) | 49 dBm (38.901) | 44dBm (38.802) |
| BS antenna element gain + connector loss | 8dBi (38.802) |  |  |
| BS receiver noise figure | 5dB |  |  |
| UE antenna configuration | Omni |  |  |
| UE antenna height | 1.0m | Same as 3D-UMa in TR36.873 |  |
| UE antenna gain | 0dBi |  |  |
| UE receiver noise figure | 9dB |  |  |
| UE Tx power | 23dBm |  |  |
| Traffic model | FTP traffic model 3 with packet size 0.5 Mbytes Ratio of DL/UL traffic = {4:1}, {3:2} |  |  |
| UE distribution | 10 users per TRP | 10 users per TRP 20% UEs are outdoor and 80% UEs are indoor |  |
| UE receiver | MMSE-IRC as the baseline receiver Note: Advanced receiver is not precluded. |  |  |
| Feedback assumption | Realistic |  |  |
| Channel estimation | Realistic |  |  |

Table 2: Simulation assumptions for SBFD FR2-1 system evaluation

| Parameters | Indoor office | Dense urban - Macro | Dense urban - Micro (Optional) |
| --- | --- | --- | --- |
| Layout | Single layer 12BSs per 120m x 50m | Single layer Macro layer: Hex. Grid | Single layer Micro layer: Hex. Grid |
| Carrier frequency | 30GHz (60kHz SCS) |  |  |
| Simulation bandwidth | [200MHz] (40MHz for UL subband) |  |  |
| Inter-BS distance | 20m | 200m | 100m |
| Minimum BS-UE (2D) distance | 0m | 35m |  |
| Minimum UE-UE (2D) distance | 1~3m (38.828) | 3m (38.802) |  |
| BS antenna configuration | (Mg,Ng,M,N,P)=(1,1,4,8,2) (dH,dV)=(0.5,0.5) λ | (Mg,Ng,M,N,P)=(1,1,8,8,2) (dH,dV)=(0.5,0.8) λ |  |
| BS antenna height | 3m | 25 m | 10m |
| BS Tx power | 24dBm (38.901) | 40 dBm (38.802) | 35dBm (38.901) |
| BS antenna element gain + connector loss | 5dBi (38.802) | 8dBi |  |
| BS receiver noise figure | 7dB |  |  |
| UE antenna configuration | Omni |  |  |
| UE antenna height | 1.0m | Same as 3D-UMa in TR36.873 |  |
| UE antenna gain | 0dBi |  |  |
| UE receiver noise figure | 13dB (baseline performance), 10dB (high performance) [TR 38.802] |  |  |
| UE Tx power | 23dBm |  |  |
| Traffic model | FTP traffic model 3 with packet size 0.5 Mbytes Ratio of DL/UL traffic = {4:1}, {3:2} |  |  |
| UE distribution | 10 users per TRP | 10 users per TRP 20% UEs are outdoor and 80% UEs are indoor |  |
| UE receiver | MMSE-IRC as the baseline receiver Note: Advanced receiver is not precluded. |  |  |
| Feedback assumption | Realistic |  |  |
| Channel estimation | Realistic |  |  |

    - Dynamic/flexible TDD

For Rel-18 dynamic/flexible TDD, the evaluation should focus on the performance gain of new CLI handling schemes and the baseline should be the existing CLI handling schemes.

Proposal 11: Performance of dynamic/flexible TDD with existing CLI handling schemes is the baseline for comparison with new CLI handling schemes.

Detailed simulation assumptions for dynamic/flexible TDD are given in Table 3.

Proposal 12: Adopt simulation assumptions in Table 3 for dynamic/flexible TDD system evaluation.

Table 3: simulation assumptions for flexible/dynamic TDD

| Parameters | Indoor office | HetNet |
| --- | --- | --- |
| Layout | Single layer 12BSs per 120m x 50m | Two layer Macro layer: Hex. Grid Indoor: the number of Indoor per macro cell (drop randomly) = 1 FR1: 6BSs per 120m x 50m FR2: 12BSs per 120m x 50m |
| Inter-BS distance | 20m | Macro-to-macro: 200m The minimum distance between Macro to Indoor: 35 m |
| Minimum BS-UE (2D) distance | 0m | Macro-to-UE:35m Indoor-to-UE: 0 m |
| Minimum UE-UE (2D) distance | 1~3m (38.828) | 3m (38.802) |
| Carrier frequency | 4GHz(30kHz SCS) for FR1, 30GHz(60kHz SCS) for FR2-1 |  |
| Simulation bandwidth | 100MHz for 4GHz and 200 MHz for 30GHz |  |
| Channel model | Reuse channel mode defined in Table A.2.1-11 in TR38.802 |  |
| BS antenna configuration | For 4G: (Mg,Ng,M,N,P)=(1,1,4,4,2) (dH,dV)=(0.5,0.5) λ For 30G: (Mg,Ng,M,N,P)=(1,1,4,8,2) (dH,dV)=(0.5,0.5) λ | For 4G: (Mg,Ng,M,N,P)=(1,1,4,4,2) (dH,dV)=(0.5,0.8) λ For 30G: (Mg,Ng,M,N,P)=(1,1,8,8,2) (dH,dV)=(0.5,0.8) λ |
| BS antenna height | 3m | 25 m |
| BS Tx power | 24dBm | Macro layer: 44 dBm(FR1), 40 dBm(FR2-1) Indoor layer: 24dBm |
| BS antenna element gain + connector loss | 8dBi for FR1, 5dBi for FR2-1 | Macro layer: 8 dBi ; indoor layer: 8dBi for FR1, 5dBi for FR2-1 |
| BS receiver noise figure | 4GHz: 5dB 30GHz: 7dB |  |
| UE antenna configuration | Omni |  |
| UE antenna height | 1.0m | Same as 3D-UMa in TR36.873 |
| UE antenna gain | 0dBi |  |
| UE receiver noise figure | 4GHz: 9dB 30GHz: 13dB (baseline performance), 10dB (high performance) [TR 38.802] |  |
| UE Tx power | 23dBm |  |
| Traffic model | FTP traffic model 3 with packet size 0.5 Mbytes Ratio of DL/UL traffic = {4:1}, {3:2} | FTP traffic model 3 with packet size 0.5 Mbytes Ratio of DL/UL traffic = {4:1} and {2: 3} |
| UE distribution | 10 users per TRP |  |
| UE receiver | MMSE-IRC as the baseline receiver Note: Advanced receiver is not precluded. |  |
| Feedback assumption | Realistic |  |
| Channel estimation | Realistic |  |

- Conclusions

In this contribution, further analyses and views on deployment scenarios, evaluation methodologies and evaluation assumptions for Rel-18 duplex enhancement are provided. Our proposals are:

Proposal 1: FR 2-2 is deprioritized in Rel-18 duplex evolution SI.

Proposal 2: The deployment scenarios for dynamic/flexible TDD evaluation at least include indoor office and heterogeneous deployment with Urban Macro and Indoor office.

  - Indoor office with dynamic TDD UL/DL assignment

  - HetNet with Urban Macro and Indoor office deployed in the same carrier

    - Macro layer use DL dominant static TDD UL/DL configuration: {DDDSU}

    - Indoor layer use UL dominant static TDD UL/DL configuration: {DSUUU}

Proposal 3: Adopt the following definitions for SLS metrics for duplex enhancement evaluation.

- UPT: defined as the size of a packet divided by the time between the arrival of the packet and the reception of the last bit of the packet.

- Latency: defined as the time between the arrival of the packet and the reception of the last bit of the packet.

- Resource utilization: defined as the number of RBs per cell used by UL/DL traffic during observation time divided by total number of RB per cell available for UL/DL traffic over observation time; or, defined as the number of RB per cell used by UL/DL traffic during observation time divided by total number of RB per cell available for UL&DL traffic over observation time.

- DL/UL received SINR: (BS/UE Tx power - coupling loss) / (noise + legacy inter-cell interference from N interference sources + CLI from M interference sources)

  - FFS whether dominant interference sources or all the interference sources are taken into account

  - Coupling loss includes all kinds of losses (e.g. path loss and penetration loss) minus gains (e.g. Tx antenna gain, Rx antenna gain).

Proposal 4: Evaluate low, medium and high RU ratios (20%, 50% and 80%) via different packet arrival rates in SBFD system evaluation.

Proposal 5: Reuse the UE-UE channel model for flexible duplex evaluation in TR 38.802 for both FR1 and FR2-1 SBFD evaluation.

Proposal 6: Reuse LOS probability in TR 38.901 and TR 38.802 for gNB-gNB and UE-UE channel model respectively.

Proposal 7: Uniform UE distribution for urban macro and dense urban scenarios is the baseline and FFS details of UE clustering.

Proposal 8: Adopt 0% grid shift and 100% grid shift for deployment Case 4 for SBFD.

Proposal 9: Use the methodology (LLS+link buget analysis) for R17 coverage enhancement for SBFD coverage evaluation.

Proposal 10: Adopt simulation assumptions in Table 1 and Table 2 for SBFD system evaluation.

Proposal 11: Performance of dynamic/flexible TDD with existing CLI handling schemes is the baseline for comparison with new CLI handling schemes.

Proposal 12: Adopt simulation assumptions in Table 3 for dynamic/flexible TDD system evaluation.

- Reference

- Chairman's Notes RAN1#109-e, 3GPP TSG RAN WG1 Meeting #109e, e-Meeting, May 9th – 20th, 2022.

- R1-2205030, On deployment scenarios and evaluation methodology for NR duplex evolution, Qualcomm Incorporated, RAN1#109-e, May 9th – 20th, 2022.

- R1-2205540, Summary# of email discussion on evaluation of NR duplex evolution, CMCC, RAN1#109-e, May 9th – 20th, 2022.

- R1-2204106, Evaluation of NR duplex evolution, Ericsson, RAN1#109-e, May 9th – 20th, 2022.

- R1-2204721, Deployment scenarios and evaluation methodology for NR duplex evolution, MediaTek, RAN1#109-e, May 9th – 20th, 2022.

- TS36.814, Evolved Universal Terrestrial Radio Access (E-UTRA); Further advancements for E-UTRA physical layer aspects, V9.2.0.