3GPP TSG-RAN WG1 Meeting #110	R1-2206483

Toulouse, France, 22nd – 26th August 2022

Agenda Item:	5

Source:	Ericsson

Title:	On the Reply LS on introduction of an offset to transmit CD-SSB and NCD-SSB at different times

Document for:	Discussion, Decision

# 1	Introduction

Based on an LS from RAN2, RAN1#109-e discussed the feasibility and necessity of introducing an offset to transmit CD-SSB and NCD-SSB at different times to RedCap UEs [1][2].

RAN2 has now sent a follow-up LS to RAN1 with the following information and questions to RAN1 and RAN4:

RAN2 would like to thank RAN1 and RAN4 for their reply LS on introduction of an offset to transmit CD-SSB and NCD-SSB at different times. RAN2 agreed to specify the offset with the following value range {sf5, sf10, sf15, spare5, spare4, spare3, spare2, spare1} and the definition below: ssb-TimeOffset The time offset between CD-SSB of the serving cell and this Non-Cell Defining SSB. Value sf5 means the first burst of Non-Cell Defining SSB is transmitted 5ms later than the first burst of CD-SSB transmitted after the first symbol of SFN=0 of the serving cell, value sf10 means the first burst of Non-Cell Defining SSB is transmitted 10ms later than the first burst of CD-SSB transmitted after the first symbol in SFN=0 of the serving cell, and so on. If the field is absent, RedCap UE considers that the time offset between the first burst of CD-SSB transmitted in the serving cell and the first burst of this Non-Cell Defining SSB transmitted is zero.. RAN2 would like to ask RAN1 and RAN4 to take the above into consideration and provide feedback on the values, i.e., confirm and/or indicate whether additional values are needed.

This contribution presents our view from RAN1 perspective. We have provided our view from RAN4 perspective in [4].

# 2	Discussion

In RAN1#109-e, following the discussion captured in [1], the following reply was sent to RAN2 in [2]:

RAN1 would like to thank RAN2 for the LS where RAN2 asked the following question: In RAN2#117-e, it was concluded that, from RAN2 signalling standpoint, CD-SSB and NCD-SSB(s) may be transmitted at different times by configuring an offset. RAN2 would like to ask RAN4 and RAN1 whether such offset is feasible/needed.. RAN1 has discussed the question in the LS and has the following response to RAN2: - There is no consensus in RAN1 regarding the potential need for configurable time offset(s) between CD-SSB and NCD-SSB(s). - It is feasible from RAN1 perspective to introduce configurable time offset(s) between CD-SSB and NCD-SSB(s) with half-frame granularity, including an offset value of zero (i.e., no offset), but the associated impacts on RAN1 specifications have not been fully analyzed and are expected to require additional RAN1 discussion and may require RAN1 specification updates. - If the suggested configurable time offset is introduced, RAN1 would like to ask RAN2 to provide the definition of such time offset for the cases that CD-SSB and NCD-SSB have the same or different periodicities.

Now RAN2 has provided the definition for ssb-TimeOffset in their follow-up LS (see extract in the previous section), and the definition and values seem to fulfil the recommendations provided in the reply from RAN1 to RAN2. So, from RAN1 perspective, we are fine with the definition and values chosen by RAN2.

From RAN4 perspective, if the network would want the UE to measure both CD-SSB and NCD-SSB(s), the time offset between the SSBs may need to consider the MGRP of the configured MG. So, RAN4’s input should also be taken into consideration, but that discussion belongs in RAN4. We have provided our view from RAN4 perspective in [4].

# 3	Conclusion

In our view, RAN1 can reply to RAN2 that from RAN1 point of view, the definition and values chosen by RAN2 for the time offset between CD-SSB and NCD-SSB are sufficient and can be confirmed. This does not preclude additional values from being introduced for RAN4 reasons.

# References

R1-2205429, “FL summary for incoming LS (R1-2203046) on introduction of an offset to transmit CD-SSB and NCD-SSB at different times”, Moderator (Ericsson)

R1-2205535, “Reply LS on introduction of an offset to transmit CD-SSB and NCD-SSB at different times”, RAN1, Ericsson

R1-2205734, “Reply LS on introduction of an offset to transmit CD-SSB and NCD-SSB at different times”, RAN2, Ericsson

R4-2212755, “Discussions on LS reply for RedCap SSB offset”, Ericsson